HPE 5140EI-CMW710-R6351P02 Release Notes

The information in this document is subject to change without notice. © Copyright 2010, 2023 Hewlett Packard Enterprise Development LP

Contents

Introduction	1

Version information	1

Version number	1

Version history	1

Hardware and software compatibility matrix	2

Upgrade restrictions and guidelines	3

Hardware feature updates	3

5140EI-CMW710-R6351P02	3

5140EI-CMW710-R6351	3

5140EI-CMW710-R6343P09	3

5140EI-CMW710-R6343	3

5140EI-CMW710-R6337P01	4

5140EI-CMW710-R6330	4

5140EI-CMW710-R6327	4

5140EI-CMW710-R6325	4

Software feature and command updates	4

MIB updates	4

Operation changes	5

Operation changes in R6351P02	5

Operation changes in R6351	5

Operation changes in R6343P09	5

Operation changes in R6343	5

Operation changes in R6337P01	6

Operation changes in R6330	6

Operation changes in R6327	6

Operation changes in R6325	6

Restrictions and cautions	6

Restrictions	6

Cautions	6

Open problems and workarounds	7

List of resolved problems	7

Resolved problems in R6351P02	7

Resolved problems in R6351	7

Resolved problems in R6343P09	9

Resolved problems in R6343	10

Resolved problems in R6337P01	11

Resolved problems in R6330	13

Resolved problems in R6327	14

Resolved problems in R6325	14

Support and other resources	14

Accessing Hewlett Packard Enterprise Support	14

Documents	15

Related documents	15

Documentation feedback	15

Appendix A Feature list	16

Hardware features	16

Software features	20

Appendix B Fixed security vulnerabilities	23

Fixed security vulnerabilities in R6343	23

Appendix C Upgrading software	24

System software file types	24

System startup process	24

Upgrade methods	25

Preparing for the upgrade	26

Verifying device status	26

Setting up the upgrade environment	26

Upgrading from the CLI	27

Preparing for the upgrade	27

Downloading software images to the master switch	28

Upgrading from the Boot menu	32

Prerequisites	32

Accessing the Boot menu	33

Accessing the extended Boot menu	34

Upgrading Comware images from the Boot menu	35

Upgrading Boot ROM from the Boot menu	43

Managing files from the Boot menu	50

Handling software upgrade failures	53

List of tables

Table 1 Version history	1

Table 2 Hardware and software compatibility matrix	2

Table 3 MIB updates	4

Table 4 5140EI series hardware features for non-PoE switch models	16

Table 5 5140EI series hardware features for non-PoE switch models(2)	17

Table 6 5140EI series hardware features for PoE switch models	18

Table 7 5140EI series hardware features for PoE switch models(2)	19

Table 8 Software features of the 5140EI series	20

Table 9 Minimum free storage space requirements	32

Table 10 Shortcut keys	33

Table 11 Extended Boot ROM menu options	34

Table 12 EXTENDED ASSISTANT menu options	35

Table 13 TFTP parameter description	36

Table 14 FTP parameter description	38

Table 15 TFTP parameter description	44

Table 16 FTP parameter description	45

Introduction

This document describes the features, restrictions and guidelines, open problems, and workarounds for version HPE 5140EI-CMW710-R6351P02. Before you use this version on a live network, back up the configuration and test the version to avoid software upgrade affecting your live network.

Use this document in conjunction with HPE 5140EI-CMW710-R6351P02 Release Notes (Software Feature Changes) and the documents listed in "Related documents."

Version information

Version number

HPE Comware Software, Version 7.1.070, Release 6351P02

Note: You can see the version number with the command display version in any view. Please see Note.

Version history

|  | IMPORTANT: The software feature changes listed in the version history table for each version are not complete. To obtain complete information about all software feature changes in each version, see the Software Feature Changes document for this release notes. |
| --- | --- |

Version history

| Version number | Last version | Release date | Release type | Remarks |
| --- | --- | --- | --- | --- |
| 5140EI-CMW710-R6351P02 | 5140EI-CMW710-R6351 | 2023-10-18 | Release version | Fixed bugs |
| 5140EI-CMW710-R6351 | 5140EI-CMW710-R6343P09 | 2023-04-01 | Release version | Fixed bugs |
| 5140EI-CMW710-R6343P09 | 5140EI-CMW710-R6343 | 2022-11-30 | Release version | Fixed bugs |
| 5140EI-CMW710-R6343 | 5140EI-CMW710-R6337P01 | 2022-06-13 | Release version | Fixed bugs |
| 5140EI-CMW710-R6337P01 | 5140EI-CMW710-R6330 | 2021-12-09 | Release version | Fixed bugs |
| 5140EI-CMW710-R6330 | 5140EI-CMW710-R6327 | 2021-05-31 | Release version | New feature |
| 5140EI-CMW710-R6327 | 5140EI-CMW710-R6325 | 2021-03-01 | Release version | Fixed bugs |
| 5140EI-CMW710-R6325 | First release | 2020-12-25 | Release version | First release |

Hardware and software compatibility matrix

|  | CAUTION: To avoid an upgrade failure, use Table 2 to verify the hardware and software compatibility before performing an upgrade. |
| --- | --- |

Hardware and software compatibility matrix

| Item | Specifications |
| --- | --- |
| Product family | 5140EI Series |
| Hardware platform | HPE 5140 24G 4SFP+ EI Sw JL828A HPE 5140 24G SFP 4SFP+ EI Sw JL826A HPE 5140 48G 4SFP+ EI Sw JL829A HPE 5140 24G PoE+ 4SFP+ EI Sw JL827A HPE 5140 48G PoE+ 4SFP+ EI Sw JL824A HPE 5140 24G PoE+ 2SFP+ 2XGT EI Sw JL823A HPE 5140 48G PoE+ 2SFP+ 2XGT EI Sw JL825A HPE 5140 24G 2SFP+ 2XGT EI Sw R8J41A HPE 5140 8G 2SFP 2GT EI Sw R8J42A |
| Memory | 512M |
| Flash | 256M |
| Boot ROM version | Version 151 or higher (Note: Use the display version command in any view to view the version information. Please see Note②) |
| Software images and their MD5 checksums | 5140EI-CMW710-R6351P02.ipe(See the MD5 file) |
| iMC版本号 | ADNET-FCAPS (E0709) AOM (E0706P01) ADCampus 6.0 (E6205) iMC BIMS 7.3 (E0506H01) iMC EAD7.3 (E0611P10) iMC QoSM 7.3 (E0505P01) iMC EIA 7.3 (E0611P13) iMC PLAT 7.3 (E0705P12) iMC NTA 7.3 (E0707L06) iMC SHM 7.3 (E0707L06) OASIS V100R001 |
| iNode 版本号 | iNode PC 7.3 (E0585) |
| Remarks | N/A |

Display the system software and Boot ROM versions of 5140EI

<HPE>display version

HPE Comware Software, Version 7.1.070, Release 6351P02    ------Note

Copyright (c) 2010-2023 Hewlett Packard Enterprise Development LP

HPE 5140 24G PoE+ 4SFP+ EI Sw uptime is 0 weeks, 0 days, 0 hours, 2 minutes

Last reboot reason : Cold reboot

Boot image: flash:/5140ei-cmw710-boot-r6351p02.bin

Boot image version: 7.1.070, Release 6351P02

Compiled Oct 10 2023 11:00:00

System image: flash:/5140ei-cmw710-system-r6351p02.bin

System image version: 7.1.070, Release 6351P02

Compiled Oct 10 2023 11:00:00

Slot 1:

Uptime is 0 weeks,0 days,0 hours,2 minutes

5140 24G PoE+ 4SFP+ EI Sw with 1 Processor

BOARD TYPE:         5140 24G PoE+ 4SFP+ EI Sw

DRAM:               512M bytes

FLASH:              256M bytes

PCB 1 Version:      VER.A

Bootrom Version:    151              ------Note

CPLD 1 Version:     001

Release Version:    HPE 5140 24G PoE+ 4SFP+ EI Sw JL827A-6351P02

Patch Version  :    None

Reboot Cause  :     ColdReboot

[SubSlot 0] 20GE+4COMBO+4SFP Plus

Upgrade restrictions and guidelines

Before performing a software upgrade, it is important to refer to the Software Feature Changes document for any feature changes in the new version. Also check the most recent version of the related documents (see "Related documents") available on the HPE website for more information about feature configuration and commands.

Hardware feature updates

5140EI-CMW710-R6351P02

None.

5140EI-CMW710-R6351

None.

5140EI-CMW710-R6343P09

None.

5140EI-CMW710-R6343

None.

5140EI-CMW710-R6337P01

None.

5140EI-CMW710-R6330

Support :

5140 24G 2SFP+ 2XGT EI Sw R8J41A

5140 8G 2SFP 2GT EI Sw R8J42A

5140EI-CMW710-R6327

None.

5140EI-CMW710-R6325

First release.

Software feature and command updates

For more information about the software feature and command update history, see HPE 5140_EI-CMW710-R6351P02 Release Notes (Software Feature Changes).

MIB updates

MIB updates

| Item | MIB file | Module | Description |
| --- | --- | --- | --- |
| 5140EI-CMW710-R6351P02 |  |  |  |
| New | None | None | None |
| Modified | None | None | None |
| 5140EI-CMW710-R6351 |  |  |  |
| New | None | None | None |
| Modified | None | None | None |
| 5140EI-CMW710-R6343P09 |  |  |  |
| New | None | None | None |
| Modified | None | None | None |
| 5140EI-CMW710-R6343 |  |  |  |
| New | None | None | None |
| Modified | None | None | None |
| 5140EI-CMW710-R6337P01 |  |  |  |
| New | None | None | None |
| Modified | None | None | None |
| 5140EI-CMW710-R6330 |  |  |  |
| New | None | None | None |
| Modified | None | None | None |
| 5140EI-CMW710-R6327 |  |  |  |
| New | None | None | None |
| Modified | None | None | None |
| 5140EI-CMW710-R6325 |  |  |  |
| New | First release | First release | First release |
| Modified | First release | First release | First release |

Operation changes

Operation changes in R6351P02

None.

Operation changes in R6351

Changed the length of the attribute name string carried in a RADIUS packet from 64 characters to 253 characters.

Operation changes in R6343P09

None.

Operation changes in R6343

The number of available ACL resources was increased from 512 to 768.

Change for the ipv6 verify source ip-address mac-address command.

Before modification: The device will generate four permit ACL rules and one deny ACL rule for each interface on which the ipv6 verify source ip-address mac-address command is executed. The total number of used resources is the number of interfaces multiplied by (4+1).

After modification: The device will generate four permit ACL rules for all configured interfaces globally and generate one deny ACL rule for each interface. The total number of used resources is the number of interfaces multiplied by 1 plus 4.

Change for the igmp-snooping source-deny command.

Before modification: The device will generate one permit ACL rule and one deny ACL rule for each interface on which the igmp-snooping source-deny command is executed. The total number of used resources is the number of interfaces multiplied by (1+1).

After modification: The device will generate one permit ACL rule for all configured interfaces globally and generate one deny ACL rule for each interface. The total number of used resources is the number of interfaces multiplied by 1 plus 1.

Change for the mld-snooping source-deny command.

Before modification: The device will generate three permit ACL rules and one deny ACL rule for each interface on which the mld-snooping source-deny command is executed. The total number of used resources is the number of interfaces multiplied by (3+1).

After modification: The device will generate three permit ACL rules for all configured interfaces globally and generate one deny ACL rule for each interface. The total number of used resources is the number of interfaces multiplied by 1 plus 3.

Operation changes in R6337P01

None.

Operation changes in R6330

On an IRF fabric that has multiple IRF physical links, the packet loss duration has decreased after you shut down and then bring up one IRF physical interface.

Before modification: The packet loss duration is longer than 2000 milliseconds.

After modification: The packet loss duration is in the range of 200 to 500 milliseconds.

Operation changes in R6327

None.

Operation changes in R6325

First release.

Restrictions and cautions

Before performing a software upgrade, it is important to refer to the Software Feature Changes document for any feature changes in the new version. Also check the most recent version of the related documents (see "Related documents") available on the HPE website for more information about feature configuration and commands.

When you use this version of software, make sure you fully understand the restrictions and cautions described in this section.

Restrictions

Release 6351P02 must use BootROM 151 or a later version.

If data packets are assigned to queue 7 and the scheduling algorithm is SP, all packets sent from the CPU are affected.

To avoid false alarms, make sure the statistics collection and comparison interval for CRC error packets configured in the ifmonitor crc-error command is greater than 15 seconds.

Cautions

None.

Open problems and workarounds

None.

List of resolved problems

Resolved problems in R6351P02

None.

Resolved problems in R6351

202303162097

Symptom: The IRF fabric reboots because the memory is exhausted.

Condition: This symptom occurs if a master/subordinate switchover is performed or a DHCP client requests multiple addresses from the IRF fabric acting as a DHCP relay.

202303151120

Symptom: The DHCP process exits unexpectedly and then recovers after DHCP relay entries are aged out.

Condition: This symptom occurs if the following conditions exist:

The switch acts as a DHCP relay.

A DHCP client obtains two IP addresses on an interface and then obtained one of the two addresses on anther interface.

The DHCP relay entries are aged out.

202303130217

Symptom: The switch reboots when LAGG/LoadSharing information is obtained through NETCONF.

Condition: This symptom occurs when LAGG/LoadSharing information is obtained through NETCONF.

202303010469

Symptom: Failed to log in to the switch through HTTP or HTTPS.

Condition: This symptom occurs if the RADIUS server issues the Login-Service attribute to the switch during RADIUS authentication.

202302280994

Symptom: The switch reboots when the display diagnostic-information command is executed.

Condition: This symptom occurs when the display diagnostic-information command is executed.

202302270992

Symptom: An IRF fabric cannot be formed.

Condition: This symptom occurs if you use 10-GE copper ports as IRF ports and set the IRF link down report delay to 0.

202302140409

Symptom: When you use SNMP to read the value of the hh3cEntityExtErrorStatus MIB object, the status of the fixed power supply is not returned.

Condition: This symptom occurs if you use SNMP to read the value of the hh3cEntityExtErrorStatus MIB object.

202302141134

Symptom: A memory alarm is generated.

Condition: This symptom occurs if you execute the vcf-fabric topology enable command and an interface goes down and comes up frequently.

202302151892

Symptom: The display power and display device manuinfo commands cannot display the corresponding output. The reboot command is stuck at Now rebooting, please wait.

Condition: This symptom occurs when you execute the display power, display device manuinfo, or reboot command.

202212070008

Symptom: After a device reboots, it does not have IPv6 connectivity.

Condition: This symptom might occur when the following conditions exist:

The device receives a large number of IPv6 packets from unknown addresses after reboot.

The device protects its CPUs against packet attacks by default.

202212070538

Symptom: The system printed the log message of deleting MAC address 0000-0000-0000.

Condition: This symptom occurs if the following conditions exist on an IRF fabric:

Port 0 exists on the device.

When a port is configured with port security and learns MAC addresses, the port flaps or receives TC packets.

202211141586

Symptom: The offline detection or ARP detection function for 802.1X users is abnormal, and users cannot age out to go offline.

Condition: This symptom occurs if the authorization VLAN is modified on the authentication server after 802.1X users successfully come online.

202210280272

Symptom: The device memory leaks.

Condition: This symptom occurs if the NETCONF controller frequently subscribes to events and cancels event subscriptions for the device.

202210141030

Symptom: The device cannot process new NETCONF requests.

Condition: This symptom occurs if an event subscription cancellation request is sent to the device when the subscribed NETCONF events are being reported.

202210120883

Symptom: The NTP packets are sent to the CPU for processing. As a result, NTP packets cannot be forwarded normally, and NTP cannot synchronize the time.

Condition: This symptom occurs if the device acts as a gateway to forward NTP packets through a Layer 3 interface.

202209220968

Symptom: When an endpoint receives an IP address allocated by the DHCP server, the device initiates gratuitous ARP packets within the network for IP address conflict detection. No replies are received from the network, which means that no address conflict exists. However, the endpoint replies to the DHCP server with a DHCP-DECLINE packet to apply for an IP address again. As a result, the endpoint repeatedly obtains an IP address.

Condition: This symptom occurs if the arp suppression or arp snooping command is executed on the device.

202208130589

Symptom: Executing the reset kernel deadloop, reset kernel exception, reset kernel reboot, or reset kernel starvation command cannot clear the corresponding information.

Condition: This symptom occurs when you execute the reset kernel deadloop, reset kernel exception, reset kernel reboot, or reset kernel starvation command.

202208050249

Symptom: The device cannot learn routing entries.

Condition: This symptom occurs if a QoS policy containing a large number of class-based accounting actions is applied.

202207010954

Symptom: The configured remote ID of Option 82 is not displayed in the display dhcp relay information command output.

Condition: This symptom occurs if you configure the padding format is hex and the remote ID starts with 0.

202206201644

Symptom: The switch fails to forward IPv6 ICMP packets at Layer 2.

Condition: This symptom occurs if the source IPv6 address of the packet contains abc0, for example, 2001:250:100d:abcd::abcd.

Resolved problems in R6343P09

202206140089

Symptom: On an IRF fabric, the MAC address of a packet forwarded across member devices cannot be learned.

Condition: This symptom occurs if the logical interface number of the service port is the same as that of the IRF port.

202208230885

Symptom: Port 80 and port 443 are not deleted after the HTTP service and HTTPS service are disabled.

Condition: This symptom occurs after the HTTP service and HTTPS service are disabled.

202208040367

Symptom: The power LED is solid yellow, and the power supply status is Fault in the display power command output.

Condition: This symptom occurs if a Great Wall 150-W power supply is used on the device.

202206210164

Symptom: Intrusion protection is not triggered when an interface receives packets from a learned MAC address.

Condition: This symptom occurs if the following operations have been performed:

              - Enable port security.

On each of two interfaces, set the maximum number of secure MAC addresses allowed to 1.

Send packets from the learned MAC address to one of the two interfaces.

Resolved problems in R6343

202204210856

Symptom: The fiber port of the combo port is up.

Condition: This symptom occurs when the copper port of the fiber converter connected to the device is down.

202205090269

Symptom: The switch fails to supply power to some nonstandard PDs (such as Cisco 7940G IP phone) through PoE.

Condition: This symptom might occur after PoE and nonstandard PD detection are enabled on the switch.

202205050956

Symptom: In the output from the display irf link command, the IRF physical interface on a standby MPU is displayed as down even if the interface is up.

Condition: This symptom might occur if the IRF port on the standby MPU flaps constantly.

202204130283

Symptom: When the display mac-address command is executed to view the MAC address table, the latest entries are not displayed. The MAC address learning limit configuration cannot be deleted. After an interface goes down, it is still displayed as up.

Condition: This symptom occurs if the MAC address learning limit is set to 1 on an interface, and a MAC address moves to the interface.

202204080241

Symptom: Failed to obtain the SN of a transceiver module.

Condition: This symptom occurs if you obtain the MIB information of a transceiver module through SNMP.

202203300985

Symptom: The subordinate IRF member device might fail to start normally.

Condition: This symptom might occur if a master/subordinate switchover is performed on an IRF fabric with a large amount of configuration.

202203170120

Symptom: The CPU usage of the device is high.

Condition: This symptom occurs if a transceiver module is repeatedly shut down and brought up.

202201140229

Symptom: The PoE interface that supplies power is not the one configured.

Condition: This symptom occurs if the following conditions exist:

The PoE daughter card used on the device is LSPPSE48A, LSPPSE24A, LSPPSE16A, or LSPPSE8A. To view the PoE daughter card model, execute the display poe pse command. The value of the PSE Model field in the command output is the PoE daughter card model.

The poe enable command has been executed.

202202231179

Symptom: Only SNMPv3 takes effect on the device after SNMPv1, SNMPv2c, and SNMPv3 are all configured.

Condition: This symptom occurs if the following conditions exist:

The device starts up with the factory defaults.

Use the snmp-agent sys-info version all command to specify SNMPv1, SNMPv2c, and SNMPv3 for the device.

202203010889

Symptom: Slow to obtain information from the lldpV2RemSysName MIB object.

Condition: This symptom occurs if you request the value of the lldpV2RemSysName MIB object remotely.

202201250424

Symptom: Unknown packets cannot be flooded to router ports, and multicast forwarding is abnormal.

Condition: This symptom occurs if you use the igmp-snooping drop-unknown command to enable dropping unknown multicast data packets and disable IGMP snooping for a VLAN on the Layer 2 device of the multicast source.

202201040567

Symptom: The flow control function cannot work when the device is connected to a PC.

Condition: This symptom occurs when the device is connected to a PC.

202103250327

Symptom: When the device is running, the network management interface might go down and cannot be recovered.

Condition: This symptom occurs if the network management interface repeatedly sends and receives packets.

202112230657

Symptom: The CPU usage is always high. The display mac-address command cannot display information normally.

Condition: This symptom occurs if MAC address entries are frequently added and deleted.

Resolved problems in R6337P01

202111150249

Symptom: The device has a deadloop reboot.

Condition: This symptom occurs if you execute shutdown and undo shutdown commands repeatedly on the peer ports, causing flapping of the local ports.

202111050868

Symptom: The CPU usage of the device reaches 75%.

Condition: This symptom occurs if you enable accounting for charging, execute the repeat command, and then restart the UCM process on the device.

202109241082

Symptom: After the device’s system time is synchronized, an SSH user fails to log in to the device and gets a prompt of “Failed to login because the idle timer expired.”

Condition: This symptom occurs when the following conditions are met:

  - The password control feature is enabled.

  - The maximum account idle time is not 0 (set by the password-control login idle-time command).

  - The system time is too early and the SSH user has logged in to the device before.

  - The system time is changed to the current time.

202110090340

Symptom: The device reboots unexpectedly.

Condition: This symptom occurs if the mac-vlan enable command is executed to enable the MAC-based VLAN feature on a port and then the mac-vlan trigger enable command is executed to enable dynamic MAC-based VLAN assignment on the port.

202110090688

Symptom: Failed to ping a PC from an interface after the network cable is removed from and then re-inserted into the interface.

Condition: This symptom occurs if port security has been configured on the interface.

202107050836

Symptom: The device CPU usage is high.

Condition: This symptom occurs if an SNMP request for transceiver module information fails.

202109011682

Symptom: Packets are lost when they are forwarded through PBR.

Condition: This symptom occurs when you specify both the input and output interfaces in the PBR policy on chip 1 of the device configured with two switching chips.

202107211516

Symptom: The Circuit ID field in the display dhcp snooping information command output is empty.

Condition: This symptom occurs if you configure the padding mode for the Circuit ID sub-option of Option 82 as normal-extended.

202106250033

Symptom: MAC addresses are not aged out based on the aging time configured in port security.

Condition: This symptom occurs if the following conditions exist:

  - Port security is enabled globally.

  - Users come online on one port and then move to another port.

202107290628

Symptom: sFlow cannot collect interface counter information in time, and the collected traffic rate for fixed-rate traffic is not fixed.

Condition: This symptom occurs if you enable sFlow on an interface and use an sFlow collector to analyze collected packets.

202104210763

Symptom: When the DHCP server cannot find an assignable IP address in the primary network segment, the DHCP client cannot obtain an IP address from the secondary network segments.

Condition: This symptom occurs when the following conditions exist:

The DHCP server is configured with an IP address pool that has a primary network segment and secondary network segments.

The DHCP client requests an IP address when the DHCP server has no assignable IP address in the primary network segment.

202105310255

Symptom: The NMS failed to deploy VLAN configuration to a device.

Condition: This symptom occurs if the VLAN configuration to be deployed by the NMS is the same as that on the device.

202104200312

Symptom: A MAC authentication user might fail to come online.

Condition: This symptom occurs if the following conditions exist:

  - Both 802.1X authentication and MAC authentication are enabled on an interface.

  - The 802.1X guest VLAN is configured on the interface.

  - After a MAC authentication successfully comes online, the user repeatedly goes offline and comes online.

Resolved problems in R6330

202104131824

Symptom: On an IRF system, after the MAC address entry for a voice VLAN ages out on the subordinate member device, the downlink traffic will be broadcast on the subordinate member device.

Condition: This symptom occurs if the traffic matching the MAC address entry for the voice VLAN is initiated on the master member device and the MAC address entry age out because no traffic matches the MAC address entry within one aging period on the subordinate member device.

202104190149

Symptom: The traffic reported by sFlow is different from the actual traffic on a port.

Condition: This symptom occurs if sFlow sampling is enabled on the device.

202105280911

Symptom: On a device with two chips, sFlow cannot sample traffic on ports of chip 1.

Condition: This symptom occurs if you first configure sFlow sampling for traffic on ports of chip 0 and then for traffic on ports of chip 1 on a device with two chips.

202101181488

Symptom: Forwarding error exists for packets with a destination MAC address that hits a multiport unicast MAC address entry on an IRF fabric.

Condition: This symptom occurs if the following conditions exist:

  - The packets are received on a subordinate device in the IRF fabric.

  - The multiport unicast MAC address entry that the packets hit is synchronized from the master device to the subordinate device.

202101181303

Symptom: On the device configured with an isolation group and ARP snooping, a port in the isolation group forwards an ARP packet received from another port in the isolation group.

Condition: This symptom occurs if the device receives an ARP packet on a port in the isolation group.

Resolved problems in R6327

202101151443

Symptom: Logging is enabled for portal user logins and logouts on the device. When the device is accessed from IMC, the IMC system log page fails to load.

Condition: This symptom might occur if the following conditions exist:

  - Logging is enabled for portal user logins and logouts on the device.

  - Transparent MAC authentication is enabled on IMC, or the iNode client is enabled to upload the client version number.

202101130494

Symptom: The display mac-address command displays duplicate MAC address entries for a MAC address.

Condition: This symptom might occur if the following operations are performed:

              - Configure a multiport MAC address entry to overwrite a dynamic MAC address entry.

              - Execute the undo mac-address command to delete the multiport MAC address entry.

              - Wait for the device to learn the MAC address dynamically.

202101080488

Symptom: If IPSG is enabled on a VLAN interface, the IPv4SG bindings fail to be issued.

Condition: This symptom occurs if IPSG is enabled on a VLAN interface.

202012241455

Symptom: A marking-type QoS policy applied globally does not take effect after its traffic behaviors are modified.

Condition: This symptom occurs if a marking-type QoS policy is applied globally and then its traffic behaviors are modified.

Resolved problems in R6325

First release.

Support and other resources

Accessing Hewlett Packard Enterprise Support

For live assistance, go to the Contact Hewlett Packard Enterprise Worldwide website:

www.hpe.com/assistance

To access documentation and support services, go to the Hewlett Packard Enterprise Support Center website:

www.hpe.com/support/hpesc

Information to collect:

Technical support registration number (if applicable).

Product name, model or version, and serial number.

Operating system name and version.

Firmware version.

Error messages.

Product-specific reports and logs.

Add-on products or components.

Third-party products or components.

Documents

To find related documents, see the Hewlett Packard Enterprise Support Center website at http://www.hpe.com/support/hpesc.

Enter your product name or number and click Go. If necessary, select your product from the resulting list.

For a complete list of acronyms and their definitions, see HPE FlexNetwork technology acronyms.

Related documents

The following documents provide related information:

HPE FlexNetwork 5140 EI Switch Series Configuration Guides-R63xx

HPE FlexNetwork 5140 EI Switch Series Command References-R63xx

HPE FlexNetwork 5140 EI Switch Series Installation Guide

Documentation feedback

Hewlett Packard Enterprise is committed to providing documentation that meets your needs. To help us improve the documentation, send any errors, suggestions, or comments to Documentation Feedback (docsfeedback@hpe.com). When submitting your feedback, include the document title, part number, edition, and publication date located on the front cover of the document. For online help content, include the product name, product version, help edition, and publication date located on the legal notices page.

Feature list

Hardware features

5140EI series hardware features for non-PoE switch models

| Item | 5140 24G 4SFP+ EI Sw | 5140 48G 4SFP+ EI Sw |  | 5140 24G SFP 4SFP+ EI Sw |
| --- | --- | --- | --- | --- |
| Dimensions (H × W × D) | 43.6 × 440 × 160 mm (1.72 × 17.32 × 6.30 in) | 43.6 × 440 × 230 mm (1.72 × 17.32 × 9.06 in) |  | 43.6 × 440 × 360 mm (1.72 × 17.32 × 14.17 in) |
| Weight | ≤ 2.5 kg (5.51 lb) | ≤ 3.5 kg (7.72 lb) |  | ≤ 6.5 kg (14.33 lb) |
| Console port | - 1 × serial console port - 1 × micro USB console port When both ports are connected, only the micro USB console port is available. | - 1 × serial console port - 1 × micro USB console port When both ports are connected, only the micro USB console port is available. |  | - 1 × serial console port - 1 × micro USB console port When both ports are connected, only the micro USB console port is available. |
| 10/100/1000BASE-T autosensing Ethernet port | 24 | 48 |  | 8 (Each and its corresponding SFP port form a combo interface.) |
| SFP port | N/A | N/A |  | 24 (The rightmost eight SFP ports form a combo interface with their corresponding 10/100/1000BASE-T autosensing Ethernet ports, respectively.) |
| SFP+ port | 4 | 4 |  | 4 |
| Power module slot | N/A | N/A |  | 2, at the rear panel |
| Input voltage | - Rated voltage: 100 VAC to 240 VAC @ 50 or 60 Hz - Max voltage: 90 VAC to 264 VAC @ 47 to 63 Hz |  |  | PSR75-12A: - Rated voltage: - 100 VAC to 240 VAC @ 50 or 60 Hz - 240 VDC - Max voltage: - 90 VAC to 290 VAC @ 47 to 63 Hz - 180 to 320 VDC PSR150-A1: - Rated voltage: 100 VAC to 240 VAC @ 50 or 60 Hz - Max voltage: 90 VAC to 264 VAC @ 47 to 63 Hz PSR150-D1: - Rated voltage: –48 VDC to –60 VDC - Max voltage: –36 VDC to –72 VDC DC power source for the PSR150-D1 power module: –48 VDC power source in the equipment room or an RPS (H3C RPS800-A or RPS1600-A) |
| Minimum power consumption | 10 W | 19 W |  | - 1 × PSR75-12A: 15 W - 2 × PSR75-12A: 17 W - 1 × PSR150-A1: 18 W - 1 × PSR150-D1: 18 W - 2 × PSR150-A1: 23 W - 2 × PSR150-D1: 22 W |
| Maximum power consumption | 24 W | 44 W |  | - 1 × PSR75-12A: 45 W - 2 × PSR75-12A: 48 W - 1 × PSR150-A1: 48 W - 1 × PSR150-D1: 51 W - 2 × PSR150-A1: 55 W - 2 × PSR150-D1: 57 W |
| Chassis leakage current compliance | UL60950-1/EN60950-1/IEC60950-1/GB4943 |  |  |  |
| Melting current of power module fuse | 2 A/250 V | 3.15 A/250 V | - PSR75-12A: 3.15 A/250 V - PSR150-A1: 6.3 A /250 V - PSR150-D1: 8 A/250 V |  |
| Cooling system | Using fixed fan trays to draw ambient air in from the left side, right side, and port side of the chassis and exhaust heated air out from the power module side | Using fixed fan trays to draw ambient air in from the left side and exhaust heated air out from the right side and power module side | Using fixed fan trays to draw ambient air in from the left side and port side and exhaust heated air from the right side |  |
| Operating temperature | -5ºC～45ºC (23°F to 113°F) |  |  |  |
| Operating humidity | 5% to 95%, noncondensing |  |  |  |
| Fire resistance compliance | UL60950-1/EN60950-1/IEC60950-1/GB4943 |  |  |  |

5140EI series hardware features for non-PoE switch models(2)

| Item | 5140 24G 2SFP+ 2XGT EI Sw | 5140 8G 2SFP 2GT EI Sw |
| --- | --- | --- |
| Dimensions (H × W × D) | 43.6 × 440 × 160 mm (1.72 × 17.32 × 6.30 in) | 43.6 × 266 × 161 mm (1.72 × 10.47 × 6.34 in) |
| Weight | ≤ 2.5 kg (5.51 lb) | ≤ 1.5 kg (3.31 lb) |
| Console port | 1 × serial console port |  |
| 10/100/1000BASE-T autosensing Ethernet port | 24 | 10 (The rightmost two form a combo interface with their corresponding SFP ports, respectively.) |
| 1/10GBase-T autosensing Ethernet port | 2 | N/A |
| SFP port | N/A | 4 (The leftmost two form a combo interface with their corresponding 10/100/1000BASE-T autosensing Ethernet ports, respectively.) |
| SFP+ port | 2 | N/A |
| Input voltage | - Rated voltage: 100 VAC to 240 VAC @ 50 or 60 Hz Max voltage: 90 VAC to 264 VAC @ 47 to 63 Hz |  |
| Minimum power consumption | 14W | 8 W |
| Maximum power consumption | 36W | 15 W |
| Chassis leakage current compliance | UL60950-1/EN60950-1/IEC60950-1/GB4943 |  |
| Melting current of power module fuse | - 2 A/250 V |  |
| Cooling system | Using fixed fan trays to draw ambient air in from the left side and exhaust heated air from the right side | Natural cooling without fan trays |
| Operating temperature | -5ºC～45ºC (23°F to 113°F) |  |
| Operating humidity | 5% to 95%, noncondensing |  |
| Fire resistance compliance | UL60950-1/EN60950-1/IEC60950-1/GB4943 |  |

5140EI series hardware features for PoE switch models

| Item | 5140 48G PoE+ 4SFP+ EI Sw | 5140 24G PoE+ 4SFP+ EI Sw |
| --- | --- | --- |
| Dimensions (H × W × D) | 43.6 × 440 × 400 mm (1.72 × 17.32 × 15.75 in) | 43.6 × 440 × 260 mm (1.72 × 17.32 ×10.24 in) |
| Weight | ≤ 6 kg (13.23 lb) | ≤ 4.5 kg (9.92 lb) |
| Console port | - 1 × serial console port - 1 × micro USB console port - When both ports are connected, only the micro USB console port is available. |  |
| 10/100/1000BASE-T autosensing Ethernet port | 48 | 24 (The four highest-numbered 10/100/1000BASE-T autosensing Ethernet ports form combo interfaces with their corresponding SFP ports, respectively.) |
| SFP port | N/A | 4 (Each and its corresponding 10/100/1000BASE-T autosensing Ethernet port form a combo interface.) |
| SFP+ port | 4 |  |
| Input voltage | AC power source: - Rated voltage: 100 VAC to 240 VAC @ 50 or 60 Hz - Max voltage: 90 VAC to 264 VAC @ 47 to 63 Hz DC power source: H3C RPS1600-A - Rated voltage: –54 VDC to –57 VDC - Max voltage: - Single DC input: –44 VDC to –60 VDC - AC and DC inputs: –54 VDC to –57 VDC |  |
| Maximum PoE power per port | 30 W |  |
| Total PoE power | AC: 370 W DC: 740 W |  |
| Minimum power consumption | AC: 37 W DC: 29 W | AC: 24 W DC: 17 W |
| Maximum power consumption (including PoE power consumption) | AC: 478 W DC: 825 W | AC: 451 W DC: 793 W |
| Chassis leakage current compliance | UL60950-1/EN60950-1/IEC60950-1/GB4943 |  |
| Melting current of power module fuse | 15 A/250 V |  |
| Cooling system | Using fixed fan trays to draw ambient air in from the left side and exhaust heated air from the right side | Using fixed fan trays to draw ambient air in from the left side and port side and exhaust heated air from the right side |
| Operating temperature | -5ºC～45ºC (23°F to 113°F) |  |
| Operating humidity | 5% to 95%, noncondensing |  |
| Fire resistance compliance | UL60950-1/EN60950-1/IEC60950-1/GB4943 |  |

5140EI series hardware features for PoE switch models(2)

| Item | 5140 24G PoE+ 2SFP+ 2XGT EI Sw | 5140 48G PoE+ 2SFP+ 2XGT EI Sw |
| --- | --- | --- |
| Dimensions (H × W × D) | 43.6 ×440 × 320 mm (1.72 × 17.32× 12.60 in) | 43.6 × 440 × 320 mm (1.72 × 17.32 ×12.60 in) |
| Weight | ≤4.5kg (9.92 lb) | ≤4.5 kg (9.92 lb) |
| Console port | - 1 × serial console port |  |
| 10/100/1000BASE-T autosensing Ethernet port | 24 | 48 |
| 1/10GBase-T autosensing Ethernet port | 2 | 2 |
| SFP+ port | 2 | 2 |
| Input voltage | - Rated voltage: 100 VAC to 240 VAC @ 50 or 60 Hz - Max voltage: 90 VAC to 264 VAC @ 47 to 63 Hz |  |
| Maximum PoE power per port | 30 W |  |
| Total PoE power | 370W |  |
| Minimum power consumption | 20W | 32W |
| Maximum power consumption (including PoE power consumption) | 450W | 470W |
| Chassis leakage current compliance | UL60950-1/EN60950-1/IEC60950-1/GB4943 |  |
| Melting current of power module fuse | 2 A/250 V | 2 A/250 V |
| Cooling system | Using fixed fan trays to draw ambient air in from the left side and exhaust heated air from the right side |  |
| Operating temperature | -5ºC～45ºC (23°F to 113°F) |  |
| Operating humidity | 5% to 95%, noncondensing |  |
| Fire resistance compliance | UL60950-1/EN60950-1/IEC60950-1/GB4943 |  |

Software features

Software features of the 5140EI series

| Feature | 5140EI series switch |
| --- | --- |
| IRF | - Ring topology - Daisy chain topology - LACP MAD - ARP MAD |
| Link aggregation | - Aggregation of 1-GE ports - Aggregation of 10-GE ports - Static link aggregation - Dynamic link aggregation - Inter-device aggregation - A maximum of 14 aggregation groups on a device - A maximum of 124 inter-device aggregation groups - A maximum of 8 ports for each aggregation group |
| Flow control | - IEEE 802.3x flow control |
| Jumbo Frame | - Supports maximum frame size of 10000 |
| MAC address table | - 16K MAC addresses - 1K static MAC addresses - Blackhole MAC addresses - MAC address learning limit on a port |
| VLAN | - Port-based VLANs (4094 VLANs) - QinQ - VLAN mapping |
| ARP | - 1K entries - 512 static entries - Gratuitous ARP - ARP black hole - ARP detection (based on DHCP snooping entries/802.1X security entries/static IP-to-MAC bindings) - ARP source suppression |
| ND | - 240 entries - 128 static entries |
| VLAN virtual interface | - 32 |
| DHCP | - DHCP client - DHCP snooping - DHCP relay - DHCP server - DHCPv6 Server - DHCPv6 relay - DHCPv6 snooping |
| UDP Helper | - UDP Helper |
| DNS | - Static DNS - Dynamic DNS - IPv4 and IPv6 DNS |
| unicast route | - IPv4 and IPv6 static routes - RIP/RIPng - OSPF/OSPFv3 - Routing policies - Policy-based routing - IPv6 policy-based routing |
| Multicast | - IGMP snooping - PIM Snooping - MLD snooping - IPv4 and IPv6 multicast VLAN - IPv6 PIM Snooping |
| Broadcast/multicast/unicast storm control | - Storm control based on port rate percentage - PPS-based storm control - Bps-based storm control |
| MSTP | - STP/RSTP/MSTP protocol - STP Root Guard - BPDU Guard - 128 PVST instances |
| QoS/ACL | - Remarking of 802.1p and DSCP priorities - Packet filtering at L2 (Layer 2) through L4 (Layer 4) - Eight output queues for each port - SP/WRR/SP+WRR queue scheduling algorithms - Port-based rate limiting - Flow-based redirection - Time range |
| Mirroring | - Stream mirroring - Port mirroring |
| Security | - Hierarchical management and password protection of users - AAA authentication - RADIUS authentication - HWTACACS - LDAP - SSH 2.0 - Port isolation - 802.1X - Portal - Port security - MAC-address-based authentication - IP Source Guard - HTTPS - PKI - IPsec - EAD - Public key management |
| 802.1X | - Up to 2K users - Port-based and MAC address-based authentication - Trunk port authentication - Dynamic 802.1X-based QoS/ACL/VLAN assignment |
| Loading and upgrading | - Loading and upgrading through XModem protocol - Loading and upgrading through FTP - Loading and upgrading through the trivial file transfer protocol (TFTP) |
| Management | - Configuration at the command line interface - Remote configuration through Telnet - Configuration through Console port - Simple network management protocol (SNMP) - Remote Monitoring(RMON) - IMC NMS - System log - Hierarchical alarms - NTP - Power supply alarm function - Fan and temperature alarms |
| Maintenance | - Debugging information output - Ping and Tracert - Remote maintenance through Telnet - NQA - 802.1ag - 802.3ah - DLDP - Virtual Cable Test |

Fixed security vulnerabilities

Fixed security vulnerabilities in R6343

      - CVE-2022-0778

A flaw was found in OpenSSL. It is possible to trigger an   infinite loop by crafting a certificate that has invalid explicit curve   parameters. Since certificate parsing happens before verification of the   certificate signature, any process that parses an externally supplied   certificate may be subject to a denial of service attack.

Upgrading software

This chapter describes types of software used on the switch and how to upgrade software while the switch is operating normally or when the switch cannot correctly start up.

System software file types

Software required for starting up the switch includes:

Boot ROM image—A .bin file that comprises a basic section and an extended section. The basic section is the minimum code that bootstraps the system. The extended section enables hardware initialization and provides system management menus. You can use these menus to load software and the startup configuration file or manage files when the switch cannot correctly start up.

Software images—Includes boot images and system images.

Boot image—A .bin file that contains the operating system kernel. It provides process management, memory management, file system management, and the emergency shell.

System image—A .bin file that contains the minimum modules required for device operation and some basic features, including device management, interface management, configuration management, and routing management.

The software images that have been loaded are called “current software images.” The software images specified to load at next startup are called “startup software images.”

These images might be released separately or as a whole in one .ipe package file. If an .ipe file is used, the system automatically decompresses the file, loads the .bin boot and system images in the file and sets them as startup software images. Typically, the Boot ROM and software images for this switch series are released in an .ipe file named main.ipe.

|  | NOTE: Boot ROM images are not released along with the boot images and system images. To get a version of Boot ROM image, contact the H3C technical support. |
| --- | --- |

System startup process

Upon power-on, the Boot ROM image runs to initialize hardware and then the software images run to start up the entire system, as shown in Figure 1.

System startup process

Upgrade methods

You can upgrade system software by using one of the following methods:

| Upgrading method | Software types | Remarks |
| --- | --- | --- |
| Upgrading from the CLI | - Boot ROM image - Software images | - You must reboot the switch to complete the upgrade. - This method can interrupt ongoing network services. |
| Upgrading from the Boot menu | - Boot ROM image - Software images | Use this method when the switch cannot correctly start up. CAUTION: Upgrading an IRF fabric from the CLI instead of the Boot menu. The Boot menu method increases the service downtime, because it requires that you upgrade the member switches one by one. |

The output in this document is for illustration only and might vary with software releases. This document uses boot.bin and system.bin to represent boot and system image names. The actual software image name format is chassis-model_Comware-version_image-type_release, for example, 5140_EI-CMW710-BOOT-R6351P02.bin and 5140_EI-CMW710-SYSM-R6351P02.bin.

Preparing for the upgrade

Verifying device status

Verify that the system state, redundancy state, and state of each slot are stable.

<Sysname> display system stable state

System state     : Stable

Redundancy state : No redundance

Slot    CPU    Role       State

1       0      Active     Stable

If the device is unstable, use the following commands to troubleshoot the issue:

Use the display device command to verify that the device is operating correctly.

Use the display ha service-group command to verify that bulk backup has been finished for all modules.

Use the display system internal process state command in probe view to verify that services are running correctly.

If a slot persists in unstable state or there are other unrecoverable issues, contact the technical support.

Setting up the upgrade environment

Before you upgrade system software, complete the following tasks:

Set up the upgrade environment as shown in Figure 2.

Configure routes to make sure that the router and the file server can reach each other.

Run a TFTP or FTP server on the file server.

Log in to the CLI of the router through the console port.

Copy the upgrade file to the file server and correctly set the working directory on the TFTP or FTP server.

Make sure that the upgrade has minimal impact on the network services. During the upgrade, the router cannot provide any services.

Setting up the upgrade environment

Upgrading from the CLI

This section uses a two-member IRF fabric as an example to describe how to upgrade software from the CLI. If you have more than two subordinate switches, repeat the steps for the subordinate switch to upgrade their software. If you are upgrading a standalone switch, ignore the steps for upgrading the subordinate switch. For more information about setting up and configuring an IRF fabric, see the installation guide and IRF configuration guide for the HPE 5140EI switch series.

Preparing for the upgrade

Before you upgrade software, complete the following tasks:

Log in to the IRF fabric through Telnet or the console port. (Details not shown.)

Identify the number of IRF members, each member switch's role, and IRF member ID.

<Sysname> display irf

MemberID   Role   Priority  CPU-Mac         Description

*+1      Master  5         0023-8927-afdc  ---

2      Standby 1         0023-8927-af43  ---

--------------------------------------------------

* indicates the device is the master.

+ indicates the device through which the user logs in.

The Bridge MAC of the IRF is: 0023-8927-afdb

Auto upgrade                : no

Mac persistent              : 6 min

Domain ID                   : 0

Verify that each IRF member switch has sufficient storage space for the upgrade images.

|  | IMPORTANT: Each IRF member switch must have free storage space that is at least two times the size of the upgrade image file. |
| --- | --- |

# Identify the free flash space of the master switch.

<Sysname> dir

Directory of flash:

0 drw-           - Jan 01 2013 00:17:27   diagfile

1 drw-           - Jan 01 2013 00:17:28   license

2 drw-           - Jan 01 2013 00:17:27   logfile

3 drw-           - Jan 01 2013 00:17:41   pki

4 -rw-     6161408 Jan 01 2013 00:17:27   boot.bin

5 -rw-    50729984 Jan 01 2013 00:17:27   system.bin

6 drw-           - Jan 01 2013 00:17:27   seclog

7 drw-           - Jan 01 2013 00:17:49   versionInfo

251904 KB total (192736 KB free)

# Identify the free flash space of each subordinate switch, for example, switch 2.

<Sysname> dir slot2#flash:/

Directory of slot2#flash:/

0 drw-           - Jan 01 2013 00:17:27   diagfile

1 drw-           - Jan 01 2013 00:17:28   license

2 drw-           - Jan 01 2013 00:17:27   logfile

3 drw-           - Jan 01 2013 00:17:41   pki

4 -rw-     6161408 Jan 01 2013 00:17:27   boot.bin

5 -rw-    50729984 Jan 01 2013 00:17:27   system.bin

6 drw-           - Jan 01 2013 00:17:27   seclog

7 drw-           - Jan 01 2013 00:17:49   versionInfo

251904 KB total (192736 KB free)

Compare the free flash space of each member switch with the size of the software file to load. If the space is sufficient, start the upgrade process. If not, go to the next step.

Delete unused files in the flash memory to free space:

|  | CAUTION: - To avoid data loss, do not delete the current configuration file. For information about the current configuration file, use the display startup command. - The delete /unreserved file-url command deletes a file permanently and the action cannot be undone. - The delete file-url command moves a file to the recycle bin and the file still occupies storage space. To free the storage space, first execute the undelete command to restore the file, and then execute the delete /unreserved file-url command. |
| --- | --- |

# Delete unused files from the flash memory of the master switch.

<Sysname> delete /unreserved flash:/backup.bin

The file cannot be restored. Delete flash:/backup.bin?[Y/N]:y

Deleting the file permanently will take a long time. Please wait...

Deleting file flash:/backup.bin...Done.

# Delete unused files from the flash memory of the subordinate switch.

<Sysname> delete /unreserved slot2#flash:/backup.bin

The file cannot be restored. Delete slot2#flash:/backup.bin?[Y/N]:y

Deleting the file permanently will take a long time. Please wait...

Deleting file slot2#flash:/backup.bin...Done.

Downloading software images to the master switch

Before you start upgrading software images packages, make sure you have downloaded the upgrading software files to the root directory in flash memory. This section describes downloading an .ipe software file as an example.

The following are ways to download, upload, or copy files to the master switch:

FTP download from a server

FTP upload from a client

TFTP download from a server

Prerequisites

If FTP or TFTP is used, the IRF fabric and the PC working as the FTP/TFTP server or FTP client can reach each other.

Prepare the FTP server or TFTP server program yourself for the PC. The switch series does not come with these software programs.

FTP download from a server

You can use the switch as an FTP client to download files from an FTP server.

To download a file from an FTP server, for example, the server at 10.10.110.1:

Run an FTP server program on the server, configure an FTP username and password, specify the working directory and copy the file, for example, newest.ipe, to the directory.

Execute the ftp command in user view on the IRF fabric to access the FTP server.

<Sysname> ftp 10.10.110.1

Press CTRL+C to abort

Connected to 10.10.110.1(10.10.110.1).

220 FTP service ready.

User (10.10.110.1:(none)):username

331 Password required for username.

Password:

230 User logged in.

Enable the binary transfer mode.

ftp> binary

200 Type is Image (Binary)

Execute the get command in FTP client view to download the file from the FTP server.

ftp> get newest.ipe

227 Entering Passive Mode (10,10,110,1,17,97).

125 BINARY mode data connection already open, transfer starting for /newest.ipe

226 Transfer complete.

32133120 bytes received in 35 seconds (896. 0 kbyte/s)

ftp> bye

221 Server closing.

FTP upload from a client

You can use the IRF fabric as an FTP server and upload files from a client to the IRF fabric.

To FTP upload a file from a client:

On the IRF fabric:

Enable FTP server.

<Sysname> system-view

[Sysname] ftp server enable

Configure a local FTP user account:

# Create the user account.

[Sysname] local-user abc

# Set its password and specify the FTP service.

[Sysname-luser-manage-abc] password simple pwd

[Sysname-luser-manage-abc] service-type ftp

# Assign the network-admin user role to the user account for uploading file to the working directory of the server.

[Sysname-luser-manage-abc] authorization-attribute user-role network-admin

[Sysname-luser-manage-abc] quit

[Sysname] quit

On the PC:

Log in to the IRF fabric (the FTP server) in FTP mode.

c:\> ftp 1.1.1.1

Connected to 1.1.1.1.

220 FTP service ready.

User(1.1.1.1:(none)):abc

331 Password required for abc.

Password:

230 User logged in.

Enable the binary file transfer mode.

ftp> binary

200 TYPE is now 8-bit binary.

Upload the file (for example, newest.ipe) to the root directory of the flash memory on the master switch.

ftp> put newest.ipe

200 PORT command successful

150 Connecting to port 10002

226 File successfully transferred

ftp: 32133120 bytes sent in 64.58 secs (497.60 Kbytes/sec).

TFTP download from a server

To download a file from a TFTP server, for example, the server at 10.10.110.1:

Run a TFTP server program on the server, specify the working directory, and copy the file, for example, newest.ipe, to the directory.

On the IRF fabric, execute the tftp command in user view to download the file to the root directory of the flash memory on the master switch.

<Sysname> tftp 10.10.110.1 get newest.ipe

Press CTRL+C to abort.

% Total    % Received % Xferd  Average Speed   Time    Time     Time  Current

Dload  Upload   Total   Spent    Left  Speed

100 30.6M    0 30.6M    0     0   143k      0 --:--:--  0:03:38 --:--:--  142k

Upgrading the software images

To upgrade the software images:

Specify the upgrade image file (newest.ipe in this example) used at the next startup for the master switch, and assign the M attribute to the boot and system images in the file.

<Sysname> boot-loader file flash:/newest.ipe slot 1 main

Verifying the file flash:/newest.ipe on slot 1......Done.

Images in IPE:

boot.bin

system.bin

This command will set the main startup software images. Continue? [Y/N]:y

Add images to slot 1.

Decompressing file boot.bin to flash:/boot.bin....................Done.

Decompressing file system.bin to flash:/system.bin................Done.

Verifying the file flash:/boot.bin on slot 1...Done.

Verifying the file flash:/system.bin on slot 1............Done.

The images that have passed all examinations will be used as the main startup software images at the next reboot on slot 1.

Specify the upgrade image file as the main startup image file for each subordinate switch. This example uses IRF member 2. (The subordinate switches will automatically copy the file to the root directory of their flash memories.)

<Sysname> boot-loader file flash:/newest.ipe slot 2 main

Verifying the file flash:/newest.ipe on slot 2......Done.

Images in IPE:

boot.bin

system.bin

This command will set the main startup software images. Continue? [Y/N]:y

Add images to slot 2.

Decompressing file boot.bin to flash:/boot.bin....................Done.

Decompressing file system.bin to flash:/system.bin................Done.

Verifying the file flash:/boot.bin on slot 2...Done.

Verifying the file flash:/system.bin on slot 2............Done.

The images that have passed all examinations will be used as the main startup software images at the next reboot on slot 2.

Enable the software auto-update function.

<Sysname> system-view

[Sysname] irf auto-update enable

[Sysname] quit

This function checks the software versions of member switches for inconsistency with the master switch. If a subordinate switch is using a different software version than the master, the function propagates the current software images of the master to the subordinate as main startup images. The function prevents software version inconsistency from causing the IRF setup failure.

Save the current configuration in any view to prevent data loss.

<Sysname> save

The current configuration will be written to the device. Are you sure? [Y/N]:y

Please input the file name(*.cfg)[flash:/startup.cfg]

(To leave the existing filename unchanged, press the enter key):

flash:/startup.cfg exists, overwrite? [Y/N]:y

Validating file. Please wait.................

Saved the current configuration to mainboard device successfully.

Slot 2:

Save next configuration file successfully.

Reboot the IRF fabric to complete the upgrade.

<Sysname> reboot

Start to check configuration with next startup configuration file, please wait.

........DONE!

This command will reboot the device. Continue? [Y/N]:y

Now rebooting, please wait...

The system automatically loads the .bin boot and system images in the .ipe file and sets them as the startup software images.

Execute the display version command in any view to verify that the current main software images have been updated (details not shown).

|  | NOTE: The system automatically checks the compatibility of the Boot ROM image and the boot and system images during the reboot. If you are prompted that the Boot ROM image in the upgrade image file is different than the current Boot ROM image, upgrade both the basic and extended sections of the Boot ROM image for compatibility. If you choose to not upgrade the Boot ROM image, the system will ask for an upgrade at the next reboot performed by powering on the switch or rebooting from the CLI (promptly or as scheduled). If you fail to make any choice in the required time, the system upgrades the entire Boot ROM image. |
| --- | --- |

Upgrading from the Boot menu

In this approach, you must access the Boot menu of each member switch to upgrade their software one by one. If you are upgrading software images for an IRF fabric, using the CLI is a better choice.

|  | TIP: Upgrading through the Ethernet port is faster than through the console port. |
| --- | --- |

Prerequisites

Make sure the prerequisites are met before you start upgrading software from the Boot menu.

Setting up the upgrade environment

Use a console cable to connect the console terminal (for example, a PC) to the console port on the switch.

Connect the Ethernet port on the switch to the file server.

|  | NOTE: The file server and the configuration terminal can be co-located. |
| --- | --- |

Run a terminal emulator program on the console terminal and set the following terminal settings:

Bits per second—9,600

Data bits—8

Parity—None

Stop bits—1

Flow control—None

Emulation—VT100

Preparing for the TFTP or FTP transfer

To use TFTP or FTP:

Run a TFTP or FTP server program on the file server or the console terminal.

Copy the upgrade file to the file server.

Correctly set the working directory on the TFTP or FTP server.

Make sure the file server and the switch can reach each other.

Verifying that sufficient storage space is available

|  | IMPORTANT: For the switch to start up correctly, do not delete the main startup software images when you free storage space before upgrading Boot ROM. On the Boot menu, the main startup software images are marked with an asterisk (*). |
| --- | --- |

When you upgrade software, make sure each member switch has sufficient free storage space for the upgrade file, as shown in Table 9.

Minimum free storage space requirements

| Upgraded images | Minimum free storage space requirements |
| --- | --- |
| Comware images | Two times the size of the Comware upgrade package file. |
| Boot ROM | Same size as the Boot ROM upgrade image file. |

If no sufficient space is available, delete unused files as described in “Managing files from the Boot menu.”

Scheduling the upgrade time

During the upgrade, the switch cannot provide any services. You must make sure the upgrade has a minimal impact on the network services.

Accessing the Boot menu

Starting......

Press Ctrl+D to access BASIC BOOT MENU

Booting Normal Extend BootWare....

********************************************************************************

*                                                                              *

*             HPE 5140 24G PoE+ 4SFP+ EI Sw BOOTROM, Version 151               *

*                                                                              *

********************************************************************************

Copyright (c) 2010-2023 Hewlett Packard Enterprise Development LP

Creation Date       : Mar 13 2023, 17:35:17

CPU Clock Speed     : 800MHz

Memory Size         : 512MB

Flash Size          : 256MB

CPLD Version        : 001

PCB Version         : Ver.A

Mac Address         : aa1122334455

Press Ctrl+B to access EXTENDED BOOT MENU...1

Press one of the shortcut key combinations at prompt.

Shortcut keys

| Shortcut keys | Prompt message | Function | Remarks |
| --- | --- | --- | --- |
| Ctrl+B | Press Ctrl+B to enter Extended Boot menu... | Accesses the extended Boot menu. | Press the keys within 1 second (in fast startup mode) or 5 seconds (in full startup mode) after the message appears. You can upgrade and manage system software and Boot ROM from this menu. |

Accessing the extended Boot menu

Press Ctrl+B within 1 second (in fast startup mode) or 5 seconds (in full startup mode) after the "Press Ctrl-B to enter Extended Boot menu..." prompt message appears. If you fail to do this, the system starts decompressing the system software.

Alternatively, you can enter 4 in the basic Boot menu to access the extended Boot menu.

The "Password recovery capability is enabled." or "Password recovery capability is disabled." message appears, followed by the extended Boot menu. Availability of some menu options depends on the state of password recovery capability (see Table 11). For more information about password recovery capability, see Fundamentals Configuration Guide in HPE FlexNetwork 5140 EI Switch Series Configuration Guides-R63xx.

Password recovery capability is enabled.

EXTENDED BOOT MENU

1. Download image to flash

2. Select image to boot

3. Display all files in flash

4. Delete file from flash

5. Restore to factory default configuration

6. Enter BootRom upgrade menu

7. Skip current system configuration

8. Set switch startup mode

0. Reboot

Ctrl+Z: Access EXTENDED ASSISTANT MENU

Ctrl+F: Format file system

Ctrl+P: Change authentication for console login

Ctrl+R: Download image to SDRAM and run

Ctrl+Y: Change Work Mode

Ctrl+C: Display Copyright

Enter your choice(0-8):

Extended Boot ROM menu options

| Option | Tasks |
| --- | --- |
| 1. Download image to flash | Download a software image file to the flash. |
| 2. Select image to boot | - Specify the main and backup software image file for the next startup. - Specify the main and backup configuration files for the next startup. This task can be performed only if password recovery capability is enabled. |
| 3. Display all files in flash | Display files on the flash. |
| 4. Delete file from flash | Delete files to free storage space. |
| 5. Restore to factory default configuration | Delete the current next-startup configuration files and restore the factory-default configuration. This option is available only if password recovery capability is disabled. |
| 6. Enter BootRom upgrade menu | Access the Boot ROM upgrade menu. |
| 7. Skip current system configuration | Start the switch without loading any configuration file. This is a one-time operation and takes effect only for the first system boot or reboot after you choose this option. This option is available only if password recovery capability is enabled. |
| 8. Set switch startup mode | Set the startup mode to fast startup mode or full startup mode. |
| 0. Reboot | Reboot the switch. |
| Ctrl+F: Format file system | Format the current storage medium. |
| Ctrl+P: Change authentication for console login | Skip the authentication for console login. This is a one-time operation and takes effect only for the first system boot or reboot after you choose this option. This option is available only if password recovery capability is enabled. |
| Ctrl+R: Download image to SDRAM and run | Download a system software image and start the switch with the image. This option is available only if password recovery capability is enabled. |
| Ctrl+Z: Access EXTENDED ASSISTANT MENU | Access the EXTENDED ASSISTANT MENU. For options in the menu, see Table 12. |
| Ctrl+Y: Change Work Mode | Change Work Mode. |
| Ctrl+C: Display Copyright | Display the copyright statement. |

EXTENDED ASSISTANT menu options

| Option | Task |
| --- | --- |
| 1. Display Memory | Display data in the memory. |
| 2. Search Memory | Search the memory for a specific data segment. |
| 0. Return to boot menu | Return to the extended Boot ROM menu. |

Upgrading Comware images from the Boot menu

You can use the following methods to upgrade Comware images:

Using TFTP to upgrade software images through the Ethernet port

Using FTP to upgrade software images through the Ethernet port

Using XMODEM to upgrade software through the console port

Using TFTP to upgrade software images through the Ethernet port

Enter 1 in the Boot menu to access the file transfer protocol submenu.

1. Set TFTP protocol parameters

2. Set FTP protocol parameters

3. Set XMODEM protocol parameters

0. Return to boot menu

Enter your choice(0-3):

Enter 1 to set the TFTP parameters.

Load File Name      :update.ipe

Server IP Address   :192.168.0.3

Local IP Address    :192.168.0.2

Subnet Mask         :255.255.255.0

Gateway IP Address  :0.0.0.0

TFTP parameter description

| Item | Description |
| --- | --- |
| Load File Name | Name of the file to download (for example, update.ipe). |
| Server IP Address | IP address of the TFTP server (for example, 192.168.0.3). |
| Local IP Address | IP address of the switch (for example, 192.168.0.2). |
| Subnet Mask | Subnet mask of the switch (for example, 255.255.255.0). |
| Gateway IP Address | IP address of the gateway (in this example, no gateway is required because the server and the switch are on the same subnet). |

|  | NOTE: - To use the default setting for a field, press Enter without entering any value. - If the switch and the server are on different subnets, you must specify a gateway address for the switch. |
| --- | --- |

Enter all required parameters, and enter Y to confirm the settings. The following prompt appears:

Are you sure to download file to flash? Yes or No (Y/N):Y

Enter Y to start downloading the image file. To return to the Boot menu without downloading the upgrade file, enter N.

Loading.........................................................................

................................................................................

................................................................................

................................................................Done!

Enter the M (main), B (backup), or N (none) attribute for the images. In this example, assign the main attribute to the images.

Please input the file attribute (Main/Backup/None) M

Image file boot.bin is self-decompressing...

Free space: 534980608 bytes

Writing flash...................................................................

................................................................................

...................................................................Done!

Image file system.bin is self-decompressing...

Free space: 525981696 bytes

Writing flash...................................................................

................................................................................

................................................................................

................................................................................

................................................................................

................................................................................

.......................................................................Done!

|  | NOTE: - The switch always attempts to boot with the main images first. If the attempt fails, for example, because the main images are not available, the switch tries to boot with the backup images. An image with the none attribute is only stored in flash memory for backup. To use it at reboot, you must change its attribute to main or backup. - If an image with the same attribute as the image you are loading is already in the flash memory, the attribute of the old image changes to none after the new image becomes valid. |
| --- | --- |

Enter 0 in the Boot menu to reboot the switch with the new software images.

EXTENDED BOOT MENU

1. Download image to flash

2. Select image to boot

3. Display all files in flash

4. Delete file from flash

5. Restore to factory default configuration

6. Enter BootRom upgrade menu

7. Skip current system configuration

8. Set switch startup mode

0. Reboot

Ctrl+Z: Access EXTENDED ASSISTANT MENU

Ctrl+F: Format file system

Ctrl+P: Change authentication for console login

Ctrl+R: Download image to SDRAM and run

Ctrl+Y: Change Work Mode

Ctrl+C: Display Copyright

Enter your choice(0-8): 0

Using FTP to upgrade software images through the Ethernet port

Enter 1 in the Boot menu to access the file transfer protocol submenu.

1. Set TFTP protocol parameters

2. Set FTP protocol parameters

3. Set XMODEM protocol parameters

0. Return to boot menu

Enter your choice(0-3):

Enter 2 to set the FTP parameters.

Load File Name      :update.ipe

Server IP Address   :192.168.0.3

Local IP Address    :192.168.0.2

Subnet Mask         :255.255.255.0

Gateway IP Address  :0.0.0.0

FTP User Name       :switch

FTP User Password   :***

FTP parameter description

| Item | Description |
| --- | --- |
| Load File Name | Name of the file to download (for example, update.ipe). |
| Server IP Address | IP address of the FTP server (for example, 192.168.0.3). |
| Local IP Address | IP address of the switch (for example, 192.168.0.2). |
| Subnet Mask | Subnet mask of the switch (for example, 255.255.255.0). |
| Gateway IP Address | IP address of the gateway (in this example, no gateway is required because the server and the switch are on the same subnet). |
| FTP User Name | Username for accessing the FTP server, which must be the same as configured on the FTP server. |
| FTP User Password | Password for accessing the FTP server, which must be the same as configured on the FTP server. |

|  | NOTE: - To use the default setting for a field, press Enter without entering any value. - If the switch and the server are on different subnets, you must specify a gateway address for the switch. |
| --- | --- |

Enter all required parameters, and enter Y to confirm the settings. The following prompt appears:

Are you sure to download file to flash? Yes or No (Y/N):Y

Enter Y to start downloading the image file. To return to the Boot menu without downloading the upgrade file, enter N.

Loading.........................................................................

................................................................................

................................................................................

................................................................Done!

Enter the M (main), B (backup), or N (none) attribute for the images. In this example, assign the main attribute to the images.

Please input the file attribute (Main/Backup/None) M

Image file boot.bin is self-decompressing...

Free space: 534980608 bytes

Writing flash...................................................................

................................................................................

...................................................................Done!

Image file system.bin is self-decompressing...

Free space: 525981696 bytes

Writing flash...................................................................

................................................................................

................................................................................

................................................................................

................................................................................

................................................................................

.......................................................................Done!

EXTENDED BOOT MENU

1. Download image to flash

2. Select image to boot

3. Display all files in flash

4. Delete file from flash

5. Restore to factory default configuration

6. Enter BootRom upgrade menu

7. Skip current system configuration

8. Set switch startup mode

0. Reboot

Ctrl+Z: Access EXTENDED ASSISTANT MENU

Ctrl+F: Format file system

Ctrl+P: Change authentication for console login

Ctrl+R: Download image to SDRAM and run

Ctrl+Y: Change Work Mode

Ctrl+C: Display Copyright

Enter your choice(0-8):0

|  | NOTE: - The switch always attempts to boot with the main images first. If the attempt fails, for example, because the main images not available, the switch tries to boot with the backup images. An image with the none attribute is only stored in flash memory for backup. To use it at reboot, you must change its attribute to main or backup. - If an image with the same attribute as the image you are loading is already in the flash memory, the attribute of the old image changes to none after the new image becomes valid. |
| --- | --- |

Enter 0 in the Boot menu to reboot the switch with the new software images.

Using XMODEM to upgrade software through the console port

XMODEM download through the console port is slower than TFTP or FTP download through the Ethernet port. To save time, use the Ethernet port as long as possible.

Enter 1 in the Boot menu to access the file transfer protocol submenu.

1. Set TFTP protocol parameters

2. Set FTP protocol parameters

3. Set XMODEM protocol parameters

0. Return to boot menu

Enter your choice(0-3):

Enter 3 to set the XMODEM download baud rate.

Please select your download baudrate:

1.* 9600

2.  19200

3.  38400

4.  57600

5.  115200

0.  Return to boot menu

Enter your choice(0-5):5

Select an appropriate download rate, for example, enter 5 to select 115200 bps.

Download baudrate is 115200 bps

Please change the terminal's baudrate to 115200 bps and select XMODEM protocol

Press enter key when ready

Set the serial port on the terminal to use the same baud rate and protocol as the console port. If you select 9600 bps as the download rate for the console port, skip this task.

Select Call > Disconnect in the HyperTerminal window to disconnect the terminal from the switch.

Disconnecting the terminal from the switch

Select File > Properties, and in the Properties dialog box, click Configure.

Properties dialog box

Select 115200 from the Bits per second list and click OK.

Modifying the baud rate

Select Call > Call to reestablish the connection.

Reestablishing the connection

Press Enter. The following prompt appears:

Are you sure to download file to flash? Yes or No (Y/N):Y

Enter Y to start downloading the file. (To return to the Boot menu, enter N.)

Now please start transfer file with XMODEM protocol

If you want to exit, Press <Ctrl+X>

Loading ...CCCCCCCCCCCCCCCCCCCCCCCCC

Select Transfer > Send File in the HyperTerminal window.

Transfer menu

In the dialog box that appears, click Browse to select the source file, and select Xmodem from the Protocol list.

File transmission dialog box

Click Send. The following dialog box appears:

File transfer progress

Enter the M (main), B (backup), or N (none) attribute for the images. In this example, assign the main attribute to the images.

Please input the file attribute (Main/Backup/None) m

The boot.bin image is self-decompressing...

# At the Load File name prompt, enter a name for the boot image to be saved to flash memory.

Load File name  : default_file boot-update.bin       (At the prompt,

Free space: 470519808 bytes

Writing flash...................................................................

.............Done!

The system-update.bin image is self-decompressing...

# At the Load File name prompt, enter a name for the system image to be saved to flash memory.

Load File name  : default_file system-update.bin

Free space: 461522944 bytes

Writing flash...................................................................

.............Done!

Your baudrate should be set to 9600 bps again!

Press enter key when ready

|  | NOTE: - The switch always attempts to boot with the main images first. If the attempt fails, for example, because the main images not available, the switch tries to boot with the backup images. An image with the none attribute is only stored in the flash memory for backup. To use it at reboot, you must change its attribute to main or backup. - If an image with the same attribute as the image you are loading is already in flash memory, the attribute of the old image changes to none after the new image becomes valid. |
| --- | --- |

If the baud rate of the HyperTerminal is not 9600 bps, restore it to 9600 bps as described in step 5.a. If the baud rate is 9600 bps, skip this step.

|  | NOTE: The console port rate reverts to 9600 bps at a reboot. If you have changed the baud rate, you must perform this step so you can access the switch through the console port after a reboot. |
| --- | --- |

EXTENDED BOOT MENU

1. Download image to flash

2. Select image to boot

3. Display all files in flash

4. Delete file from flash

5. Restore to factory default configuration

6. Enter BootRom upgrade menu

7. Skip current system configuration

8. Set switch startup mode

0. Reboot

Ctrl+Z: Access EXTENDED ASSISTANT MENU

Ctrl+F: Format file system

Ctrl+P: Change authentication for console login

Ctrl+R: Download image to SDRAM and run

Ctrl+Y: Change Work Mode

Ctrl+C: Display Copyright

Enter your choice(0-8): 0

Enter 0 in the Boot menu to reboot the system with the new software images.

Upgrading Boot ROM from the Boot menu

You can use the following methods to upgrade the Boot ROM image:

Using TFTP to upgrade Boot ROM through the Ethernet port

Using FTP to upgrade Boot ROM through the Ethernet port

Using XMODEM to upgrade Boot ROM through the console port

Using TFTP to upgrade Boot ROM through the Ethernet port

Enter 6 in the Boot menu to access the Boot ROM update menu.

1. Update full BootRom

2. Update extended BootRom

3. Update basic BootRom

0. Return to boot menu

Enter your choice(0-3):

Enter 1 in the Boot ROM update menu to upgrade the full Boot ROM.

The file transfer protocol submenu appears:

1. Set TFTP protocol parameters

2. Set FTP protocol parameters

3. Set XMODEM protocol parameters

0. Return to boot menu

Enter your choice(0-3):

Enter 1 to set the TFTP parameters.

Load File Name      :update.btm

Server IP Address   :192.168.0.3

Local IP Address    :192.168.0.2

Subnet Mask         :255.255.255.0

Gateway IP Address  :0.0.0.0

TFTP parameter description

| Item | Description |
| --- | --- |
| Load File Name | Name of the file to download (for example, update.btm). |
| Server IP Address | IP address of the TFTP server (for example, 192.168.0.3). |
| Local IP Address | IP address of the switch (for example, 192.168.0.2). |
| Subnet Mask | Subnet mask of the switch (for example, 255.255.255.0). |
| Gateway IP Address | IP address of the gateway (in this example, no gateway is required because the server and the switch are on the same subnet). |

|  | NOTE: - To use the default setting for a field, press Enter without entering any value. - If the switch and the server are on different subnets, you must specify a gateway address for the switch. |
| --- | --- |

Enter all required parameters and press Enter to start downloading the file.

Loading.................................................Done!

Enter Y at the prompt to upgrade the basic Boot ROM section.

Will you Update Basic BootRom? (Y/N):Y

Updating Basic BootRom...........Done.

Enter Y at the prompt to upgrade the extended Boot ROM section.

Updating extended BootRom? (Y/N):Y

Updating extended BootRom.........Done.

Enter 0 in the Boot ROM update menu to return to the Boot menu.

1. Update full BootRom

2. Update extended BootRom

3. Update basic BootRom

0. Return to boot menu

Enter your choice(0-3):

Enter 0 in the Boot menu to reboot the switch with the new Boot ROM image.

Using FTP to upgrade Boot ROM through the Ethernet port

Enter 6 in the Boot menu to access the Boot ROM update menu.

1. Update full BootRom

2. Update extended BootRom

3. Update basic BootRom

0. Return to boot menu

Enter your choice(0-3):

Enter 1 in the Boot ROM update menu to upgrade the full Boot ROM.

The file transfer protocol submenu appears:

1. Set TFTP protocol parameters

2. Set FTP protocol parameters

3. Set XMODEM protocol parameters

0. Return to boot menu

Enter your choice(0-3):

Enter 2 to set the FTP parameters.

Load File Name     :update.btm

Server IP Address  :192.168.0.3

Local IP Address   :192.168.0.2

Subnet Mask        :255.255.255.0

Gateway IP Address :0.0.0.0

FTP User Name      :switch

FTP User Password  :123

FTP parameter description

| Item | Description |
| --- | --- |
| Load File Name | Name of the file to download (for example, update.btm). |
| Server IP Address | IP address of the FTP server (for example, 192.168.0.3). |
| Local IP Address | IP address of the switch (for example, 192.168.0.2). |
| Subnet Mask | Subnet mask of the switch (for example, 255.255.255.0). |
| Gateway IP Address | IP address of the gateway (in this example, no gateway is required because the server and the switch are on the same subnet). |
| FTP User Name | Username for accessing the FTP server, which must be the same as configured on the FTP server. |
| FTP User Password | Password for accessing the FTP server, which must be the same as configured on the FTP server. |

|  | NOTE: - To use the default setting for a field, press Enter without entering any value. - If the switch and the server are on different subnets, you must specify a gateway address for the switch. |
| --- | --- |

Enter all required parameters and press Enter to start downloading the file.

Loading.................................................Done!

Enter Y at the prompt to upgrade the basic Boot ROM section.

Will you Update Basic BootRom? (Y/N):Y

Updating Basic BootRom...........Done.

Enter Y at the prompt to upgrade the extended Boot ROM section.

Updating extended BootRom? (Y/N):Y

Updating extended BootRom.........Done.

Enter 0 in the Boot ROM update menu to return to the Boot menu.

1. Update full BootRom

2. Update extended BootRom

3. Update basic BootRom

0. Return to boot menu

Enter your choice(0-3):

Enter 0 in the Boot menu to reboot the switch with the new Boot ROM image.

Using XMODEM to upgrade Boot ROM through the console port

XMODEM download through the console port is slower than TFTP or FTP download through the Ethernet port. To save time, use the Ethernet port as long as possible.

Enter 6 in the Boot menu to access the Boot ROM update menu.

1. Update full BootRom

2. Update extended BootRom

3. Update basic BootRom

0. Return to boot menu

Enter your choice(0-3):

Enter 1 in the Boot ROM update menu to upgrade the full Boot ROM.

The file transfer protocol submenu appears:

1. Set TFTP protocol parameters

2. Set FTP protocol parameters

3. Set XMODEM protocol parameters

0. Return to boot menu

Enter your choice(0-3):

Enter 3 to set the XMODEM download baud rate.

Please select your download baudrate:

1.* 9600

2.  19200

3.  38400

4.  57600

5.  115200

0.  Return to boot menu

Enter your choice(0-5):5

Select an appropriate download rate, for example, enter 5 to select 115200 bps.

Download baudrate is 115200 bps

Please change the terminal's baudrate to 115200 bps and select XMODEM protocol

Press enter key when ready

Set the serial port on the terminal to use the same baud rate and protocol as the console port. If you select 9600 bps as the download rate for the console port, skip this task.

Select Call > Disconnect in the HyperTerminal window to disconnect the terminal from the switch.

Disconnecting the terminal from the switch

Select File > Properties, and in the Properties dialog box, click Configure.

Properties dialog box

Select 115200 from the Bits per second list and click OK.

Modifying the baud rate

Select Call > Call to reestablish the connection.

Reestablishing the connection

Press Enter to start downloading the file.

Now please start transfer file with XMODEM protocol

If you want to exit, Press <Ctrl+X>

Loading ...CCCCCCCCCCCCCCCCCCCCCCCCC

Select Transfer > Send File in the HyperTerminal window.

Transfer menu

In the dialog box that appears, click Browse to select the source file, and select Xmodem from the Protocol list.

File transmission dialog box

Click Send. The following dialog box appears:

File transfer progress

Enter Y at the prompt to upgrade the basic Boot ROM section.

Loading ...CCCCCCCCCCCCCC  ...Done!

Will you Update Basic BootRom? (Y/N):Y

Updating Basic BootRom...........Done.

Enter Y at the prompt to upgrade the extended Boot ROM section.

Updating extended BootRom? (Y/N):Y

Updating extended BootRom.........Done.

If the baud rate of the HyperTerminal is not 9600 bps, restore it to 9600 bps at the prompt, as described in step 4.a. If the baud rate is 9600 bps, skip this step.

Please change the terminal's baudrate to 9600 bps, press ENTER when ready.

|  | NOTE: The console port rate reverts to 9600 bps at a reboot. If you have changed the baud rate, you must perform this step so you can access the switch through the console port after a reboot. |
| --- | --- |

Press Enter to access the Boot ROM update menu.

Enter 0 in the Boot ROM update menu to return to the Boot menu.

1. Update full BootRom

2. Update extended BootRom

3. Update basic BootRom

0. Return to boot menu

Enter your choice(0-3):

Enter 0 in the Boot menu to reboot the switch with the new Boot ROM image.

Managing files from the Boot menu

From the Boot menu, you can display files in flash memory to check for obsolete files, incorrect files, or space insufficiency, delete files to release storage space, or change the attributes of software images.

Displaying all files

Enter 3 in the Boot menu to display all files in flash memory and identify the free space size.

EXTENDED BOOT MENU

1. Download image to flash

2. Select image to boot

3. Display all files in flash

4. Delete file from flash

5. Restore to factory default configuration

6. Enter BootRom upgrade menu

7. Skip current system configuration

8. Set switch startup mode

0. Reboot

Ctrl+Z: Access EXTENDED ASSISTANT MENU

Ctrl+F: Format file system

Ctrl+P: Change authentication for console login

Ctrl+R: Download image to SDRAM and run

Ctrl+Y: Change Work Mode

Ctrl+C: Display Copyright

Enter your choice(0-8): 3

The following is a sample output:

Display all file(s) in flash:

File Number    File Size(bytes)     File Name

================================================================================

1              8177                 flash:/testbackup.cfg

2(*)           53555200             flash:/system.bin

3(*)           9959424              flash:/boot.bin

4              3678                 flash:/startup.cfg_backup

5              30033                flash:/default.mdb

6              42424                flash:/startup.mdb

7              18                   flash:/.pathfile

8              232311               flash:/logfile/logfile.log

9              5981                 flash:/startup.cfg_back

10(*)          6098                 flash:/startup.cfg

11             20                   flash:/.snmpboots

Free space: 464298848 bytes

The current image is boot.bin

(*)-with main attribute

(b)-with backup attribute

(*b)-with both main and backup attribute

Deleting files

If storage space is insufficient, delete obsolete files to free up storage space.

To delete files:

Enter 4 in the Boot menu:

Deleting the file in flash:

File Number    File Size(bytes)     File Name

================================================================================

1              8177                 flash:/testbackup.cfg

2(*)           53555200             flash:/system.bin

3(*)           9959424              flash:/boot.bin

4              3678                 flash:/startup.cfg_backup

5              30033                flash:/default.mdb

6              42424                flash:/startup.mdb

7              18                   flash:/.pathfile

8              232311               flash:/logfile/logfile.log

9              5981                 flash:/startup.cfg_back

10(*)          6098                 flash:/startup.cfg

11             20                   flash:/.snmpboots

Free space: 464298848 bytes

The current image is boot.bin

(*)-with main attribute

(b)-with backup attribute

(*b)-with both main and backup attribute

Enter the number of the file to delete. For example, enter 1 to select the file testbackup.cfg.

Please input the file number to change: 1

Enter Y at the confirmation prompt.

The file you selected is testbackup.cfg,Delete it? (Y/N):Y

Deleting....................................Done!

Changing the attribute of software images

Software image attributes include main (M), backup (B), and none (N). System software and boot software can each have multiple none-attribute images but only one main image and one backup image on the switch. You can assign both the M and B attributes to one image. If the M or B attribute you are assigning has been assigned to another image, the assignment removes the attribute from that image. If the removed attribute is the sole attribute of the image, its attribute changes to N.

For example, the system image system.bin has the M attribute and the system image system-update.bin has the B attribute. After you assign the M attribute to system-update.bin, the attribute of system-update.bin changes to M+B and the attribute of system.bin changes to N.

To change the attribute of a system or boot image:

Enter 2 in the Boot menu.

EXTENDED BOOT MENU

1. Download image to flash

2. Select image to boot

3. Display all files in flash

4. Delete file from flash

5. Restore to factory default configuration

6. Enter BootRom upgrade menu

7. Skip current system configuration

8. Set switch startup mode

0. Reboot

Ctrl+Z: Access EXTENDED ASSISTANT MENU

Ctrl+F: Format file system

Ctrl+P: Change authentication for console login

Ctrl+R: Download image to SDRAM and run

Ctrl+Y: Change Work Mode

Ctrl+C: Display Copyright

Enter your choice(0-8): 2

1 or 2 at the prompt to set the attribute of a software image. (The following output is based on the option 2. To set the attribute of a configuration file, enter 3.)

1. Set image file

2. Set bin file

3. Set configuration file

0. Return to boot menu

Enter your choice(0-3): 2

File Number    File Size(bytes)     File Name

================================================================================

1(*)              53555200              flash:/system.bin

2(*)              9959424               flash:/boot.bin

3                 13105152              flash:/boot-update.bin

4                 91273216              flash:/system-update.bin

Free space: 417177920 bytes

(*)-with main attribute

(b)-with backup attribute

(*b)-with both main and backup attribute

Note:Select .bin files. One but only one boot image and system image must be included.

Enter the number of the file you are working with. For example, enter 3 to select the boot image boot-update.bin. and enter 4 to select the system image system-update.bin.

Enter file No.(Allows multiple selection):3

Enter another file No.(0-Finish choice):4

Enter 0 to finish the selection.

Enter another file No.(0-Finish choice):0

You have selected:

flash:/boot-update.bin

flash:/system-update.bin

Enter M or B to change its attribute to main or backup. If you change its attribute to M, the attribute of boot.bin changes to none.

Please input the file attribute (Main/Backup) M

This operation may take several minutes. Please wait....

Next time, boot-update.bin will become default boot file!

Next time, system-update.bin will become default boot file!

Set the file attribute success!

Handling software upgrade failures

If a software upgrade fails, the system runs the old software version.

To handle a software upgrade failure:

Verify that the software release is compatible with the switch model and the correct file is used.

Verify that the software release and the Boot ROM release are compatible. For software and Boot ROM compatibility, see the hardware and software compatibility matrix in the correct release notes.

Check the physical ports for a loose or incorrect connection.

If you are using the console port for file transfer, check the HyperTerminal settings (including the baud rate and data bits) for any wrong setting.

Check the file transfer settings:

If XMODEM is used, you must set the same baud rate for the terminal as for the console port.

If TFTP is used, you must enter the same server IP addresses, file name, and working directory as set on the TFTP server.

If FTP is used, you must enter the same FTP server IP address, source file name, working directory, and FTP username and password as set on the FTP server.

Check the FTP or TFTP server for any incorrect setting.

Check that the storage device has sufficient space for the upgrade file.