HPE 5140_EI-CMW710-R6351P02 Release Notes

Software Feature Changes

The information in this document is subject to change without notice. © Copyright 2023 Hewlett Packard Enterprise Development LP

Contents

Release 6351P02	1

Release 6351	2

New feature: Advertising proprietary TLVs on an interface	3

Advertising proprietary TLVs on an interface	3

Command reference	3

lldp tlv-enable private-tlv	3

New feature: Configuring a dynamic aggregation group to use port speed as the prioritized criterion for reference port selection	4

Configuring a dynamic aggregation group to use port speed as the prioritized criterion for reference port selection	4

Command reference	4

lacp select speed	4

New feature: Loopback test on an interface	5

Performing a loopback test on an interface	5

About this task	5

Restrictions and guidelines	6

Procedure	6

Command reference	6

New command: loopback-test	6

New feature: Configuring disk usage monitoring	6

Command changes	7

monitor disk-usage disk	7

monitor disk-usage interval	8

New feature: Enabling the portal fail-permit feature for portal Web servers	8

Enabling the portal fail-permit feature for portal Web servers	8

Command reference	9

portal fail-permit web-server	9

New feature: Configuring packet detection for 802.1X authentication	10

Configuring packet detection for 802.1X authentication	10

Command reference	11

New command: dot1x packet-detect enable	11

New command: dot1x packet-detect retry	12

Modified command: display dot1x connection	13

New feature: Configuring packet detection for MAC authentication	14

Configuring packet detection for MAC authentication	14

Command reference	16

New command: mac-authentication packet-detect enable	16

New command: mac-authentication packet-detect retry	17

Modified command: display mac-authentication connection	17

New feature: Specifying an IP address and mask for calculating the source IP of ARP detection packets	18

Specifying an IP address and mask for calculating the source IP of ARP detection packets	18

Command reference	19

port-security packet-detect arp-source-ip factor	19

New feature: Associating PoE with Track	20

Associating PoE with Track	20

Command reference	21

poe track	21

New feature: many-to-one VLAN mapping	22

Configuring many-to-one VLAN mapping	22

About many-to-one VLAN mapping	22

About many-to-one VLAN mapping	22

Command reference	23

vlan mapping uni	23

New feature: Disabling receiving a specific type of ICMP messages	24

Disabling receiving a specific type of ICMP messages	24

Command reference	25

ip icmp receive enable	25

New feature: Disabling sending a specific type of ICMP messages	26

Disabling sending a specific type of ICMP messages	26

Command reference	27

ip icmp send enable	27

New feature: MAC swap loopback test configuration	29

Configuring a MAC swap loopback test	29

Command reference	29

loopback local swap-mac	29

loopback remote swap-mac	31

loopback swap-mac	32

display loopback swap-mac information	32

New feature: Configuring the TMPDO for the MPS	34

Configuring the TMPDO for the MPS	34

Command reference	34

poe mps	34

New feature: Configuring port collaboration	35

Configuring port collaboration	35

Command reference	36

cfd port-trigger	36

New feature: Disabling PoE power supply on shutdown interfaces	37

Disabling PoE power supply on shutdown interfaces	37

Command reference	38

poe track-shutdown	38

New feature: Configuring PoE delay	38

Configuring PoE delay	38

Command reference	39

poe power-delay	39

New feature: Setting the PoE guard band	40

Setting the PoE guard band	40

Command reference	40

poe guard-band	40

New feature: Configuring a PD disconnection detection mode	41

Configuring a PD disconnection detection mode	41

Command reference	41

poe disconnect	41

New feature: Ignoring the PD power class	42

Ignoring the PD power class	42

Command reference	43

poe class-detect ignore	43

Modified feature: Applying a portal Web server to an interface	43

Feature change description	43

Command changes	44

Modified command: portal apply web-server	44

Modified feature: Displaying portal configuration and running information on an interface	44

Feature change description	44

Command changes	44

Modified command: display portal	44

Modified feature: Display power supply information	46

Feature change description	46

Command changes	46

Modified command: display power	46

Modified feature: Configuring the padding mode and padding format for the Circuit ID sub-option in VLAN view	46

Feature change description	46

Command changes	47

Modified command: dhcp snooping information circuit-id	47

Modified feature: Enabling DHCP snooping to support Option 82 in VLAN view	47

Feature change description	47

Command changes	47

Modified command: dhcp snooping information enable	47

Modified feature: Configuring the padding mode and padding format for the Remote ID sub-option in VLAN view	48

Feature change description	48

Command changes	48

Modified command: dhcp snooping information remote-id	48

Modified feature: Configuring the Option 82 handling strategy for DHCP request messages in VLAN view	48

Feature change description	48

Command changes	49

Modified command: dhcp snooping information strategy	49

Modified feature: Configuring the padding mode for the Vendor-Specific sub-option in VLAN view	49

Feature change description	49

Command changes	49

Modified command: dhcp snooping information vendor-specific	49

Modified feature: Changing the default settings for outputting port state transition information	50

Feature change description	50

Command changes	50

Modified command: stp port-log	50

Modified feature: Support of 10G fiber ports for 2.5G transceiver modules	50

Feature change description	50

Command changes	50

Modified command: speed	50

Modified feature: PD detection mode	51

Feature change description	51

Command changes	51

Modified command: poe detection-mode	51

Modified feature: Enabling recording user IP address conflicts	51

Feature change description	51

Command changes	51

Modified command: arp user-ip-conflict record enable	51

Modified feature: Enabling IP conflict notification	52

Feature change description	52

Command changes	52

Modified command: arp ip-conflict log prompt	52

Modified feature: Testing the cable connection of an Ethernet interface	52

Feature change description	52

Modified command: virtual-cable-test	52

New command: display virtual-cable-test	53

reset interface virtual-cable-test	55

Modified feature: Allowing inrush currents of PDs	55

Feature change description	55

Command changes	55

Modified command: poe high-inrush enable	55

Release 6343P09	1

Modified feature: Factory defaults change for console login and password control settings	1

Feature change description	1

Command changes	2

Release 6343	1

New feature: Configuring interface alarm functions	1

Command changes	2

ifmonitor input-error	2

ifmonitor output-error	3

port ifmonitor input-error	4

port ifmonitor output-error	5

Modified command: snmp-agent trap enable ifmonitor	6

New feature: Configuring the aging timer for temporary MAC address entries for Web authentication	7

Command reference	8

New command: web-auth timer temp-entry-aging	8

Modified command: display web-auth	9

Modified feature: Displaying the running configuration	9

Feature change description	9

Command changes	9

Modified command: display current-configuration	9

Modified feature: Displaying the running configuration in current view	10

Feature change description	10

Command changes	10

Modified command: display this	10

Modified feature: 802.1X periodic reauthentication timer	10

Feature change description	10

Command changes	10

Modified command: dot1x timer reauth-period (system view)	10

Modified command: dot1x timer reauth-period (interface view)	11

Modified feature: Periodic MAC reauthentication timer	11

Feature change description	11

Command changes	11

Modified command: mac-authentication timer (system view)	11

Modified command: mac-authentication timer (interface view)	11

Modified feature: Configuring the padding mode and padding format for the Remote ID sub-option of Option 82	12

Feature change description	12

Command changes	12

Modified command: dhcp relay information remote-id	12

Modified feature: Configuring gRPC collectors	12

Feature change description	12

Command changes	13

New command: domain-name	13

New command: ipv6 domain-name	14

Modified command: display grpc	15

Modified feature: Displaying detailed information about 802.1X online users	16

Feature change description	16

Command changes	16

Modified command: display dot1x connection	16

Release 6337P01	1

New feature: Configuring SmartMC	1

About SmartMC	1

SmartMC network framework	1

SmartMC network establishment	2

SmartMC features	3

Restrictions: Hardware compatibility with SmartMC	6

Restrictions and guidelines: SmartMC configuration	6

SmartMC tasks at a glance	6

Prerequisites for SmartMC	7

Enabling SmartMC	7

Setting the file server information	8

Configuring an outgoing interface for the SmartMC network	9

Enabling automatic Ethernet link aggregation	9

Modifying the password of the default user for members	9

Creating a SmartMC group	10

Creating a VLAN for members	10

Deploying a batch file to members	11

Configuring a batch file for ports connecting APs or IP phones	11

Backing up configuration files	11

Configuring resource monitoring	12

Upgrading the startup software and configuration file on members	13

About upgrading the startup software and configuration file on members	13

Restrictions and guidelines for startup software and configuration file upgrade	13

Prerequisites	13

Upgrading the startup software and configuration file on members	13

Upgrading the startup software and configuration file on all members in SmartMC groups	14

Managing the network topology	15

Refreshing the network topology	15

Saving the network topology	16

Replacing faulty members	16

Display and maintenance commands for SmartMC	17

SmartMC configuration examples	17

Example: Configuring SmartMC	17

Command reference	20

boot-loader file	20

create batch-file	21

display smartmc backup configuration status	22

display smartmc batch-file status	23

display smartmc configuration	24

display smartmc device-link	26

display smartmc group	26

display smartmc replace status	28

display smartmc resource-monitor	28

display smartmc resource-monitor configuration	29

display smartmc tc	30

display smartmc tc log buffer	32

display smartmc tc log restart	33

display smartmc upgrade status	34

display smartmc vlan	35

match	36

smartmc auto-link-aggregation enable	36

smartmc auto-replace enable	37

smartmc backup configuration	38

smartmc backup configuration max-number	38

smartmc backup configuration interval	39

smartmc batch-file apply	40

smartmc batch-file deploy	41

smartmc batch-file-apply enable	41

smartmc enable	42

smartmc { ftp-server| sftp-server }	43

smartmc group	44

smartmc outbound	45

smartmc resource-monitor	45

smartmc resource-monitor interval	47

smartmc resource-monitor max-age	47

smartmc replace	48

smartmc tc boot-loader	49

smartmc tc device-type	49

smartmc tc password	50

smartmc tc startup-configuration	51

smartmc topology-refresh	51

smartmc topology-refresh interval	52

smartmc topology-save	52

smartmc upgrade boot-loader	53

smartmc upgrade startup-configuration	54

smartmc vlan	55

startup-configuration	56

New feature: Configuring interface alarm functions	58

Configuring interface alarm functions	58

Command reference	59

ifmonitor crc-error	59

port ifmonitor crc-error	60

snmp-agent trap enable ifmonitor	61

New feature: Configuring Option 60 for DHCP requests	61

Configuring Option 60 for DHCP requests	61

Command reference	62

dhcp client class-id	62

New feature: Configuring the type of port ID TLVs advertised by LLDP	63

Configuring the type of port ID TLVs advertised by LLDP	63

Command reference	64

lldp global tlv-config basic-tlv port-id	64

lldp tlv-config basic-tlv port-id	64

New feature: Enabling displaying LLDP local information about all interfaces	65

Enabling displaying LLDP local information about all interfaces	65

Command reference	66

lldp local-information all-interface	66

New feature: PoE forced power supply	67

Enabling PoE forced power supply	67

Command reference	67

poe force-power	67

Command changes	68

Modified command: display poe pse	68

New feature: Interval at which the SNMP module examines the system configuration for changes	69

Setting the interval at which the SNMP module examines the system configuration for changes	69

Command reference	69

snmp-agent configuration-examine interval	69

New feature: Enabling generation of dynamic IPSG binding entries for 802.1X authenticated users	70

Enabling generation of dynamic IPSG binding entries for 802.1X authenticated users	70

dot1x { ip-verify-source | ipv6-verify-source } enable	71

New feature: Automated IPv6 underlay network deployment for VCF fabric	71

About automated IPv6 underlay network deployment	71

Command reference	72

Modified feature: Setting the port status detection timer	72

Feature change description	72

Command changes	72

Modified command: shutdown-interval	72

Modified feature: 802.1X EAD assistant	72

Feature change description	72

Command changes	72

New command: dot1x ead-assistant permit authentication-escape	72

Modified feature: Displaying information about online 802.1X users	73

Feature change description	73

Command changes	73

Modified command: display dot1x connection	73

Modified feature: Displaying information about online MAC authentication users	74

Feature change description	74

Command changes	74

Modified command: display mac-authentication connection	74

Modified feature: L2PT for CFD	75

Feature change description	75

Command changes	75

Modified command: l2protocol type tunnel-dmac	75

Modified command: l2protocol tunnel dot1q	76

Modified command: display l2protocol statistics	77

Release 6330	79

New feature: Enabling fast PoE for a PSE	79

Enabling fast PoE for a PSE	79

Command reference	79

poe fast-on enable	79

Modified feature: L2PT for CFD and DTP	80

Feature change description	80

Command changes	80

New command: l2protocol type tunnel-dmac	80

Modified command: l2protocol tunnel dot1q	81

Modified command: display l2protocol statistics	82

Modified feature: Displaying information about online 802.1X users	83

Feature change description	83

Command changes	83

Modified command: display dot1x connection	83

Modified feature: Displaying information about online MAC authentication users	84

Feature change description	84

Command changes	84

Modified command: display mac-authentication connection	84

Release 6351P02

This release has no feature changes.

Release 6351

This release has the following changes:

- New feature: Advertising proprietary TLVs on an interface

New feature: Configuring a dynamic aggregation group to use port speed as the prioritized criterion for reference port selection

New feature: Loopback test on an interface

New feature: Configuring disk usage monitoring

New feature: Enabling the portal fail-permit feature for portal Web servers

New feature: Configuring packet detection for 802.1X authentication

New feature: Configuring packet detection for MAC authentication

New feature: Specifying an IP address and mask for calculating the source IP of ARP detection packets

New feature: Associating PoE with Track

New feature: many-to-one VLAN mapping

New feature: Disabling receiving a specific type of ICMP messages

New feature: Disabling sending a specific type of ICMP messages

New feature: MAC swap loopback test configuration

New feature: Configuring the TMPDO for the MPS

- New feature: Configuring port collaboration

New feature: Disabling PoE power supply on shutdown interfaces

New feature: Configuring PoE delay

New feature: Setting the PoE guard band

New feature: Configuring a PD disconnection detection mode

New feature: Ignoring the PD power class

Modified feature: Applying a portal Web server to an interface

Modified feature: Displaying portal configuration and running information on an interface

Modified feature: Display power supply information

Modified feature: Configuring the padding mode and padding format for the Circuit ID sub-option in VLAN view

Modified feature: Enabling DHCP snooping to support Option 82 in VLAN view

Modified feature: Configuring the padding mode and padding format for the Remote ID sub-option in VLAN view

Modified feature: Configuring the Option 82 handling strategy for DHCP request messages in VLAN view

Modified feature: Configuring the padding mode for the Vendor-Specific sub-option in VLAN view

Modified feature: Changing the default settings for outputting port state transition information

Modified feature: Support of 10G fiber ports for 2.5G transceiver modules

Modified feature: PD detection mode

Modified feature: Enabling recording user IP address conflicts

Modified feature: Enabling IP conflict notification

Modified feature: Testing the cable connection of an Ethernet interface

Modified feature: Allowing inrush currents of PDs

New feature: Advertising proprietary TLVs on an interface

Advertising proprietary TLVs on an interface

As from this version, proprietary TLVs can be advertised on an interface. Only actual power TLVs are supported. This type of TLVs provides PoE power information about an interface for the peer to obtain the actual power that can be provided by the local.

Command reference

lldp tlv-enable private-tlv

Use lldp tlv-enable private-tlv to specify types of proprietary TLVs advertisable on an interface.

Use undo lldp tlv-enable private-tlv to disabling advertising proprietary TLVs on an interface.

Syntax

lldp [ agent { nearest-customer | nearest-nontpmr } ] tlv-enable private-tlv actual-power

undo lldp [ agent { nearest-customer | nearest-nontpmr } ] tlv-enable private-tlv actual-power

Default

No proprietary TLVs can be advertised on an interface.

Views

Layer 2 Ethernet interface view

Layer 3 Ethernet interface view

Predefined user roles

network-admin

Parameters

agent: Specifies an agent type. If you do not specify an agent type, the command specifies types of  proprietary TLVs that can be advertised by nearest bridge agents.

nearest-customer: Specifies nearest customer bridge agents.

nearest-nontpmr: Specifies nearest non-TPMR bridge agents.

actual-power: Specifies actual power TLVs.

Usage guidelines

proprietary TLVs are used to meet specific transmission requirements on network management. Other vendors cannot identify proprietary TLVs carried in LLDPDUs.

Only actual power TLVs are supported in the current software version. This type of TLVs provides PoE power information about an interface.

Examples

# Configure nearest customer bridge agents to advertise actual power TLVs on interface GigabitEthernet 1/0/1.

<Sysname> system-view

[Sysname] interface gigabitethernet 1/0/1

[Sysname-GigabitEthernet1/0/1] lldp agent neareset-customer tlv-enable private-tlv actual-power

New feature: Configuring a dynamic aggregation group to use port speed as the prioritized criterion for reference port selection

Configuring a dynamic aggregation group to use port speed as the prioritized criterion for reference port selection

      - About this task

Perform this task to ensure that a dynamic aggregation group selects a high-speed member port as the reference port. After you perform this task, the reference port will be selected based on the criteria in order of device ID, port speed, and port ID.

      - Restrictions and guidelines

Changing reference port selection criteria might cause transient traffic interruption. Make sure you understand the impact of this task on your network.

You must perform this task at both ends of the aggregate link so the peer aggregation systems use the same criteria for reference port selection.

As a best practice, shut down the peer aggregate interfaces before you execute this command and bring up the interfaces after this command is executed on both of them.

      - Procedure

            - Enter system view.

system-view

            - Enter Layer 2 aggregate interface view.

interface bridge-aggregation interface-number

            - Specify port speed as the prioritized criterion for reference port selection.

lacp select speed

By default, port ID is the prioritized criterion for reference port selection of a dynamic aggregation group.

Command reference

    - lacp select speed

Use lacp select speed to configure a dynamic aggregation group to use port speed as the prioritized criterion for reference port selection.

Use undo lacp select speed to restore the default.

Syntax

lacp select speed

undo lacp select speed

Default

Port ID is the prioritized criterion for reference port selection in a dynamic aggregation group.

Views

Layer 2 aggregate interface view

Predefined user roles

network-admin

Usage guidelines

|  | CAUTION: Changing reference port selection criteria might cause transient traffic interruption. When you use this command, make sure you understand its impact on your network. |
| --- | --- |

This command enables a dynamic aggregation group to select a high-speed member port as the reference port.

You must execute this command at both ends of the aggregate link so the peer aggregation systems use the same criteria for reference port selection.

As a best practice, shut down the peer aggregate interfaces before you execute this command and bring up the interfaces after this command is executed on both of them.

This command takes effect only on dynamic aggregate interfaces. On a static aggregate interface, you can execute this command, but the setting cannot take effect.

Examples

# Specify port speed as the prioritized criterion for reference port selection on Layer 2 dynamic aggregate interface Bridge-Aggregation 1.

<Sysname> system-view

[Sysname] interface bridge-aggregation 1

[Sysname-Bridge-Aggregation1] link-aggregation mode dynamic

[Sysname-Bridge-Aggregation1] lacp select speed

- New feature: Loopback test on an interface

  - Performing a loopback test on an interface

    - About this task

Perform this task to determine whether an Ethernet link works correctly. Loopback tests includes the following types:

- Internal loopback test—Tests the device where the Ethernet interface resides. The Ethernet interface sends outgoing packets back to the local device. If the device fails to receive the packets, the device fails.

- External loopback test—Tests the inter-device link. The Ethernet interface sends incoming packets back to the remote device. If the remote device fails to receive the packets, the inter-device link fails.

    - Restrictions and guidelines

The shutdown, port up-mode, loopback, and loopback-test commands are mutually exclusive on an interface.

    - Procedure

            - Enter system view.

system-view

            - Enter Ethernet interface view.

interface interface-type interface-number

            - Perform a loopback test.

loopback-test{ external | internal }

  - Command reference

    - New command: loopback-test

Use loopback-test to perform a loopback test.

Syntax

loopback-test { external | internal }

Views

Ethernet interface view

Predefined user roles

network-admin

Parameters

external: Performs an external loopback test.

internal: Performs an internal loopback test.

Usage guidelines

The shutdown, port up-mode, loopback, and loopback-test commands are mutually exclusive on an interface.

Examples

# Perform an internal loopback test on GigabitEthernet 1/0/1.

<Sysname> system-view

[Sysname] interface gigabitethernet 1/0/1

[Sysname-GigabitEthernet1/0/1] loopback-test internal

New feature: Configuring disk usage monitoring

      - About this task

This feature enables the device to periodically sample the usage of a disk and compare the usage with the threshold. If the disk usage exceeds the threshold, the device sends a high disk usage alarm to the NETCONF module. For more information about the NETCONF module, see Network Management and Monitoring Configuration Guide.

      - Procedure

            - Enter system view.

system-view

            - Set the disk usage sampling interval.

monitor disk-usage interval interval

By default, the disk usage sampling interval is 300 seconds.

            - Set the usage threshold for a disk.

monitor disk-usage [ slot slot-number  ] disk disk-name threshold threshold-value

By default, the disk usage threshold is 95%.

Command changes

    - monitor disk-usage disk

Use monitor disk-usage disk to set the usage threshold for a disk.

Use undo monitor disk-usage disk to restore the default.

Syntax

monitor disk-usage [ slot slot-number  ] disk disk-name threshold threshold-value

undo monitor disk-usage [ slot slot-number  ] disk disk-name threshold

Default

By default, the disk usage threshold is 95%.

Views

System view

Predefined user roles

network-admin

Parameters

slot slot-number: Specifies an IRF member device by its member ID. If you do not specify an IRF member device, the command applies to the master device.

disk disk-name: Specifies a storage medium by its name. This option is case sensitive. The system will prompt a parameter error if you enter this option incorrectly.

threshold threshold-value: Specifies the disk usage threshold in percentage, in the range of 1 to 100.

Usage guidelines

After you configure the usage threshold for a disk, the device compares the usage of the disk with the threshold at each sampling. If the usage exceeds the threshold, the device sends a high disk usage alarm to the NETCONF module. For more information about the NETCONF module see Network Management and Monitoring Configuration Guide.

Examples

# Set the disk usage threshold to 80% for a storage medium.

<Sysname> system-view

[Sysname] monitor disk-usage disk flash threshold 80

Related commands

monitor disk-usage interval

    - monitor disk-usage interval

Use monitor disk-usage interval to set the disk usage sampling interval.

Use undo monitor disk-usage interval to restore the default.

Syntax

monitor disk-usage interval interval

undo monitor disk-usage interval

Default

The disk usage sampling interval is 300 seconds.

Views

System view

Predefined user roles

network-admin

Parameters

interval interval-time: Specifies the disk usage sampling interval in seconds, a multiple of five in the range of 5 to 1800.

Usage guidelines

After you set the disk usage sampling interval, the device samples disk usages at the specified intervals.

Examples

# Set the disk usage sampling interval to 120 seconds.

<Sysname> system-view

[Sysname] monitor disk-usage interval 120

Related commands

monitor disk-usage disk

New feature: Enabling the portal fail-permit feature for portal Web servers

Enabling the portal fail-permit feature for portal Web servers

      - About this task

You can configure the portal fail-permit feature on an interface. When the access device detects that the portal authentication server or portal Web server is unreachable, it allows users to have network access without portal authentication.

If you enable fail-permit for both the portal authentication server and the portal Web servers, the device does the following:

- Disables portal authentication when the portal authentication server is unreachable or all the portal Web servers are unreachable.

- Resumes portal authentication when both the portal authentication and Web servers are reachable.

After portal authentication resumes, unauthenticated users must pass portal authentication to access the network. Users who have passed portal authentication before the fail-permit event can continue accessing the network.

      - Procedure

            - Enter system view.

system-view

            - Enter Layer 3 interface view.

interface interface-type interface-number

            - Enable portal fail-permit for portal Web servers on the interface.

portal [ ipv6 ] fail-permit web-server

By default, portal fail-permit is disabled for portal Web servers on an interface.

    - Command reference

portal fail-permit web-server

Use portal fail-permit web-server to enable the portal fail-permit feature for portal Web servers.

Use undo portal fail-permit web-server to disable the portal fail-permit feature for portal Web servers.

Syntax

portal [ ipv6 ] fail-permit web-server

undo portal [ ipv6 ] fail-permit web-server

Default

Portal fail-permit is disabled for portal Web servers.

Views

Interface view

Predefined user roles

network-admin

Parameters

ipv6: Specifies IPv6 portal Web servers. To specify IPv4 portal Web servers, do not specify this keyword.

Usage guidelines

On an interface enabled with portal fail-permit for a portal authentication server and portal Web servers, portal authentication on the interface is disabled in either of the following conditions:

- All portal Web servers are unreachable.

- The specified portal authentication server is unreachable.

Portal authentication resumes on the interface when the specified portal authentication server and a minimum of one portal Web server becomes reachable. After portal authentication resumes, unauthenticated portal users need to pass authentication to access network resources. Portal users who have passed authentication can continue accessing network resources.

Examples

# Enable portal fail-permit for the portal Web servers on VLAN-interface 100.

<Sysname> system-view

[Sysname] interface vlan-interface 100

[Sysname–Vlan-interface100] portal fail-permit web-server

Related commands

display portal

New feature: Configuring packet detection for 802.1X authentication

Configuring packet detection for 802.1X authentication

About this task

When packet detection for 802.1X authentication is enabled on a port, the device sends detection packets to 802.1X users connected to that port at offline detection intervals set by using the offline detect timer. If the device does not receive a response from a user after it has made the maximum packet transmission attempts within an offline detection interval, it logs off that user and requests the RADIUS server to stop accounting for the user.

When packet detection for 802.1X authentication and 802.1X offline detection are both enabled, the device processes an 802.1X user as follows:

If 802.1X offline detection determines that a user is online, the device does not send detection packets to that user.

If 802.1X offline detection determines that a user is offline, the device does not immediately logs off that user. Instead, the device sends a detection packet to that user. It will log off that user if it does not receive a response from that user after it has made the maximum packet transmission attempts within an offline detection interval.

802.1X uses ARP request packets to detect the online status of IPv4 users and uses NS packets to detect the online status of IPv6 users.

Packet detection adopts the principle of counting prior to judging. The device decreases the detection attempts (packet transmission attempts) by 1 only after it determines that it does not receive a response from a user. The device stops the detection process when the number of detection attempts becomes 0. The duration from the time when the user sends the last packet to the time when the user is logged off is calculated by using the following formula: duration = (retries + 1) * T + X. Figure 1 shows the packet detection process. In this example, the device sends a detection packet to an 802.1X user for a maximum of two times.

Network diagram for packet detection process

Restrictions and guidelines

To ensure that the device is aware of user IP address changes, enable ARP snooping and ND snooping in conjunction with packet detection for 802.1X authentication. If you do not enable ARP snooping or ND snooping, the device is unaware of user IP address changes. As a result, the device still sends detection packets to the users' original IP addresses and falsely log off these users.

Procedure

Enter system view.

system-view

Set the offline detect timer.

dot1x timer offline-detect offline-detect-value

By default, the offline detect timer expires in 300 seconds.

Enter interface view.

interface interface-type interface-number

Enable packet detection for 802.1X authentication.

dot1x packet-detect enable

By default, packet detection for 802.1X authentication is disabled.

Set the maximum number of attempts for sending a detection packet to an 802.1X user.

dot1x packet-detect retry retries

By default, the device sends a detection packet to an 802.1X user for a maximum of two times.

Command reference

New command: dot1x packet-detect enable

Use dot1x packet-detect enable to enable packet detection for 802.1X authentication.

Use undo dot1x packet-detect enable to restore the default.

Syntax

dot1x packet-detect enable

undo dot1x packet-detect enable

Default

Packet detection for 802.1X authentication is disabled.

Views

Layer 2 Ethernet interface view

Predefined user roles

network-admin

Usage guidelines

When packet detection for 802.1X authentication is enabled on a port, the device sends detection packets to 802.1X users connected to that port at offline detection intervals set by using the offline detect timer. If the device does not receive a response from a user after it has made the maximum packet transmission attempts within an offline detection interval, it logs off that user and requests the RADIUS server to stop accounting for the user.

When packet detection for 802.1X authentication and 802.1X offline detection are both enabled, the device processes an 802.1X user as follows:

If 802.1X offline detection determines that a user is online, the device does not send detection packets to that user.

If 802.1X offline detection determines that a user is offline, the device does not immediately logs off that user. Instead, the device sends a detection packet to that user. It will log off that user if it does not receive a response from that user after it has made the maximum packet transmission attempts within an offline detection interval.

To ensure that the device is aware of user IP address changes, enable ARP snooping and ND snooping in conjunction with packet detection for 802.1X authentication. If you do not enable ARP snooping or ND snooping, the device is unaware of user IP address changes. As a result, the device still sends detection packets to the users' original IP addresses and falsely log off these users.

Examples

# Enable packet detection for 802.1X authentication.

<Sysname> system-view

[Sysname] interface gigabitethernet 1/0/1

[Sysname-GigabitEthernet1/0/1] dot1x packet-detect enable

Related commands

dot1x timer offline-detect

port-security packet-detect arp-source-ip factor

dot1x packet-detect retry

New command: dot1x packet-detect retry

Use dot1x packet-detect retry to set the maximum number of attempts for sending a detection packet to an 802.1X user.

Use undo dot1x packet-detect retry to restore the default.

Syntax

dot1x packet-detect retry retries

undo dot1x packet-detect retry

Default

The device sends a detection packet to an 802.1X user for a maximum of two times.

Views

Layer 2 Ethernet interface view

Predefined user roles

network-admin

Parameters

retries: Sets the maximum number of attempts for sending a detection packet to an 802.1X user. The value range is 1 to 10.

Usage guidelines

When packet detection for 802.1X authentication is enabled on a port, the device sends detection packets to 802.1X users connected to that port at offline detection intervals set by using the offline detect timer. If the device does not receive a response from a user after it has made the maximum packet transmission attempts within an offline detection interval, it logs off that user and requests the RADIUS server to stop accounting for the user.

If the device fails to send a detection packet to an 802.1X user because it does not obtain the IP address of that user when that user just comes online, the device still decreases the maximum packet transmission attempts by 1. To prevent an 802.1X user from being logged off because the device does not obtain the IP address of that user when that user just comes online, the device increases the maximum number of packet transmission attempts by 10 on the basis of the original configuration.

Examples

# Set the maximum number of attempts to 8 for sending a detection packet to an 802.1X user.

<Sysname> system-view

[Sysname] interface gigabitethernet 1/0/1

[Sysname-GigabitEthernet1/0/1] dot1x packet-detect retry 8

Related commands

dot1x packet-detect enable

Modified command: display dot1x connection

Syntax

display dot1x connection [ open ] [ interface interface-type interface-number | slot slot-number | user-mac mac-address | user-name name-string ]

Views

Any view

Change description

Packet detection fields were added to the command output. The following information shows a sample command output:

# Display information about all online 802.1X users.

<Sysname> display dot1x connection

Total connections: 1

Slot ID: 1

User MAC address: 0015-e9a6-7cfe

Access interface: GigabitEthernet1/0/1

Username: ias

User access state: Successful

Authentication domain: aaa

IPv4 address: 192.168.1.1

IPv6 address: 2000:0:0:0:1:2345:6789:abcd

Authentication method: CHAP

AAA authentication method: Local

Initial VLAN: 1

Authorization untagged VLAN: 6

Authorization tagged VLAN list: 1 to 5 7 9 11 13 15 17 19 21 23 25 27 29 31 33 29 31 33

35 37 40 to 100

Authorization VSI: N/A

Authorization ACL number/name: 3001

Authorization dynamic ACL name: N/A

Authorization user profile: N/A

Authorization CAR: N/A

Authorization URL: N/A

Termination action: Default

Session timeout period: 2 s

Packet detection:

Max attempts: 5

Remaining attempts: 3

Source IPv4 address: 192.168.1.3

Source IPv4 mask: 255.255.0.0

Online from: 2013/03/02  13:14:15

Online duration: 0h 2m 15s

New feature: Configuring packet detection for MAC authentication

Configuring packet detection for MAC authentication

About this task

When packet detection for MAC authentication is enabled on a port, the device sends detection packets to MAC authentication users connected to that port at offline detection intervals set by using the offline detect timer. If the device does not receive a response from a user after it has made the maximum packet transmission attempts within an offline detection interval, it logs off that user and requests the RADIUS server to stop accounting for the user.

When packet detection for MAC authentication and MAC authentication offline detection are both enabled, the device processes a MAC authentication user as follows:

If MAC authentication offline detection determines that a user is online, the device does not send detection packets to that user.

If MAC authentication offline detection determines that a user is offline, the device does not immediately logs off that user. Instead, the device sends a detection packet to that user. It will log off that user if it does not receive a response from that user after it has made the maximum packet transmission attempts within an offline detection interval.

MAC authentication uses ARP request packets to detect the online status of IPv4 users and uses NS packets to detect the online status of IPv6 users.

Packet detection adopts the principle of counting prior to judging. The device decreases the detection attempts (packet transmission attempts) by 1 only after it determines that it does not receive a response from a user. The device stops the detection process when the number of detection attempts becomes 0. The duration from the time when the user sends the last packet to the time when the user is logged off is calculated by using the following formula: duration = (retries + 1) * T + X. Figure 2 shows the packet detection process. In this example, the device sends a detection packet to a MAC authentication user for a maximum of two times.

Network diagram for packet detection process

Restrictions and guidelines

To ensure that the device is aware of user IP address changes, enable ARP snooping and ND snooping in conjunction with packet detection for MAC authentication. If you do not enable ARP snooping or ND snooping, the device is unaware of user IP address changes. As a result, the device still sends detection packets to the users' original IP addresses and falsely log off these users.

Procedure

Enter system view.

system-view

Set the offline detect timer.

mac-authentication timer offline-detect offline-detect-value

By default, the offline detect timer expires in 300 seconds.

Enter interface view.

interface interface-type interface-number

Enable packet detection for MAC authentication.

mac-authentication packet-detect enable

By default, packet detection for MAC authentication is disabled.

Set the maximum number of attempts for sending a detection packet to a MAC authentication user.

mac-authentication packet-detect retry retries

By default, the device sends a detection packet to a MAC authentication user for a maximum of two times.

Command reference

- New command: mac-authentication packet-detect enable

Use mac-authentication packet-detect enable to enable packet detection for MAC authentication.

Use undo mac-authentication packet-detect enable to restore the default.

Syntax

mac-authentication packet-detect enable

undo mac-authentication packet-detect enable

Default

Packet detection for MAC authentication is disabled.

Views

Layer 2 Ethernet interface view

Predefined user roles

network-admin

Usage guidelines

When packet detection for MAC authentication is enabled on a port, the device sends detection packets to MAC authentication users connected to that port at offline detection intervals set by using the offline detect timer. If the device does not receive a response from a user after it has made the maximum packet transmission attempts within an offline detection interval, it logs off that user and requests the RADIUS server to stop accounting for the user.

When packet detection for MAC authentication and MAC authentication offline detection are both enabled, the device processes a MAC authentication user as follows:

If MAC authentication offline detection determines that a user is online, the device does not send detection packets to that user.

If MAC authentication offline detection determines that a user is offline, the device does not immediately logs off that user. Instead, the device sends a detection packet to that user. It will log off that user if it does not receive a response from that user after it has made the maximum packet transmission attempts within an offline detection interval.

MAC authentication uses ARP request packets to detect the online status of IPv4 users and uses NS packets to detect the online status of IPv6 users.

To ensure that the device is aware of user IP address changes, enable ARP snooping and ND snooping in conjunction with packet detection for MAC authentication. If you do not enable ARP snooping or ND snooping, the device is unaware of user IP address changes. As a result, the device still sends detection packets to the users' original IP addresses and falsely log off these users.

Examples

# Enable packet detection for MAC authentication on GigabitEthernet 1/0/1.

<Sysname> system-view

[Sysname] interface gigabitethernet 1/0/1

[Sysname-GigabitEthernet1/0/1] mac-authentication packet-detect enable

Related commands

mac-authentication timer offline-detect

port-security packet-detect arp-source-ip factor

mac-authentication packet-detect retry

- New command: mac-authentication packet-detect retry

Use mac-authentication packet-detect retry to set the maximum number of attempts for sending a detection packet to a MAC authentication user.

Use undo mac-authentication packet-detect retry to restore the default.

Syntax

mac-authentication packet-detect retry retries

undo mac-authentication packet-detect retry

Default

The device sends a detection packet to a MAC authentication user for a maximum of two times.

Views

Layer 2 Ethernet interface view

Predefined user roles

network-admin

Parameters

retries: Sets the maximum number of attempts for sending a detection packet to a MAC authentication user. The value range is 1 to 10.

Usage guidelines

When packet detection for MAC authentication is enabled on a port, the device sends detection packets to MAC authentication users connected to that port at offline detection intervals set by using the offline detect timer. If the device does not receive a response from a user after it has made the maximum packet transmission attempts within an offline detection interval, it logs off that user and requests the RADIUS server to stop accounting for the user.

To prevent a MAC authentication user from being logged off because the device does not obtain the IP address of that user when that user just comes online, the device increases the maximum number of packet transmission attempts by 10 on the basis of the original configuration.

Examples

# On GigabitEthernet 1/0/1, set the maximum number of attempts to 8 for sending a detection packet to a MAC authentication user.

<Sysname> system-view

[Sysname] interface gigabitethernet 1/0/1

[Sysname-GigabitEthernet1/0/1] mac-authentication packet-detect retry 8

Related commands

mac-authentication packet-detect enable

- Modified command: display mac-authentication connection

Syntax

display mac-authentication connection [ open ] [ interface interface-type interface-number | slot slot-number | user-mac mac-address | user-name user-name ]

Views

Any view

Change description

Packet detection fields were added to the command output. The following information shows a sample command output:

# Display information about all online MAC authentication users.

<Sysname> display mac-authentication connection

Total connections: 1

Slot ID: 0

User MAC address: 0015-e9a6-7cfe

Access interface: GigabitEthernet1/0/1

Username: ias

User access state: Successful

Authentication domain: macusers

IPv4 address: 192.168.1.1

IPv6 address: 2000:0:0:0:1:2345:6789:abcd

Initial VLAN: 1

Authorization untagged VLAN: 100

Authorization tagged VLAN: N/A

Authorization VSI: N/A

Authorization ACL number/name: 3001

Authorization dynamic ACL name: N/A

Authorization user profile: N/A

Authorization CAR: N/A

Authorization URL: N/A

Termination action: Radius-request

Session timeout period: 2 sec

Offline detection: 100 sec (server-assigned)

Packet detection:

Max attempts: 5

Remaining attempts: 3

Source IPv4 address: 192.168.1.3

Source IPv4 mask: 255.255.0.0

Online from: 2013/03/02  13:14:15

Online duration: 0h 2m 15s

New feature: Specifying an IP address and mask for calculating the source IP of ARP detection packets

- Specifying an IP address and mask for calculating the source IP of ARP detection packets

About this task

By default, the device uses 0.0.0.0 as the source IP address of ARP detection packets. The network might have users that cannot respond to ARP detection packets with source IP address 0.0.0.0. As a result, the device inadequately determines that these users have gone offline. To resolve the issue, use this feature to specify an IP address and mask for calculating the source IP of ARP detection packets sent to a user in conjunction with the user's IP address.

The device uses the following formula to calculate the source IP address of ARP detection packets: source IP = (user IP & specified mask) | (specified IP & ~specified mask). The ~mask parameter represents the reverse of a mask. For example, the reverse mask of 255.255.255.0 is 0.0.0.255. If the IP address of a user is 192.168.8.1/24 and the IP address and mask specified by using this feature is 1.1.1.11/255.255.255.0, the source IP address of ARP detection packets is 192.168.8.11/24.

Restrictions and guidelines

This feature takes effect only on users that come online after this feature is configured.

Procedure

Enter system view.

system-view

Specify an IP address and mask for calculating the source IP of ARP detection packets.

port-security packet-detect arp-source-ip factor ip-address { mask | mask-length }

By default, no IP address or mask is specified for calculating the source IP of ARP detection packets. The source IP of ARP detection packets is 0.0.0.0.

Command reference

port-security packet-detect arp-source-ip factor

Use port-security packet-detect arp-source-ip factor to specify an IP address and mask for calculating the source IP of ARP detection packets.

Use undo port-security packet-detect arp-source-ip factor to restore the default.

Syntax

port-security packet-detect arp-source-ip factor ip-address { mask | mask-length }

undo port-security packet-detect arp-source-ip factor

Default

No IP address or mask is specified for calculating the source IP of ARP detection packets. The source IP of ARP detection packets is 0.0.0.0.

Views

System view

Predefined user roles

network-admin

Parameters

ip-address { mask | mask-length }: Specifies an IP address and mask for calculating the source IP of ARP detection packets. The mask argument represents the IP address mask, in dotted decimal notation. The mask cannot be 255.255.255.255. The mask-length argument represents the IP address mask length, in the range of 0 to 31.

Usage guidelines

By default, the device uses 0.0.0.0 as the source IP address of ARP detection packets. The network might have users that cannot respond to ARP detection packets with source IP address 0.0.0.0. As a result, the device inadequately determines that these users have gone offline. To resolve the issue, use this command to specify an IP address and mask for calculating the source IP of ARP detection packets sent to a user in conjunction with the user's IP address.

The device uses the following formula to calculate the source IP address of ARP detection packets: source IP = (user IP & specified mask) | (specified IP & ~specified mask). The ~mask parameter represents the reverse of a mask. For example, the reverse mask of 255.255.255.0 is 0.0.0.255. If the IP address of a user is 192.168.8.1/24 and the IP address and mask specified by using this command is 1.1.1.11/255.255.255.0, the source IP address of ARP detection packets is 192.168.8.11/24.

To avoid the source IP address of ARP detection packets being the same as the destination IP address, follow these restrictions and guidelines:

The mask length specified by using this command must be equal to or longer than the mask length of users' IP addresses.

The mask cannot be 255.255.255.255.

This command takes effect only on users that come online after this command is executed.

Examples

# Specify 0.0.0.11/24 for calculating the source IP of ARP detection packets.

<Sysname> system-view

[Sysname] port-security packet-detect arp-source-ip factor 0.0.0.11 24

Related commands

mac-authentication packet-detect retry

dot1x packet-detect retry

New feature: Associating PoE with Track

  - Associating PoE with Track

      - About this task

The PoE module can collaborate with the Track module to monitor the link status between the device and a PD. For example, if the PD supports the NQA ICMP echo test, you can specify a track entry associated with NQA to test the reachability of the PD. The NQA ICMP echo test must be configured on a Layer 3 interface. The PI is a Layer 2 interface. You are required to create a VLAN interface for the ICMP echo test and assign the PI to the VLAN.

The Track module notifies the PoE module of the following monitoring results:

- Positive—The monitored object is reachable.

- Negative—The monitored object is unreachable.

- NotReady—The monitoring result is not ready because of reasons such as nonexistence of the NQA group associated with the track entry.

When the Track module detects failure of the link, it changes the track entry state from positive to negative, which triggers the PoE module to take the following actions:

- alarm-only: Outputs an SNMP notification and log.

- alarm-reboot-pd: Outputs an SNMP notification and log and reboots the PD connected to the PI.

For information about SNMP notifications, see SNMP configuration in Network Management and Monitoring Configuration Guide.

For information about logs, see information center configuration in Network Management and Monitoring Configuration Guide.

For information about the Track module, see track configuration in High Availability Configuration Guide.

      - Procedure

            - Enter system view.

system-view

            - Enter PI view.

interface interface-type interface-number

            - Associate the PI with a track entry.

poe track track-entry-number action { alarm | alarm-reboot-pd }

By default, a PI is not associated with a track entry.

Command reference

    - poe track

Use poe track to associate a PI to a track entry.

Use undo poe track to restore the default.

Syntax

poe track track-entry-number action { alarm | alarm-reboot-pd }

undo poe track

Default

A PI is not associated with any track entry.

Views

PI view

Predefined user roles

network-admin

Parameters

track-entry-number: Specify a track entry ID in the range of 1 to 1024.

action: Specifies the action to be taken when the track entry state changes from positive to negative.

alarm: Outputs an SNMP notification and log.

alarm-reboot-pd: Outputs an SNMP notification and log and reboots the PD connected to the PI.

Usage guidelines

This command uses a track entry to monitor the link status between the device and a PD and triggers the specified action when the track entry state changes from positive to negative. For more information about Track, see Track configuration in High Availability Configuration Guide.

If you configure this command multiple times in PI view, the most recent configuration takes effect.

Examples

# Associate GigabitEthernet 1/0/1 with track entry 1 and enable the system to output an SNMP notification and log when the track entry state changes from positive to negative.

<Sysname> system-view

[Sysname] interface gigabitethernet 1/0/1

[Sysname-GigabitEthernet1/0/1] poe track 1 action alarm

- New feature: many-to-one VLAN mapping

  - Configuring many-to-one VLAN mapping

    - About many-to-one VLAN mapping

Configure many-to-one VLAN mapping to transmit the same type of traffic from different users in one VLAN.

    - About many-to-one VLAN mapping

- To ensure correct traffic forwarding from the service provider network to the customer network, do not configure many-to-one VLAN mapping together with the following reatures:

  - Disabling MAC address learning.

  - Setting the MAC learning limit.

For more information about MAC address learning, see "Configuring the MAC address table."

- To avoid network connection failure in a many-to-one VLAN mapping environment, make sure that an ARP request has been sent from the customer-side port for the connection to the network-side port.

      - Many-to-one VLAN mapping tasks at a glance

            - Configuring the customer-side port

            - Configuring the network-side port

      - Configuring the customer-side port

            - Enter system view.

system-view

            - Enter interface view.

  - Enter Layer 2 Ethernet interface view.

interface interface-type interface-number

  - Enter Layer 2 aggregate interface view.

interface bridge-aggregation interface-number

            - Set the link type of the port.

port link-type { hybrid | trunk }

By default, the link type of a port is access.

            - Assign the port to the original VLANs.

  - Assign the trunk port to the original VLANs

port trunk permit vlan vlan-id-list

By default, a trunk port is assigned to VLAN 1.

  - Assign the hybrid port to the original VLANs as a tagged member.

port hybrid vlan vlan-id-list tagged

By default, a hybrid port is an untagged member of the VLAN to which the port belongs when its link type is access.

            - Configure a many-to-one VLAN mapping.

vlan mapping uni { range vlan-range-list | single vlan-id-list } translated-vlan vlan-id

By default, no VLAN mapping is configured on an interface.

      - Configuring the network-side port

            - Enter system view.

system-view

            - Enter interface view.

  - Enter Layer 2 Ethernet interface view.

interface interface-type interface-number

  - Enter Layer 2 aggregate interface view.

interface bridge-aggregation interface-number

            - Set the link type of the port.

port link-type { hybrid | trunk }

By default, the link type of a port is access.

            - Assign the port to the translated VLAN.

  - Assign the trunk port to the translated VLAN.

port trunk permit vlan vlan-id-list

By default, a trunk port is assigned to VLAN 1.

  - Assign the hybrid port to the translated VLAN as a tagged member.

port hybrid vlan vlan-id-list tagged

By default, a hybrid port is an untagged member of the VLAN to which the port belongs when its link type is access.

  - Command reference

    - vlan mapping uni

Use vlan mapping uni to configure many-to-one VLAN mapping on an interface.

Use undo vlan mapping uni to cancel the many-to-one VLAN mapping configuration.

Syntax

vlan mapping uni { range vlan-range-list | single vlan-id-list } translated-vlan vlan-id

undo vlan mapping uni { range vlan-range-list | single vlan-id-list } translated-vlan vlan-id

Default

No many-to-one VLAN mapping is configured on an interface.

Views

Layer 2 Ethernet interface view

Layer 2 aggregate interface view

Predefined user roles

network-admin

Parameters

uni range vlan-range-list translated-vlan vlan-id: Specifies the original VLAN ranges and the translated VLAN for a many-to-one VLAN mapping on the customer-side port. The vlan-range-list argument specifies a space-separated list of up to 10 VLAN items. Each item specifies a VLAN ID or a range of VLAN IDs in the form of vlan-id1 to vlan-id2. The value range for VLAN IDs is 1 to 4094. The value for the vlan-id2 argument must be greater than the value for the vlan-id1 argument. The value range for the vlan-id argument is 1 to 4094. Different VLAN ranges cannot overlap. Any of the original VLANs cannot be the same as the translated VLAN.

uni single vlan-id-list translated-vlan vlan-id: Specifies the original VLANs and the translated VLAN for a many-to-one VLAN mapping on the customer-side port. The vlan-id-list argument specifies a space-separated list of up to 10 VLAN IDs, each of which is in the range of 1 to 4094. The value range for the vlan-id argument is 1 to 4094. Any of the original VLANs cannot be the same as the translated VLAN.

Examples

# Configure many-to-one VLAN mappings on the customer-side port to map VLANs 1 through 50 and VLAN 80 to VLAN 101.

<Sysname> system-view

[Sysname] interface gigabitethernet 1/0/2

[Sysname-GigabitEthernet1/0/2] vlan mapping uni range 1 to 50 translated-vlan 101

[Sysname-GigabitEthernet1/0/2] vlan mapping uni single 80 translated-vlan 101

New feature: Disabling receiving a specific type of ICMP messages

  - Disabling receiving a specific type of ICMP messages

About this task

By default, the device receives all types of ICMP messages. Such a setting might affect device performance if a large number of ICMP responses are received within a short time. To solve this issue, you can perform this task to disable the device from receiving a specific type of ICMP messages.

Restrictions and guidelines

Disabling receiving ICMP messages of a specific type might affect network operation. Please use this feature with caution.

Procedure

Enter system view.

system-view

Disable the device from receiving a specific type of ICMP messages.

undo ip icmp { name icmp-name | type icmp-type code icmp-code } receive enable

By default, the device receives all types of ICMP messages.

  - Command reference

    - ip icmp receive enable

Use ip icmp receive enable to enable the device to receive a specific type of ICMP messages.

Use undo ip icmp receive enable to disable the device from receiving a specific type of ICMP messages.

Syntax

ip icmp { name icmp-name | type icmp-type code icmp-code } receive enable

undo ip icmp { name icmp-name | type icmp-type code icmp-code } receive enable

Default

The device can receive all types of ICMP messages.

Views

System view

Predefined user roles

network-admin

Parameters

name icmp-name: Specifies an ICMP message name, a case-insensitive string of 1 to 20 characters.

type icmp-type: Specifies an ICMP message type. The value range for the icmp-type argument is 0 to 255.

code icmp-code: Specifies an ICMP message code. The value range for the icmp-code argument is 0 to 255.

Usage guidelines

|  | CAUTION: Disabling receiving ICMP messages of a specific type might affect network operation. Please use this feature with caution. |
| --- | --- |

By default, the device receives all types of ICMP messages. Such a setting might affect device performance if a large number of ICMP responses are received within a short time. To solve this issue, you can perform this task to disable the device from receiving a specific type of ICMP messages.

Table 1 shows common ICMP messages and the information they carry.

Common ICMP messages

| Name | Type | Code | Description |
| --- | --- | --- | --- |
| echo | 8 | 0 | Echo request used to ping a target node. |
| echo-reply | 0 | 0 | Echo reply sent by a target node after receiving an echo request. |
| fragmentneed-dfset | 3 | 4 | Packets that need fragmentation but have the DF bit set. |
| host-redirect | 5 | 1 | Host redirection. |
| host-tos-redirect | 5 | 3 | Host ToS redirection. |
| host-unreachable | 3 | 1 | Unreachable host. |
| information-reply | 16 | 0 | Information reply. |
| information-request | 15 | 0 | Information request. |
| net-redirect | 5 | 0 | Network redirection. |
| net-tos-redirect | 5 | 2 | Network ToS redirection. |
| net-unreachable | 3 | 0 | Unreachable network. |
| parameter-problem | 12 | 0 | Invalid parameter. |
| port-unreachable | 3 | 3 | Unreachable port. |
| protocol-unreachable | 3 | 2 | Unreachable protocol. |
| reassembly-timeout | 11 | 1 | Fragment reassembly timeout. |
| source-quench | 4 | 0 | Source quench message. |
| source-route-failed | 3 | 5 | Source route failure. |
| timestamp-reply | 14 | 0 | Timestamp reply. |
| timestamp-request | 13 | 0 | Timestamp request. |
| ttl-exceeded | 11 | 0 | TTL exceeded in transit. |

‌

Examples

# Enable the device to receive ICMP echo reply messages.

<Sysname> system-view

[Sysname] ip icmp name echo-reply receive enable

New feature: Disabling sending a specific type of ICMP messages

  - Disabling sending a specific type of ICMP messages

About this task

By default, the device sends all types of ICMP messages except the Destination Unreachable, Time Exceeded, and Redirect types. Attackers might obtain information from specific types of ICMP messages, causing security issues.

For security purposes, you can perform this task disable sending ICMP messages of specific types.

Restrictions and guidelines

Disabling sending ICMP messages of a specific type might affect network operation. Please use this feature with caution.

To enable sending Destination Unreachable, Time Exceeded, or Redirect messages, you can perform one of the following operations:

Execute the ip icmp send enable command.

Execute one command as needed:

ip unreachables enable

ip ttl-expires enable

ip redirects enable

Procedure

Enter system view.

system-view

Disable the device from sending a specific type of ICMP messages.

undo ip icmp { name icmp-name | type icmp-type code icmp-code } send enable

By default, the device sends all types of ICMP messages except the Destination Unreachable, Time Exceeded, and Redirect types.

  - Command reference

    - ip icmp send enable

Use ip icmp send enable to enable the device to send a specific type of ICMP messages.

Use undo ip icmp send enable to disable the device from sending a specific type of ICMP messages.

Syntax

ip icmp { name icmp-name | type icmp-type code icmp-code  } send enable

undo ip icmp { name icmp-name | type icmp-type code icmp-code } send enable

Default

The device can send all types of ICMP messages except the following ICMP messages:

Destination Unreachable.

Time Exceeded.

Redirect.

Views

System view

Predefined user roles

network-admin

Parameters

name icmp-name: Specifies an ICMP message name, a case-insensitive string of 1 to 20 characters.

type icmp-type: Specifies an ICMP message type. The value range for the icmp-type argument is 0 to 255.

code icmp-code: Specifies an ICMP message code. The value range for the icmp-code argument is 0 to 255.

Usage guidelines

|  | CAUTION: Disabling sending ICMP messages of a specific type might affect network operation. Please use this feature with caution. |
| --- | --- |

By default, the device sends all types of ICMP messages except the Destination Unreachable, Time Exceeded, and Redirect types. Attackers might obtain information from specific types of ICMP messages, causing security issues.

For security purposes, you can use this command to disable the device from sending ICMP messages of specific types.

To enable sending Destination Unreachable, Time Exceeded, or Redirect messages, you can perform one of the following operations:

Execute the ip icmp send enable command.

Execute one command as needed:

ip unreachables enable

ip ttl-expires enable

ip redirects enable

Table 2 shows common ICMP messages and the information they carry.

Common ICMP messages

| Name | Type | Code | Description |
| --- | --- | --- | --- |
| echo | 8 | 0 | Echo request used to ping a target node. |
| echo-reply | 0 | 0 | Echo reply sent by a target node after receiving an echo request. |
| fragmentneed-dfset | 3 | 4 | Packets that need fragmentation but have the DF bit set. |
| host-redirect | 5 | 1 | Host redirection. |
| host-tos-redirect | 5 | 3 | Host ToS redirection. |
| host-unreachable | 3 | 1 | Unreachable host. |
| information-reply | 16 | 0 | Information reply. |
| information-request | 15 | 0 | Information request. |
| net-redirect | 5 | 0 | Network redirection. |
| net-tos-redirect | 5 | 2 | Network ToS redirection. |
| net-unreachable | 3 | 0 | Unreachable network. |
| parameter-problem | 12 | 0 | Invalid parameter. |
| port-unreachable | 3 | 3 | Unreachable port. |
| protocol-unreachable | 3 | 2 | Unreachable protocol. |
| reassembly-timeout | 11 | 1 | Fragment reassembly timeout. |
| source-quench | 4 | 0 | Source quench message. |
| source-route-failed | 3 | 5 | Source route failure. |
| timestamp-reply | 14 | 0 | Timestamp reply. |
| timestamp-request | 13 | 0 | Timestamp request. |
| ttl-exceeded | 11 | 0 | TTL exceeded in transit. |

‌

Examples

# Enable the device to send ICMP echo reply messages.

<Sysname> system-view

[Sysname] ip icmp name echo-reply send enable

New feature: MAC swap loopback test configuration

Configuring a MAC swap loopback test

About this task

A MAC swap loopback test is an interruptive performance test. The following MAC swap loopback tests are available based on the usage scenario:

Local MAC swap loopback test—Checks connectivity and performance of the network between a tester and a tested switch, and performance of the tested switch. After Ethernet frames sent by a tester reach a downlink interface of a switch on which the local MAC swap loopback test is configured, the downlink interface swaps source and destination MAC addresses of the Ethernet frames. Then, the switch sends the Ethernet frames back to the tester through a specified interface for the tester to obtain information about the network connectivity and performance.

Remote MAC swap loopback test—Checks connectivity and performance of the network between a tester and a tested switch without checking performance of the tested switch. After Ethernet frames sent by a tester reach an uplink interface of a switch on which the remote MAC swap loopback test is configured, the uplink interface swaps source and destination MAC addresses of the Ethernet frames. Then, the switch sends the Ethernet frames back to the tester through the uplink interface for the tester to obtain information about the uplink network connectivity and performance.

Restrictions and guidelines

The MAC swap loopback test feature interrupts all services of a tested interface. Make sure you are fully aware of the impacts of this feature when you configure it on a live network.

Procedure

Enter Ethernet interface view.

interface interface-type interface-number

Configure MAC swap loopback tests.

Configure the local MAC swap loopback test.

loopback local swap-mac source-mac source-mac-address dest-mac dest-mac-address vlan vlan-id [ inner-vlan inner-vlan-id ] interface interface-type interface-number [ timeout { time-value | none } ]

By default, the local MAC swap loopback test is disabled on an interface.

Configure the remote MAC swap loopback test.

loopback remote swap-mac source-mac source-mac-address dest-mac dest-mac-address vlan vlan-id [ inner-vlan inner-vlan-id ] [ timeout { time-value | none } ]

By default, the remote MAC swap loopback test is disabled on an interface.

Start MAC swap loopback tests.

loopback swap-mac start

By default, MAC swap loopback tests are stopped on an interface.

Command reference

    - loopback local swap-mac

Use loopback local swap-mac to enable the local MAC swap loopback test.

Use undo loopback local swap-mac to disable the local MAC swap loopback test.

Syntax

loopback local swap-mac source-mac source-mac-address dest-mac dest-mac-address vlan vlan-id [ inner-vlan inner-vlan-id ] interface interface-type interface-number [ timeout { time-value | none } ]

undo loopback local swap-mac

Default

The local MAC swap loopback test is disabled on an interface.

Views

Ethernet interface view

Predefined user roles

network-admin

network-operator

Parameters

source-mac-address: Specifies a source MAC address. The source MAC address must be a unicast MAC address.

dest-mac-address: Specifies a destination MAC address. The destination MAC address must be a unicast MAC address.

vlan-id: Specifies a VLAN ID in the range of 1 to 4094.

inner-vlan-id: Specifies an inner VLAN ID in the range of 1 to 4094.

time-value: Sets the loopback test timeout timer in the range of 5 to 300 seconds. The default value is 60. When the timer expires, the system automatically stops MAC swap loopback tests.

none: Disables the loopback test timeout timer. MAC swap loopback tests can only be disabled manually.

Usage guidelines

The local MAC swap loopback test checks connectivity and performance of the network between a tester and a tested switch, and performance of the tested switch.

After Ethernet frames sent by a tester reach a downlink interface of a switch on which the local MAC swap loopback test is configured, the downlink interface swaps source and destination MAC addresses of the Ethernet frames. Then, the switch sends the Ethernet frames back to the tester through the downlink interface for the tester to obtain information about the network connectivity and performance.

Examples

# Configure the local MAC swap loopback test.

<Sysname> system-view

[Sysname] interface gigabitethernet 1/0/1

[Sysname-GigabitEthernet1/0/1] loopback local swap-mac source-mac 00e0-fc00-0085 dest-mac 00e0-fc00-1004 vlan 100 interface gigabitethernet 1/0/2

Related commands

loopback remote swap-mac

loopback swap-mac

display loopback swap-mac information

    - loopback remote swap-mac

Use loopback remote swap-mac to enable the remote MAC swap loopback test.

Use undo loopback remote swap-mac to disable the remote MAC swap loopback test.

Syntax

loopback remote swap-mac source-mac source-mac-address dest-mac dest-mac-address vlan vlan-id [ inner-vlan inner-vlan-id ] [ timeout { time-value | none } ]

undo loopback remote swap-mac

Default

The remote MAC swap loopback test is disabled on an interface.

Views

Ethernet interface view

Predefined user roles

network-admin

network-operator

Parameters

source-mac-address: Specifies a source MAC address. The source MAC address must be a unicast MAC address.

dest-mac-address: Specifies a destination MAC address. The destination MAC address must be a unicast MAC address.

vlan-id: Specifies a VLAN ID in the range of 1 to 4094.

inner-vlan-id: Specifies an inner VLAN ID in the range of 1 to 4094.

time-value: Sets the loopback test timeout timer in the range of 5 to 300 seconds. The default value is 60. When the timer expires, the system automatically stops MAC swap loopback tests.

none: Disables the loopback test timeout timer. MAC swap loopback tests can only be disabled manually.

Usage guidelines

The remote MAC swap loopback test checks connectivity and performance of the network between a tester and a tested switch without checking performance of the tested switch.

After Ethernet frames sent by a tester reach an uplink interface of a switch on which the remote MAC swap loopback test is configured, the uplink interface swaps source and destination MAC addresses of the Ethernet frames. Then, the switch sends the Ethernet frames back to the tester through the uplink interface for the tester to obtain information about the uplink network connectivity and performance.

Examples

# Configure the remote MAC swap loopback test.

<Sysname> system-view

[Sysname] interface gigabitethernet 1/0/1

[Sysname-GigabitEthernet1/0/1] loopback local swap-mac source-mac 00e0-fc00-0085 dest-mac 00e0-fc00-1004 vlan 100

Related commands

loopback local swap-mac

loopback swap-mac

display loopback swap-mac information

    - loopback swap-mac

Use loopback swap-mac to start or stop MAC swap loopback tests.

Syntax

loopback swap-mac [ start | stop ]

Default

MAC swap loopback tests are stopped on an interface.

Views

Ethernet interface view

Predefined user roles

network-admin

network-operator

Parameters

start: Starts MAC swap loopback tests.

stop: Stops MAC swap loopback tests.

Usage guidelines

Both the remote and local MAC swap loopback tests affect operation of the network. To minimize the impact of the loopback tests on the network, plan the testing window according to your services and perform the loopback tests within the testing window.

MAC swap loopback tests can be stopped manually or automatically upon expiration of the loopback test timeout timer. To start the loopback tests again, execute the loopback swap-mac start command.

Examples

# Start MAC swap loopback tests.

<Sysname> system-view

[Sysname] interface gigabitethernet 1/0/1

[Sysname-GigabitEthernet1/0/1] loopback swap-mac start

Related commands

loopback local swap-mac

loopback remote swap-mac

display loopback swap-mac information

    - display loopback swap-mac information

Use display loopback swap-mac information to display configuration of MAC swap loopback tests.

Syntax

display loopback swap-mac information

Views

Any view

Predefined user roles

network-admin

network-operator

Examples

# Display configuration of the local MAC swap loopback test.

<Sysname> display loopback swap-mac information

Loopback type             : local

Loopback state            : running

Loopback test times(s)    : 60

Loopback interface        : GigabitEthernet1/0/1

Loopback output interface : GigabitEthernet1/0/2

Loopback source MAC       : 0001-0001-0001

Loopback destination MAC  : 0002-0002-0002

Loopback vlan             : 10

Loopback inner vlan       : 0

Loopback packets          : 0

Drop packets              : 3

# Display configuration of the remote MAC swap loopback test.

<Sysname> display loopback swap-mac information

Loopback type             : remote

Loopback state            : running

Loopback test time(s)     : 60

Loopback interface        : GigabitEthernet1/0/1

Loopback source MAC       : 0001-0001-0001

Loopback destination MAC  : 0002-0002-0002

Loopback vlan             : 10

Loopback inner vlan       : 0

Loopback packets          : 0

Drop packets              : 3

Command output

| Field | Description |
| --- | --- |
| Loopback type | Type of the MAC swap loopback test: Local. Remote. |
| Loopback state | State of the loopback test: Running—The loopback test is running. Stop—The loopback test is stopped. |
| Loopback test time(s) | Value of the loopback test timeout timer. This field displays None if the timeout timer is disabled. |
| Loopback interface | Interface under loopback test. |
| Loopback output interface | Interface sending backup loopback testing frames. |
| Loopback source MAC | Source MAC address in the loopback testing frames. |
| Loopback destination MAC | Destination MAC address in the loopback testing frames. |
| Loopback vlan | VLAN ID of the loopback testing frames. |
| Loopback inner vlan | Inner VLAN ID of the loopback testing frames. |
| Loopback packets | Number of received loopback testing frames. |
| Drop packets | Number of discarded loopback testing packets that do not meet the requirements. This field is displayed only for the local MAC swap loopback test. |

Related commands

loopback local swap-mac

loopback remote swap-mac

loopback swap-mac

New feature: Configuring the TMPDO for the MPS

  - Configuring the TMPDO for the MPS

About this task

The Maintain Power Signature (MPS) is an electrical signature provided by a PD. The PD uses this signature to maintain connection to the PSE in sleep mode. The PD sends a PoE-compliant pulse current to the PSE periodically. If the PSE detects the PoE-compliant pulse current from the PD within the TMPDO, it supplies power to the PD. If the PSE does not detect the PoE-compliant pulse current from the PD within the TMPDO, it will not supply power to the PD.

To send pulse currents at larger intervals for lower standby power, you can use this command to change the TMPDO to be longer.

Restrictions and guidelines

Only PSE modules that have a model name of LSPPSE**A support this feature. To view the PSE models, execute the display poe pse command.

If you execute the command multiple times, the most recent configuration takes effect.

Procedure

Enter System view.

system-view

Set the TMPDO for the MPS.

poe mps pse pse-id tmpdo { timer | long |normal }

By default, the normal TMPDO mode is used for the MPS. The TMPDO for the MPS is 324 milliseconds.

  - Command reference

    - poe mps

Use poe mps to set the TMPDO for MPS.

Use undo poe mps to restore the default.

Syntax

poe mps pse pse-id tmpdo { timer | long |normal }

undo poe mps pse pse-id tmpdo

Default

The normal TMPDO mode is used for the MPS. The TMPDO is 324 milliseconds.

Views

System view

Predefined user roles

network-admin

Parameters

pse-id: Specifies a PSE by its ID.

timer: Sets the TMPDO for the MPS. The value is a multiple of 4 in the range of 300 to 400, in milliseconds.

long: Specify the long TMPDO mode. The TMPDO is 360 milliseconds.

normal: Specifies the normal TMPDO mode. The TMPDO is 324 milliseconds.

Usage guidelines

The MPS is an electrical signature provided by a PD. The PD uses this signature to maintain connection to the PSE in sleep mode. The PD sends a PoE-compliant pulse current to the PSE periodically. If the PSE detects the PoE-compliant pulse current from the PD within the TMPDO, it supplies power to the PD. If the PSE does not detect the PoE-compliant pulse current from the PD within the TMPDO, it will not supply power to the PD.

To send pulse currents at larger intervals for lower standby power, you can use this command to change the TMPDO to be longer.

Only PSE modules that have a model name of LSPPSE**A support this feature. To view the PSE models, execute the display poe pse command.

If you execute the command multiple times, the most recent configuration takes effect.

Examples

# Set the TMPDO for the MPS to 350 milliseconds.

<Sysname> system-view

[Sysname] poe mps pse 1 tmpdo 350

New feature: Configuring port collaboration

Configuring port collaboration

Restrictions and guidelines

Port collaboration takes effect only on the ports with outward-facing MEPs configured.

      - Procedure

Enter system view.

system-view

Enter Layer 2 Ethernet interface view or Layer 2 aggregate interface view。

interface interface-type interface-number

Configure port collaboration.

cfd port-trigger { cc-expire | rdi } action { block | shutdown }

By default, port collaboration is not configured.

Command reference

    - cfd port-trigger

Use cfd port-trigger to specify the triggering event and triggered action for port collaboration.

Use undo cfd port-trigger to cancel the triggering event and triggered action for port collaboration.

Syntax

cfd port-trigger { cc-expire | rdi } action { block | shutdown }

undo cfd port-trigger { cc-expire | rdi } action

cfd port-trigger remote-status action shutdown

undo cfd port-trigger remote-status action

Default

The triggering event and triggered action are not specified for port collaboration.

Views

Layer 2 Ethernet interface view

Layer 2 aggregate interface view

Predefined user roles

network-admin

Parameters

cc-expire: Triggers port collaboration when continuity check expires.

rdi: Triggers port collaboration when the CCMs with the RDI flag bit set are received.

remote-status: Triggers port collaboration when the inward-facing MEP detects that the remote MEP interface is down.

block: Blocks the port by changing its link layer protocol state to DOWN (CFD). The port cannot send or receive any data packets.

shutdown: Shuts down the port by changing its physical state to CFD DOWN. The port cannot send or receive any data packets or protocol packets.

Usage guidelines

After you execute this command, CFD blocks or shuts down the associated port upon detecting a link failure.

If a port is blocked by CFD, it can automatically come up when the link recovers.

If a port is shut down by CFD, it cannot automatically come up when the link recovers.

- If port collaboration is triggered by continuity check expiration, you must execute the undo shutdown or undo cfd port-trigger { cc-expire | rdi } action command to bring up the port.

- If port collaboration is triggered by CCMs with the RDI flag bits set, you must execute the undo cfd port-trigger { cc-expire | rdi } action command to bring up the port.

- If port collaboration is triggered by remote interface failure, you must execute the undo cfd port-trigger remote-status action command to bring up the port. In this triggering event, port collaboration shuts down only the inward-facing MEP interface in up state on the local end.

You can specify multiple triggering events for an interface. All the triggering events can take effect. When you specify multiple triggered actions for a triggering event on an interface, the most recent configuration takes effect.

The command takes effect only on the ports with outward-facing MEPs configured.

Configurations in Ethernet interface view take effect only on the current interface.

Configurations in aggregate interface view take effect only on the current aggregate interface.

If the MEP belongs to an MA that does not carry the VLAN attribute, configurations on a member port of an aggregation group take effect only on the current member port.

If the MEP belongs to an MA that carries the VLAN attribute, configurations on a member port of an aggregation group take effect only when the member port leaves the aggregation group.

Examples

# Specify the triggering event as cc-expire and the triggered action as block for port collaboration on GigabitEthernet 1/0/1.

<Sysname> system-view

[Sysname] interface gigabitethernet 1/0/1

[Sysname-GigabitEthernet1/0/1] cfd port-trigger cc-expire action block

# Specify the triggering event as cc-expire and the triggered action as shutdown for port collaboration on GigabitEthernet 1/0/1.

<Sysname> system-view

[Sysname] interface gigabitethernet 1/0/1

[Sysname-GigabitEthernet1/0/1] cfd port-trigger cc-expire action shutdown

Related commands

cfd cc enable

cfd mep

- New feature: Disabling PoE power supply on shutdown interfaces

  - Disabling PoE power supply on shutdown interfaces

About this task

By default, the device continues supplying power to an interface after the interface is shut down by the shutdown command or by an upper layer module such as monitor link. As a result, the PD connected to the shutdown interface operates continuously but fails to access the network.

This task disables the PoE module from supplying power to an interface after the interface is shut down. After the interface comes up, the PoE module resumes power supply to the interface.

Restrictions and guidelines

The command does not power off an interface that has been shut down but is supplying power to a PD.

Procedure

Enter system view.

system-view

Disable PoE power supply on shutdown interfaces.

poe track-shutdown

By default, the device continues supplying power to an interface after the interface is shut down.

  - Command reference

    - poe track-shutdown

Use poe track-shutdown to disable PoE power supply on shutdown interfaces.

Use undo poe track-shutdown to restore the default.

Syntax

poe track-shutdown

undo poe track-shutdown

Default

The device continues supplying power to an interface after the interface is shut down.

Views

System view

Predefined user roles

network-admin

Usage guidelines

By default, the device continues supplying power to an interface after the interface is shut down by the shutdown command or by an upper layer module such as monitor link. As a result, the PD connected to the shutdown interface operates continuously but fails to access the network.

This command disables the PoE module from supplying power to an interface after the interface is shut down. After the interface comes up, the PoE module resumes power supply to the interface.

The command does not power off an interface that has been shut down but is supplying power to a PD.

Examples

# Disable PoE power supply on interfaces after they shut down.

<Sysname> system-view

[Sysname] poe track-shutdown

Related commands

poe enable

- New feature: Configuring PoE delay

  - Configuring PoE delay

About this task

By default, the device executes the poe enable command and supplies power to an interface immediately when any one of the following conditions is met:

The poe enable command is configured.

The device reboots with the poe enable command in the configuration file.

The interface comes up and the PoE module resumes PoE power supply to the interface.

This task creates a PoE delay timer after the poe enable command is executed and allows the PoE module to supply power to the PI only after the timer expires.

Restrictions and guidelines

The undo poe enable command is executed immediately upon configuration and is not affected by this task.

Procedure

Enter system view.

system-view

Enable PoE delay.

poe power-delay time

By default, PoE delay is disabled.

  - Command reference

    - poe power-delay

Use poe power-delay to enable PoE delay.

Use undo poe power-delay to disable PoE delay.

Syntax

poe power-delay time

undo poe power-delay

Default

PoE delay is disabled.

Views

System view

Predefined user roles

network-admin

Parameters

time: Specifies the delay time in the range of 1 to 3600, in seconds.

Usage guidelines

By default, the device executes the poe enable command and supplies power to an interface immediately when any one of the following conditions is met:

The poe enable command is configured.

The device reboots with the poe enable command in the configuration file.

The interface comes up and the PoE module resumes PoE power supply to the interface.

The poe power-delay creates a PoE delay timer after the poe enable command is executed and allows the PoE module to supply power to the PI only after the timer expires.

The undo poe enable command is executed immediately upon configuration and is not affected by the poe power-delay command.

Examples

# Enable PoE delay and set the delay time to 30 seconds.

<Sysname> system-view

[Sysname] poe power-delay 30

Related commands

poe enable

- New feature: Setting the PoE guard band

  - Setting the PoE guard band

About this task

PoE guard band is a specified amount of power reserved to prevent PSE power overload in case of PD power jitter. For example, if the maximum power of the PSE is 200 W and the guard band is 30 W, the PSE can provide a maximum power of 170 W to all PDs. If the total power of PDs reaches 170 W, no power will be supplied to new PDs.

When the PD operates stably with low power jitter (less than the default guard band), you can use the poe pse guard-band command to decrease the guard band and use the released PSE power to power the new PDs.

Procedure

Enter system view.

system-view

Set the PoE guard band.

poe pse pse-id guard-band power

undo poe pse pse-id guard-band power

By default, the PoE guard band is 30 W.

  - Command reference

    - poe guard-band

Use poe guard-band to set the PoE guard band.

Use undo poe guard-band to restore the default.

Syntax

poe pse pse-id guard-band power

undo poe pse pse-id guard-band power

Default

By default, the PoE guard band is 30 W.

Views

System view

Predefined user roles

network-admin

Parameters

pse-id: Specifies a PSE by its ID.

power: Specifies the PoE guard band in the range of 0 to 30, in watts.

Usage guidelines

Application scenarios

PoE guard band is a specified amount of power reserved to prevent PSE power overload in case of PD power jitter. For example, if the maximum power of the PSE is 200 W and the guard band is 30 W, the PSE can provide a maximum power of 170 W to all PDs. If the total power of PDs reaches 170 W, no power will be supplied to new PDs.

When the PD operates stably with low power jitter (less than the default guard band), you can use the poe pse guard-band command to reduce the guard band and use the released PSE power to power the new PDs.

Restrictions and guidelines

Before executing this command, execute the display poe interface command to check the value of the Remaining field. If the guard band is greater than the value of Remaining field, some PDs being powered might have a power down. For example, the current guard band is 10 W, and the value of the Remaining field is 10 W. If the network administrator changes the guard band to 30 W, some PDs that are currently being powered might have a power down to make up for the 20 W power gap.

Examples

# Set the PoE guard band to 30 W.

<Sysname> system-view

[Sysname] poe pse-id 1 guard-band 30

New feature: Configuring a PD disconnection detection mode

  - Configuring a PD disconnection detection mode

|  | CAUTION: If you change the PD disconnection detection mode when the device is running, the connected PDs are powered off. Be aware of the impact of this operation before you perform it. |
| --- | --- |

About this task

A PSE detects PD disconnection in AC mode or DC mode. The AC mode uses less power than the DC mode.

Procedure

Enter system view.

system-view

Configure a PD disconnection detection mode.

poe disconnect { ac | dc }

The default PD disconnection detection mode is AC.

  - Command reference

    - poe disconnect

Use poe disconnect to configure a PD disconnection detection mode.

Use undo poe disconnect to restore the default.

Syntax

poe disconnect { ac | dc }

undo poe disconnect

Default

The default PD disconnection detection mode is AC.

Views

System view

Predefined user roles

network-admin

Parameters

ac: Specifies the PD disconnection detection mode as ac.

dc: Specifies the PD disconnection detection mode as dc.

Usage guidelines

If you change the PD disconnection detection mode while the device is running, the connected PDs are powered off.

Examples

# Set the PD disconnection detection mode to dc.

<Sysname> system-view

[Sysname] poe disconnect dc

Related commands

display poe pse

New feature: Ignoring the PD power class

Ignoring the PD power class

About this task

Before supplying power to a PD, the device detects the power class of the PD by default. For example, if the device detects the power class of a PD as 1, the device can provide a maximum power of 12.95 W to the PD. If the power required by a PD is greater than 12.95 W, the device will stop supplying power to the PD.

Restrictions and guidelines

After configuring this task, the device will continue to detect the PD power class before supplying power to a PD. As long as the power required by the PD does not exceed the maximum power on the interface (configured by using the poe max power command in interface view), the device can supply power to the PD.

If you execute the poe class-detect ignore command multiple times, and the most recent configuration will take effect.

Procedure

Enter system view.

system-view

Enter PI view.

interface interface-type interface-number

Ignore the PD power class.

poe class-detect ignore

By default, the device supplies power to PDs based on the detected PD power class.

  - Command reference

poe class-detect ignore

Use poe class-detect ignore to ignore the PD power class.

Use undo poe class-detect to restore the default.

Syntax

poe class-detect ignore

undo poe class-detect

Default

The device supplies power to a PD based on the detected power class of the PD.

Views

PI view

Predefined user roles

network-admin

Usage guidelines

Before supplying power to a PD, the device detects the power class of the PD by default. For example, if the device detects the power class of 1 for a PD, the device can provide a maximum power of 12.95 W to the PD. If the power required by a PD is greater than 12.95 W, the device will stop supplying power to the PD.

After configuring this command, the device will continue to detect the PD power class before supplying power to a PD. As long as the power required by the PD does not exceed the maximum power on the interface (configured by using the poe max power command in interface view), the device can supply power to the PD.

If you execute the poe class-detect ignore command multiple times, and the most recent configuration will take effect.

Examples

# Configure the GigabitEthernet1/0/1 to ignore the PD power class.

<Sysname> system-view

[Sysname] interface gigabitethernet 1/0/1

[Sysname-GigabitEthernet1/0/1] poe class-detect ignore

Modified feature: Applying a portal Web server to an interface

Feature change description

As from this release, the device supports applying a secondary portal Web server to an interface.

Command changes

Modified command: portal apply web-server

Old syntax

portal [ ipv6 ] apply web-server server-name [ fail-permit ]

undo portal [ ipv6 ] apply web-server

New syntax

portal [ ipv6 ] apply web-server server-name [ fail-permit | secondary ]

undo portal [ ipv6 ] apply web-server [ server-name ]

Views

Interface view

Change description

The secondary keyword was added to the command to support applying a secondary portal Web server to the interface.

Modified feature: Displaying portal configuration and running information on an interface

Feature change description

As from this release, the device supports displaying secondary portal Web server information on an interface.

Command changes

Modified command: display portal

Syntax

display portal interface interface-type interface-number

Views

Any view

Change description

Before modification: This command does not display information about secondary portal Web servers configured on interfaces.

After modification: This command can display information about secondary portal Web servers configured on interfaces, for example:

# Display portal configuration and running information on VLAN-interface 2.

<Sysname> display portal interface vlan-interface 2

Portal information of Vlan-interface2

NAS-ID profile: aaa

VSRP instance : instance1

VSRP status   : Master

Authorization : Strict checking

ACL           : Enabled

User profile  : Disabled

Dual stack    : Disabled

Max users     : Not configured

IPv4:

Portal status: Enabled

Portal authentication method: Direct

Portal VSRP status: M_Delay

Portal web server: wbs(active)

Secondary portal Web server: wbsec

Portal mac-trigger-server: mts

Authentication domain: my-domain

Pre-auth domain: abc

User-dhcp-only: Enabled

Pre-auth IP pool: ab

Max users: Not configured

Bas-ip: Not configured

User detection: Type: ICMP  Interval: 300s  Attempts: 5   Idle time: 180s

Action for server detection:

Server type    Server name                        Action

Web server     wbs                                fail-permit

Portal server  pts                                fail-permit

Layer3 source network:

IP address               Mask

1.1.1.1                  255.255.0.0

Destination authentication subnet:

IP address               Mask

2.2.2.2                  255.255.255.0

IPv6:

portal status: Disabled

Portal authentication method: Disabled

Portal VSRP status: M_Alone

Portal web server: Not configured

Secondary portal Web server: Not configured

Portal mac-trigger-server: Not configured

Authentication domain: Not configured

Pre-auth domain: Not configured

User-dhcp-only: Disabled

Pre-auth IP pool: Not configured

Max users: Not configured

Bas-ipv6:Not configured

User detection: Not configured

Action for server detection:

Server type    Server name                        Action

--             --                                 --

Layer3 source network:

IP address                                        Prefix length

Destination authentication subnet:

IP address                                        Prefix length

Modified feature: Display power supply information

Feature change description

As from this release, if pluggable power supplies except PSR180-56A are installed on PoE models, the display power command displays the output current, voltage, and power information of the power supplies. The command output from the display power command is for reference only. The actual data is subject to professional testing.

Command changes

- Modified command: display power

Syntax

display power [ slot slot-number [ power-id ] ]

Views

Any view

Change description

Before modification: The Current(A), Voltage(V), and Power(W) fields in the command output are displayed two hyphens (--). The device does not support collection information about these fields.

After modification: If pluggable power supplies except PSR180-56A are installed on PoE models, the display power command displays the output current, voltage, and power information of the power supplies. The Current(A), Voltage(V), Power(W) fields in the command output are for reference only. The actual data is subject to professional testing.

Examples

# Display power supply information.

<Sysname> display power

PowerID State    Mode   Current(A)  Voltage(V)  Power(W)

1       Normal   AC        5.95       11.95        72

2       Absent   --        --          --          --

Modified feature: Configuring the padding mode and padding format for the Circuit ID sub-option in VLAN view

Feature change description

As from this release, you can configure the padding mode and padding format for the Circuit ID sub-option in VLAN view.

Command changes

Modified command: dhcp snooping information circuit-id

Syntax

dhcp snooping information circuit-id { normal-extended | [ vlan vlan-id ] string circuit-id | { normal | verbose [ node-identifier { mac | sysname | user-defined node-identifier } ] } [ format { ascii | hex } ] }

undo dhcp snooping information circuit-id [ vlan vlan-id ]

Views

Layer 2 Ethernet interface view

Layer 2 aggregate interface view

VLAN view

Change description

Before modification: You cannot configure the padding mode and padding format for the Circuit ID sub-option in VLAN view.

After modification: You can configure the padding mode and padding format for the Circuit ID sub-option in VLAN view, but the vlan vlan-id option is not supported in VLAN view.

Modified feature: Enabling DHCP snooping to support Option 82 in VLAN view

  - Feature change description

As from this release, you can enable DHCP snooping to support Option 82 in VLAN view.

  - Command changes

    - Modified command: dhcp snooping information enable

Syntax

dhcp snooping information enable

undo dhcp snooping information enable

Views

VLAN view

Change description

Before modification: You cannot enable DHCP snooping to support Option 82 in VLAN view.

After modification: You can enable DHCP snooping to support Option 82 in VLAN view.

Modified feature: Configuring the padding mode and padding format for the Remote ID sub-option in VLAN view

  - Feature change description

As from this release, you can configure the padding mode and padding format for the Remote ID sub-option in VLAN view. To specify the hexadecimal padding format, you can specify the hex keyword.

  - Command changes

    - Modified command: dhcp snooping information remote-id

Old syntax

dhcp snooping information remote-id { normal [ format ascii ] | { hex remote-id | string remote-id | sysname } }

undo dhcp snooping information remote-id

New syntax

dhcp snooping information remote-id { normal [ format { ascii | hex } ] | { hex remote-id | string remote-id | sysname } }

undo dhcp snooping information remote-id

Views

VLAN view

Change description

Before modification: In VLAN view, you cannot configure the padding mode and padding format for the Remote ID sub-option and the hex keyword is not supported.

After modification: In VLAN view, you can configure the padding mode and padding format for the Remote ID sub-option and the hex keyword is supported. To specify the hexadecimal padding format, you can specify the hex keyword.

Modified feature: Configuring the Option 82 handling strategy for DHCP request messages in VLAN view

  - Feature change description

As from this release, you can configure the Option 82 handling strategy for DHCP request messages in VLAN view.

  - Command changes

    - Modified command: dhcp snooping information strategy

Syntax

dhcp snooping information strategy { append | drop | keep | replace }

undo dhcp snooping information strategy

Views

VLAN view

Change description

Before modification: You cannot configure the Option 82 handling strategy for DHCP request messages in VLAN view.

After modification: You can configure the Option 82 handling strategy for DHCP request messages in VLAN view.

Modified feature: Configuring the padding mode for the Vendor-Specific sub-option in VLAN view

  - Feature change description

As from this release, you can configure the padding mode for the Vendor-Specific sub-option in VLAN view.

  - Command changes

    - Modified command: dhcp snooping information vendor-specific

syntax

dhcp snooping information vendor-specific bas [ node-identifier { mac | sysname | user-defined string } ]

undo dhcp snooping information vendor-specific

Views

VLAN view

Change description

Before modification: You cannot configure the padding mode for the Vendor-Specific sub-option in VLAN view.

After modification: You can configure the padding mode for the Vendor-Specific sub-option in VLAN view.

Modified feature: Changing the default settings for outputting port state transition information

Feature change description

As from this release, If the device starts up the factory defaults, outputting port state transition information is enabled.

Command changes

Modified command: stp port-log

Syntax

stp port-log { all | instance instance-list | vlan vlan-id-list }

undo stp port-log { all | instance instance-list | vlan vlan-id-list }

Views

System view

Change description

Before modification: By default, if the device starts up the factory defaults, outputting port state transition information is disabled.

After modification: By default, if the device starts up the factory defaults, outputting port state transition information is enabled.

Modified feature: Support of 10G fiber ports for 2.5G transceiver modules

  - Feature change description

As from this version, 10G fiber ports support 2.5G transceiver modules. You can use the speed 2500 command to configure a port to operate at 2500 Mbps.

  - Command changes

    - Modified command: speed

Old syntax

speed { 1000 | 10000 | auto }

New syntax

speed { 1000 | 2500 | 10000 | auto }

Views

10G interface view

Change description

Before modification: The 2500 keyword is not supported.

After modification: The 2500 keyword is supported.

2500: Sets the speed to 2500 Mbps.

Modified feature: PD detection mode

  - Feature change description

As from this version, if the device starts up with factory default settings, it supplies power to PDs that comply with basic requirements of 802.3af or 802.3at.

  - Command changes

    - Modified command: poe detection-mode

Syntax

poe detection-mode { none | simple | strict }

Views

PI view

Change description

Before modification: By default, the device supplies power to PDs that comply with all requirements of 802.3af or 802.3at if it starts up with factory default settings.

After modification: By default, the device supplies power to PDs that comply with basic requirements of 802.3af or 802.3at if it starts up with factory default settings.

none: Enables the device to supply power to PDs that are correctly connected to the device without causing short circuit.

simple: Enables the device to supply power to PDs that comply with basic requirements of 802.3af or 802.3at.

strict: Enables the device to supply power to PDs that comply with all requirements of 802.3af or 802.3at.

Modified feature: Enabling recording user IP address conflicts

Feature change description

As from this release, If the device starts up with the factory defaults, recording user IP address conflicts is enabled.

Command changes

Modified command: arp user-ip-conflict record enable

Syntax

arp user-ip-conflict record enable

undo arp user-ip-conflict record enable

Views

System view

Change description

Before modification: By default, the device starts up with the factory defaults, recording user IP address conflicts is disabled.

After modification: By default, If the device starts up with the factory defaults, recording user IP address conflicts is enabled.

Modified feature: Enabling IP conflict notification

Feature change description

As from this release, If the device starts up with the factory defaults, IP conflict notification is disabled.

Command changes

Modified command: arp ip-conflict log prompt

Syntax

arp ip-conflict log prompt

undo arp ip-conflict log prompt

Views

System view

Change description

Before modification: By default, If the device starts up with the factory defaults, IP conflict notification is disabled.

After modification: By default, if the device starts up with the factory defaults, IP conflict notification is enabled.

- Modified feature: Testing the cable connection of an Ethernet interface

  - Feature change description

As from this version, the interface [ interface-type interface-number | interface-name  ] parameter is added to the virtual-cable-test command, and the view where the command is available is changed. The display virtual-cable-test  command and the reset interface virtual-cable-test commands are added.

    - Modified command: virtual-cable-test

Old syntax

Layer 2 Ethernet interface view:

virtual-cable-test

New syntax

Any view:

virtual-cable-test interface [ interface-type interface-number | interface-name ]

Change description

Before modification: This command is supported only in Layer 2 Ethernet interface view. The interface interface-type interface-number parameter is not supported.

After modification: This command is supported in any view. The interface interface-type interface-number parameter is supported.

interface interface-type interface-number: Specifies an interface by its type and number.

    - New command: display virtual-cable-test

Use display virtual-cable-test to display the cable connection test results of Ethernet interfaces.

Syntax

display virtual-cable-test interface [ interface-type interface-number | interface-name ]

Views

Any view

Predefined user roles

network-admin

network-operator

Parameters

interface interface-type interface-number: Specifies an interface by its type and number. If you do not specify this keyword, this command displays the cable connection test results of all Ethernet interfaces.

Usage guidelines

The test results are for reference only. The maximum length error is 10 meters.

When the obtained cable length is 0 to 3 meters, the length is displayed as Invalid.

Interfaces operating at 10 Mbps or 100 Mbps and in up state do not support this command. Fiber ports do not support this command.

When the local interface is shut down, this command is not supported. When the peer interface is shut down, the cable length cannot be obtained.

Examples

# Display the summary cable connection test results of all Ethernet interfaces.

<Sysname>display virtual-cable-test

Interface                     Result          Length(meters)      Date

GigabitEthernet1/0/1          Not test

GigabitEthernet1/0/2          Abnormal(open)  <50                 2013-03-06 23:01:34

# Display the detailed cable connection test results of Ethernet interface GigabitEthernet 1/0/2.

<Sysname> display virtual-cable-test interface GigabitEthernet 1/0/2

Cable status:

Pair A length: Invalid meters

Pair B length: Invalid meters

Pair C length: Invalid meters

Pair D length: Invalid meters

Pair A state: Abnormal(open)

Pair B state: Abnormal(open)

Pair C state: Abnormal(open)

Pair D state: Abnormal(open)

Pair Impedance mismatch: no

Pair skew: - ns

Pair swap: -

Pair polarity: -

Insertion loss: - db

Return loss: - db

Near-end crosstalk: - db

        - Command output

| Field | Description |
| --- | --- |
| Result | Test result: - Abnormal (open)—An open circuit is detected. - Abnormal (short)—A short circuit is detected. - Not test—The test results of the interface are not obtained. |
| Length(meters) | Total length of the cable pair, in meters. |
| Date | Time when the test was performed. |
| Cable status | Cable state. |
| Pair x length | When the cable pair state is OK, this field displays the total length of the cable pair. When the cable pair is in any other state, this field displays the length from the local interface to the faulty point. |
| Pair x state | Cable pair state: - OK—The cable pair is in good condition. - Abnormal—The cable pair is abnormal. - Abnormal (open)—An open circuit is detected. - Abnormal (short)—A short circuit is detected. - Invalid—Detection failed. |
| Pair Impedance mismatch | Pair impedance mismatch: - yes—Impedance match. - no—Impedance mismatch. |
| Pair skew | Pair skew. A hyphen (-) is displayed when this test item is not supported. |
| Pair swap | Pair swap. A hyphen (-) is displayed when this test item is not supported. |
| Pair polarity | Pair polarity swap. A hyphen (-) is displayed when this test item is not supported. |
| Insertion loss | Insertion signal loss. A hyphen (-) is displayed when this test item is not supported. |
| Return loss | Return signal loss. A hyphen (-) is displayed when this test item is not supported. |
| Near-end crosstalk | Near-end crosstalk. A hyphen (-) is displayed when this test item is not supported. |

Related commands

virtual-cable-test

    - reset interface virtual-cable-test

Use reset interface virtual-cable-test to clear the cable connection test results of Ethernet interfaces.

Syntax

reset interface [ interface-type interface-number | interface-name ] virtual-cable-test

Views

User view

System view

Predefined user roles

network-admin

Parameters

interface interface-type interface-number: Specifies an interface by its type and number.

Examples

# Clear the cable connection test results of Ethernet interface GigabitEthernet 1/0/1.

<Sysname> reset interface GigabitEthernet 1/0/1 virtual-cable-test

Related commands

virtual-cable-test

- Modified feature: Allowing inrush currents of PDs

  - Feature change description

As from this release, the command for allowing inrush current of PDs can be configured only in PI view and does not support the pse pse-id option.

  - Command changes

    - Modified command: poe high-inrush enable

Use poe high-inrush enable to allow high inrush currents of PDs.

Use undo poe high-inrush enable to restore the default.

Old syntax

poe high-inrush enable pse pse-id

undo poe high-inrush enable pse pse-id

System view

New syntax

poe high-inrush enable pse pse-id

undo poe high-inrush enable pse pse-id

PoE interface view

Change description

Before modification: This command can be configured only in system view and cannot be configured in PI view. The pse pse-id option is supported.

After modification: This command can be configured only in PI view and cannot be configured in system view. The pse pse-id option is not supported.

pse pse-id: Specifies a PSE by its ID.

Release 6343P09

This release has the following changes:

- Modified feature: Factory defaults change for console login and password control settings

  - Modified feature: Factory defaults change for console login and password control settings

    - Feature change description

Factory defaults are custom basic settings that came with the device. You can use the display default-configuration command to display factory defaults.

The device starts up with the factory defaults if no next-startup configuration files are available.

In this version, the following factory default settings are added:

#

password-control enable

#

local-user admin

service-type terminal

authorization-attribute user-role network-admin

#

line class aux

authentication-mode scheme

#

undo password-control aging enable

#

undo password-control composition enable

#

undo password-control history enable

#

undo password-control length enable

#

password-control login idle-time 0

#

password-control login-attempt 3 exceed unlock

#

password-control update-interval 0

#

The output shows that the factory defaults for console login and password control settings change:

- The device performs local AAA authentication for console users. A console user must use the username admin without any password to log in to the device for the first time. The user role network-admin is assigned to the login console user.

- By default, the global password control and password change at first login are both enabled. Users must change the password at first login before they can access the system. The new password must contain a minimum of four different characters.

- The default maximum account idle time is 0 days. The system has no restriction for the account idle time.

- The default minimum password update interval is 0 hours. The system has no requirement for the password update interval.

- The default maximum number of consecutive login failures is 3. When console user fails the maximum number of login attempts, the console user can continue using this user account to make login attempts.

- After a console user modifies the password after first login, if you want to delete the default user account admin, make sure either of the following conditions are met before deleting the user account admin:

  - Another user account with the highest permissions exists.

  - The authentication-mode none command has been configured for AUX user lines.

- If you add or modify security configurations, make sure they do not conflict with the factory defaults or will not lead login failures. For more information about factory defaults, see configuration file management in Fundamentals Configuration Guide for the product. For more information about AAA authentication and password control, see Security Configuration Guide.

- After the global password control is enabled, the device generates an lauth.dat file to save the authentication and login information for local users.Do not edit or delete this file to ensure the authentication and login of the local users.

  - If you execute the restore factory-default command in user view to restore the factory defaults, the lauth.dat file will be deleted. After the device reboots, you can use the username admin without any password to log in to the device, and you are required to change the password.

  - If you restore the factory defaults through Restore to factory default configuration on the boot menu, the lauth.dat file will not be deleted. After the device reboots, you must use the latest password to log in to the device.

  - Command changes

None.

Release 6343

This release has the following changes:

New feature: Configuring interface alarm functions

New feature: Configuring the aging timer for temporary MAC address entries for Web authentication

Modified feature: Displaying the running configuration

Modified feature: Displaying the running configuration in current view

Modified feature: 802.1X periodic reauthentication timer

Modified feature: Periodic MAC reauthentication timer

Modified feature: Configuring the padding mode and padding format for the Remote ID sub-option of Option 82

Modified feature: Configuring gRPC collectors

Modified feature: Displaying detailed information about 802.1X online users

  - New feature: Configuring interface alarm functions

        - About this task

With the interface alarm functions enabled, when the number of error packets on an interface in normal state within the specified interval exceeds the upper threshold, the interface generates an upper threshold exceeding alarm and enters the alarm state. When the number of error packets on an interface in the alarm state within the specified interval drops below the lower threshold, the interface generates a recovery alarm and restores to the normal state.

        - Restrictions and guidelines

You can configure the error packet alarm parameters in system view and interface view.

- The configuration in system view takes effect on all interfaces of the specified slot. The configuration in interface view takes effect only on the current interface.

- For an interface, the configuration in interface view takes priority, and the configuration in system view is used only when no configuration is made in interface view.

An interface that is shut down because of error packet alarms cannot automatically recover. To bring up the interface, execute the undo shutdown command on the interface.

        - Configuring input error packet alarm parameters

            - Enter system view.

system-view

            - Configure global input error packet alarm parameters.

ifmonitor input-error slot slot-number high-threshold high-value low-threshold low-value interval interval [ shutdown ]

By default, the upper threshold is 1000, the lower threshold is 100, and the statistics collection and comparison interval is 10 seconds for input error packets.

            - Enter Ethernet interface view.

interface interface-type interface-number

            - Configure input error packet alarm parameters for the interface.

port ifmonitor input-error high-threshold high-value low-threshold low-value interval interval [ shutdown ]

By default, an interface uses the global input error packet alarm parameters.

        - Configuring output error packet alarm parameters

            - Enter system view.

system-view

            - Configure global output error packet alarm parameters.

ifmonitor output-error slot slot-number high-threshold high-value low-threshold low-value interval interval [ shutdown ]

By default, the upper threshold is 1000, the lower threshold is 100, and the statistics collection and comparison interval is 10 seconds for output error packets.

            - Enter Ethernet interface view.

interface interface-type interface-number

            - Configure output error packet alarm parameters.

port ifmonitor output-error high-threshold high-value low-threshold low-value interval interval [ shutdown ]

By default, an interface uses the global output error packet alarm parameters.

    - Command changes

      - ifmonitor input-error

Use ifmonitor input-error to configure global input error packet alarm parameters.

Use undo ifmonitor input-error to restore the default.

Syntax

ifmonitor input-error slot slot-number high-threshold high-value low-threshold low-value interval interval [ shutdown ]

undo ifmonitor input-error slot slot-number

Default

The upper threshold is 1000, the lower threshold is 100, and the statistics collection and comparison interval is 10 seconds for input error packet alarms.

Views

System view

Predefined user roles

network-admin

Parameters

high-threshold high-value: Specifies the upper threshold for input error packet alarms, in the range of 1 to 4294967295 packets.

low-threshold low-value: Specifies the lower threshold for input error packet alarms, in the range of 1 to 4294967295 packets.

interval interval: Specifies the statistics collection and comparison interval for input error packets, in the range of 1 to 65535 seconds.

shutdown: Shuts down an interface when the number of input error packets on the interface exceeds the upper threshold. Then, the interface stops forwarding all packets. To recover the interface, execute the undo shutdown command on the interface. If you do not specify this keyword, an upper threshold exceeding alarm is generated and the interface enters the alarm state when the number of input error packets exceeds the upper threshold on the interface.

slot slot-number: Specifies an IRF member device by its member ID.

Usage guidelines

With the input error packet alarm function enabled, when the number of input error packets on an interface in normal state within the specified interval exceeds the upper threshold, the interface generates an upper threshold exceeding alarm and enters the alarm state. When the number of input error packets on an interface in the alarm state within the specified interval drops below the lower threshold, the interface generates a recovery alarm and restores to the normal state.

You can configure the input error packet alarm parameters in system view and interface view.

- The configuration in system view takes effect on all interfaces of the specified slot. The configuration in interface view takes effect only on the current interface.

- For an interface, the configuration in interface view takes priority, and the configuration in system view is used only when no configuration is made in interface view.

When you execute this command multiple times, the most recent configuration takes effect.

Examples

# Set the upper threshold to 5000, lower threshold to 400, and statistics collection and comparison interval to 6 seconds for input error packet alarms.

<Sysname> system-view

[Sysname] ifmonitor input-error slot 1 high-threshold 5000 low-threshold 400 interval 6

Related commands

snmp-agent trap enable ifmonitor

      - ifmonitor output-error

Use ifmonitor output-error to configure global output error packet alarm parameters.

Use undo ifmonitor output-error to restore the default.

Syntax

ifmonitor output-error slot slot-number high-threshold high-value low-threshold low-value interval interval [ shutdown ]

undo ifmonitor output-error slot slot-number

Default

The upper threshold is 1000, the lower threshold is 100, and the statistics collection and comparison interval is 10 seconds for output error packet alarms.

Views

System view

Predefined user roles

network-admin

Parameters

high-threshold high-value: Specifies the upper threshold for output error packet alarms, in the range of 1 to 4294967295 packets.

low-threshold low-value: Specifies the lower threshold for output error packet alarms, in the range of 1 to 4294967295 packets.

interval interval: Specifies the statistics collection and comparison interval for output error packets, in the range of 1 to 65535 seconds.

shutdown: Shuts down an interface when the number of output error packets on the interface exceeds the upper threshold. Then, the interface stops forwarding all packets. To recover the interface, execute the undo shutdown command on the interface. If you do not specify this keyword, an upper threshold exceeding alarm is generated and the interface enters the alarm state when the number of output error packets exceeds the upper threshold on the interface.

slot slot-number: Specifies an IRF member device by its member ID.

Usage guidelines

With the output error packet alarm function enabled, when the number of output error packets on an interface in normal state within the specified interval exceeds the upper threshold, the interface generates an upper threshold exceeding alarm and enters the alarm state. When the number of output error packets on an interface in the alarm state within the specified interval drops below the lower threshold, the interface generates a recovery alarm and restores to the normal state.

You can configure the output error packet alarm parameters in system view and interface view.

- The configuration in system view takes effect on all interfaces of the specified slot. The configuration in interface view takes effect only on the current interface.

- For an interface, the configuration in interface view takes priority, and the configuration in system view is used only when no configuration is made in interface view.

When you execute this command multiple times, the most recent configuration takes effect.

Examples

# Set the upper threshold to 5000, lower threshold to 400, and statistics collection and comparison interval to 6 seconds for output error packet alarms.

<Sysname> system-view

[Sysname] ifmonitor output-error slot 1 high-threshold 5000 low-threshold 400 interval 6

Related commands

snmp-agent trap enable ifmonitor

      - port ifmonitor input-error

Use port ifmonitor input-error to configure input error packet alarm parameters for an interface.

Use undo port ifmonitor input-error to restore the default.

Syntax

port ifmonitor input-error high-threshold high-value low-threshold low-value interval interval [ shutdown ]

undo port ifmonitor input-error

Default

An interface uses the global input error packet alarm parameters.

Views

Ethernet interface view

Predefined user roles

network-admin

Parameters

high-threshold high-value: Specifies the upper threshold for input error packet alarms, in the range of 1 to 4294967295 packets.

low-threshold low-value: Specifies the lower threshold for input error packet alarms, in the range of 1 to 4294967295 packets.

interval interval: Specifies the statistics collection and comparison interval for input error packets, in the range of 1 to 65535 seconds.

shutdown: Shuts down an interface when the number of input error packets on the interface exceeds the upper threshold. Then, the interface stops forwarding all packets. To recover the interface, execute the undo shutdown command on the interface. If you do not specify this keyword, an upper threshold exceeding alarm is generated and the interface enters the alarm state when the number of input error packets exceeds the upper threshold on the interface.

Usage guidelines

With the input error packet alarm function enabled, when the number of input error packets on an interface in normal state within the specified interval exceeds the upper threshold, the interface generates an upper threshold exceeding alarm and enters the alarm state. When the number of input error packets on an interface in the alarm state within the specified interval drops below the lower threshold, the interface generates a recovery alarm and restores to the normal state.

You can configure the input error packet alarm parameters in system view and interface view.

- The configuration in system view takes effect on all interfaces of the specified slot. The configuration in interface view takes effect only on the current interface.

- For an interface, the configuration in interface view takes priority, and the configuration in system view is used only when no configuration is made in interface view.

When you execute this command multiple times, the most recent configuration takes effect.

Examples

# Set the upper threshold to 5000, lower threshold to 400, and statistics collection and comparison interval to 6 seconds for input error packet alarms on GigabitEthernet 1/0/1.

<Sysname> system-view

[Sysname] interface gigabitethernet 1/0/1

[Sysname-GigabitEthernet1/0/1] port ifmonitor input-error high-threshold 5000 low-threshold 400 interval 6

Related commands

snmp-agent trap enable ifmonitor

      - port ifmonitor output-error

Use port ifmonitor output-error to configure output error packet alarm parameters for an interface.

Use undo port ifmonitor output-error to restore the default.

Syntax

port ifmonitor output-error high-threshold high-value low-threshold low-value interval interval [ shutdown ]

undo port ifmonitor output-error

Default

An interface uses the global output error packet alarm parameters.

Views

Ethernet interface view

Predefined user roles

network-admin

Parameters

high-threshold high-value: Specifies the upper threshold for output error packet alarms, in the range of 1 to 4294967295 packets.

low-threshold low-value: Specifies the lower threshold for output error packet alarms, in the range of 1 to 4294967295 packets.

interval interval: Specifies the statistics collection and comparison interval for output error packets, in the range of 1 to 65535 seconds.

shutdown: Shuts down an interface when the number of output error packets on the interface exceeds the upper threshold. Then, the interface stops forwarding all packets. To recover the interface, execute the undo shutdown command on the interface. If you do not specify this keyword, an upper threshold exceeding alarm is generated and the interface enters the alarm state when the number of output error packets exceeds the upper threshold on the interface.

Usage guidelines

With the output error packet alarm function enabled, when the number of output error packets on an interface in normal state within the specified interval exceeds the upper threshold, the interface generates an upper threshold exceeding alarm and enters the alarm state. When the number of output error packets on an interface in the alarm state within the specified interval drops below the lower threshold, the interface generates a recovery alarm and restores to the normal state.

You can configure the output error packet alarm parameters in system view and interface view.

- The configuration in system view takes effect on all interfaces. The configuration in interface view takes effect only on the current interface. (Devices that do not support the slot keyword.)

- The configuration in system view takes effect on all interfaces of the specified slot. The configuration in interface view takes effect only on the current interface.

- For an interface, the configuration in interface view takes priority, and the configuration in system view is used only when no configuration is made in interface view.

When you execute this command multiple times, the most recent configuration takes effect.

Examples

# Set the upper threshold to 5000, lower threshold to 400, and statistics collection and comparison interval to 6 seconds for output error packet alarms on GigabitEthernet 1/0/1.

<Sysname> system-view

[Sysname] interface gigabitethernet 1/0/1

[Sysname-GigabitEthernet1/0/1] port ifmonitor output-error high-threshold 5000 low-threshold 400 interval 6

Related commands

snmp-agent trap enable ifmonitor

Modified command: snmp-agent trap enable ifmonitor

Old syntax

snmp-agent trap enable ifmonitor [ crc-error ]

undo snmp-agent trap enable ifmonitor [ crc-error ]

New syntax

snmp-agent trap enable ifmonitor [ crc-error | input-error | output-error ] *

undo snmp-agent trap enable ifmonitor [ crc-error | input-error | output-error ] *

Views

System view

Change description

Before modification: SNMP notifications for input and output error packets is not supported.

After modification: SNMP notifications for input and output error packets is supported.

New feature: Configuring the aging timer for temporary MAC address entries for Web authentication

      - About this task

If Web authentication is enabled, the device generates a temporary MAC address entry when it detects traffic from a user for the first time. The entry records the MAC address, access interface, and VLAN ID of the user, as well as the aging time of the entry.

The aging timer works as follows:

- If the user does not initiate authentication when the aging timer expires, the device deletes the temporary entry.

- If the user passes authentication before the aging timer expires, the device delete the aging timer and records online information for the Web authentication user.

- If the user fails authentication before the aging timer expires and an Auth-Fail VLAN is specified for Web authentication, the device binds the MAC address of the user to the Auth-fail VLAN and reset the aging timer. If the user still fails authentication when the aging timer expires, the device deletes the temporary entry for the user.

      - Restrictions and guidelines

As a best practice, change the aging timer to a bigger value in the following cases:

- Web authentication users without access rights frequently send traffic in a short time. As a result, the access device continuously initiates the web authentication process, increasing the load on the device.

- When a user fails authentication, the user does not have enough time to obtain resources from the Auth-Fail VLAN, for example, it failed to download the virus patches.

      - Procedure

            - Enter system view.

system-view

            - Configure the aging timer for temporary MAC address entries.

web-auth timer temp-entry-aging aging-time-value

By default, the aging timer for temporary MAC address entries is 60 seconds.

Command reference

New command: web-auth timer temp-entry-aging

Use web-auth timer temp-entry-aging to configure the aging timer for temporary MAC address entries.

Use undo web-auth timer temp-entry-aging to restore the default.

Syntax

web-auth timer temp-entry-aging aging-time-value

undo web-auth timer temp-entry-aging

Default

The aging timer for temporary MAC address entries is 60 seconds.

Views

System view

Default command level

network-admin

Parameters

aging-time-value: Specifies the aging timer in seconds for temporary MAC address entries, in the range of 60 to 2147483647.

Usage guidelines

If Web authentication is enabled, the device generates a temporary MAC address entry when it detects traffic from a user for the first time. The entry records the MAC address, access interface, and VLAN ID of the user, as well as the aging time of the entry.

The aging timer works as follows:

- If the user does not initiate authentication when the aging timer expires, the device deletes the temporary entry.

- If the user passes authentication before the aging timer expires, the device delete the aging timer and records online information for the Web authentication user.

- If the user fails authentication before the aging timer expires and an Auth-Fail VLAN is specified for Web authentication, the device binds the MAC address of the user to the Auth-fail VLAN and reset the aging timer. If the user still fails authentication when the aging timer expires, the device deletes the temporary entry for the user.

As a best practice, change the aging timer to a bigger value in the following cases:

- Web authentication users without access rights frequently send traffic in a short time. As a result, the access device continuously initiates the web authentication process, increasing the load on the device.

- When a user fails authentication, the user does not have enough time to obtain resources from the Auth-Fail VLAN, for example, it failed to download the virus patches.

Examples

# Set the aging timer for temporary MAC address entries to 500 seconds.

<Sysname> system-view

[Sysname] web-auth timer temp-entry-aging 500

Modified command: display web-auth

Syntax

display web-auth [ interface interface-type interface-number ]

Views

Any view

Change description

The command output added support for the Temp entry aging time field.

# Display Web authentication configuration on GigabitEthernet 1/0/1.

<Sysname> display web-auth interface gigabitethernet 1/0/1

Global Web-auth parameters:

Temp entry aging time      : 500 s

HTTP proxy port numbers    : Not configured

HTTPS proxy port numbers   : Not configured

Total online web-auth users  : 1

GigabitEthernet1/0/1  is link-up

Port role                  : Authenticator

Web-auth domain            : my-domain

Auth-Fail VLAN             : Not configured

Offline-detect             : Not configured

Max online users           : 1024

Web-auth enable            : Enabled

Host mode                  : Single-VLAN

Primary Web server         : aaa

Secondary Web server       : Not configured

Total online web-auth users: 1

- Modified feature: Displaying the running configuration

  - Feature change description

As from this release, the display current-configuration command supports displaying all configuration information.

Command changes

Modified command: display current-configuration

Old syntax

display current-configuration [ [ configuration [ module-name ] | interface [ interface-type [ interface-number ] ] ] | slot slot-number ]

New syntax

display current-configuration [ [ configuration [ module-name ] | interface [ interface-type [ interface-number ] ] ] [ all ] | slot slot-number ]

Views

System view

Change description

The all keyword was added for the command to support displaying all configuration information.

- Modified feature: Displaying the running configuration in current view

  - Feature change description

As from this release, the display this command supports displaying all configuration information in current view.

Command changes

Modified command: display this

Old syntax

display this

New syntax

display this [ all ]

Views

System view

Change description

The all keyword was added for the command to support displaying all configuration information in current view.

Modified feature: 802.1X periodic reauthentication timer

Feature change description

In this release, the maximum value for the 802.1X periodic reauthentication timer changed from 7200 to 86400.

Command changes

Modified command: dot1x timer reauth-period (system view)

Syntax

dot1x timer reauth-period reauth-period-value

Views

System view

Change description

Before modification: The value range for the periodic reauthentication timer is 60 to 7200 seconds in system view.

After modification: The value range for the periodic reauthentication timer is 60 to 86400 seconds in system view.

Modified command: dot1x timer reauth-period (interface view)

Syntax

dot1x timer reauth-period reauth-period-value

Views

Layer 2 Ethernet interface view

Change description

Before modification: The value range for the periodic reauthentication timer is 60 to 7200 seconds in interface view.

After modification: The value range for the periodic reauthentication timer is 60 to 86400 seconds in interface view.

Modified feature: Periodic MAC reauthentication timer

Feature change description

In this release, the maximum value for the periodic MAC reauthentication timer changed from 7200 to 86400.

Command changes

Modified command: mac-authentication timer (system view)

Syntax

mac-authentication timer reauth-period reauth-period-value

Views

System view

Change description

Before modification: The value range for the periodic reauthentication timer is 60 to 7200 seconds in system view.

After modification: The value range for the periodic reauthentication timer is 60 to 86400 seconds in system view.

Modified command: mac-authentication timer (interface view)

Syntax

mac-authentication timer reauth-period reauth-period-value

Views

Layer 2 Ethernet interface view

Change description

Before modification: The value range for the periodic reauthentication timer is 60 to 7200 seconds in interface view.

After modification: The value range for the periodic reauthentication timer is 60 to 86400 seconds in interface view.

Modified feature: Configuring the padding mode and padding format for the Remote ID sub-option of Option 82

Feature change description

As from this release, you can configure the hex remote-id option when you configure the padding mode and padding format for the Remote ID sub-option of Option 82.

Command changes

Modified command: dhcp relay information remote-id

Old syntax

dhcp relay information remote-id { normal [ format { ascii | hex } ] | string remote-id | sysname }

New syntax

dhcp relay information remote-id { hex remote-id | normal [ format { ascii | hex } ] | string remote-id | sysname }

Views

Interface view

Change description

Before modification: The hex remote-id option is not supported in this command.

After modification: The hex remote-id option is supported in this command.

Parameters

hex remote-id: Pads the Remote ID sub-option with a user-defined hexadecimal string of 2 to 256 characters. The number of characters in the string must be even.

Modified feature: Configuring gRPC collectors

Feature change description

As from this release, you can add collectors to a destination group by their domain names. When you specify collectors by their domain names, use the following restrictions and guidelines:

You must configure DNS to make sure the device can translate the domain names of the collectors to IP addresses. For more information about DNS, see Layer 3—IP Services Configuration Guide.

To view domain name and IP address mappings, use the display dns host command. If a domain name maps to multiple IP addresses, the device will push data to the first reachable IP address.

Command changes

    - New command: domain-name

Use domain-name to add the domain name of an IPv4 collector to a destination group.

Use undo domain-name to remove the domain name of an IPv4 collector from a destination group.

Syntax

domain-name domain-name [ port port-number ] [ vpn-instance vpn-instance-name ] [ tls ]

undo domain-name domain-name [ port port-number ] [ vpn-instance vpn-instance-name ] [ tls ]

Default

A destination group does not contain IPv4 collectors.

Views

Destination group view

Predefined user roles

network-admin

Parameters

domain-name: Domain name mapped to the IPv4 address of a collector. It can be a case-insensitive string of 1 to 253 characters and can contain letters, digits, hyphens (-), and dots (.).

port port-number: Specifies the service port number on which the collector receives data. The value range is 1 to 65535 and the default is 50051. To have the collector receive data, make sure the specified service port number is the same as the one used on the collector.

vpn-instance vpn-instance-name: Specifies the MPLS L3VPN instance to which the collector belongs. The vpn-instance-name argument is a case-sensitive string of 1 to 31 characters. Make sure the specified VPN instance already exists. If the collector is on the public network, do not specify this option.

tls: Enables Transport Layer Security (TLS) to encrypt the gRPC connection between the device and the specified collector. The device will then use a root TLS certificate that came with it for encryption. By default, the gRPC connection between the device and a collector is unencrypted.

Usage guidelines

If you specify collectors by their domain names, you must configure DNS to make sure the device can translate the domain names of the collectors to IPv4 addresses. For more information about DNS, see Layer 3—IP Services Configuration Guide.

To view domain name and IP address mappings, use the display dns host command. If a domain name maps to multiple IP addresses, the device will push data to the first reachable IP address.

To add multiple collectors, repeat this command.

A collector is uniquely identified by a three-tuple of domain name, port number, and VPN instance name. One collector must have a different domain name, port number, or VPN instance name than the other collectors.

If you execute this command multiple times to change the TLS enabling state for a collector, the most recent configuration takes effect.

A destination group can have a maximum of five collectors.

You can enable TLS encryption globally by executing the grpc pki domain command in system view or enable collector-specific TLS encryption by specifying the tls keyword when you specify the collector. For a collector, the setting in system view has higher priority than the collector-specific setting.

To modify the collector configuration for a destination group that is already used by a subscription, you must remove the destination group from the subscription first.

Examples

# Add the IPv4 collector at sample.com to destination group collector1.

<Sysname> system-view

[Sysname] telemetry

[Sysname-telemetry] destination-group collector1

[Sysname-telemetry-destination-group-collector1] domain-name sample.com

Related commands

destination-group (subscription view)

subscription

display dns host (Layer 3—IP Services Command Reference)

    - New command: ipv6 domain-name

Use ipv6 domain-name to add the domain name of an IPv6 collector to a destination group.

Use undo ipv6 domain-name to remove the domain name of an IPv6 collector from a destination group.

Syntax

ipv6 domain-name domain-name [ port port-number ] [ vpn-instance vpn-instance-name ] [ tls ]

undo ipv6 domain-name domain-name [ port port-number ] [ vpn-instance vpn-instance-name ] [ tls ]

Default

A destination group does not contain IPv6 collectors.

Views

Destination group view

Predefined user roles

network-admin

Parameters

domain-name: Domain name mapped to the IPv6 address of a collector. It can be a case-insensitive string of 1 to 253 characters and can contain letters, digits, hyphens (-), and dots (.).

port port-number: Specifies the service port number on which the collector receives data. The value range is 1 to 65535 and the default is 50051. To have the collector receive data, make sure the specified service port number is the same as the one used on the collector.

vpn-instance vpn-instance-name: Specifies the MPLS L3VPN instance to which the collector belongs. The vpn-instance-name argument is a case-sensitive string of 1 to 31 characters. Make sure the specified VPN instance already exists. If the collector is on the public network, do not specify this option.

tls: Enables Transport Layer Security (TLS) to encrypt the gRPC connection between the device and the specified collector. The device will then use a root TLS certificate that came with it for encryption. By default, the gRPC connection between the device and a collector is unencrypted.

Usage guidelines

If you specify IPv6 collectors by their domain names, you must configure DNS to make sure the device can translate the domain names of the collectors to IPv6 addresses. For more information about DNS, see Layer 3—IP Services Configuration Guide.

To  view domain name and IP address mappings, use the display dns host command. If a domain name maps to multiple IP addresses, the device will push data to the first reachable IP address.

To add multiple collectors, repeat this command.

A collector is uniquely identified by a three-tuple of domain name, port number, and VPN instance name. One collector must have a different domain name, port number, or VPN instance name than the other collectors.

If you execute this command multiple times to change the TLS enabling state for a collector, the most recent configuration takes effect.

A destination group can have a maximum of five collectors.

You can enable TLS encryption globally by executing the grpc pki domain command in system view or enable collector-specific TLS encryption by specifying the tls keyword when you specify the collector. For a collector, the setting in system view has higher priority than the collector-specific setting.

To modify the collector configuration for a destination group that is already used by a subscription, you must remove the destination group from the subscription first.

Examples

# Add the IPv6 collector at sample.com to destination group collector1.

<Sysname> system-view

[Sysname] telemetry

[Sysname-telemetry] destination-group collector1

[Sysname-telemetry-destination-group-collector1] ipv6 domain-name sample.com

Related commands

destination-group (subscription view)

subscription

display dns host (Layer 3—IP Services Command Reference)

    - Modified command: display grpc

Syntax

display grpc [ verbose ]

Views

Any view

Change description

Before modification: The output from this command contains the Encoding and Telemetry data mode field. However, the command output does not contain the Domain name field.

After modification: The Domain name field is available in the command output. The Encoding and Telemetry data mode fields are not available in the command output.

- Modified feature: Displaying detailed information about 802.1X online users

  - Feature change description

As from this version, the display dot1x connection command can display the AAA authentication method used by each user when they come online.

  - Command changes

    - Modified command: display dot1x connection

Syntax

display dot1x connection [ open ] [ interface interface-type interface-number | slot slot-number | user-mac mac-address | user-name name-string ]

Views

Any view

Change description

The AAA authentication method field was added to the command output. The value for this field can be Local, HWTACACS, RADIUS, or None.

The following shows an example:

# Display information about all 802.1X online users.

<Sysname> display dot1x connection

Total connections: 1

Slot ID: 1

User MAC address: 0015-e9a6-7cfe

Access interface: GigabitEthernet1/0/1

Username: ias

User access state: Successful

Authentication domain: aaa

IPv4 address: 192.168.1.1

IPv6 address: 2000:0:0:0:1:2345:6789:abcd

Authentication method: CHAP

AAA authentication method: Local

Initial VLAN: 1

Authorization untagged VLAN: 6

Authorization tagged VLAN list: 1 to 5 7 9 11 13 15 17 19 21 23 25 27 29 31 33 29 31 33

35 37 40 to 100

Authorization ACL number/name: 3001

Authorization dynamic ACL name: N/A

Authorization user profile: N/A

Authorization CAR: N/A

Authorization URL: N/A

Termination action: Default

Session timeout period: 2 s

Online from: 2013/03/02  13:14:15

Online duration: 0h 2m 15s

Release 6337P01

This release has the following changes:

New feature: Configuring SmartMC

New feature: Configuring interface alarm functions

New feature: Configuring Option 60 for DHCP requests

New feature: Configuring the type of port ID TLVs advertised by LLDP

New feature: Enabling displaying LLDP local information about all interfaces

New feature: PoE forced power supply

New feature: Interval at which the SNMP module examines the system configuration for changes

New feature: Enabling generation of dynamic IPSG binding entries for 802.1X authenticated users

New feature: Automated IPv6 underlay network deployment for VCF fabric

Modified feature: Setting the port status detection timer

Modified feature: 802.1X EAD assistant

Modified feature: Displaying information about online MAC authentication users

Modified feature: L2PT for CFD

- New feature: Configuring SmartMC

  - About SmartMC

Smart Management Center (SmartMC) centrally manages and maintains dispersed network devices at network edges. In a SmartMC network, only one device acts as the commander and the remaining devices all act as members. SmartMC provides the following features for you to manage the members from the commander:

- Configuration file backup and download.

- Software upgrade.

- Configuration deployment.

- Faulty member replacement.

    - SmartMC network framework

Figure 1 shows the basic framework of a SmartMC network.

          - SmartMC network framework

‌

The SmartMC network contains the following elements:

- Commander—Also called topology master (TM), which manages all members in the SmartMC network.

- Member—Also called topology client (TC), which is managed by the commander.

- File server—Stores startup software images and configuration files for the commander and members.

    - SmartMC network establishment

A SmartMC network can be established automatically or manually. In an automatically established SmartMC network, the commander obtains member information through NETCONF sessions to form the network topology. The member information includes port information, LLDP neighbor information, STP information, device type, and software version. In a manually established SmartMC network, the commander obtains member's LLDP neighbor information through NETCONF sessions and member's hardware information through SNMP Get operations.

      - Automatic SmartMC network establishment

The commander and members use the following procedure to establish a SmartMC network:

            - After SmartMC is enabled, the commander broadcasts a SmartMC packet at an interval of 15 seconds to detect members in the network. The SmartMC packet contains information of the commander, such as its bridge MAC address and the IP address of VLAN-interface 1.

            - When a member receives the packet, it records the commander information, and returns a response packet to the commander. The response packet contains information of the member, such as its bridge MAC address and the IP address of VLAN-interface 1.

            - When the commander receives the response packet, it initiates a NETCONF session to the member with the default username admin and the default password admin. The commander then obtains detailed information about the member through the session, including port information, LLDP neighbor information, STP information, device type, and software version.

            - The commander establishes a connection to the member for tracking the liveliness of the member, and adds the member to the SmartMC network.

            - Based on the LLDP neighbor information obtained from all members, the commander forms a SmartMC topology.

After the SmartMC network is established, the commander and members check for the existence of each other by exchanging SmartMC packets.

- When a member receives a SmartMC broadcast packet from the commander, it compares the bridge MAC address in the packet with the recorded bridge MAC address. If the two bridge MAC addresses are the same, the member returns a response packet to the commander. If the member does not receive a broadcast packet from the commander within the time limit, the member determines that the commander does not exist in the network anymore. Then, the member clears the commander information. The time limit is a random value in the range of 60 to 120 seconds.

- When the commander receives a response packet from a member, it compares the bridge MAC address in the packet with the recorded bridge MAC address. If the two bridge MAC addresses are the same, the commander determines that the member still exists in the network. If the commander does not receive a response packet from a member within 150 seconds, the commander determines that the member is offline. Then, the commander sets the status of the member to offline.

      - Manual SmartMC network establishment

You can log in to the Web interface of the commander, and enter the IP address, username, and password of the members to manually add them to the network. The members can join the network without exchanging SmartMC packets with the commander. For more information, see Comware 7 Web-Based Products User Guide.

After you specify the information of a member on the commander, the commander performs the following operations to add the member to the network:

- Verify that the member can be accessed through Telnet.

- Obtain basic member information, including LLDP neighbor information through NETCONF.

- Obtain hardware information through SNMP Get operations.

    - SmartMC features

      - Bulk configuration deployment for members

This feature allows you to deploy multiple command lines to members from the commander, eliminating the need to log in to members and configure the command one by one.

The procedure for bulk configuration deployment is as follows:

            - The commander acts as a Telnet client and establishes Telnet connections to the members.

            - The commander deploys a batch file to the members through Telnet connections. The batch file is created on the commander and contains command lines to be deployed.

            - The members run the command lines in the file.

      - Bulk configuration deployment for ports connecting APs and IP phones

With batch file deployment enabled, the commander automatically deploys configurations in the specified batch file to a port connecting an AP or IP phone, simplifying access port configuration.

When the commander first detects the association of an AP or IP phone on a port through LLDP, it deploys the command lines in the specified batch file to the port. If no batch file is specified for the device type, the configurations on the port remain unchanged.

If the AP or IP phone disconnects from the port, the configurations on the port remain. When a new device comes online from the port, configurations used by the port depend on the new device type.

- If the new device is an AP or IP phone and has the same type as the disconnected device, the configurations on the port remain unchanged.

- If the new device is an AP or IP phone but has a different type as the disconnected device, the commander deploys the command lines in the specified batch file to the port. If no batch file is specified for the device type, the configurations on the port remain unchanged.

- If the new device is neither an AP nor an IP phone, the configurations on the port remain unchanged.

To disable the commander from deploying a batch file to ports, remove the specified batch file or execute the undo smartmc batch-file-apply enable command to disable batch file deployment.

      - Configuration file backup

You can use the following methods to back up the next-startup configuration file on the commander and members:

- Automatic backup—Enable this feature for the commander and all members in the network to immediately perform a backup. After that, the commander and members back up the configuration file at a user-specified interval.

- Manual backup—Manually trigger a backup on the commander or the specified members or SmartMC groups.

To back up the configuration file on a member, the commander instructs the member by unicasting a SmartMC packet to them. When a member receives the packet, it saves the running configuration to the next-startup configuration file and uploads the file to the file server.

      - Startup software and configuration file upgrade

This feature enables users to upgrade startup software and the configuration file of member devices from the commander.

Before upgrade, you must upload the upgrade files from the commander to the file server and specify the upgrade files on the file server for the members to download.

The procedure for startup software and configuration file upgrade is as follows:

            - The commander instructs the members (or SmartMC group) to download the upgrade files from the file server.

            - The members download the upgrade files from the file server.

            - The members upgrade the startup software and configuration file as follows:

  - Startup software upgrade—Uses the boot loader method to perform the software upgrade. The members might be restarted during the upgrade process.

  - Configuration file upgrade—Replaces the current configuration file with the upgrade configuration file. The members will not be restarted during the upgrade process.

      - Faulty member replacement

You can use the following methods to replace a faulty member:

- Automatic replacement—Enables the commander to record the positions of all members in the topology for replacement. When the commander discovers that the new member has physically replaced the faulty member, it compares the new member with the faulty one. The commander performs a replacement if the following requirements are met:

  - The new member is deployed at the same topological position as the faulty one.

  - The models of the new member and faulty member are the same.

The commander then instructs the new member to download the configuration file of the faulty member from the file server. After downloading the configuration file, the new member runs the configuration file to complete the replacement.

- Manual replacement—After the faulty member is physically replaced, you manually trigger a configuration replacement. The new member will download the configuration file of the faulty member from the file server and run the file to complete the replacement.

      - Outgoing interface for a SmartMC network

The outgoing interface feature allows hosts connecting to an outgoing interface to access all the members in a SmartMC network. You can configure multiple outgoing interfaces for a SmartMC network.

As shown in Figure 2, the host is connected to port 1 on the TM and TC 1 and TC 2 are in a different network segment than the host. The host can access the Web interface of the TM but cannot access the Web interface of any member.

If port 1 on the TM is configured as the outgoing interface, the system mirrors the IP address of each member to a new address. The new address contains the IP address of the outgoing interface and the port number assigned by the commander to the member in the format of IP address:Port number. This enables the host to access the Web interfaces of members from the Web interface of the TM.

To access the Web interface of a member, enter the Web interface of the commander, and click Visibility from the navigation pane. Then, click the Topology tab, select the target member, and click Login to Web interface.

          - SmartMC network

‌

      - Automatic link aggregation

Automatic link aggregation automatically bundles multiple physical Ethernet links between two members into one logical link, called an aggregate link. This feature provides increased link bandwidth and improved link reliability.

|  | NOTE: - Automatic link aggregation cannot be performed between the commander and a member, or between a member and a device outside the SmartMC network. You can aggregate the links between the commander and a member manually. For more information about manual link aggregation, see Ethernet link aggregation in Layer 2—LAN Switching Configuration Guide. - If a member enabled with automatic link aggregation joins a SmartMC network whose commander is disabled with the aggregation feature, the feature will be disabled for the member as well. This might affect service traffic forwarding on the member. |
| --- | --- |

‌

      - VLAN creation for members

To simplify configuration and management, you can create a VLAN for members. Then, all access ports on a member that are not connected to other members or the commander are assigned to the VLAN.

If a member has access ports that are connected to offline devices, you must remove the offline devices before creating a VLAN for the member.

The VLAN creation fails for a member if one or more access ports cannot be assigned to the VLAN. If the VLAN creation fails, the VLAN memberships for the access ports are restored to the state before the VLAN was created.

The failure to create a VLAN for a member does not affect the VLAN creation for other members.

      - Resource monitoring

Resource monitoring allows you to view resource usage, memory usage, temperature information, and packet dropping information of commanders and members on the commander.

You can view the usage and temperature information on the commander, and view packet dropping information from the SmartMC > Intelligent O&M > Resource monitoring page of the commander's Web interface.

  - Restrictions: Hardware compatibility with SmartMC

The HPE 5140 EI switch series cannot act as the commander. The switches only support SmartMC members.

  - Restrictions and guidelines: SmartMC configuration

You need to enable SmartMC on both the commander and members and perform all the other tasks only on the commander.

The following features take effect only on members added to the SmartMC network automatically:

- Configuration file backup.

- Faulty member replacement.

- Startup software and configuration file upgrade.

- Automatic link aggregation.

A SmartMC network is established in VLAN 1. For the network to work correctly, do not configure security settings in VLAN 1.

  - SmartMC tasks at a glance

To configure SmartMC, perform the following tasks:

            - Enabling SmartMC

            - Setting the file server information

This task is required for configuring automatic configuration file backup, replacing faulty members, and upgrading the startup software and configuration file on members.

            - (Optional.) Configuring an outgoing interface for the SmartMC network

            - (Optional.) Enabling automatic Ethernet link aggregation

            - (Optional.) Modifying the password of the default user for members

            - Creating a SmartMC group

This task is required for upgrading the startup software and configuration file on members and deploying a batch file to a SmartMC group.

            - (Optional.) Deploying and managing configuration

  - Creating a VLAN for members

  - Deploying a batch file to members

  - Configuring a batch file for ports connecting APs or IP phones

  - Backing up configuration files

            - (Optional.) Monitoring and maintaining the SmartMC network

  - Configuring resource monitoring

  - Upgrading the startup software and configuration file on members

  - Managing the network topology

  - Replacing faulty members

  - Prerequisites for SmartMC

Before you configure SmartMC, perform the following tasks on the commander and members:

- Enable the Telnet service, and configure scheme authentication for VTY user lines. For information about Telnet service and VTY user lines, see CLI login configuration in Fundamentals Configuration Guide.

- Configure a local user.

  - Specify the username and password.

    - On the commander, the username and password must be the same as the username and password configured by using the smartmc tm username username password { cipher | simple } string enable command.

    - On a member, set both the username and password to admin, and execute the password-control length 4, password-control composition type-number 1 type-length 1, and undo password-control complexity user-name check commands to lower the password complexity requirements.

This is because SmartMC requires that the commander use username admin and password admin to communicate with members, which does not meet the default password complexity requirements. For more information about these commands, see password control commands in Security Command Reference.

- After the SmartMC network is established, you can increase the password complexity requirements and use the smartmc tc password command to modify the username and password.

  - Specify the Telnet, HTTP, and HTTPS services for the user.

  - Set the RBAC role of the local user to network-admin.

For information about local users, see AAA configuration in Security Configuration Guide. For information about user roles, see RBAC configuration in Fundamentals Configuration Guide.

- Enable NETCONF over SOAP over HTTP. For information about NETCONF over SOAP, see NETCONF configuration in Network Management and Monitoring Configuration Guide.

- Enable LLDP globally. For information about LLDP, see Layer 2—LAN Switching Configuration Guide.

- To manage the commander and members through a Web interface, you must enable the HTTP and HTTPS services, and set the service type to HTTP and HTTPS for the local user. For information about Web login, HTTP, and HTTPS, see Fundamentals Configuration Guide.

- To manually establish a SmartMC network, you must configure the snmp-agent community read public and snmp-agent sys-info version v2c commands on the members. For information about SNMP, see Network Management and Monitoring Configuration Guide.

  - Enabling SmartMC

      - About SmartMC

Enable this feature on both the commander and members to enable management of members from the commander.

      - Restrictions and guidelines

A SmartMC network must have one and only one commander.

If you change the role of the commander to member or disable SmartMC on the commander, all SmartMC settings in its running configuration will be cleared.

SmartMC fails to be enabled if ACL resources are insufficient. If ACL resources are insufficient, use the undo acl command to delete unnecessary ACLs and then enable SmartMC. You can execute the display acl command to view ACL configuration and match statistics. For more information about ACLs, see ACL and QoS Configuration Guide.

SmartMC fails to be enabled if ports 80 and 443 have been used.

If you execute the smartmc enable command multiple times, the most recent configuration takes effect. You can execute the command to change the device role or the password.

      - Procedure

            - Enter system view.

system-view

            - Enable SmartMC and set the device role.

smartmc { tc | tm username username password { cipher | simple } string } enable

By default, SmartMC is disabled.

  - Setting the file server information

      - About files stored on the file server

In a SmartMC network, a file server is used to store the following files:

- Upgrade startup software files and upgrade configuration file for members.

- Backup configuration files of the commander and members.

For information about FTP servers, see configuring FTP in Fundamentals Configuration Guide. For information about SFTP servers, see configuring SSH in Security Configuration Guide.

      - Restrictions and guidelines

You can use the following methods to specify a file server:

- Specify the IP address of a file server.

- Specify the IP address of the commander. The commander will act as a file server.

To configure the commander to act as a file server, make sure the commander has enough storage space for storing the files required by members.

To use an independent file server, connect the file server to the commander instead of the members as a best practice. The file server uses VLAN 1 to communicate with the SmartMC network. If you connect the file server to members, creating a VLAN for members will assign member interfaces connecting to the file server to the created VLAN, causing file server disconnection. For more information about member VLAN creation, see "Creating a VLAN for members."

      - Procedure

            - Enter system view.

system-view

            - Set the file server information.

smartmc { ftp-server | sftp-server } { ipv4-address | ipv6 ipv6-address } [ port port ] [ vpn-instance vpn-instance-name ] [ directory directory ] username username password { cipher | simple } string

By default, no file server information is set.

  - Configuring an outgoing interface for the SmartMC network

      - Restrictions and guidelines

VLAN interface 1 cannot be used as an outgoing interface, because the SmartMC network is established in VLAN 1.

      - Procedure

            - Enter system view.

system-view

            - Enter VLAN interface view.

interface vlan interface-number

            - Configure the interface as an outgoing interface.

smartmc outbound

By default, no interface is used as an outgoing interface.

  - Enabling automatic Ethernet link aggregation

      - Restrictions and guidelines

Enabling or disabling automatic link aggregation might cause network flapping, and the members might go offline for a short period of time.

      - Procedure

            - Enter system view.

system-view

            - Enable automatic Ethernet link aggregation.

smartmc auto-link-aggregation enable

By default, automatic Ethernet link aggregation is disabled.

  - Modifying the password of the default user for members

      - About modifying the password of the default user for members

During SmartMC network establishment, the commander uses the default username and password to establish NETCONF sessions to members automatically added to the network. The default username and password of the members for NETCONF session establishment are admin and admin.

To enhance security, you can perform this task to change the password for the default user admin of the members after the commander adds the members to the network.

      - Restrictions and guidelines

Do not modify the password for members that are manually added to the SmartMC network. If you modify the password for a manually added member, you will not be able to manage that member from the commander.

You can use the display smartmc tc verbose command to identify the method used to add the members.

      - Procedure

            - Enter system view.

system-view

            - Modify the password of the default user for members.

smartmc tc password [ cipher ] string

  - Creating a SmartMC group

      - About SmartMC groups

This feature allows you to create a SmartMC group on the commander and add members to the group. When you perform the following operations, you can specify a SmartMC group to apply the operations to all members in the group:

- Startup software upgrade.

- Configuration file upgrade.

- Configuration deployment.

      - Procedure

            - Enter system view.

system-view

            - Create a SmartMC group and enter its view.

smartmc group group-name

            - (Optional.) Display predefined device types.

match device-type ?

If the device type of the members is not predefined on the commander, you must manually add the device type to the commander. This enables members of an undefined type to join a SmartMC group created on the commander.

            - Set a match criterion.

match { device-type device-type | ip-address ip-address { ip-mask-length | ip-mask } | mac-address mac-address mac-mask-length }

By default, no match criterion is set.

            - If the device type of the members is not predefined on the commander, perform the following tasks to manually define the device type on the commander:

              - Return to system view.

quit

              - Define a device type on the commander.

smartmc tc sysoid sysoid device-type device-type

To obtain the SYSOID of a member, execute the display smartmc tc verbose command.

You cannot define a predefined member type as another type.

  - Creating a VLAN for members

      - Restrictions and guidelines

If you perform this task multiple times to create a VLAN for members, the most recent configuration takes effect.

      - Procedure

            - Enter system view.

system-view

            - Creating a VLAN for members and assign access ports on the members to the VLAN.

smartmc vlan vlan-id { group group-name-list | tc tc-id-list }

  - Deploying a batch file to members

            - Execute the following command in user view to create a batch file and edit the command lines to be deployed to members.

create batch-file cmd-filename

Each command occupies a line in the batch file. When you finish editing, enter a percent sign (%) to return to user view.

Make sure the command lines that you enter are correct because the system does not verify whether the command lines are correct.

            - Enter system view.

system-view

            - Deploy the batch file to a list of members or SmartMC groups.

smartmc batch-file cmd-filename deploy { group group-name-list | tc tc-id-list }

  - Configuring a batch file for ports connecting APs or IP phones

      - Restrictions and guidelines

All commands in the batch file must be commands used in interface view.

The size of the batch file cannot exceed 8190 characters.

Make sure the file name is correct when specifying the batch file because the system does not verify whether the file name is correct. After specifying the batch file, do not delete the file or rename the file.

      - Procedure

            - (Optional.) Execute the following command in user view to create a batch file and edit the command lines to be deployed to members.

create batch-file cmd-filename

Each command occupies a line in the batch file. When you finish editing, enter a percent sign (%) to return to user view.

Make sure the command lines that you enter are correct because the system does not verify whether the command lines are correct.

            - Enter system view.

system-view

            - Specify the batch file for ports connecting APs or IP phones.

smartmc batch-file batch-file-name apply { ap | phone }

            - (Optional.) Disable batch file deployment.

undo smartmc batch-file-apply enable

By default, batch file deployment is enabled.

  - Backing up configuration files

      - About backing up configuration files

Perform this task to back up the configuration file of the commander or the specified members. Configuration files automatically backed up to the file server are named in the format of device bridge MAC address_backup.cfg.

      - Restrictions and guidelines

When you change the commander in the SmartMC network, make sure the backup configuration file of the original commander on the file server is deleted. If the file still exists, the new commander might download the file and run the settings. This will cause a conflict in the network.

The maximum number of members that can perform automatic configuration at the same time is limited by the performance of the file server. If automatic configuration backup fails, set the maximum number of members to a smaller value.

      - Prerequisites

Before performing this task, you must set the file server information (see "Setting the file server information").

      - Procedure

            - Enter system view.

system-view

            - Set the maximum number of members that can perform configuration file backup at the same time.

smartmc backup configuration max-number max-number

By default, a maximum of five members can perform automatic configuration backup at the same time.

            - Back up configuration files.

Choose one option as needed:

  - Enable automatic configuration file backup and set the backup interval.

smartmc backup startup-configuration interval interval-time

By default, automatic configuration file backup is disabled.

  - Manually back up the configuration file on members.

smartmc backup configuration { group group-name-list | tc [ tc-id-list ] }

TC ID 0 represents the commander.

  - Configuring resource monitoring

            - Enter system view.

system-view

            - Set the interval for the commander to obtain resource monitoring information.

smartmc resource-monitor interval interval

The default setting is 1 minute.

            - Set the aging time for resource monitoring information.

smartmc resource-monitor max-age max-age

The default setting is 24 hours.

            - Enable resource monitoring.

smartmc resource-monitor [ cpu | memory | packet-drop | temperature ] * [ group group-name-list | tc { tc-id-list | mac-address mac-address } | tm ]

By default, resource monitoring is disabled.

If you do not specify a resource type, this command enables resource monitoring for all resource types.

If you do not specify a device to monitor (member or commander), this command enables resource monitoring on the commander and all members.

  - Upgrading the startup software and configuration file on members

    - About upgrading the startup software and configuration file on members

You can use the following methods to upgrade the startup software and configuration file on members:

- Schedule an upgrade by specifying an upgrade time or upgrade delay.

- Upgrade immediately by not specifying an upgrade time or upgrade delay.

    - Restrictions and guidelines for startup software and configuration file upgrade

A member can perform only one upgrade task at a time.

An immediate upgrade cannot be cancelled. If you specify a delay time or upgrade time to perform a scheduled upgrade, the upgrade operation can be cancelled by using the undo smartmc upgrade command before it starts.

    - Prerequisites

Before performing this task, you must set the file server information (see "Setting the file server information").

    - Upgrading the startup software and configuration file on members

      - Upgrading the startup software and configuration file in one step

            - Enter system view.

system-view

            - Upgrade the startup software on members in one step.

smartmc upgrade boot-loader tc { tc-id-list { boot boot-filename system system-filename | file ipe-filename } }&<1-40> [ delay delay-time | time in-time ]

|  | CAUTION: Upgrading the startup software might interrupt services. Before upgrading the startup software, make sure no services will be interrupted. |
| --- | --- |

            - Upgrade the configuration file on members in one step.

smartmc upgrade startup-configuration tc { tc-id-list cfg-filename }&<1-40> [ delay delay-time | time in-time ]

|  | CAUTION: After you update the configuration file, the configuration in the new configuration file will become the running configuration. Before upgrading the configuration file, make sure the contents of the new configuration file are correct. |
| --- | --- |

      - Configuring startup software and configuration file upgrade step by step

            - Enter system view.

system-view

            - Configure startup software upgrade for members step by step:

              - Specify the upgrade startup software files.

smartmc tc tc-id boot-loader { ipe-filename | boot boot-filename system system-filename }

              - Upgrade the startup software on members.

smartmc upgrade boot-loader tc tc-id-list

|  | CAUTION: Upgrading the startup software might interrupt services. Before upgrading the startup software, make sure no services will be interrupted. |
| --- | --- |

            - Configure configuration file upgrade for members step by step:

              - Specify the upgrade configuration file.

smartmc tc tc-id startup-configuration cfg-filename

              - Upgrade the configuration file on members.

smartmc upgrade startup-configuration tc tc-id-list

|  | CAUTION: After you update the configuration file, the configuration in the new configuration file will become the running configuration. Before upgrading the configuration file, make sure the contents of the new configuration file are correct. |
| --- | --- |

    - Upgrading the startup software and configuration file on all members in SmartMC groups

      - Upgrading the startup software and configuration file in one step

            - Enter system view.

system-view

            - Upgrade the startup software on all members in SmartMC groups in one step.

smartmc upgrade boot-loader group { group-name-list [ boot boot-filename system system-filename | file ipe-filename ] }&<1-40> [ delay minutes | time in-time ]

|  | CAUTION: Upgrading the startup software might interrupt services. Before upgrading the startup software, make sure no services will be interrupted. |
| --- | --- |

            - Upgrade the configuration file on all members in SmartMC groups in one step.

smartmc upgrade startup-configuration group { group-name-list file cfg-filename }&<1-40> [ delay minutes | time in-time ]

|  | CAUTION: After you update the configuration file, the configuration in the new configuration file will become the running configuration. Before upgrading the configuration file, make sure the contents of the new configuration file are correct. |
| --- | --- |

      - Configuring startup software and configuration file upgrade step by step

            - Enter system view.

system-view

            - Enter SmartMC group view.

smartmc group group-name

            - Specify the upgrade startup software files for the SmartMC group.

boot-loader file { ipe-filename | boot boot-filename system system-filename }

By default, no upgrade startup software files are specified for a SmartMC group.

            - Specify the upgrade configuration file for the SmartMC group.

startup-configuration cfgfile

By default, no upgrade configuration file is specified for a SmartMC group.

            - Return to system view.

quit

            - Upgrade the startup software and configuration file on all members in the SmartMC group.

Choose one option as needed:

  - Upgrade the startup software.

smartmc upgrade boot-loader group group-name-list [ delay minutes | time in-time ]

|  | CAUTION: Upgrading the startup software might interrupt services. Before upgrading the startup software, make sure no services will be interrupted. |
| --- | --- |

  - Upgrade the configuration file.

smartmc upgrade startup-configuration group group-name-list [ delay minutes | time in-time ]

|  | CAUTION: After you update the configuration file, the configuration in the new configuration file will become the running configuration. Before upgrading the configuration file, make sure the contents of the new configuration file are correct. |
| --- | --- |

  - Managing the network topology

    - Refreshing the network topology

      - About refreshing the network topology

You can use the following methods to refresh the network topology:

- Automatic topology refresh—Specify the refresh interval to allow the commander to refresh the network topology periodically.

- Manual topology refresh—Execute the smartmc topology-refresh command to manually refresh the network topology.

      - Restrictions and guidelines

The topology refresh time depends on the number of members in the network.

      - Procedure

Choose one option as needed:

- Manually refresh the network topology in any view.

smartmc topology-refresh

- Configure automatic network topology refresh.

              - Enter system view.

system-view

              - Set the automatic topology refresh interval.

smartmc topology-refresh interval interval

By default, the automatic topology refresh interval is 60 seconds.

    - Saving the network topology

      - About saving the network topology

This task allows you to save the current network topology to the topology.dba file in the flash memory. After the commander reboots, it uses the topology.dba file to restore the network topology.

      - Procedure

            - Enter system view.

system-view

            - Save the network topology.

smartmc topology-save

  - Replacing faulty members

      - Restrictions and guidelines

Make sure the new member for replacement and the faulty member have the same neighbor relationship, device model, and IRF member ID.

Make sure the new member has a different member ID than all the members in the SmartMC network, including offline members. Faulty members are considered offline.

To automatically replace a faulty member, first enable automatic replacement, and then install the new member at the location where the faulty member was installed and connect all cables.

To manually replace a faulty member, first install the new member at the location where the faulty member was installed, connect all cables, and then execute the manual replacement command.

      - Prerequisites

Before you replace a faulty member, set the file server information (see "Setting the file server information").

      - Procedure

            - Enter system view.

system-view

            - Replace faulty members.

Choose one option as needed:

  - Enable automatic faulty member replacement.

smartmc auto-replace enable

By default, automatic faulty member replacement is disabled.

  - Manually replace a faulty member.

smartmc replace tc tc-id1 faulty-tc tc-id2

  - Display and maintenance commands for SmartMC

Execute display commands in any view.

| Task | Command |
| --- | --- |
| Display the backup status on members. | display smartmc backup configuration status |
| Display the batch file execution results. | display smartmc batch-file status [ ap ¦ last number ¦ phone ] |
| Display SmartMC configuration. | display smartmc configuration |
| Display connections between the devices in the SmartMC network. | display smartmc device-link |
| Display SmartMC group information. | display smartmc group [ group-name ] [ verbose ] |
| Display the faulty member replacement status. | display smartmc replace status |
| Display resource monitoring information. | display smartmc resource-monitor [ cpu ¦ memory ¦ temperature ] * [ tc tc-id ¦ tm ] |
| Display resource monitoring configuration. | display smartmc resource-monitor configuration |
| Display member information. | display smartmc tc [ tc-id ] [ verbose ] |
| Display log information in the log buffer on a member. | display smartmc tc tc-id log buffer [ module module-name [ mnemonic mnemonic-value ] ] |
| Display restart log information for a member. | display smartmc tc tc-id log restart |
| Display VLAN creation results for members. | display smartmc vlan |
| Display member upgrade status. | display smartmc upgrade status |

‌

  - SmartMC configuration examples

    - Example: Configuring SmartMC

      - Network configuration

As shown in Figure 3, member 1, member 2, and member 3 belong to the same device type: HPE FlexNetwork 5140 EI series. The IP address of the FTP server is 192.168.2.1. The FTP username is admin and the FTP password is hello12345.

Perform the following tasks to establish a SmartMC network and upgrade the configuration file on the members:

            - Configure the commander and members to automatically establish a SmartMC network.

            - Configure interface GigabitEthernet 1/0/1 as the outgoing interface for the SmartMC network.

            - Create a SmartMC group and add the members to the group.

            - Upgrade the configuration file on all members in the SmartMC group.

            - Save configuration file startup.cfg on the FTP server.

          - Network diagram

‌

      - Procedure

            - Configure TC 1:

# Configure VLAN-interface 1.

<TC1> system-view

[TC1] interface vlan-interface 1

[TC1-Vlan-interface1] ip address 192.168.2.3 24

[TC1-Vlan-interface1] quit

# Enable HTTP and HTTPS.

[TC1] ip http enable

[TC1] ip https enable

# Enable the Telnet service.

[TC1] telnet server enable

# Enable NETCONF over SOAP over HTTP.

[TC1] netconf soap http enable

# Enable LLDP globally.

[TC1] lldp global enable

# Create a user named admin.

[TC1] local-user admin

# Lower password complexity requirements. For more information about these commands, see password control commands in Security Command Reference.

[TC1-luser-manage-admin] password-control length 4

[TC1-luser-manage-admin] password-control composition type-number 1 type-length 1

[TC1-luser-manage-admin] undo password-control complexity user-name check

# Set the password to admin, add the telnet, http, and https service types, and authorize the user to use the network-admin user role.

[TC1-luser-manage-admin] password simple admin

[TC1-luser-manage-admin] service-type telnet http https

[TC1-luser-manage-admin] authorization-attribute user-role network-admin

[TC1-luser-manage-admin] quit

# Set scheme authentication for VTY user lines 0 to 63.

[TC1] line vty 0 63

[TC1-line-vty0-63] authentication-mode scheme

[TC1-line-vty0-63] quit

# Enable SmartMC and set the device role to tc.

[TC1] smartmc tc enable

            - Configure TC 2 and TC 3 in the same way TC 1 is configured. (Details not shown.)

            - Configure the TM:

# Configure GigabitEthernet 1/0/1.

<TM> system-view

[TM] interface gigabitethernet 1/0/1

[TM-GigabitEthernet1/0/1] port link-mode route

[TM-GigabitEthernet1/0/1] ip address 192.168.52.2 24

[TM-GigabitEthernet1/0/1] quit

# Configure VLAN-interface 1.

[TM] interface vlan-interface 1

[TM-Vlan-interface1] ip address 192.168.2.2 24

[TM-Vlan-interface1] quit

# Enable HTTP and HTTPS.

[TM] ip http enable

[TM] ip https enable

# Enable the Telnet service.

[TM] telnet server enable

# Enable NETCONF over SOAP over HTTP.

[TM] netconf soap http enable

# Enable LLDP globally.

[TM] lldp global enable

# Create a user. Set the username to admin and the password to hello12345, add the telnet, http, and https service types, and authorize the user to use the network-admin user role.

[TM] local-user admin

[TM-luser-manage-admin] password simple hello12345

[TM-luser-manage-admin] service-type telnet http https

[TM-luser-manage-admin] authorization-attribute user-role network-admin

[TM-luser-manage-admin] quit

# Set scheme authentication for VTY user lines 0 to 63.

[TM] line vty 0 63

[TM-line-vty0-63] authentication-mode scheme

[TM-line-vty0-63] quit

# Enable SmartMC, set the device role to commander, and set the username to admin and the password (plaintext) to hello12345.

[TM] smartmc tm username admin password simple hello12345 enable

# Specify GigabitEthernet 1/0/1 as the outgoing interface.

[TM] interface gigabitethernet 1/0/1

[TM-GigabitEthernet1/0/1] smartmc outbound

[TM-GigabitEthernet1/0/1] quit

# Set the FTP server IP address, username, and plaintext password to 192.168.2.1, admin, and hello12345, respectively.

[TM] smartmc ftp-server 192.168.2.1 username admin password simple hello12345

# Create SmartMC group S1 and enter its view.

[TM] smartmc group S1

# Create an IP address match criterion to add all members in the specified network segment to SmartMC group S1.

[TM-smartmc-group-S1] match ip-address 192.168.2.0 24

# Specify the upgrade configuration file startup.cfg for SmartMC group S1.

[TM-smartmc-group-S1] startup-configuration startup.cfg

[TM-smartmc-group-S1] quit

# Upgrade the configuration file on all members in SmartMC group S1.

[TM] smartmc upgrade startup-configuration group S1 file startup.cfg

      - Verifying the configuration

# Display brief information about all members after the SmartMC network is established.

[TM] display smartmc tc

TCID DeviceType        Sysname  IpAddress       MacAddress     Status   Version

1    5140 24G 4SFP+ EI  TC1      192.168.2.3     201c-e7c3-0300 Normal   COMWAREV700R001

2    5140 24G 4SFP+ EI  TC2      192.168.2.4     201c-e7c3-0301 Normal   COMWAREV700R001

3    5140 24G 4SFP+ EI  TC3      192.168.2.5     201c-e7c3-0302 Normal   COMWAREV700R001

# Display the configuration file upgrade status on the members.

<TM> display smartmc upgrade status

ID    IpAddress        MacAddress      Status      UpdateTime        UpdateFile

1     192.168.2.3      201c-e7c3-0300  Finished    Immediately       startup.cfg

2     192.168.2.4      201c-e7c3-0301  Finished    Immediately       startup.cfg

3     192.168.2.5      201c-e7c3-0302  Finished    Immediately       startup.cfg

Command reference

    - boot-loader file

Use boot-loader file to specify the upgrade startup software files for a SmartMC group.

Use undo boot-loader to restore the default.

Syntax

boot-loader file { ipe-filename | boot boot-filename system system-filename }

undo boot-loader

Default

No upgrade startup software files are specified for a SmartMC group.

Views

SmartMC group view

Predefined user roles

network-admin

Parameters

ipe-filename: Specifies an IPE software file by its name, a case-insensitive string of 5 to 45 characters. The file name must include the .ipe extension.

boot boot-filename: Specifies a boot image file by its name, a case-insensitive string of 5 to 45 characters. The file name must include the .bin extension.

system system-filename: Specifies a system image file by its name, a case-insensitive string of 5 to 45 characters. The file name must include the .bin extension.

Usage guidelines

If you execute this command multiple times, the most recent configuration takes effect.

Examples

# Specify IPE software file device.ipe for SmartMC group testgroup.

<Sysname> system-view

[Sysname] smartmc group testgroup

[Sysname-smartmc-group-testgroup] boot-loader file device.ipe

Related commands

smartmc group

smartmc upgrade boot-loader

    - create batch-file

Use create batch-file to create a batch file.

Syntax

create batch-file batch-file-name

Default

No batch files exist.

Views

User view

Predefined user roles

network-admin

Parameters

batch-file-name: Specifies the name of the batch file, a case-insensitive string of 1 to 255 characters. If you do not specify a file extension when specifying a file name, the default extension .cmdset is used.

Usage guidelines

After executing this command, you will enter the batch edit mode. In this mode, each command occupies a line. When you finish editing all command lines, enter a percent sign (%) to return to user view.

Make sure the command lines that you enter are correct because the system does not verify whether the command lines are correct.

Examples

# Create a batch file named test.cmdset, and enter the command lines for specifying the device name as Sysname and enabling Telnet.

<Sysname> create batch-file test.cmdset

Begin to edit batch commands, and quit with the character '%'.

system-view

sysname Sysname

telnet server enable%

<Sysname>

Related commands

display smartmc batch-file status

smartmc batch-file deploy

    - display smartmc backup configuration status

Use display smartmc backup configuration status to display the backup status on members.

Syntax

display smartmc backup configuration status

Views

Any view

Predefined user roles

network-admin

Usage guidelines

This command displays the status of the ongoing backup task or the most recent backup task if the member is not performing backup.

Examples

# Display the backup status on members.

<Sysname> display smartmc backup configuration status

ID     IpAddress        MacAddress          Status       Time

1      192.168.56.30    08d2-38ff-0300      Finished     2017-04-05 11:30:35

2      192.168.56.40    62d2-c21c-0400      Finished     2017-04-05 11:30:40

        - Command output

| Field | Description |
| --- | --- |
| ID | ID of the member. |
| IpAddress | IP address of the member. |
| MacAddress | MAC address of the member. |
| Status | Backup status: - Waiting—The member is waiting for configuration backup. - Processing—The member is backing up the configuration. - Finished—The member has finished backing up the configuration. - Timeout—Configuration backup times out. - Failed—The member failed to back up the configuration. |
| Time | Time when the member finished backing up the configuration. If the member has not finished backing up the configuration, this field displays a hyphen (-). |

Related commands

smartmc backup configuration

smartmc backup configuration interval

smartmc backup configuration max-number

    - display smartmc batch-file status

Use display smartmc batch-file status to display the batch file deployment result.

Syntax

display smartmc batch-file status [ ap | last number | phone ]

Views

Any view

Predefined user roles

network-admin

Parameters

ap: Displays the result of the most recent batch file deployment for ports connected to APs.

last number: Specifies a batch file deployment (performed by using the smartmc batch-file deploy command) by its number counting from the most recent batch file deployment. The value range for the number argument is 1 to 5.

phone: Displays the result of the most recent batch file deployment for ports connected to IP phones.

Usage guidelines

If you do not specify any parameters, this command displays the result of the most recent batch file deployment performed by using the smartmc batch-file deploy command.

Examples

# Display the result of the most recent batch file deployment. In this example, the batch file contains the display smartmc configuration command.

<Sysname> display smartmc batch-file status last 1

TC ID 1

Device MAC : 8a73-60c3-0200

Start Time : 2018-12-24 14:55:39

End   Time : 2018-12-24 14:55:43

Result     :

<Sysname>display smartmc configuration

Device role              : TC

TM IP                    : 192.168.22.103

TM MAC                   : 8a73-4faa-0100

TM sysname               : Sysname

<Sysname>

TC ID 2

Device MAC : 8a73-6b31-0300

Start Time : 2018-12-24 14:55:43

End   Time : 2018-12-24 14:55:48

Result     :

<Sysname>display smartmc configuration

Device role              : TC

TM IP                    : 192.168.22.103

TM MAC                   : 8a73-4faa-0100

TM sysname               : Sysname

<Sysname>

        - Command output

| Field | Description |
| --- | --- |
| TC ID | ID of the member. |
| Device MAC | MAC address of the member. |
| Start Time | Batch file deployment start time. |
| End Time | Batch file deployment end time. |
| Result | Batch file deployment result in details. |

Related commands

create batch-file

smartmc batch-file apply

smartmc batch-file deploy

    - display smartmc configuration

Use display smartmc configuration to display the SmartMC configuration.

Syntax

display smartmc configuration

Views

Any view

Predefined user roles

network-admin

Examples

# Display the SmartMC configuration on the commander.

<Sysname> display smartmc configuration

Device role                           : TM

File server:

Type: FTP

IP address: 192.168.22.103

Username: admin

Port: 21

VPN instance: N/A

Directory: /FTP

Topology-refresh interval             : 60(s)

Backup startup-configuration interval : N/A

Sync backup number                    : 5

Device status                         : Lack

Some configurations are absent on the TM, such as Telnet or LLDP configuration.

# Display the commander information on a member.

<Sysname> display smartmc configuration

Device role       : TC

TM IP             : 192.168.22.103

TM MAC            : 8288-468d-0100

TM sysname        : Sysname

        - Command output

| Field | Description |
| --- | --- |
| Device role | Role of the device. |
| File server | File server configuration. |
| Type | File server type. If no file server is specified, this field displays N/A. |
| IP address | File server IP address. If no file server is specified, this field displays N/A. |
| Username | File server username. If no file server is specified, this field displays N/A. |
| Port | File server port. If no file server is specified, this field displays N/A. |
| VPN instance | VPN instance to which the file server belongs. If no file server is specified, this field displays N/A. |
| Directory | Storage directory of files on the file server. If no file server is specified, this field displays N/A. |
| Topology-refresh interval | Topology refresh interval, in seconds. |
| Backup startup-configuration interval | Automatic configuration file backup interval, in hours. If no interval is set, this field displays N/A. |
| Sync backup number | Number of members that can perform configuration backup at the same time. |
| Device status | Commander status: - Normal. - Lack—Lack of configuration, such as NETCONF, Telnet, local user, and LLDP. |
| TM IP | IP address of the commander. If the member failed to obtain the commander IP address, this field displays N/A. |
| TM MAC | MAC address of the commander, If the member failed to obtain the commander MAC address, this field displays N/A. |
| TM sysname | Name of the commander. If the member failed to obtain the commander name, this field displays N/A. |
| Some configurations are absent on the TM, such as XXX. | This field is available only when the Device status field displays Lack. Lack of configuration will affect SmartMC functions. Please follow the prompt to complete the configuration. |

Related commands

smartmc backup configuration interval

smartmc backup configuration max-number

smartmc enable

smartmc { ftp-server | sftp-server }

smartmc topology-refresh interval

    - display smartmc device-link

Use display smartmc device-link to display connections between devices in the SmartMC network.

Syntax

display smartmc device-link

Views

Any view

Predefined user roles

network-admin

Examples

# Display connections between devices in the SmartMC network.

<Sysname> display smartmc device-link

(TM IP)[192.168.56.20]

ID    Hop  LocalPort            LocalIP           PeerPort               PeerIP

0     0    GigabitEthernet1/0/2  192.168.56.20     GigabitEthernet1/0/1   192.168.56.30

1     1    GigabitEthernet1/0/1  192.168.56.30     GigabitEthernet1/0/2   192.168.56.20

1     2    GigabitEthernet1/0/2  192.168.56.30     GigabitEthernet1/0/1   192.168.56.40

2     3    GigabitEthernet1/0/1  192.168.56.40     GigabitEthernet1/0/2   192.168.56.30

        - Command output

| Field | Description |
| --- | --- |
| TM IP | IP address of the commander. |
| ID | ID of the commander or member. |
| Hop | Number of hops between the commander and member. |
| LocalPort | Local port. |
| LocalIP | IP address of the local device. |
| PeerPort | Peer port. |
| PeerIP | IP address of the peer port. |

Related commands

smartmc topology-refresh

smartmc topology-refresh interval

    - display smartmc group

Use display smartmc group to display SmartMC group information.

Syntax

display smartmc group [ group-name ] [ verbose ]

Views

Any view

Predefined user roles

network-admin

Parameters

group-name: Specifies a SmartMC group by its name, a case-sensitive string of 1 to 31 characters. If you do not specify this argument, the command displays information about all SmartMC groups.

Verbose: Displays detailed SmartMC group information. If you do not specify this keyword, the command displays brief SmartMC group information.

Examples

# Display detailed SmartMC group information.

<Sysname> display smartmc group verbose

Group name                   : test

TC count                     : 3

Boot-loader file             :

Startup-configuration file   :

Rule:

Match Device-type 5140 24G 4SFP+ EI

TCID DeviceType        Sysname  IpAddress       MacAddress     Status   Version

1    5140 24G 4SFP+ EI  S1       192.168.56.103  0e74-e2fb-0400 Normal   COMWAREV700R001

2    5140 24G 4SFP+ EI  S2       192.168.56.102  0e74-ea13-0500 Normal   COMWAREV700R001

3    5140 24G 4SFP+ EI  S3       192.168.56.104  0e74-db54-0300 Normal   COMWAREV700R001

        - Command output

| Field | Description |
| --- | --- |
| GroupName | Name of the SmartMC group. |
| TC count | Number of members in the SmartMC group. |
| Boot-loader file | Names of the upgrade startup software files for upgrading the SmartMC group. If no upgrade startup software files are specified, this field displays null. |
| Startup-configuration file | Name of the configuration file for upgrading the SmartMC group. If no configuration file is specified, this field displays null. |
| Rule | Match criteria of the SmartMC group. |
| Match | Match type and its value. The match types include the following: - Device-type—Matches members by device type. - IP-address—Matches members by IP address. - MAC-address—Matches members by MAC address. |
| TCID | ID of the member. |
| DeviceType | Device type of the member. |
| Sysname | Device name of the member. |
| IpAddress | IP address of the member. |
| MacAddress | MAC address of the member. |
| Version | Software version of the member. |
| Status | Operating status of the member: - Offline—The member is offline. - Normal—The member is online. |

Related commands

match

smartmc group

    - display smartmc replace status

Use display smartmc replace status to display faulty member replacement status.

Syntax

display smartmc replace status

Views

Any view

Predefined user roles

network-admin

Examples

# Display faulty member replacement status.

<Sysname> display smartmc replace status

Faulty ID      : 2

Faulty MAC     : 94e2-cdcb-0600

Replacement ID : 3

Replacement MAC: 2443-5f8c-0200

Mode           : Manual

Status         : Successful

Start time     : 2017-03-21 15:01:31

End time       : 2017-03-21 15:01:40

        - Command output

| Field | Description |
| --- | --- |
| Faulty ID | ID of the faulty member. |
| Faulty MAC | MAC address of the faulty member. |
| Replacement ID | ID of the new member. |
| Replacement MAC | MAC address of the new member. |
| Mode | Replacement method, which can be Manual or Auto. |
| Status | Replacement status: - Successful. - Failed. - Replacing. - Timeout. |
| Start time | Replacement start time |
| End time | Replacement end time. |

Related commands

smartmc auto-replace enable

smartmc replace

    - display smartmc resource-monitor

Use display smartmc resource-monitor to display resource monitoring information.

Syntax

display smartmc resource-monitor [ cpu | memory | temperature ] * [ tc tc-id | tm ]

Views

Any view

Predefined user roles

network-admin

Parameters

cpu: Displays CPU usage.

memory: Displays memory usage.

temperature: Displays temperature information.

tc tc-id: Specify a member by its ID in the range of 1 to 255.

tm: Specify the commander.

Usage guidelines

This command displays CPU usage, memory usage, and temperature information of the commander and members on the commander. For packet dropping information, log in to the Web interface of the commander and access the SmartMC > Intelligent O&M > Resource monitoring page.

If you do not specify a resource type, this command displays the resource monitoring information of all types.

If you do not specify a member or the commander, this command displays the resource monitoring information for the commander and all members.

Examples

# Display the resource monitoring information for member 1.

<Sysname> display smartmc resource-monitor tc 1

TC 1

Collection time : 2017-07-25 18:02:30

Slot 1:

CPU 0 CPU usage: 1%

Memory usage   : 587076/903332

Temperature    : 30

        - Command output

| Field | Description |
| --- | --- |
| Collection time | Time when the resource monitoring information was collected. |

Related commands

smartmc resource-monitor

    - display smartmc resource-monitor configuration

Use display smartmc resource-monitor configuration to display resource monitoring configuration.

Syntax

display smartmc resource-monitor configuration

Views

Any view

Predefined user roles

network-admin

Usage guidelines

This command displays CPU usage, memory usage, and temperature monitoring configuration of the commander. You can view the status of packet dropping monitoring by using the display current-configuration | include smartmc command.

Examples

# Display resource monitoring configuration.

<Sysname> display smartmc resource-monitor configuration

ID  MacAddress      CPU  Memory  Temperature

1   1234-2222-3333  Y    N       N

2   1234-2222-3334  Y    N       N

3   1234-2222-3335  Y    N       N

        - Command output

| Field | Description |
| --- | --- |
| ID | Device ID. |
| MacAddress | MAC address of the device. |
| CPU | CPU usage monitoring status: - Y—CPU usage monitoring is enabled. - N—CPU usage monitoring is disabled. - -—The device does not support CPU usage monitoring. |
| Memory | Memory usage monitoring status: - Y—Memory usage monitoring is enabled. - N—Memory usage monitoring is disabled. - -—The device does not support memory usage monitoring. |
| Temperature | Temperature monitoring status: - Y—Temperature monitoring is enabled. - N—Temperature monitoring is disabled. - -—The device does not support temperature monitoring. |

Related commands

smartmc resource-monitor

    - display smartmc tc

Use display smartmc tc to display member information.

Syntax

display smartmc tc [ tc-id ] [ verbose ]

Views

Any view

Predefined user roles

network-admin

Parameters

tc-id: Specifies a member by its ID in the range of 1 to 255. If you do not specify a member, this command displays information about all members.

verbose: Displays detailed member information. If you do not specify this keyword, the command displays brief member information.

Examples

# Display brief information about all members.

<Sysname> display smartmc tc

TCID DeviceType        Sysname  IpAddress       MacAddress     Status   Version

1    5140 24G 4SFP+ EI  S1       192.168.22.104  201c-e7c3-0300 Normal   COMWAREV700R001

        - Command output

| Field | Description |
| --- | --- |
| TCID | ID of the member. |
| DeviceType | Device type of the member. |
| Sysname | Device name of the member. |
| IpAddress | IP address of the member. |
| MacAddress | MAC address of the member. |
| Status | Operating status of the member: - Normal—The member is operating correctly. - Offline—The member is offline. |
| Version | Software version of the member. |

# Display detailed information about all members.

<Sysname> display smartmc tc verbose

TC ID                        : 1

Adding method                : Manual

Sysname                      : S1

Model                        : 5140 24G 4SFP+ EI

Device type                  : 5140-EI

SYSOID                       : 1.3.6.1.4.1.25506

MAC address                  : 0e74-e2fb-0400

IP address                   : 192.168.56.103

Boot image                   :

Boot image version           :

System image                 :

System image version         :

Current-configuration file   :

Uptime                       : 2 days, 3 hours, 4 minutes

System CPU usage             : 0%

System memory usage          : 0%

Status                       : Offline

Boot-loader file             :

Startup-configuration file   :

        - Command output

| Field | Description |
| --- | --- |
| TC ID | ID of the member. |
| Adding method | Method through which the member is added to the SmartMC network: - Manual. - Auto. |
| Sysname | Device name of the member. |
| Model | Device model of the member. |
| Device type | Device type of the member. |
| SYSOID | SYSOID of the member. |
| MAC address | MAC address of the member. |
| IP address | IP address of the member. |
| Boot image | Boot image file that the member runs. |
| Boot image version | Version of the boot image file. |
| System image | System image file that the member runs. |
| System image version | Version of the system image file. |
| Current-configuration file | Current startup configuration file used by the member. |
| Uptime | Operation duration of the member. |
| System CPU usage | CPU usage on the member. |
| System memory usage | Memory usage on the member. |
| Status | Operating status of the member: - Normal—The member is operating correctly. - Offline—The member is offline. |
| Boot-loader file | Upgrade startup software files. |
| Startup-configuration file | Upgrade configuration file. |

    - display smartmc tc log buffer

Use display smartmc tc log buffer to display log information in the log buffer on a member.

Syntax

display smartmc tc tc-id log buffer [ module module-name [ mnemonic mnemonic-value ] ]

Views

Any view

Predefined user roles

network-admin

Parameters

tc-id: Specifies a member by its ID in the range of 1 to 255.

module module-name: Specifies a module by its name, a case-insensitive string of 1 to 8 characters. To display module names, use the info-center source command (see information center commands in Network Management and Monitoring Command Reference).

mnemonic mnemonic-value: Specifies a mnemonic, a case-insensitive string of 1 to 32 characters.

Examples

# Display the log information for the SHELL module with the SHELL_CMD mnemonic for member 1.

<Sysname> display smartmc tc 1 log buffer module SHELL mnemonic SHELL_CMD

Time     : 2017-07-15 13:51:46

Level    : Informational

Module   : SHELL

Mnemonic : SHELL_CMD

Content  : -Line=con0-IPAddr=**-User=**; Command is qu

Time     : 2017-07-15 13:51:39

Level    : Informational

Module   : SHELL

Mnemonic : SHELL_CMD

Content  : -Line=con0-IPAddr=**-User=**; Command is local-user admin

        - Command output

| Field | Description |
| --- | --- |
| Time | Time when the log was generated. |
| Level | Log level. |

    - display smartmc tc log restart

Use display smartmc tc log restart to display the restart log information for a member.

Syntax

display smartmc tc tc-id log restart

Views

Any view

Predefined user roles

network-admin

Parameters

tc-id: Specifies a member by its ID in the range of 1 to 255.

Usage guidelines

In addition to saving the logs generated by modules to the log buffer, a member sends restart logs to the commander. The commander creates a restart log buffer for each member to store their restart logs.

The commander stores a maximum of 10 restart logs for each member. The most recent restart log overwrites the oldest one when there are more than 10 restart logs for a member.

You can also use the display smartmc tc tc-id log buffer module SYSLOG mnemonic SYSLOG_RESTART command to display the restart log information.

Examples

# Display the restart log information for member 1.

<Sysname> display smartmc tc 1 log restart

Time     : 2017-07-15 13:51:46

Level    : Informational

Module   : SYSLOG

Mnemonic : SYSLOG_RESTART

Content  : System restarted -- HPE Comware Software.

        - Command output

| Field | Description |
| --- | --- |
| Time | Time when the log was generated. |
| Level | Log level. |

Related commands

display smartmc tc log buffer

    - display smartmc upgrade status

Use display smartmc upgrade status to display member upgrade status.

Syntax

display smartmc upgrade status

Views

Any view

Predefined user roles

network-admin

Examples

# Display member upgrade status.

<Sysname> display smartmc upgrade status

ID    IpAddress         MacAddress      Status      UpdateTime        UpdateFile

1     192.168.56.1      82dd-a434-0200  Finished    Immediately       bootloader.ipe

2     192.168.56.103    5caf-2e5f-0100  Finished    Immediately       bootloader.ipe

        - Command output

| Field | Description |
| --- | --- |
| ID | ID of the member. |
| MacAddress | MAC address of the member. |
| IpAddress | IP address of the member. |
| Status | Upgrade status: - Waiting—The member is waiting for downloading the upgrade file. - Download-failed—The member failed to download the upgrade file. - Download-finished—The member has downloaded the upgrade file. - Downloading—The member is downloading the upgrade file. - Updating—The member is upgrading. - Finished—The member has finished upgrading. - Failed—The member failed to upgrade. - Unknown—The upgrade status of the member is unknown. |
| Updated File | Name of the upgrade file. |
| UpdateTime | Upgrade time: - Immediately—Upgrade at once. - Delay(m)—Upgrade after the specified delay. - Time(HH:MM)—Upgrade at the specified time. |

Related commands

smartmc upgrade group

smartmc upgrade tc

    - display smartmc vlan

Use display smartmc vlan to display VLAN creation results for members.

Syntax

display smartmc vlan

Views

Any view

Predefined user roles

network-admin

Examples

# Display VLAN creation results.

<Sysname> display smartmc vlan

ID     IpAddress         MacAddress        Vlan      Status

1      192.168.22.222    703d-15ad-cd02    2         Success

2      192.168.22.3      24ff-2264-0100    2         Success

3      192.168.22.4      24ff-2f74-0200    2         Success

4      192.168.22.223    487a-dac8-29ba    2         Success

        - Command output

| Field | Description |
| --- | --- |
| ID | Member ID. |
| IpAddress | IP address of the member. |
| MacAddress | MAC address of the member. |
| Vlan | VLAN created for the member. |
| Status | VLAN creation status: - Processing—The VLAN is being created. - Success—The VLAN has been created successfully. - Failure. The port xxx is not an access port—The VLAN fails to be created, because ports connected to non-SmartMC devices are not access ports. - Failure. xxx not exist—The VLAN fails to be created, because all access ports are connected to SmartMC devices. |

Related commands

smartmc vlan

    - match

Use match to set a match criterion to add all matching members to a SmartMC group.

Use undo match to delete a match criterion.

Syntax

match { device-type device-type | ip-address ip-address { ip-mask-length | ip-mask } | mac-address mac-address mac-mask-length }

undo match { device-type device-type | ip-address ip-address { ip-mask-length | ip-mask } | mac-address mac-address mac-mask-length }

Default

No match criterion is set.

Views

SmartMC group view

Predefined user roles

network-admin

Parameters

device-type device-type: Sets a device type match criterion.

ip-address ip-address { ip-mask-length | ip-mask }: Sets an IP address match criterion. The ip-address argument specifies an IP address in dotted decimal notation. The ip-mask argument specifies the subnet mask in dotted decimal notation. The ip-mask-length argument specifies the subnet mask length in the range of 1 to 32.

mac-address mac-address mac-mask-length: Sets a MAC address match criterion. The mac-address argument specifies a MAC address in the format of H-H-H. The mac-mask-length argument specifies the mask length in the range of 1 to 48.

Examples

# Create a SmartMC group named a and add members in subnet 192.168.1.0/24 to the group.

<Sysname> system-view

[Sysname] smartmc group a

[Sysname-smartmc-group-a] match ip-address 192.168.1.0 24

Related commands

smartmc group

display smartmc group

    - smartmc auto-link-aggregation enable

Use smartmc auto-link-aggregation enable to enable automatic Ethernet link aggregation.

Use undo smartmc auto-link-aggregation enable to disable automatic Ethernet link aggregation.

Syntax

smartmc auto-link-aggregation enable

undo smartmc auto-link-aggregation enable

Default

Automatic Ethernet link aggregation is disabled.

Views

System view

Predefined user roles

network-admin

Usage guidelines

Automatic Ethernet link aggregation is performed only between member devices.

Enabling or disabling automatic Ethernet link aggregation might cause network flapping, and the members might go offline for a short period of time.

Examples

# Enable automatic Ethernet link aggregation.

<Sysname> system-view

[Sysname] smartmc auto-link-aggregation enable

    - smartmc auto-replace enable

Use smartmc auto-replace enable to enable the automatic faulty member replacement feature.

Use undo smartmc auto-replace enable to disable the automatic faulty member replacement feature.

Syntax

smartmc auto-replace enable

undo smartmc auto-replace enable

Default

The automatic faulty member replacement feature is disabled.

Views

System view

Predefined user roles

network-admin

Usage guidelines

To perform an automatic fault replacement, first enable this feature on the commander, and then perform the following tasks:

            - Install the new member at the location where the faulty member was installed.

            - Connect all cables to the new member.

Make sure the new member and the faulty member have the same neighbor relationship, device model, and IRF member ID.

Examples

# Enable the automatic faulty member replacement feature.

<Sysname> system-view

[Sysname] smartmc auto-replace enable

Related commands

smartmc replace

    - smartmc backup configuration

Use smartmc backup configuration to manually back up the configuration file on members.

Syntax

smartmc backup configuration { group group-name-list | tc [ tc-id-list ] }

Views

System view

Predefined user roles

network-admin

Parameters

group group-name-list: Specifies a space-separated list of up to 10 SmartMC groups. The group name is a case-sensitive string of 1 to 31 characters.

tc tc-id-list: Specifies a space-separated list of up to 10 member items. Each item specifies a device or a range of devices in the form of tc-id1 to tc-id2. The value for tc-id2 must be greater than or equal to the value for tc-id1. The value range for the tc-id argument is 0 to 255, with 0 representing the commander and 1 to 255 representing members. If you do not specify the commander or any members, all devices will perform configuration backup.

Usage guidelines

After you execute this command, the members immediately save the running configuration to the next-startup configuration files and upload the configuration files to the file server.

The backup configuration files are named in the format of bridge MAC address_backup.cfg.

Examples

# Back up the configuration file on member 1, member 2, member 3, and member 4.

<Sysname> system-view

[Sysname] smartmc backup configuration tc 1 to 4

# Back up the configuration file on all members in SmartMC groups test1, test2, and test3.

<Sysname> system-view

[Sysname] smartmc backup configuration group test1 test2 test3

Related commands

display smartmc configuration

smartmc backup configuration interval

    - smartmc backup configuration max-number

Use smartmc backup configuration max-number to set the maximum number of members that can perform automatic configuration backup at the same time.

Use undo smartmc backup configuration max-number to restore the default.

Syntax

smartmc backup configuration max-number max-number

undo smartmc backup configuration max-number

Default

A maximum of five members can perform automatic configuration backup at the same time.

Views

System view

Predefined user roles

network-admin

Parameters

max-number: Specifies the maximum number of members that can perform automatic configuration backup at the same time, in the range of 2 to 20.

Usage guidelines

The maximum number of members that can perform automatic configuration at the same time is limited by the performance of the file server. If automatic configuration backup fails, set the maximum number of members to a smaller value.

Examples

# Specify that a maximum of 10 members can perform automatic configuration backup at the same time.

<Sysname> system-view

[Sysname] smartmc backup configuration max-number 10

Related commands

display smartmc configuration

smartmc backup configuration

smartmc backup configuration interval

    - smartmc backup configuration interval

Use smartmc backup configuration interval to enable the automatic configuration file backup feature and set the automatic backup interval.

Use undo smartmc backup configuration interval to restore the default.

Syntax

smartmc backup configuration interval interval

undo smartmc backup configuration interval

Default

The automatic configuration file backup feature is disabled.

Views

System view

Predefined user roles

network-admin

Parameters

interval: Specifies the automatic configuration file backup interval in the range of 1 to 720 hours.

Usage guidelines

This command enables the commander and members to back up their configuration files by saving the running configuration to the files and then uploading them to the file server. When you execute this command, the commander and members immediately perform a backup. After that, they back up the configuration files at the specified interval. The backup configuration files are named in the format of bridge MAC address_backup.cfg.

Examples

# Enable the automatic configuration file backup feature and set the backup interval to 24 hours.

<Sysname> system-view

[Sysname] smartmc backup configuration interval 24

Related commands

display smartmc configuration

smartmc backup configuration

    - smartmc batch-file apply

Use smartmc batch-file apply to specify a batch file to deploy to ports connecting APs or IP phones.

Use undo smartmc batch-file apply to remove a batch file specified for ports connecting APs or IP phones.

Syntax

smartmc batch-file batch-file-name apply { ap | phone }

undo smartmc batch-file apply { ap | phone }

Default

No batch file is specified for ports connecting APs or IP phones.

Views

System view

Predefined user roles

network-admin

Parameters

batch-file-name: Specifies a batch file by its name, a case-insensitive string of 1 to 255 characters.

ap: Specifies ports connecting APs.

phone: Specifies ports connecting IP phones.

Usage guidelines

With batch file deployment enabled, the commander automatically deploys configurations in the specified batch file to a port connecting an AP or IP phone, simplifying access port configuration.

When the commander first detects the association of an AP or IP phone on a port through LLDP, it deploys the command lines in the specified batch file to the port. If no batch file is specified for the device type, the configurations on the port remain unchanged.

If the AP or IP phone disconnects from the port, the configurations on the port remain. When a new device comes online from the port, configurations used by the port depend on the new device type.

- If the new device is an AP or IP phone and has the same type as the disconnected device, the configurations on the port remain unchanged.

- If the new device is an AP or IP phone but has a different type as the disconnected device, the commander deploys the command lines in the specified batch file to the port. If no batch file is specified for the device type, the configurations on the port remain unchanged.

- If the new device is neither an AP nor an IP phone, the configurations on the port remain unchanged.

To disable the commander from deploying a batch file to ports, remove the specified batch file or execute the undo smartmc batch-file-apply enable command to disable batch file deployment.

Examples

# Specify batch file ap.cmdset for ports connecting APs or IP phones.

<Sysname> system-view

[Sysname] smartmc batch-file ap.cmdset apply ap

Related commands

create batch-file

smartmc batch-file-apply enable

    - smartmc batch-file deploy

Use smartmc batch-file deploy to deploy bulk command lines to a list of members or SmartMC groups.

Syntax

smartmc batch-file batch-file-name deploy { group group-name-list | tc tc-id-list }

Views

System view

Predefined user roles

network-admin

Parameters

batch-file-name: Specifies the name of a batch file, a case-insensitive string of 1 to 255 characters.

group group-name-list: Specifies a space-separated list of up to 10 SmartMC groups. The group name is a case-sensitive string of 1 to 31 characters.

tc tc-id-list: Specifies a space-separated list of up to 10 member items. Each item specifies a member or a range of members in the form of tc-id1 to tc-id2. The value for tc-id2 must be greater than or equal to the value for tc-id1. The value range for the tc-id argument is 1 to 255.

Examples

# Deploy batch file startup.cmdset to SmartMC group testgroup.

<Sysname> system-view

[Sysname] smartmc batch-file startup.cmdset deploy group testgroup

Related commands

create batch-file

display smartmc batch-file status

    - smartmc batch-file-apply enable

Use smartmc batch-file-apply enable to enable batch file deployment.

Use undo smartmc batch-file-apply enable to disable batch file deployment.

Syntax

smartmc batch-file-apply enable

undo smartmc batch-file-apply enable

Default

Batch file deployment is enabled.

Views

System view

Predefined user roles

network-admin

Usage guidelines

With batch file deployment enabled, the commander automatically deploys configurations in the specified batch file to a port connecting an AP or IP phone, simplifying access port configuration. To disable the commander from deploying a batch file to ports, remove the specified batch file or disable batch file deployment.

Examples

# Disable batch file deployment.

<Sysname> system-view

[Sysname] undo smartmc batch-file-apply enable

Related commands

smartmc batch-file apply

    - smartmc enable

Use smartmc enable to enable SmartMC and set the device role.

Use undo smartmc enable to disable SmartMC.

Syntax

smartmc { tc | tm username username password { cipher | simple } string } enable

undo smartmc enable

Default

SmartMC is disabled.

Views

System view

Predefined user roles

network-admin

Parameters

tc: Enables SmartMC and sets the device role to member.

tm: Enables SmartMC and sets the device role to commander.

username username: Specifies a username for the local user, a case-sensitive string of 1 to 55 characters.

password: Specifies a password for the local user.

cipher: Specifies a password in encrypted form.

simple: Specifies a password in plaintext form.

string: Specifies the password. Its plaintext form is a case-sensitive string of 1 to 63 characters. Its encrypted form is a case-sensitive string of 1 to 117 characters.

Usage guidelines

A SmartMC network must have one and only one commander.

To enable SmartMC, execute this command on both the commander and members. To configure the other SmartMC features, execute associated commands only on the commander.

If you change the role of the commander to member or disable SmartMC on the commander, all SmartMC settings in its running configuration will be cleared.

SmartMC fails to be enabled if ACL resources are insufficient. If ACL resources are insufficient, use the undo acl command to delete unnecessary ACLs and then enable SmartMC. You can execute the display acl command to view ACL configuration and match statistics. For more information about ACLs, see ACL and QoS Configuration Guide.

SmartMC fails to be enabled if ports 80 and 443 have been used.

If you execute this command multiple times, the most recent configuration takes effect. You can execute the command to change the device role or the password.

Examples

# Enable SmartMC and set the device role to member.

<Sysname> system-view

[Sysname] smartmc tc enable

    - smartmc { ftp-server| sftp-server }

Use smartmc { ftp-server | sftp-server } to configure the file server information.

Use undo smartmc { ftp-server | sftp-server } to delete the file server information.

Syntax

smartmc { ftp-server | sftp-server } { ipv4-address | ipv6 ipv6-address } [ port port ] [ vpn-instance vpn-instance-name ] [ directory directory ] username username password { cipher | simple } string

undo smartmc { ftp-server | sftp-server }

Default

No file server information is configured.

Views

System view

Predefined user roles

network-admin

Parameters

ftp-server: Specifies an FTP server.

sftp-server: Specifies an SFTP server.

ipv4-address: Specifies the IPv4 address of the file server.

ipv6 ipv6-address: Specifies the IPv6 address of the file server.

port port: Specifies the port number of the file server, in the range of 1 to 65535. The default pot for an FTP server and an SFTP server is 21 and 22, respectively.

vpn-instance vpn-instance-name: Specifies the name of the MPLS L3VPN instance to which the file server belongs, a case-sensitive string of 1 to 31 characters. If you do not specify this option, the command considers that the file server is in the public network.

directory directory: Specifies the working directory of the file server, a case-insensitive string. By default, the root directory is used.

username username: Specifies the file server username, a case-sensitive string of 1 to 55 characters.

password: Specifies the file server password.

cipher: Specifies a password in encrypted form.

simple: Specifies a password in plaintext form.

string: Specifies the password. Its plaintext form is a case-sensitive string of 1 to 63 characters. Its encrypted form is a case-sensitive string of 1 to 117 characters.

Usage guidelines

You can specify only one file server. If you execute this command multiple times, the most recent configuration takes effect.

Examples

# Set the file server type to FTP, and specify the server IP address, username, and password as 192.168.22.19, admin, and hello12345, respectively.

<Sysname> system-view

[Sysname] smartmc ftp-server 192.168.22.19 username admin password simple hello12345

Related commands

display smartmc configuration

    - smartmc group

Use smartmc group to create a SmartMC group and enter its view, or enter the view of an existing SmartMC group.

Use undo smartmc group to delete a SmartMC group.

Syntax

smartmc group group-name

undo smartmc group group-name

Default

No SmartMC groups exist.

Views

System view

Predefined user roles

network-admin

Parameters

group-name: Specifies the name of the SmartMC group, a case-sensitive string of 1 to 31 characters.

Usage guidelines

When you perform the following operations, you can specify a SmartMC group to apply the operations to all members in the group:

- Startup software upgrade.

- Configuration file upgrade.

- Configuration deployment.

Examples

# Create SmartMC group testgroup.

<Sysname> system-view

[Sysname] smartmc group testgroup

[Sysname-smartmc-group-testgroup]

Related commands

match

    - smartmc outbound

Use smartmc outbound to configure an outgoing interface for the SmartMC network.

Use undo smartmc outbound to restore the default.

Syntax

smartmc outbound

undo smartmc outbound

Default

No interface is used as an outgoing interface, and the SmartMC network cannot communicate with outside networks.

Views

VLAN interface view

Predefined user roles

network-admin

Usage guidelines

VLAN interface 1 cannot be used as an outgoing interface, because the SmartMC network is established in VLAN 1.

Examples

# Configure GigabitEthernet 1/0/1 as an outgoing interface for the SmartMC network.

<Sysname> system-view

[Sysname] interface gigabitethernet 1/0/1

[Sysname-GigabitEthernet1/0/1] smartmc outbound

    - smartmc resource-monitor

Use smartmc resource-monitor to enable resource monitoring.

Use undo smartmc resource-monitor to disable resource monitoring.

Syntax

smartmc resource-monitor [ cpu | memory | packet-drop | temperature ] * [ group group-name-list | tc { tc-id-list | mac-address mac-address } | tm ]

undo smartmc resource-monitor [ cpu | memory | packet-drop | temperature ] * [ group group-name-list | tc { tc-id-list | mac-address mac-address } | tm ]

Default

Resource monitoring is disabled.

Views

System view

Predefined user roles

network-admin

Parameters

cpu: Enables CPU usage monitoring.

memory: Enables memory usage monitoring.

packet-drop: Enables packet dropping monitoring.

temperature: Enables temperature monitoring.

group group-name-list: Specifies the SmartMC groups to monitor. You can specify a space-separated list of up to 10 SmartMC groups. The group name is a case-sensitive string of 1 to 31 characters.

tc: Specifies the members to monitor.

tc-id-list: Specifies a space-separated list of up to 10 member items. Each item specifies a member or a range of members in the form of tc-id1 to tc-id2. The value for tc-id2 must be greater than or equal to the value for tc-id1. The value range for the tc-id argument is 1 to 255.

mac-address mac-address: Specifies a member by its MAC address in the format of H-H-H.

tm: Enables resource monitoring on the commander.

Usage guidelines

Packet dropping monitoring monitors packet dropping on members and on interfaces.

If you do not specify a resource type, this command enables resource monitoring for all resource types.

If you do not specify a device to monitor (member or the commander), this command enables resource monitoring on the commander and all members.

Examples

# Enable resource monitoring for all resource types on member 1 through member 3.

<Sysname> system-view

[Sysname] smartmc resource-monitor tc 1 to 3

Related commands

display smartmc resource-monitor

smartmc resource-monitor interval

smartmc resource-monitor max-age

    - smartmc resource-monitor interval

Use smartmc resource-monitor interval to set the interval for the commander to obtain resource monitoring information.

Use undo smartmc resource-monitor interval to restore the default.

Syntax

smartmc resource-monitor interval interval

undo smartmc resource-monitor interval

Default

The interval for the commander to obtain resource monitoring information is 1 minute.

Views

System view

Predefined user roles

network-admin

Parameters

interval: Specifies the interval for the commander to obtain resource monitoring information, in the range of 1 to 60 minutes.

Usage guidelines

For packet dropping monitoring, the specified interval applies only to obtaining of member packet dropping information. Because of the great amount of interface information, the commander obtains interface packet dropping information from members only when Web displaying is requested.

Examples

# Set the interval for the commander to obtain resource monitoring information to 5 minutes.

<Sysname> system-view

[Sysname] smartmc resource-monitor interval 5

Related commands

display smartmc resource-monitor

smartmc resource-monitor

    - smartmc resource-monitor max-age

Use smartmc resource-monitor max-age to set the aging time for resource monitoring information.

Use undo smartmc resource-monitor max-age to restore the default.

Syntax

smartmc resource-monitor max-age max-age

undo smartmc resource-monitor max-age

Default

The aging time for resource monitoring information is 24 hours.

Views

System view

Predefined user roles

network-admin

Parameters

max-age: Specifies the aging time for resource monitoring information, in the range of 1 to 168 hours.

Usage guidelines

For packet dropping monitoring, the specified aging time applies only to member packet dropping information. Each member saves its interface packet dropping information for as long as 30 days.

To view interface packet dropping information, log in to the Web interface of the commander and access the SmartMC > Intelligent O&M > Resource monitoring page. You can view information in the past 1 hour, 1 day, or 30 days.

Examples

# Set the aging time for resource monitoring information to 1 hour.

<Sysname> system-view

[Sysname] smartmc resource-monitor max-age 1

Related commands

display smartmc resource-monitor

smartmc resource-monitor

    - smartmc replace

Use smartmc replace to manually replace a faulty member.

Syntax

smartmc replace tc tc-id1 faulty-tc tc-id2

Views

System view

Predefined user roles

network-admin

Parameters

tc tc-id1: Specifies the ID of the new member, in the range of 1 to 255.

faulty-tc tc-id2: Specifies the ID of the faulty member, in the range of 1 to 255.

Usage guidelines

Before you execute this command, perform the following tasks:

            - Install the new member at the location where the faulty member was installed.

            - Connect all cables to the new member.

Make sure the new member and the faulty member have the same neighbor relationship, device model, and IRF member ID.

Examples

# Replace faulty member 5 with new member 10.

<Sysname> system-view

[Sysname] smartmc replace tc 10 faulty-tc 5

Related commands

display smartmc replace status

smartmc auto-replace enable

    - smartmc tc boot-loader

Use smartmc tc boot-loader to specify the upgrade startup software files for a member.

Use undo smartmc tc boot-loader to remove the configuration.

Syntax

smartmc tc tc-id boot-loader { ipe-filename | boot boot-filename system system-filename }

undo smartmc tc tc-id boot-loader

Views

System view

Predefined user roles

network-admin

Parameters

tc tc-id: Specifies a member by its ID in the range of 1 to 255.

ipe-filename: Specifies an IPE software file by its name, a case-insensitive string of 5 to 45 characters. The file name must include the .ipe extension.

boot boot-filename: Specifies a boot image file by its name, a case-insensitive string of 5 to 45 characters. The file name must include the .bin extension.

system system-filename: Specifies a system image file by its name, a case-insensitive string of 5 to 45 characters. The file name must include the .bin extension.

Examples

# Specify upgrade boot image boot.bin and upgrade system image system.bin for member 1.

<Sysname> system-view

[Sysname] smartmc tc 1 boot-loader boot boot.bin system system.bin

Related commands

display smartmc tc

    - smartmc tc device-type

Use smartmc tc device-type to define a member type on the commander.

Use undo smartmc tc device-type to delete a member type.

Syntax

smartmc tc sysoid sysoid device-type device-type

undo smartmc tc sysoid sysoid device-type device-type

Views

System view

Predefined user roles

network-admin

Parameters

sysoid sysoid: Specifies the SYSOID of a member.

device-type device-type: Specifies a member type.

Usage guidelines

A device type can correspond to multiple device models. You can identify different device models with different SYSOIDs by specifying a SYSOID for each device model. The commander identifies member types by SYSOID.

The system predefines the device types for some device models based on SYSOIDs. For device models without predefined device types, you must define their member types by SYSOID manually. If you do not do so, the commander cannot identify the types of such devices.

You cannot modify the predefined device types.

Before defining a device type for a member, you can use the display smartmc tc command to determine whether the member has a predefined one.

- If the member has been predefined with one device type, the DeviceType field displays the actual predefined device type.

- If the member does not have a predefined device type, the DeviceType field displays unknown.

To obtain the SYSOID of a member, use the display smartmc tc verbose command.

Examples

# Define a member type by specifying the SYSOID as 1.3.6.1.4.1.25506.1.1588 and the member type as SW.

<Sysname> system-view

[Sysname] smartmc tc sysoid 1.3.6.1.4.1.25506.1.1588 device-type SW

    - smartmc tc password

Use smartmc tc password to modify the password for the default user (admin) on members.

Use undo smartmc tc password to restore the default.

Syntax

smartmc tc password [ cipher ] string

undo smartmc tc password

Default

The password for the default user on members is admin.

Views

System view

Predefined user roles

network-admin

Parameters

cipher: Specifies a password in encrypted form. If you do not specify this keyword, the command creates a password in plaintext form. For security purposes, the password specified in plaintext form will be stored in encrypted form.

string: Specifies the password. Its plaintext form is a case-sensitive string of 1 to 63 characters. Its encrypted form is a case-sensitive string of 33 to 117 characters.

Usage guidelines

During SmartMC network establishment, the commander establishes NETCONF sessions to members and adds them to the network. The default username and password on the members for NETCONF session establishment are admin and admin. To enhance security, you can perform this task to change the password for the default user admin of the members after the commander adds the members to the network.

If the default password cannot meet the password complexity requirements on members, you cannot execute the undo smartmc tc password command to restore the default.

Do not modify the password for members that are manually added to the SmartMC network. If you modify the password for a manually added member, you will not be able to manage that member from the commander.

Examples

# Configure default user admin on members to use plaintext password hello12345.

<Sysname> system-view

[Sysname] smartmc tc password hello12345

    - smartmc tc startup-configuration

Use smartmc tc startup-configuration to specify the upgrade configuration file for a member.

Use undo smartmc tc startup-configuration to remove the configuration.

Syntax

smartmc tc tc-id startup-configuration cfg-filename

undo smartmc tc tc-id startup-configuration

Views

System view

Predefined user roles

network-admin

Parameters

tc tc-id: Specifies a member by its ID in the range of 1 to 255.

cfg-filename: Specifies a configuration file by its name, a case-insensitive string of 5 to 45 characters. The file name must include the .cfg extension.

Examples

# Specify upgrade configuration file startup.cfg for member 1.

<Sysname> system-view

[Sysname] smartmc tc 1 startup-configuration startup.cfg

Related commands

display smartmc tc

    - smartmc topology-refresh

Use smartmc topology-refresh to manually refresh the network topology.

Syntax

smartmc topology-refresh

Views

Any view

Predefined user roles

network-admin

Usage guidelines

To display topology changes, use this command to manually refresh the topology.

Examples

# Manually refresh the network topology.

<Sysname> smartmc topology-refresh

Related commands

display smartmc device-link

    - smartmc topology-refresh interval

Use smartmc topology-refresh interval to set the automatic network topology refresh interval.

Use undo smartmc topology-refresh interval to restore the default.

Syntax

smartmc topology-refresh interval interval

undo smartmc topology-refresh interval

Default

The automatic network topology refresh interval is 60 seconds.

Views

System view

Predefined user roles

network-admin

Parameters

interval: Specifies the automatic network topology refresh interval in the range of 60 to 300 seconds.

Examples

# Set the automatic network topology refresh interval to 100 seconds.

<Sysname> system-view

[Sysname] smartmc topology-refresh interval 100

Related commands

display smartmc device-link

    - smartmc topology-save

Use smartmc topology-save to save the current network topology.

Syntax

smartmc topology-save

Views

System view

Predefined user roles

network-admin

Usage guidelines

This task allows you to save the current network topology to the topology.dba file in the flash memory. After the commander reboots, it uses the topology.dba file to restore the network topology.

Examples

# Save the current network topology

<Sysname> system-view

[Sysname] smartmc topology-save

Related commands

display smartmc device-link

    - smartmc upgrade boot-loader

Use smartmc upgrade boot-loader to upgrade the startup software on a list of members or SmartMC groups.

Use undo smartmc upgrade delete the startup software upgrade task.

Syntax

smartmc upgrade boot-loader { group | tc } list [ delay minutes | time time ]

smartmc upgrade boot-loader { group | tc } { list { boot boot-filename system system-filename | file ipe-filename } }&<1-40> [ delay delay-time | time time ]

undo smartmc upgrade

Views

System view

Predefined user roles

network-admin

Parameters

group: Specifies the SmartMC groups to be upgraded.

tc: Specifies the members to be upgraded.

list: Specifies a space-separated list of up to 10 member items or SmartMC group items.

- SmartMC group—Each item specifies a SmartMC group name, a case-sensitive string of 1 to 31 characters.

- Member—Each item specifies a member ID or a range of member IDs in the form of tc-id1 to tc-id2. The value for tc-id2 must be greater than or equal to the value for tc-id1. The value range for the tc-id argument is 1 to 255.

boot boot-filename: Specifies a boot image by its name.

system system-filename: Specifies a system image by its name.

file ipe-filename: Specifies an IPE file by its name, a case-insensitive string of 5 to 45 characters.

delay delay-time: Specifies the upgrade delay time in the range of 1 to 1440 minutes.

time in-time: Specifies the upgrade time in the format of hh:mm. The value range for the hh argument is 0 to 23 hours. The value range for the mm argument is 0 to 59 minutes.

Usage guidelines

|  | CAUTION: Upgrading the startup software might interrupt services. Before upgrading the startup software, make sure no services will be interrupted. |
| --- | --- |

To use this command to upgrade the startup software on members without specifying the upgrade files, you must first perform one of the following tasks:

- Execute the smartmc tc boot-loader command to specify the upgrade files for members.

- Execute the boot-loader command to specify the upgrade files for a SmartMC group.

A member can perform only one upgrade task at a time.

If you execute this command without specifying the delay time or update time, the members or SmartMC group immediately upgrades the startup software and the upgrade operation cannot be cancelled. If you specify a delay time or upgrade time to perform a scheduled upgrade, the upgrade operation can be cancelled by using the undo smartmc upgrade command before it starts.

Examples

# Upgrade startup software images boot.bin and sys.bin on all members in SmartMC groups test1 and test2.

<Sysname> system-view

[Sysname] smartmc upgrade boot-loader group test1 test2 boot boot.bin system sys.bin

Related commands

boot-loader

startup-configuration

    - smartmc upgrade startup-configuration

Use smartmc upgrade startup-configuration to upgrade the configuration file on a list of members or on all members in SmartMC groups.

Use undo smartmc upgrade delete the configuration file upgrade task.

Syntax

smartmc upgrade startup-configuration { group | tc } list [ delay minutes | time time ]

smartmc upgrade startup-configuration group { list file cfg-filename }&<1-40> [ delay delay-time | time time ]

smartmc upgrade startup-configuration tc { list cfg-filename }&<1-40> [ delay delay-time | time time ]

undo smartmc upgrade

Views

System view

Predefined user roles

network-admin

Parameters

group: Specifies the SmartMC groups to be upgraded.

tc: Specifies the members to be upgraded.

list: Specifies a space-separated list of up to 10 member items or SmartMC group items.

- SmartMC group—Each item specifies a SmartMC group name, a case-sensitive string of 1 to 31 characters.

- Member—Each item specifies a member ID or a range of member IDs in the form of tc-id1 to tc-id2. The value for tc-id2 must be greater than or equal to the value for tc-id1. The value range for the tc-id argument is 1 to 255.

file cfg-filename: Specifies a configuration file by its name.

delay delay-time: Specifies the upgrade delay time in the range of 1 to 1440 minutes.

time in-time: Specifies the upgrade time in the format of hh:mm. The value range for the hh argument is 0 to 23 hours. The value range for the mm argument is 0 to 59 minutes.

Usage guidelines

|  | CAUTION: After you update the configuration file, the configuration in the new configuration file will become the running configuration. Before upgrading the configuration file, make sure the contents of the new configuration file are correct. |
| --- | --- |

To use this command to upgrade the configuration file on members without specifying the upgrade file, you must first perform one of the following tasks:

- Execute the smartmc tc startup-configuration command to specify the upgrade file for members.

- Execute the startup-configuration command to specify the upgrade file for a SmartMC group.

A member can perform only one upgrade task at a time.

If you execute this command without specifying the delay time or update time, the members or SmartMC group immediately upgrades the configuration file and the upgrade operation cannot be cancelled. If you specify a delay time or upgrade time to perform a scheduled upgrade, the upgrade operation can be cancelled by using the undo smartmc upgrade command before it starts.

Examples

# Upgrade configuration file startup.cfg on all members in SmartMC groups test1 and test2.

<Sysname> system-view

[Sysname] smartmc upgrade boot-loader group test1 test2 file startup.cfg

Related commands

boot-loader

startup-configuration

    - smartmc vlan

Use smartmc vlan to create a VLAN for members.

Syntax

smartmc vlan vlan-id { group group-name-list | tc tc-id-list }

Views

System view

Predefined user roles

network-admin

Parameters

vlan-id: Specifies the VLAN ID in the range of 1 to 4094.

group group-name-list: Specifies the SmartMC groups for which the VLAN is created. You can specify a space-separated list of up to 10 SmartMC groups. The group name is a case-sensitive string of 1 to 31 characters.

tc tc-id-list: Specifies the members for which the VLAN is created. You can specify a space-separated list of up to 10 member items. Each item specifies a member or a range of members in the form of tc-id1 to tc-id2. The value for tc-id2 must be greater than or equal to the value for tc-id1. The value range for the tc-id argument is 1 to 255.

Usage guidelines

Execute this command when the network topology is stable. As a best practice, use the smartmc topology-refresh command to refresh the network topology before executing this command.

After you execute this command, all access ports on members except the following access ports are assigned to the VLAN:

- Access ports connecting to the commander.

- Access ports connecting to other members.

- Access ports connecting to offline devices. Remove offline devices before configuring this command.

If the VLAN is successfully created but some access ports of a member cannot be assigned to the VLAN, the VLAN memberships of the member is restored to the state before the VLAN is created.

The failure to assign an access port of a member to the created VLAN does not affect the VLAN assignment for other members.

After command execution, you can use the display smartmc vlan command to examine the VLAN creation result.

Examples

# Create a VLAN for member 1 and member 2.

<Sysname> system-view

[Sysname] smartmc vlan 2 tc 1 to 2

As a best practice, execute the display smartmc vlan command to verify that the VLAN has been created successfully.

    - startup-configuration

Use startup-configuration to specify an upgrade configuration file for a SmartMC group .

Use undo startup-configuration to restore the default.

Syntax

startup-configuration cfgfile

undo startup-configuration

Default

No upgrade configuration file is specified for the SmartMC group.

Views

SmartMC group view

Predefined user roles

network-admin

Parameters

cfgfile: Specifies a configuration file by its name, a string of 5 to 45 characters. The file name must include the .cfg extension.

Usage guidelines

If you execute this command multiple times, the most recent configuration takes effect.

Examples

# Specify configuration file startup.cfg for SmartMC group testgroup.

<Sysname> system-view

[Sysname] smartmc group testgroup

[Sysname-smartmc-group-testgroup] startup-configuration startup.cfg

New feature: Configuring interface alarm functions

Configuring interface alarm functions

About this task

With the interface alarm functions enabled, when the number of error packets on an interface in normal state within the specified interval exceeds the upper threshold, the interface generates an upper threshold exceeding alarm and enters the alarm state. When the number of error packets on an interface in the alarm state within the specified interval drops below the lower threshold, the interface generates a recovery alarm and restores to the normal state.

Restrictions and guidelines

You can configure the error packet alarm parameters in system view and interface view.

- The configuration in system view takes effect on all interfaces of the specified slot. The configuration in interface view takes effect only on the current interface.

- For an interface, the configuration in interface view takes priority, and the configuration in system view is used only when no configuration is made in interface view.

An interface that is shut down because of error packet alarms cannot automatically recover. To bring up the interface, execute the undo shutdown command on the interface.

To ensure that error packet statistics are accurate, make sure the value for the interval interval option is greater than 7.

Enabling interface alarm functions

            - Enter system view.

system-view

            - Enable alarm functions for the interface monitoring module.

snmp-agent trap enable ifmonitor [ crc-error ]

By default, all alarm functions are enabled for interfaces.

Configuring CRC error packet parameters

            - Enter system view.

system-view

            - Configure global CRC error packet alarm parameters.

ifmonitor crc-error slot slot-number high-threshold high-value low-threshold low-value interval interval [ shutdown ]

By default, the upper threshold is 1000, the lower threshold is 100, and the statistics collection and comparison interval is 10 seconds for CRC error packets.

            - Enter Ethernet interface view.

interface interface-type interface-number

            - Configure CRC error packet alarm parameters for the interface.

port ifmonitor crc-error high-threshold high-value low-threshold low-value interval interval [ shutdown ]

By default, an interface uses the global CRC error packet alarm parameters.

Command reference

    - ifmonitor crc-error

Use ifmonitor crc-error to configure global CRC error packet alarm parameters.

Use undo ifmonitor crc-error to restore the default.

Syntax

ifmonitor crc-error slot slot-number high-threshold high-value low-threshold low-value interval interval [ shutdown ]

undo ifmonitor crc-error slot slot-number

Default

The upper threshold is 1000, the lower threshold is 100, and the statistics collection and comparison interval is 10 seconds for CRC error packet alarms.

Views

System view

Predefined user roles

network-admin

Parameters

high-threshold high-value: Specifies the upper threshold for CRC error packet alarms, in the range of 1 to 4294967295 packets.

low-threshold low-value: Specifies the lower threshold for CRC error packet alarms, in the range of 1 to 4294967295 packets.

interval interval: Specifies the statistics collection and comparison interval for CRC error packets, in the range of 1 to 65535 seconds.

shutdown: Shuts down an interface when the number of incoming CRC error packets on the interface exceeds the upper threshold. Then, the interface stops forwarding all packets. To recover the interface, execute the undo shutdown command on the interface. If you do not specify this keyword, an upper threshold exceeding alarm is generated and the interface enters the alarm state when the number of incoming CRC error packets exceeds the upper threshold on the interface.

slot slot-number: Specifies an IRF member device by its member ID.

Usage guidelines

With the CRC error packet alarm function enabled, when the number of incoming CRC error packets on an interface in normal state within the specified interval exceeds the upper threshold, the interface generates an upper threshold exceeding alarm and enters the alarm state. When the number of incoming CRC error packets on an interface in the alarm state within the specified interval drops below the lower threshold, the interface generates a recovery alarm and restores to the normal state.

You can configure the CRC error packet alarm parameters in system view and interface view.

- The configuration in system view takes effect on all interfaces of the specified slot. The configuration in interface view takes effect only on the current interface.

- For an interface, the configuration in interface view takes priority, and the configuration in system view is used only when no configuration is made in interface view.

When you execute this command multiple times, the most recent configuration takes effect.

Examples

# Set the upper threshold to 5000, lower threshold to 400, and statistics collection and comparison interval to 6 seconds for CRC error packet alarms.

<Sysname> system-view

[Sysname] ifmonitor crc-error slot 1 high-threshold 5000 low-threshold 400 interval 6

Related commands

snmp-agent trap enable ifmonitor

    - port ifmonitor crc-error

Use port ifmonitor crc-error to configure CRC error packet alarm parameters for an interface.

Use undo port ifmonitor crc-error to restore the default.

Syntax

port ifmonitor crc-error high-threshold high-value low-threshold low-value interval interval [ shutdown ]

undo port ifmonitor crc-error

Default

An interface uses the global CRC error packet alarm parameters.

Views

Ethernet interface view

Predefined user roles

network-admin

Parameters

high-threshold high-value: Specifies the upper threshold for CRC error packet alarms, in the range of 1 to 4294967295 packets.

low-threshold low-value: Specifies the lower threshold for CRC error packet alarms, in the range of 1 to 4294967295 packets.

interval interval: Specifies the statistics collection and comparison interval for CRC error packets, in the range of 1 to 65535 seconds.

shutdown: Shuts down an interface when the number of incoming CRC error packets on the interface exceeds the upper threshold. Then, the interface stops forwarding all packets. To recover the interface, execute the undo shutdown command on the interface. If you do not specify this keyword, an upper threshold exceeding alarm is generated and the interface enters the alarm state when the number of incoming CRC error packets exceeds the upper threshold on the interface.

Usage guidelines

With the CRC error packet alarm function enabled, when the number of incoming CRC error packets on an interface in normal state within the specified interval exceeds the upper threshold, the interface generates an upper threshold exceeding alarm and enters the alarm state. When the number of incoming CRC error packets on an interface in the alarm state within the specified interval drops below the lower threshold, the interface generates a recovery alarm and restores to the normal state.

You can configure the CRC error packet alarm parameters in system view and interface view.

- The configuration in system view takes effect on all interfaces of the specified slot. The configuration in interface view takes effect only on the current interface.

- For an interface, the configuration in interface view takes priority, and the configuration in system view is used only when no configuration is made in interface view.

When you execute this command multiple times, the most recent configuration takes effect.

Examples

# Set the upper threshold to 5000, lower threshold to 400, and statistics collection and comparison interval to 6 seconds for CRC error packet alarms on GigabitEthernet 1/0/1.

<Sysname> system-view

[Sysname] interface gigabitethernet 1/0/1

[Sysname-GigabitEthernet1/0/1] port ifmonitor crc-error high-threshold 5000 low-threshold 400 interval 6

Related commands

snmp-agent trap enable ifmonitor

    - snmp-agent trap enable ifmonitor

Use snmp-agent trap enable ifmonitor to enable interface alarm functions.

Use undo snmp-agent trap enable ifmonitor to disable interface alarm functions.

Syntax

snmp-agent trap enable ifmonitor [ crc-error ]

undo snmp-agent trap enable ifmonitor [ crc-error ]

Default

Interface alarm functions are enabled.

Views

System view

Predefined user roles

network-admin

Parameters

crc-error: Enables the CRC error packet alarm function for interfaces.

Examples

# Enable the CRC error packet alarm function for interfaces.

<Sysname> system-view

[Sysname] snmp-agent trap enable ifmonitor crc-error

New feature: Configuring Option 60 for DHCP requests

Configuring Option 60 for DHCP requests

About this task

Option 60 acts as a vendor class identifier (VCI). You can configure a DHCP client to send a request with Option 60 for the DHCP server to make class-based IP address assignment. When the DHCP server receives a request with Option 60 from a client, the server identifies the user class of the client. Then, the server assigns the client an IP address from the IP range specified for the user class.

By default, Option 60 contains the vendor name and the product name. To define this option for DHCP requests, perform this task.

Procedure

Enter system view.

system-view

Enter interface view.

interface interface-type interface-number

Configure Option 60 for DHCP requests.

dhcp client class-id { ascii ascii-string | hex hex-string }

By default, Option 60 contains the vendor name and the product name.

Command reference

dhcp client class-id

Use dhcp client class-id to configure Option 60.

Use undo dhcp client class-id to restore the default.

Syntax

dhcp client class-id { ascii ascii-string | hex hex-string }

undo dhcp client class-id

Default

Option 60 contains the vendor name and the product name.

Views

Interface view

Predefined user roles

network-admin

Parameters

ascii ascii-string: Specifies a case-sensitive ASCII string of 1 to 63 characters as the content in Option 60.

hex hex-string: Specifies a case-sensitive hexadecimal string of 4 to 64 characters as the value in Option 60.

Usage guidelines

Option 60 acts as a vendor class identifier (VCI). You can configure a DHCP client to send a request with Option 60 for the DHCP server to make class-based IP address assignment. When the DHCP server receives a request with Option 60 from a client, the server identifies the user class of the client. Then, the server assigns the client an IP address from the IP range specified for the user class.

By default, Option 60 contains the vendor name and the product name. To customize this option, use this command.

Examples

# Configure FFFFFFFF as the content of Option 60 on VLAN-interface 10.

<Sysname> system-view

[Sysname] interface vlan-interface 10

[Sysname-Vlan-interface10] dhcp client class-id hex FFFFFFFF

New feature: Configuring the type of port ID TLVs advertised by LLDP

- Configuring the type of port ID TLVs advertised by LLDP

      - About this task

An HPE device determines whether it has an MED neighbor based on received LLDPDUs. If the LLDPDUs contain LLDP-MED TLVs, the device determines that it has an MED neighbor. By default, the device advertises port ID TLVs that contain interface MAC addresses out of interfaces that have MED neighbors. If no MED neighbor exists on an interface, the device advertises port ID TLVs that contain interface names through the interface.

This task enables an HPE device to advertise only port ID TLVs that contain interface names. The media devices from some vendors can obtain interface information from HPE devices only through LLDP. For the media devices to obtain interface names, you must configure HPE devices to generate port ID TLVs based on interface names.

      - Restrictions and guidelines

Perform this task only when LLDP neighbors must obtain interface names from LLDPDUs. Do not perform this task in any other scenarios.

You can configure the port ID TLV type in system view or interface view. The interface-specific setting takes precedence over the global setting.

      - Configuring the type of port ID TLVs advertised by LLDP globally

            - Enter system view.

system-view

Configure the type of port ID TLVs advertised by LLDP.

lldp [ agent { nearest-customer | nearest-nontpmr } ] global tlv-config basic-tlv port-id type-id

By default, an interface advertises port ID TLVs that contain interface MAC addresses if it receives LLDP-MED TLVs and advertises port ID TLVs that contain interface names if no LLDP-MED TLVs are received.

      - Configuring the type of port ID TLVs advertised by LLDP on an interface

            - Enter system view.

system-view

            - Enter interface view.

interface interface-type interface-number

Configure the type of port ID TLVs advertised by LLDP.

lldp [ agent { nearest-customer | nearest-nontpmr } ] tlv-config basic-tlv port-id type-id

By default, an interface advertises port ID TLVs that contain interface MAC addresses if it receives LLDP-MED TLVs and advertises port ID TLVs that contain interface names if no LLDP-MED TLVs are received.

- Command reference

- lldp global tlv-config basic-tlv port-id

Use lldp global tlv-config basic-tlv port-id to set the type of port ID TLVs advertised by LLDP globally.

Use undo lldp global tlv-config basic-tlv port-id to restore the default.

Syntax

lldp [ agent { nearest-customer | nearest-nontpmr } ] global tlv-config basic-tlv port-id type-id

undo lldp [ agent { nearest-customer | nearest-nontpmr } ] global tlv-config basic-tlv port-id

Default

An interface advertises port ID TLVs that contain interface MAC addresses if it receives LLDP-MED TLVs and advertises port ID TLVs that contain interface names if no LLDP-MED TLVs are received.

Views

System view

Predefined user roles

network-admin

Parameters

agent: Specifies an LLDP agent type. If you do not specify an agent type, the command sets the port ID TLV type for nearest bridge agents.

nearest-customer: Specifies nearest customer bridge agents.

nearest-nontpmr: Specifies nearest non-TPMR bridge agents.

type-id: Specifies a port ID TLV type. The available value is 5, which represents that port ID TLVs contain interface names.

Usage guidelines

This command enables the device to advertise only port ID TLVs that contain interface names. Execute this command if LLDP neighbors must obtain interface names from LLDPDUs.

You can configure the port ID TLV type in system view or interface view. The interface-specific setting takes precedence over the global setting.

Examples

# Enable the device to advertise port ID TLVs that contain interface names.

<Sysname> system-view

[Sysname] lldp global tlv-config basic-tlv port-id 5

Related commands

lldp tlv-config basic-tlv port-id

- lldp tlv-config basic-tlv port-id

Use lldp tlv-config basic-tlv port-id to set the type of port ID TLVs advertised by LLDP on an interface.

Use undo lldp tlv-config basic-tlv port-id to restore the default.

Syntax

lldp [ agent { nearest-customer | nearest-nontpmr } ] tlv-config basic-tlv port-id type-id

undo lldp [ agent { nearest-customer | nearest-nontpmr } ] tlv-config basic-tlv port-id

Default

An interface advertises port ID TLVs that contain interface MAC addresses if it receives LLDP-MED TLVs and advertises port ID TLVs that contain interface names if no LLDP-MED TLVs are received.

Views

Layer 2 Ethernet interface view

Management Ethernet interface view

Layer 2 aggregate interface view

Predefined user roles

network-admin

Parameters

agent: Specifies an LLDP agent type. If you do not specify an agent type, the command sets the port ID TLV type for nearest bridge agents.

nearest-customer: Specifies nearest customer bridge agents.

nearest-nontpmr: Specifies nearest non-TPMR bridge agents.

type-id: Specifies a port ID TLV type. The available value is 5, which represents that port ID TLVs contain interface names.

Usage guidelines

This command enables an interface to advertise only port ID TLVs that contain interface names. Execute this command if LLDP neighbors must obtain interface names from LLDPDUs.

You can configure the port ID TLV type in system view or interface view. The interface-specific setting takes precedence over the global setting.

Examples

# Enable GigabitEthernet 1/0/1 to advertise port ID TLVs that contain interface names.

<Sysname> system-view

[Sysname] interface gigabitethernet 1/0/1

[Sysname-GigabitEthernet1/0/1] lldp tlv-config basic-tlv port-id 5

Related commands

lldp global tlv-config basic-tlv port-id

New feature: Enabling displaying LLDP local information about all interfaces

- Enabling displaying LLDP local information about all interfaces

About this task

This task enables the display lldp local-information command to display LLDP local information about all interfaces.

By default, the display lldp local-information command displays information about physically up interfaces. The media devices from some vendors can obtain interface information from HPE devices only through LLDP. For the media devices to obtain all interface information, enable the display lldp local-information command to display LLDP local information about all interfaces.

Restrictions and guidelines

Perform this task only when LLDP neighbors must obtain all interface information from the device through LLDP.

Procedure

            - Enter system view.

system-view

Enable displaying LLDP local information about all interfaces.

lldp local-information all-interface

By default, the display lldp local-information command displays information about physically up interfaces.

- Command reference

- lldp local-information all-interface

Use lldp local-information all-interface to enable displaying LLDP local information about all interfaces.

Use undo lldp local-information all-interface to disable displaying LLDP local information about interfaces not in physically up state.

Syntax

lldp local-information all-interface

undo lldp local-information all-interface

Default

The display lldp local-information command displays information about physically up interfaces.

Views

System view

Predefined user roles

network-admin

Usage guidelines

This command enables the display lldp local-information command to display LLDP local information about all interfaces.

By default, the display lldp local-information command displays information about physically up interfaces. The media devices from some vendors can obtain interface information from HPE devices only through LLDP. For the media devices to obtain all interface information, enable the display lldp local-information command to display LLDP local information about all interfaces.

Examples

# Enable displaying LLDP local information about all interfaces.

<Sysname> system-view

[Sysname] lldp local-information all-interface

Related commands

display lldp local-information

New feature: PoE forced power supply

Enabling PoE forced power supply

About this task

Before supplying power to a PD, the device performs a detection of the PD. It supplies power to the PD only after the PD passes the detection. If the PD fails the detection but the power provided by the device meets the PD specifications, you can perform this task to enable forced power supply to the PD.

Restrictions and guidelines

This feature enables the device to supply power to a PD directly without performing a detection of the PD. To avoid damaging the PD, make sure the power provided by the device meets the PD specifications before performing this task.

After enabling PoE forced power supply on a PI, the system reserves the maximum power for the PI even if no PD is attached to the PI or the PI is not enabled with PoE. For the maximum power that a PI can deliver, execute the display poe pse pse-id interface power command. For the maximum power that the PSE can allocate, execute the display poe pse command.

Procedure

Enter system view.

system-view

Enter PI view.

interface interface-type interface-number

Enable PoE forced power supply.

poe force-power

By default, PoE forced power supply is disabled.

Command reference

poe force-power

Use poe force-power to enable PoE forced power supply.

Use undo poe force-power to disable PoE forced power supply.

Syntax

poe force-power

undo poe force-power

Default

PoE forced power supply is disabled.

Views

PI view

Predefined user roles

network-admin

Usage guidelines

|  | CAUTION: This command enables the device to supply power to a PD directly without performing a detection of the PD. To avoid damaging the PD, make sure the power provided by the device meets the PD specifications before executing this command. |
| --- | --- |

Before supplying power to a PD, the device performs a detection of the PD. It supplies power to the PD only after the PD passes the detection. If the PD fails the detection but the power provided by the device meets the PD specifications, you can execute this command to enable forced power supply to the PD.

After enabling PoE forced power supply on a PI, the system reserves the maximum power for the PI even if no PD is attached to the PI or the PI is not enabled with PoE. For the maximum power that a PI can deliver, execute the display poe pse pse-id interface power command. For the maximum power that the PSE can allocate, execute the display poe pse command.

Examples

# Enable PoE forced power supply.

<Sysname> system-view

[Sysname] interface gigabitethernet 1/0/1

[Sysname-GigabitEthernet1/0/1] poe force-power

The PD might be damaged if the power provided by the device does not meet the PD power specifications. Continue? [Y/N]:y

Command changes

Modified command: display poe pse

Syntax

display poe pse

Views

Any view

Change description

Before modification: The output from the display poe pse command does not contain the Max Allocable Power field.

After modification: The Max Allocable Power field was added to the output from the display poe pse command. The value for the field is equal to the maximum power of the PSE minus the sum of the maximum powers of all PIs on which PoE forced power supply is enabled.

  - New feature: Interval at which the SNMP module examines the system configuration for changes

    - Setting the interval at which the SNMP module examines the system configuration for changes

      - About this task

This task enables the SNMP module to examine the system configuration for changes at the specified interval and generate a trap and a log if any change is found.

      - Procedure

            - Enter system view.

system-view

            - Set the interval at which the SNMP module examines the system configuration for changes.

snmp-agent configuration-examine interval interval

By default, the SNMP module examines the system configuration for changes at intervals of 600 seconds.

    - Command reference

      - snmp-agent configuration-examine interval

Use snmp-agent configuration-examine interval to set the interval at which the SNMP module examines the system configuration for changes.

Use undo snmp-agent configuration-examine interval to restore the default.

Syntax

snmp-agent configuration-examine interval interval

undo snmp-agent configuration-examine interval

Default

By default, the SNMP module examines the system configuration for changes at intervals of 600 seconds.

Views

System view

Predefined user roles

network-admin

Parameters

interval: Specifies the interval at which the SNMP module examines the system configuration for changes. The value is in the range of 1 to 86400, in seconds.

Usage guidelines

This command enables the SNMP module to examine the system configuration for changes at the specified interval and generate a trap and a log if any change is found.

Examples

# Set the interval at which the SNMP module examines the system configuration for changes to 600 seconds.

<sysname> system-view

[sysname] snmp-agent configuration-examine interval 600

New feature: Enabling generation of dynamic IPSG binding entries for 802.1X authenticated users

  - Enabling generation of dynamic IPSG binding entries for 802.1X authenticated users

      - About this task

|  | IMPORTANT: This feature must operate in conjunction with the IP source guard (IPSG) feature. |
| --- | --- |

By default, the device generates a dynamic IPv4SG or IPv6SG binding entry for an 802.1X authenticated user after the user obtains a static or DHCP assigned IP address.

To allow only 802.1X users with DHCP assigned IP addresses to access the network, perform the following operations:

- Enable IPSG.

- Disable generation of dynamic IPv4SG or IPv6SG binding entries for 802.1X authenticated users.

- Enable DHCP snooping. The device will generate IPv4SG or IPv6SG binding entries for the users based on DHCP snooping.

For more information about IPSG, see IP source guard in Security Configuration Guide.

      - Restrictions and guidelines

This feature takes effect only on 802.1X users that come online after the feature is enabled. If the IP address of an online 802.1X user changes, the device will update the dynamic IPv4SG or IPv6SG binding entry for the user.

Disabling this feature does not delete the existing dynamic IPv4SG or IPv6SG binding entries for online 802.1X users. If the IP address of an online 802.1X user changes after the feature is disabled, the device will delete the dynamic IPv4SG or IPv6SG binding entry for the user.

      - Procedure

            - Enter system view.

system-view

            - Enter interface view.

interface interface-type interface-number

            - Enable generation of dynamic IPv4SG or IPv6SG binding entries for 802.1X authenticated users.

dot1x { ip-verify-source | ipv6-verify-source } enable

By default, generation of dynamic IPv4SG or IPv6SG binding entries for 802.1X authenticated users is enabled.

    - dot1x { ip-verify-source | ipv6-verify-source } enable

Use dot1x { ip-verify-source | ipv6-verify-source } enable to enable generation of dynamic IPv4SG or IPv6SG binding entries for 802.1X authenticated users.

Use undo dot1x { ip-verify-source | ipv6-verify-source } enable to disable generation of dynamic IPv4SG or IPv6SG binding entries for 802.1X authenticated users.

Syntax

dot1x { ip-verify-source | ipv6-verify-source } enable

undo dot1x { ip-verify-source | ipv6-verify-source } enable

Default

Generation of dynamic IPv4SG or IPv6SG binding entries for 802.1X authenticated users is enabled.

Views

Layer 2 Ethernet interface view

Layer 2 aggregate interface view

Predefined user roles

network-admin

mdc-admin

Usage guidelines

|  | IMPORTANT: This feature must operate in conjunction with the IP source guard (IPSG) feature. |
| --- | --- |

The dot1x { ip-verify-source | ipv6-verify-source } enable command takes effect only on 802.1X users that come online after the command is used. If the IP address of an online 802.1X user changes, the device will update the dynamic IPv4SG or IPv6SG binding entry for the user.

The undo dot1x { ip-verify-source | ipv6-verify-source } enable command does not delete the existing dynamic IPv4SG or IPv6SG binding entries for online 802.1X users. If the IP address of an online 802.1X user changes after the command is used, the device will delete the dynamic IPv4SG or IPv6SG binding entry for the user.

Examples

# Disable generation of dynamic IPv4SG or IPv6SG binding entries for 802.1X authenticated users on GigabitEthernet 1/0/1.

<Sysname> system-view

[Sysname] interface gigabitethernet 1/0/1

[Sysname-GigabitEthernet1/0/1] undo dot1x ip-verify-source enable

New feature: Automated IPv6 underlay network deployment for VCF fabric

About automated IPv6 underlay network deployment

As from this software version, the VCF fabric feature supports automated IPv6 underlay network deployment. The deployment procedure is the same as that of automated IPv4 underlay network deployment.

In an IPv6 VCF fabric, the controller collects the topology automatically. You do not need to specify a master spine node.

Command reference

None.

Modified feature: Setting the port status detection timer

Feature change description

As from this release, the value range for the port status detection timer is changed to 0 to 3600 seconds.

The device starts a port status detection timer when a port is shut down by a protocol such as LLDP and loop detection. Once the timer expires, the device brings up the port so the port status reflects the port's physical status. For example, loop detection shuts down a looped interface to disable the interface from receiving or sending frames. The device automatically sets the interface to the forwarding state after the port status detection timer expires.

Command changes

Modified command: shutdown-interval

Syntax

shutdown-interval interval

undo shutdown-interval

Views

System view

Change description

Before modification: The value range for the interval argument is 0 to 300.

After modification: The value range for the interval argument is 0 to 3600.

Modified feature: 802.1X EAD assistant

Feature change description

As from this version, you can use the dot1x ead-assistant permit authentication-escape command to enable support for 802.1X Auth-Fail and critical VLANs in 802.1X EAD assistant.

Command changes

New command: dot1x ead-assistant permit authentication-escape

Use dot1x ead-assistant permit authentication-escape to enable support for 802.1X Auth-Fail and critical VLANs in 802.1X EAD assistant.

Use undo dot1x ead-assistant permit authentication-escape to restore the default.

Syntax

dot1x ead-assistant permit authentication-escape

undo dot1x ead-assistant permit authentication-escape

Default

802.1X Auth-Fail and critical VLANs cannot take effect when 802.1X EAD assistant is enabled.

Views

System view

Predefined user roles

network-admin

Usage guidelines

This command enables the device to remove the EAD entries of users before it assigns the users to 802.1X Auth-Fail and critical VLANs.

Examples

# Enable support for 802.1X Auth-Fail and critical VLANs in 802.1X EAD assistant.

<Sysname> system-view

[Sysname] dot1x ead-assistant permit authentication-escape

Related commands

dot1x ead-assistant enable

Modified feature: Displaying information about online 802.1X users

Feature change description

As from this version, the Authorization dynamic ACL name field is added to the command output from the display dot1x connection command.

Command changes

Modified command: display dot1x connection

Syntax

display dot1x connection [ open ] [ interface interface-type interface-number | slot slot-number | user-mac mac-address | user-name name-string ]

Views

Any view

Change description

The Authorization dynamic ACL name field was added to the command output from this command. If no dynamic ACL is assigned, this field displays N/A. If a dynamic ACL is assigned but the assignment fails, this field displays (NOT effective) next to the dynamic ACL name.

The following is a sample output from the display dot1x connection command:

<Sysname> display dot1x connection

Total connections: 1

Slot ID: 1

User MAC address: 0015-e9a6-7cfe

Access interface: GigabitEthernet1/0/1

Username: ias

User access state: Successful

Authentication domain: aaa

IPv4 address: 192.168.1.1

IPv6 address: 2000:0:0:0:1:2345:6789:abcd

Authentication method: CHAP

Initial VLAN: 1

Authorization untagged VLAN: 6

Authorization tagged VLAN list: 1 to 5 7 9 11 13 15 17 19 21 23 25 27 29 31 33 29 31 33

35 37 40 to 100

Authorization VSI: N/A

Authorization ACL number/name: 3001

Authorization dynamic ACL name: N/A

Authorization user profile: N/A

Authorization CAR: N/A

Authorization URL: N/A

Termination action: Default

Session timeout period: 2 s

Online from: 2013/03/02  13:14:15

Online duration: 0h 2m 15s

Modified feature: Displaying information about online MAC authentication users

Feature change description

As from this version, the Authorization dynamic ACL name field is added to the command output from the display mac-authentication connection command.

Command changes

Modified command: display mac-authentication connection

Syntax

display mac-authentication connection [ open ] [ interface interface-type interface-number | slot slot-number | user-mac mac-address | user-name user-name ]

Views

Any view

Change description

The Authorization dynamic ACL name field was added to the command output from this command. If no dynamic ACL is assigned, this field displays N/A. If a dynamic ACL is assigned but the assignment fails, this field displays (NOT effective) next to the dynamic ACL name.

The following is a sample output from the display mac-authentication connection command:

<Sysname> display mac-authentication connection

Total connections: 1

Slot ID: 1

User MAC address: 0015-e9a6-7cfe

Access interface: GigabitEthernet1/0/1

Username: ias

User access state: Successful

Authentication domain: macusers

IPv4 address: 192.168.1.1

IPv6 address: 2000:0:0:0:1:2345:6789:abcd

Initial VLAN: 1

Authorization untagged VLAN: 100

Authorization tagged VLAN: N/A

Authorization VSI: N/A

Authorization ACL number/name: 3001

Authorization dynamic ACL name: N/A

Authorization user profile: N/A

Authorization CAR: N/A

Authorization URL: N/A

Termination action: Radius-request

Session timeout period: 2 sec

Offline detection: 100 sec (server-assigned)

Online from: 2013/03/02  13:14:15

Online duration: 0h 2m 15s

Modified feature: L2PT for CFD

Feature change description

As from this version, the device supports enabling L2TP for CFD and configuring the destination multicast MAC address for tunneled packets of the specified protocol.

Command changes

Modified command: l2protocol type tunnel-dmac

Old syntax

l2protocol type { cdp | dldp | dtp | eoam | gvrp | lacp | lldp | mvrp | pagp | pvst | stp | udld | vtp } tunnel-dmac mac-address

undo l2protocol type { cdp | dldp | dtp | eoam | gvrp | lacp | lldp | mvrp | pagp | pvst | stp | udld | vtp } tunnel-dmac

New syntax

l2protocol type { cdp | cfd | dldp | dtp | eoam | gvrp | lacp | lldp | mvrp | pagp | pvst | stp | udld | vtp } tunnel-dmac mac-address

undo l2protocol type { cdp | cfd | dldp | dtp | eoam | gvrp | lacp | lldp | mvrp | pagp | pvst | stp | udld | vtp } tunnel-dmac

Views

System view

Change description

Before modification: The cfd keyword is not supported.

After modification: The cfd keyword is supported.

Modified command: l2protocol tunnel dot1q

Old syntax

In Layer 2 Ethernet interface view:

l2protocol { cdp | dldp | dtp | eoam | gvrp | lacp | lldp | mvrp | pagp | pvst | stp | udld | vtp } tunnel dot1q

undo l2protocol { cdp | dldp | dtp | eoam | gvrp | lacp | lldp | mvrp | pagp | pvst | stp | udld | vtp } tunnel dot1q

In Layer 2 aggregate interface view:

l2protocol { cdp | gvrp | lacp | lldp | mvrp | pagp | pvst | stp | udld | vtp } tunnel dot1q

undo l2protocol { cdp | gvrp | lacp | lldp | mvrp | pagp | pvst | stp | udld |  vtp } tunnel dot1q

New syntax

In Layer 2 Ethernet interface view:

l2protocol { cdp | cfd | dldp | dtp | eoam | gvrp | lacp | lldp | mvrp | pagp | pvst | stp | udld | vtp } tunnel dot1q

undo l2protocol { cdp | cfd | dldp | dtp | eoam | gvrp | lacp | lldp | mvrp | pagp | pvst | stp | udld | vtp } tunnel dot1q

In Layer 2 aggregate interface view:

l2protocol { cdp | cfd | gvrp | lacp | lldp | mvrp | pagp | pvst | stp | udld | vtp } tunnel dot1q

undo l2protocol { cdp | cfd | gvrp | lacp | lldp | mvrp | pagp | pvst | stp | udld |  vtp } tunnel dot1q

Views

Layer 2 Ethernet interface view

Layer 2 aggregate interface view

Change description

Before modification: The cfd keyword is not supported.

After modification: The cfd keyword is supported.

Modified command: display l2protocol statistics

Syntax

display l2protocol statistics [ interface interface-type interface-number ]

Views

Any view

Change description

Before modification: The device does not support displaying L2TP statistics for CFD protocol packets.

After modification: The device supports displaying L2TP statistics for CFD protocol packets.

# Display L2PT statistics for all Layer 2 Ethernet and aggregate interfaces.

<Sysname> display l2protocol statistics

L2PT statistics information on interface Bridge-Aggregation1:

Protocol  Encapsulated      Decapsulated      Forwarded         Dropped

CDP       0                 0                 0                 0

DLDP      0                 3                 0                 0

EOAM      0                 2                 0                 0

GVRP      8                 4                 9                 2

LACP      0                 0                 0                 0

LLDP      0                 3                 0                 0

MVRP      0                 0                 0                 0

PAGP      0                 1                 0                 0

PVST      0                 0                 0                 0

STP       5                 5                 5                 0

Tunnel    N/A               N/A               100               10

VTP       0                 6                 0                 0

UDLD      0                 0                 0                 0

DTP       0                 0                 0                 0

CFD       0                 0                 0                 0

L2PT statistics information on interface GigabitEthernet1/0/1:

Protocol  Encapsulated      Decapsulated      Forwarded         Dropped

CDP       0                 0                 0                 0

DLDP      2                 3                 3                 0

EOAM      5                 2                 9                 0

GVRP      8                 4                 9                 2

LACP      0                 0                 0                 0

LLDP      3                 3                 3                 3

MVRP      0                 0                 0                 0

PAGP      5                 1                 7                 3

PVST      0                 0                 0                 0

STP       5                 5                 5                 0

Tunnel    N/A               N/A               100               10

VTP       0                 6                 0                 0

UDLD      0                 0                 0                 0

DTP       0                 0                 0                 0

CFD       0                 0                 0                 0

Release 6330

This release has the following changes:

New feature: Enabling fast PoE for a PSE

Modified feature: L2PT for CFD and DTP

Modified feature: Displaying information about online 802.1X users

Modified feature: Displaying information about online MAC authentication users

New feature: Enabling fast PoE for a PSE

Enabling fast PoE for a PSE

About this task

This feature enables PIs on a PSE to supply power to PDs immediately after the PSE is powered on.

      - Restrictions and guidelines

You must re-configure this feature if you changed other PoE settings after configuring this feature.

      - Procedure

            - Enter system view.

system-view

            - Enable fast PoE for a PSE.

poe fast-on enable pse pse-id

By default, fast PoE is disabled for a PSE.

Command reference

    - poe fast-on enable

Use poe fast-on enable to enable fast PoE for a PSE.

Use undo poe fast-on enable to disable fast PoE for a PSE.

Syntax

poe fast-on enable pse pse-id

undo poe fast-on enable pse pse-id

Default

Fast PoE is disabled for a PSE.

Views

System view

Predefined user roles

network-admin

Parameters

pse pse-id: Specifies a PSE by its ID.

Usage guidelines

Fast PoE enables PIs on a PSE to supply power to PDs immediately after the PSE is powered on.

You must re-configure this command if you changed other PoE settings after configuring this command.

Examples

# Enable fast PoE for PSE 4.

<Sysname> system-view

[Sysname] poe fast-on enable pse 4

Modified feature: L2PT for CFD and DTP

Feature change description

As from this version, the device supports enabling L2TP for CFD and DTP and configuring the destination multicast MAC address for tunneled packets of the specified protocol.

Command changes

New command: l2protocol type tunnel-dmac

Use l2protocol type tunnel-dmac to set the destination multicast MAC address for tunneled packets of the specified protocol.

Use undo l2protocol type tunnel-dmac to restore the default.

Syntax

l2protocol type { cdp | cfd | dldp | dtp | eoam | gvrp | lacp | lldp | mvrp | pagp | pvst | stp | udld | vtp } tunnel-dmac mac-address

undo l2protocol type { cdp | cfd | dldp | dtp | eoam | gvrp | lacp | lldp | mvrp | pagp | pvst | stp | udld | vtp } tunnel-dmac

Default

The tunneled packets of all protocols use 010f-e200-0003 as the destination multicast MAC address.

Views

System view

Predefined user roles

network-admin

Parameters

cdp: Specifies CDP.

cfd: Specifies CFD.

dldp: Specifies DLDP.

dtp: Specifies DTP.

eoam: Specifies EOAM.

gvrp: Specifies GVRP.

lacp: Specifies LACP.

lldp: Specifies LLDP.

mvrp: Specifies MVRP.

pagp: Specifies PAgP.

pvst: Specifies PVST.

stp: Specifies STP.

udld: Specifies UDLD.

vtp: Specifies VTP.

mac-address: Specifies a destination multicast MAC address for tunneled packets of the specified protocol, in the range of 0100-0000-0000 to 01ff-ffff-ffff.

Usage guidelines

As a best practice, set different destination multicast MAC addresses on PEs connected to different customer networks. It prevents L2PT from sending packets of a customer network to another customer network.

The l2protocol tunnel-dmac command sets the destination multicast MAC address for tunneled packets of all protocols. This command sets the destination multicast MAC address for tunneled packets of the specified protocol. If both commands are executed, the l2protocol type tunnel-dmac command takes priority.

For tunneled packets to be recognized, set the same destination multicast MAC address for packets of the same protocol on PEs that are connected to the same customer network.

If you execute this command multiple times for a protocol, the most recent configuration takes effect.

Examples

# Set the destination multicast MAC address to 0100-0ccd-cddc for tunneled packets of CFD.

<Sysname> system-view

[Sysname] l2protocol type cfd tunnel-dmac 0100-0ccd-cddc

Modified command: l2protocol tunnel dot1q

Old syntax

In Layer 2 Ethernet interface view:

l2protocol { cdp | dldp | eoam | gvrp | lacp | lldp | mvrp | pagp | pvst | stp | udld | vtp } tunnel dot1q

undo l2protocol { cdp | dldp | eoam | gvrp | lacp | lldp | mvrp | pagp | pvst | stp | udld | vtp } tunnel dot1q

In Layer 2 aggregate interface view:

l2protocol { cdp | gvrp | lacp | lldp | mvrp | pagp | pvst | stp | udld | vtp } tunnel dot1q

undo l2protocol { cdp | gvrp | lacp | lldp | mvrp | pagp | pvst | stp | udld |  vtp } tunnel dot1q

New syntax

In Layer 2 Ethernet interface view:

l2protocol { cdp | cfd | dldp | dtp | eoam | gvrp | lacp | lldp | mvrp | pagp | pvst | stp | udld | vtp } tunnel dot1q

undo l2protocol { cdp | cfd | dldp | dtp | eoam | gvrp | lacp | lldp | mvrp | pagp | pvst | stp | udld | vtp } tunnel dot1q

In Layer 2 aggregate interface view:

l2protocol { cdp | cfd | gvrp | lacp | lldp | mvrp | pagp | pvst | stp | udld | vtp } tunnel dot1q

undo l2protocol { cdp | cfd | gvrp | lacp | lldp | mvrp | pagp | pvst | stp | udld |  vtp } tunnel dot1q

Views

layer 2 Ethernet interface view

layer 2 aggregate interface view

Change description

Before modification: L2TP cannot be enabled for CFD or DTP.

After modification: L2TP can be enabled for CFD and DTP.

Modified command: display l2protocol statistics

Syntax

display l2protocol statistics [ interface interface-type interface-number ]

Views

Any view

Change description

Before modification: The device does not support displaying L2TP statistics for CFD or DTP protocol packets.

After modification: The device supports displaying L2TP statistics for CFD and DTP protocol packets.

# Display L2PT statistics for all Layer 2 Ethernet and aggregate interfaces.

<Sysname> display l2protocol statistics

L2PT statistics information on interface Bridge-Aggregation1:

Protocol  Encapsulated      Decapsulated      Forwarded         Dropped

CDP       0                 0                 0                 0

DLDP      0                 3                 0                 0

EOAM      0                 2                 0                 0

GVRP      8                 4                 9                 2

LACP      0                 0                 0                 0

LLDP      0                 3                 0                 0

MVRP      0                 0                 0                 0

PAGP      0                 1                 0                 0

PVST      0                 0                 0                 0

STP       5                 5                 5                 0

Tunnel    N/A               N/A               100               10

VTP       0                 6                 0                 0

UDLD      0                 0                 0                 0

DTP       0                 0                 0                 0

CFD       0                 0                 0                 0

L2PT statistics information on interface GigabitEthernet1/0/1:

Protocol  Encapsulated      Decapsulated      Forwarded         Dropped

CDP       0                 0                 0                 0

DLDP      2                 3                 3                 0

EOAM      5                 2                 9                 0

GVRP      8                 4                 9                 2

LACP      0                 0                 0                 0

LLDP      3                 3                 3                 3

MVRP      0                 0                 0                 0

PAGP      5                 1                 7                 3

PVST      0                 0                 0                 0

STP       5                 5                 5                 0

Tunnel    N/A               N/A               100               10

VTP       0                 6                 0                 0

UDLD      0                 0                 0                 0

DTP       0                 0                 0                 0

CFD       0                 0                 0                 0

Modified feature: Displaying information about online 802.1X users

Feature change description

As from this version, the Authorization dynamic ACL name field is added to the command output from the display dot1x connection command.

Command changes

Modified command: display dot1x connection

Syntax

display dot1x connection [ open ] [ interface interface-type interface-number | slot slot-number | user-mac mac-address | user-name name-string ]

Views

Any view

Change description

The Authorization dynamic ACL name field was added to the command output from this command. If no dynamic ACL is assigned, this field displays N/A. If a dynamic ACL is assigned but the assignment fails, this field displays (NOT effective) next to the dynamic ACL name.

The following is a sample output from the display dot1x connection command:

<Sysname> display dot1x connection

Total connections: 1

Slot ID: 1

User MAC address: 0015-e9a6-7cfe

Access interface: GigabitEthernet1/0/1

Username: ias

User access state: Successful

Authentication domain: aaa

IPv4 address: 192.168.1.1

IPv6 address: 2000:0:0:0:1:2345:6789:abcd

Authentication method: CHAP

Initial VLAN: 1

Authorization untagged VLAN: 6

Authorization tagged VLAN list: 1 to 5 7 9 11 13 15 17 19 21 23 25 27 29 31 33 29 31 33

35 37 40 to 100

Authorization VSI: N/A

Authorization ACL number/name: 3001

Authorization dynamic ACL name: N/A

Authorization user profile: N/A

Authorization CAR: N/A

Authorization URL: N/A

Termination action: Default

Session timeout period: 2 s

Online from: 2013/03/02  13:14:15

Online duration: 0h 2m 15s

Modified feature: Displaying information about online MAC authentication users

Feature change description

As from this version, the Authorization dynamic ACL name field is added to the command output from the display mac-authentication connection command.

Command changes

Modified command: display mac-authentication connection

Syntax

display mac-authentication connection [ open ] [ interface interface-type interface-number | slot slot-number | user-mac mac-address | user-name user-name ]

Views

Any view

Change description

The Authorization dynamic ACL name field was added to the command output from this command. If no dynamic ACL is assigned, this field displays N/A. If a dynamic ACL is assigned but the assignment fails, this field displays (NOT effective) next to the dynamic ACL name.

The following is a sample output from the display mac-authentication connection command:

<Sysname> display mac-authentication connection

Total connections: 1

Slot ID: 1

User MAC address: 0015-e9a6-7cfe

Access interface: GigabitEthernet1/0/1

Username: ias

User access state: Successful

Authentication domain: macusers

IPv4 address: 192.168.1.1

IPv6 address: 2000:0:0:0:1:2345:6789:abcd

Initial VLAN: 1

Authorization untagged VLAN: 100

Authorization tagged VLAN: N/A

Authorization VSI: N/A

Authorization ACL number/name: 3001

Authorization dynamic ACL name: N/A

Authorization user profile: N/A

Authorization CAR: N/A

Authorization URL: N/A

Termination action: Radius-request

Session timeout period: 2 sec

Offline detection: 100 sec (server-assigned)

Online from: 2013/03/02  13:14:15

Online duration: 0h 2m 15s