HPE 5950-CMW710-R6301P02 Release Notes

Software Feature Changes

The information in this document is subject to change without notice. © Copyright 2023 Hewlett Packard Enterprise Development LP

Contents

About software feature changes	1

Release 6301P02	2

Release 6301P01	3

Release 6301	4

New features: Fundamentals features	6

New features: Virtual technologies features	7

New features: Layer 2—LAN switching features	7

New features: Layer 3—IP services features	9

New features: Layer 3—IP routing features	13

New features: IP multicast features	18

New features: MPLS features	20

New features: ACL and QoS features	21

New features: Security features	23

New features: High availability features	28

New features: Network management and monitoring features	29

New features: FC and FCoE features	31

New features: OpenFlow features	31

New features: VXLAN features	32

New features: EVPN features	32

Modified feature: ISSU by using issu commands	33

Feature change description	33

Command changes	34

Modified command: display version comp-matrix	34

Modified command: issu load	34

Modified command: issu one-step	35

Modified feature: Automatic configuration	35

Feature change description	35

Command changes	35

Modified feature: Setting the timestamp format for logs sent to log hosts	36

Feature change description	36

Command changes	36

Modified command: info-center timestamp loghost	36

Modified feature: Configuring an interface event for a CLI-defined EAA monitor policy	36

Feature change description	36

Command changes	36

Modified command: event-interface	36

Modified feature: Configuring an EAA monitor policy by using Tcl	37

Feature change description	37

Command changes	37

Modified feature: Displaying the operating status and information of an interface	37

Feature change description	37

Command changes	37

Modified command: display interface	37

Modified feature: Displaying PFC information of all interfaces	38

Feature change description	38

Command changes	38

Modified command: display priority-flow-control interface	38

Modified feature: Link state change suppression on an interface	39

Feature change description	39

Command changes	39

Modified command: link-delay	39

Modified feature: MAC-to-VLAN entries	40

Feature change description	40

Command changes	40

Modified command: mac-vlan mac-address	40

Modified command: display mac-vlan	40

Modified feature: Collision handling process of LACP MAD and BFD MAD	41

Feature change description	41

Command changes	41

Modified feature: Collision handling process of ND MAD	42

Feature change description	42

Command changes	42

Modified feature: Displaying global link aggregation load sharing modes	42

Feature change description	42

Command changes	42

Modified command: display link-aggregation load-sharing mode	42

Modified feature: Configuring a link aggregation load sharing hash seed	43

Feature change description	43

Command changes	43

Modified command: link-aggregation global load-sharing seed	43

Modified feature: Displaying detailed information about the IPP and DR interfaces of DRNI	44

Feature change description	44

Command changes	44

Modified command: display drni verbose	44

Modified feature: Configuring the advertisable TLVs for LLDP	45

Feature change description	45

Command changes	46

Modified command: lldp tlv-enable	46

Modified feature: Setting the aging timer for dynamic ARP entries	51

Feature change description	51

Command changes	51

Modified command: arp timer aging	51

Modified feature: ARP snooping	52

Feature change description	52

Command changes	52

Modified command: arp snooping enable	52

Modified command: display arp snooping	52

Modified command: reset arp snooping	53

Modified feature: Specifying DHCP servers for a DHCP relay address pool	53

Feature change description	53

Command changes	53

Modified command: remote-server	53

Modified feature: Displaying DHCP snooping and DHCPv6 snooping trusted ports	54

Feature change description	54

Command changes	54

Modified command: display dhcp snooping trust	54

Modified command: display ipv6 dhcp snooping trust	55

Modified feature: Displaying DHCP address pool information	57

Feature change description	57

Command changes	57

Modified command: display dhcp server pool	57

Modified feature: Displaying DHCPv6 address pool information	59

Feature change description	59

Command changes	59

Modified command: display ipv6 dhcp pool	59

Modified feature: Enabling recording DHCPv6 snooping address entries in VLAN view	61

Feature change description	61

Command changes	62

Modified command: ipv6 dhcp snooping binding record	62

Modified feature: Setting the aging timer for ND entries in stale state	62

Feature change description	62

Command changes	62

Modified feature: Displaying member interfaces shut down by Monitor Link	62

Feature change description	62

Command changes	63

Modified command: display monitor-link group	63

Modified feature: Displaying DLDP configuration	63

Feature change description	63

Command changes	63

Modified command: display dldp	63

Modified feature: Configuring routing policy-based recursive lookup	64

Feature change description	64

Command changes	64

Modified command: protocol nexthop recursive-lookup	64

Modified feature: Associating Track with the output interface for a static route	65

Feature change description	65

Command changes	65

Modified command: ip route-static	65

Modified feature: Configuring OSPF area authentication	66

Feature change description	66

Command changes	66

Modified command: authentication-mode	66

Modified feature: Configuring OSPF interface authentication	67

Feature change description	67

Command changes	67

Modified command: ospf authentication-mode	67

Modified feature: Configuring a virtual link	68

Feature change description	68

Command changes	68

Modified command: vlink-peer	68

Modified feature: Setting the number of OSPF logs	68

Feature change description	68

Command changes	69

Modified command: event-log	69

Modified feature: Displaying IS-IS LSP log information	69

Feature change description	69

Command changes	69

Modified feature: Clearing IS-IS LSP log information	70

Feature change description	70

Command changes	70

Modified feature: Specifying a label allocation mode	70

Feature change description	70

Command changes	70

Modified command: label-allocation-mode	70

Modified feature: Displaying BGP peer or peer group information	71

Feature change description	71

Command changes	71

Modified command: display bgp peer	71

Modified feature: Displaying AS path information for BGP routes	71

Feature change description	71

Command changes	72

Modified feature: Displaying RPF information for an IPv6 multicast source	74

Feature change description	74

Command changes	74

Modified command: display ipv6 multicast rpf-info	74

Modified feature: Displaying statistics for the IGMP messages and PIMv2 hello messages learned through IGMP snooping	75

Feature change description	75

Command changes	75

Modified command: display igmp-snooping statistics	75

Modified feature: Displaying statistics for the MLD messages and IPv6 PIM hello messages learned through MLD snooping	77

Feature change description	77

Command changes	77

Modified command: display mld-snooping statistics	77

Modified feature: Enabling a VPN instance to replicate routes from the public network or other VPN instances	79

Feature change description	79

Command changes	79

Modified command: route-replicate	79

Modified feature: Specifying outgoing labels for a static SRLSP	79

Feature change description	79

Command changes	80

Modified command: static-sr-mpls lsp	80

Modified feature: Creating an ACL	80

Feature change description	80

Command changes	80

Modified command: acl	80

Modified feature: Referencing an ACL in QoS or packet filtering	81

Feature change description	81

Command changes	81

Modified command: none	81

Modified feature: Enabling hardware-count for the packet filtering default action	82

Feature change description	82

Command changes	82

Modified command: packet-filter default hardware-count	82

Modified feature: Configuring drop-level-based parameters for a queue in a WRED table	82

Feature change description	82

Command changes	83

Modified command: queue	83

Modified feature: Configuring binding attributes for a local user	83

Feature change description	83

Command changes	83

Modified command: bind-attribute	83

Modified feature: Password handling when global password control is enabled	84

Feature change description	84

Command changes	85

Modified feature: Setting the quiet timer for RADIUS servers	85

Feature change description	85

Command changes	85

Modified command: timer quiet (RADIUS scheme view)	85

Modified feature: Configuring MAC-based MAC authentication user accounts	85

Feature change description	85

Command changes	85

Modified command: mac-authentication user-name-format	85

Modified feature: MAC authentication VLAN mode	86

Feature change description	86

Command changes	86

Modified command: mac-authentication host-mode	86

Modified feature: Port security MAC move	87

Feature change description	87

Command changes	87

Modified command: port-security mac-move permit	87

Modified feature: Web authentication support for HTTPS redirection	88

Modified feature: RSA key modulus length	88

Feature change description	88

Command changes	88

Modified command: public-key local create	88

Modified feature: RSA key modulus length used for certification request in a PKI domain	89

Feature change description	89

Command changes	89

Modified command: public-key rsa	89

Modified feature: Displaying IPv4SG bindings	89

Feature change description	89

Command changes	90

Modified command: display ip source binding	90

Modified feature: Displaying IPv6SG bindings	90

Feature change description	90

Command changes	90

Modified command: display ipv6 source binding	90

Modified feature: Displaying the MFF configuration for a VLAN	91

Feature change description	91

Command changes	91

Modified command: display mac-forced-forwarding vlan	91

Modified feature: Associating Track with application modules	92

Feature change description	92

Command changes	92

Modified command: track bfd ctrl	92

Modified command: track bfd echo	93

Modified command: track cfd	93

Modified command: track interface	93

Modified command: track interface physical	94

Modified command: track interface protocol	94

Modified command: track ip route reachability	94

Modified command: track lldp neighbor	95

Modified command: track nqa	95

Modified feature: Specifying the length of ICMP echo requests sent by an IPv4 or IPv6 ping operation	95

Feature change description	95

Command changes	96

Modified command: ping	96

Modified command: ping ipv6	96

Modified feature: Removing a TCP or UDP listening service for a VPN instance	96

Feature change description	96

Command changes	96

Modified command: nqa server tcp-connect	96

Modified command: nqa server udp-echo	97

Modified feature: Specifying the source IP address for NTP messages	97

Feature change description	97

Command changes	97

Modified command: ntp-service source	97

New feature: Specifying the NTP time-offset thresholds for log and trap outputs	98

Specifying the NTP time-offset thresholds for log and trap outputs	98

Command reference	98

ntp-service time-offset-threshold	98

New feature: Specifying the SNTP time-offset thresholds for log and trap outputs	99

Specifying the SNTP time-offset thresholds for log and trap outputs	99

Command reference	100

sntp time-offset-threshold	100

Modified feature: Creating a sampler	100

Feature change description	100

Command changes	101

Modified command: sampler	101

Modified feature: sFlow counter sampling	101

Feature change description	101

Command changes	101

Modified command: sflow counter collector	101

Modified feature: sFlow flow sampling	102

Feature change description	102

Command changes	102

Modified command: sflow counter collector	102

Modified feature: Configuring a backup PW for a cross-connect	102

Feature change description	102

Command changes	102

Modified command: backup-peer	102

Modified feature: Configuring a backup PW for a VSI	103

Feature change description	103

Command changes	103

Modified command: backup-peer	103

Modified feature: Change of the bandwidth limit value range for VSIs	103

Feature change description	103

Command changes	103

Modified command: bandwidth	103

Modified feature: Value range change for the broadcast, multicast, or unknown unicast restraint bandwidth of VSIs	104

Feature change description	104

Command changes	104

Modified command: restrain	104

Modified feature: Frame match criteria of VXLAN Ethernet service instances	104

Feature change description	104

Command changes	104

Modified command: encapsulation	104

Modified feature: Displaying EVPN routing table information	105

Feature change description	105

Command changes	105

Modified command: display evpn routing-table	105

Modified feature: NETCONF logging	106

Feature change description	106

Command changes	106

Modified command: netconf log	106

Modified feature: Specifying the role of the device in the VCF fabric	106

Feature change description	106

Command reference	107

Modified command: vcf-fabric role	107

Modified command: display vcf-fabric role	107

Modified command: display vcf-fabric underlay autoconfigure	108

About software feature changes

This document contains feature changes in the software versions listed in Table 1. For information about software feature changes in HPE 5950-CMW710-F6207 (or earlier), see their respective release notes (software feature changes).

Software feature change summary

| Section | Software feature changes |
| --- | --- |
| Release 6301P02 | Contains changes in HPE 5950-CMW710-R6301P02 over HPE 5950-CMW710-R6301P01. |
| Release 6301P01 | Contains changes in HPE 5950-CMW710-R6301P01 over HPE 5950-CMW710-R6301. |
| Release 6301 | Contains changes in HPE 5950-CMW710-R6301 over HPE 5950-CMW710-F6207. |

Release 6301P02

This release has no feature changes.

Release 6301P01

This release has no feature changes.

Release 6301

This release has the following changes:

New features: Fundamentals features

New features: Virtual technologies features

New features: Layer 2—LAN switching features

New features: Layer 3—IP services features

New features: Layer 3—IP routing features

New features: IP multicast features

New features: MPLS features

New features: ACL and QoS features

New features: Security features

New features: High availability features

New features: Network management and monitoring features

New features: FC and FCoE features

New features: OpenFlow features

New features: VXLAN features

New features: EVPN features

Modified feature: ISSU by using issu commands

Modified feature: Automatic configuration

Modified feature: Setting the timestamp format for logs sent to log hosts

Modified feature: Configuring an interface event for a CLI-defined EAA monitor policy

Modified feature: Configuring an EAA monitor policy by using Tcl

Modified feature: Displaying the operating status and information of an interface

Modified feature: Displaying PFC information of all interfaces

Modified feature: Link state change suppression on an interface

Modified feature: MAC-to-VLAN entries

Modified feature: Collision handling process of LACP MAD and BFD MAD

Modified feature: Collision handling process of ND MAD

Modified feature: Displaying global link aggregation load sharing modes

Modified feature: Configuring a link aggregation load sharing hash seed

Modified feature: Displaying detailed information about the IPP and DR interfaces of DRNI

Modified feature: Configuring the advertisable TLVs for LLDP

Modified feature: Setting the aging timer for dynamic ARP entries

Modified feature: ARP snooping

Modified feature: Specifying DHCP servers for a DHCP relay address pool

Modified feature: Displaying DHCP snooping and DHCPv6 snooping trusted ports

Modified feature: Displaying DHCP address pool information

Modified feature: Displaying DHCPv6 address pool information

Modified feature: Enabling recording DHCPv6 snooping address entries in VLAN view

Modified feature: Setting the aging timer for ND entries in stale state

Modified feature: Displaying member interfaces shut down by Monitor Link

Modified feature: Displaying DLDP configuration

Modified feature: Configuring routing policy-based recursive lookup

Modified feature: Associating Track with the output interface for a static route

Modified feature: Configuring OSPF area authentication

Modified feature: Configuring OSPF interface authentication

Modified feature: Configuring a virtual link

Modified feature: Setting the number of OSPF logs

Modified feature: Displaying IS-IS LSP log information

Modified feature: Clearing IS-IS LSP log information

Modified feature: Specifying a label allocation mode

Modified feature: Displaying BGP peer or peer group information

Modified feature: Displaying AS path information for BGP routes

Modified feature: Displaying RPF information for an IPv6 multicast source

Modified feature: Displaying statistics for the IGMP messages and PIMv2 hello messages learned through IGMP snooping

Modified feature: Displaying statistics for the MLD messages and IPv6 PIM hello messages learned through MLD snooping

Modified feature: Enabling a VPN instance to replicate routes from the public network or other VPN instances

Modified feature: Specifying outgoing labels for a static SRLSP

Modified feature: Creating an ACL

Modified feature: Referencing an ACL in QoS or packet filtering

Modified feature: Enabling hardware-count for the packet filtering default action

Modified feature: Configuring drop-level-based parameters for a queue in a WRED table

Modified feature: Configuring binding attributes for a local user

Modified feature: Password handling when global password control is enabled

Modified feature: Setting the quiet timer for RADIUS servers

Modified feature: Configuring MAC-based MAC authentication user accounts

Modified feature: MAC authentication VLAN mode

Modified feature: Port security MAC move

Modified feature: Web authentication support for HTTPS redirection

Modified feature: RSA key modulus length

Modified feature: RSA key modulus length used for certification request in a PKI domain

Modified feature: Displaying IPv4SG bindings

Modified feature: Displaying IPv6SG bindings

Modified feature: Displaying the MFF configuration for a VLAN

Modified feature: Associating Track with application modules

Modified feature: Specifying the length of ICMP echo requests sent by an IPv4 or IPv6 ping operation

Modified feature: Removing a TCP or UDP listening service for a VPN instance

Modified feature: Specifying the source IP address for NTP messages

New feature: Specifying the NTP time-offset thresholds for log and trap outputs

New feature: Specifying the SNTP time-offset thresholds for log and trap outputs

Modified feature: Creating a sampler

Modified feature: sFlow counter sampling

Modified feature: sFlow flow sampling

Modified feature: Configuring a backup PW for a cross-connect

Modified feature: Configuring a backup PW for a VSI

Modified feature: Change of the bandwidth limit value range for VSIs

Modified feature: Value range change for the broadcast, multicast, or unknown unicast restraint bandwidth of VSIs

Modified feature: Frame match criteria of VXLAN Ethernet service instances

Modified feature: Displaying EVPN routing table information

Modified feature: NETCONF logging

Modified feature: Specifying the role of the device in the VCF fabric

New features: Fundamentals features

Table 1 describes the fundamentals features added in this software version. For more information about the features and commands, see HPE FlexFabric 5950 Switch Series Fundamentals Configuration Guide and HPE FlexFabric 5950 Switch Series Fundamentals Command Reference.

Fundamentals features added in version R6301

| Feature | Command changes |
| --- | --- |
| RBAC: Automatically obtaining the login username when a login user requests temporary user role authorization from a remote authentication server | The super use-login-username command was added. |
| Login management: Logging for Telnet login attempts that are denied by the Telnet login control ACL | The telnet server acl-deny-log enable command was added. |
| Login management: Setting the Telnet service port numbers | The following commands were added: telnet server port port-number telnet server ipv6 port port-number |
| FTP: Logging for FTP login attempts that are denied by the FTP login control ACL | The ftp server acl-deny-log enable command was added. |
| File system management: Executing a batch file | The execute filename command was added. |
| Configuration file management: Automatic system-wide next-startup configuration file operations | The standby auto-update config command was added. |
| Configuration file management: Archiving the running configuration to a remote SCP server | The following commands were added: archive configuration server archive configuration server user archive configuration server password |
| ISSU: Terminating the ongoing ISSU process forcibly | The issu quit command was added. |
| Device management: Parity error alarm | The following commands were added: parity-error monitor log enable parity-error monitor period parity-error monitor threshold |
| Device management: Resource usage monitoring | The following commands were added: resource-monitor minor resend enable resource-monitor output resource-monitor resource |
| Device management: Setting the memory usage thresholds | The ratio, early-warning, and secure options were added to the memory-threshold command. |
| Device management: Displaying memory alarm thresholds and statistics | Early-warning threshold information was added to output from the display memory-threshold command. |
| Device management: Displaying the current CPU usage statistics | The core option was added to the display cpu-usage command. |
| Device management: Locating devices | The locator blink command was added. |
| Device management: Rebooting a subcard | The subslot subslot-number option was added to the reboot command. |
| Python: Comware extended Python API channel | N/A |
| Python: Comware extended Python API send | N/A |
| Python: Comware extended Python API SYSLOG | N/A |
| License management | All license management commands were newly added. |

New features: Virtual technologies features

Table 2 describes the virtual technologies features added in this software version. For more information about the features and commands, see HPE FlexFabric 5950 Switch Series Virtual technologies Configuration Guide and HPE FlexFabric 5950 Switch Series Virtual technologies Command Reference.

Virtual technologies features added in version R6301

| Feature | Command changes |
| --- | --- |
| Configuring ND MAD that uses management Ethernet ports | The mad nd enable command was added in management Ethernet interface view. |

New features: Layer 2—LAN switching features

Table 3 describes the Layer 2—LAN switching features added in this software version. For more information about the features and commands, see HPE FlexFabric 5950 Switch Series Layer 2—LAN Switching Configuration Guide and HPE FlexFabric 5950 Switch Series Layer 2—LAN Switching Command Reference.

Layer 2—LAN switching features added in version R6301

| Feature | Command changes |
| --- | --- |
| Ethernet interfaces: Displaying the status and packet statistics of interfaces | The display interface link-info command was added. |
| Ethernet interfaces: Displaying operating status and information of all interfaces except subinterfaces | The display interface main command was added. |
| Ethernet interfaces: Configuring PFC on Ethernet interfaces | The following commands were added to system view: priority-flow-control priority-flow-control no-drop dot1p |
| Ethernet interfaces: Configuring PFC deadlock detection | The following commands were added: priority-flow-control deadlock auto-recover action priority-flow-control deadlock auto-recover cos priority-flow-control deadlock threshold |
| Ethernet interfaces: Enabling fast retrain. | The port fast-retrain enable command was added. |
| Ethernet interfaces: Setting the interface connection distance | The port connection-distance command was added. |
| Ethernet interfaces: Enabling link flapping protection on an interface | The following commands were added: display link-flap protection link-flap protect enable port link-flap protect enable |
| Ethernet link aggregation: Disabling the default port selection action for dynamic aggregation groups | The lacp default-selected-port disable command was added. |
| Ethernet link aggregation: Configuring a dynamic aggregation group to use port speed as the prioritized criterion for reference port selection | The lacp select speed command was added. |
| Ethernet link aggregation: Setting a hash offset to adjust the load balancing hash results on link aggregations | The link-aggregation global load-sharing offset command was added. |
| Ethernet link aggregation: Setting the load sharing mode for tunneled traffic on link aggregations. | The link-aggregation global load-sharing tunnel command was added. |
| Ethernet link aggregation: Enabling an aggregation group to distribute traffic across the member links on a per-packet basis. | The per-packet keyword was added to the link-aggregation load-sharing mode command. |
| Ethernet link aggregation: Setting the minimum percentage of Selected ports in an aggregation group | The percentage number option was added to the link-aggregation selected-port minimum command. |
| Ethernet link aggregation: Setting the physical state change suppression interval on an aggregate interface | The link-delay command was added. |
| VLAN mapping: Configuring two-to-one VLAN mapping | The vlan mapping egress command was added. |
| VLAN mapping: Configuring network-side many-to-one VLAN mapping | The nni keyword was added to the vlan mapping command. |
| DRNI: Enabling DR system auto-recovery and set the reload delay timer | The drni auto-recovery reload-delay command was added. |
| DRNI: Excluding an interface from the shutdown action by DRNI MAD | The drni mad exclude interface command was added. |
| Spanning tree: Enabling dispute guard | The stp dispute-protection command was added. |
| LLDP: Setting the timeout for receiving LLDP frames and enable the device to report no LLDP neighbor events | The lldp timer rx-timeout command was added. |
| LLDP: Enabling advertisement of the management address TLV globally and set the management address to be advertised | The lldp global tlv-enable basic-tlv management-address-tlv command was added. |
| LLDP: Clearing LLDP statistics on interfaces | The reset lldp statistics command was added. |

New features: Layer 3—IP services features

Table 4 describes the Layer 3—IP services features added in this software version. For more information about the features and commands, see HPE FlexFabric 5950 Switch Series Layer 3—IP Services Configuration Guide and HPE FlexFabric 5950 Switch Series Layer 3—IP Services Command Reference.

Layer 3—IP services features added in version R6301

| Feature | Command changes |
| --- | --- |
| ARP: Enabling interface consistency check between ARP and MAC address entries | The arp mac-interface-consistency check enable command was added. |
| ARP: Setting the maximum number of probes for dynamic ARP entries | The arp timer aging probe-count command was added. |
| ARP: Setting the interval for probing dynamic ARP entries | The arp timer aging probe-interval command was added. |
| ARP: Enabling recording user IP address conflicts | The arp user-ip-conflict record enable command was added. |
| ARP: Enabling recording user port migrations | The arp user-move record enable command was added. |
| ARP: Displaying user IP address conflict records | The display arp user-ip-conflict record command was added. |
| ARP: Displaying user port migration records | The display arp user-move record command wadded. |
| ARP: Enabling ARP snooping in a VXLAN | The arp snooping enable command was added to VSI view. The vsi [vsi-name ] option was added to the following commands: display arp snooping reset arp snooping |
| ARP: Configuring a static ARP entry for a local site associated with the VXLAN gateway | The arp static ip-address mac-address vsi-interface vsi-interface-id interface-type interface-number service-instance instance-id vsi vsi-name [ vpn-instance vpn-instance-name ] command was added. |
| ARP: Setting the times and the interval for retransmitting a gratuitous ARP packet for the device MAC address change | The gratuitous-arp mac-change retransmit times interval seconds command was added. |
| DHCP server: Enabling the DHCP server to return a DHCP-NAK message if the client notions of their IP addresses are incorrect | The dhcp server request-ip-address check command was added. |
| DHCP: Binding the gateways to the DHCP server's MAC address in the address management module when the DHCP server assigns gateways to clients | The export-route keyword was added to the gateway-list command. |
| DHCP: Advertising the subnet assigned to DHCP clients when the DHCP server dynamically assigns IP addresses in the subnet to clients | The export-route keyword was added to the network command. |
| DHCP relay agent: Enabling the DHCP relay agent to insert Option 60 into DHCP requests | The dhcp relay insert option60 command was added. |
| DHCP relay agent: Specifying a DHCP relay address pool for DHCP clients | The dhcp relay pool command was added. |
| DHCP snooping: Specifying the append handling strategy for Option 82 in request messages | The append keyword was added to the dhcp snooping information strategy command. |
| DHCP snooping: Configuring the padding mode for the Vendor-Specific sub-option | The dhcp snooping information vendor-specific command was added. |
| DHCP relay agent: Configuring the DHCP relay agent to perform a MAC address table lookup for a DHCP reply if it does not have the request forwarding information for the reply | The dhcp relay mac-forward enable [ broadcast ] command was added. |
| DHCP snooping: Enabling DHCP snooping for a VLAN | The dhcp snooping enable vlan command was added. |
| DHCP snooping: Enabling recording of client information in DHCP snooping entries for a VLAN | The dhcp snooping binding record command was added to VLAN view. |
| DHCP snooping: Disabling DHCP snooping on an interface | The dhcp snooping disable command was added. |
| DHCP snooping: Configuring an interface in a VLAN as a DHCP snooping trusted port | The dhcp snooping trust interface command was added. |
| IP forwarding basics: Enabling symmetric load sharing | The ip load-sharing symmetric enable command was added. |
| IP addressing: Displaying IP configuration and statistics for interfaces of the specified interface type. | The display ip interface interface-type command was added. |
| IP performance optimization: Enabling the device to encapsulate the TCP Timestamps option in outgoing TCP packets | The tcp timestamps enable command was added. |
| DNS: Enabling DNS proxy | The dns proxy enable command was added. |
| DNS: Enabling DNS spoofing and specifying the IPv4 address for spoofing DNS requests | The dns spoofing ip-address [ vpn-instance vpn-instance-name ] command was added. |
| DNS: Enabling DNS spoofing and specifying the IPv6 address for spoofing DNS requests | The ipv6 dns spoofing ipv6-address [ vpn-instance vpn-instance-name ] command was added. |
| IPv6 basics: Configuring ND snooping | The following command were added: ipv6 nd snooping dad retrans-timer interval ipv6 nd snooping enable global ipv6 nd snooping enable link-local ipv6 nd snooping glean source ipv6 nd snooping lifetime { invalid invalid-lifetime ¦ valid valid-lifetime } ipv6 nd snooping max-learning-num max-number ipv6 nd snooping uplink display ipv6 nd snooping display ipv6 nd snooping count reset ipv6 nd snooping [ [ vlan vlan-id ] [ global ¦ link-local ] ¦ vlan vlan-id ipv6-address ] |
| IPv6 basics: Configuring ND snooping in a VXLAN | The following commands were added: ipv6 nd snooping enable global (VSI view) ipv6 nd snooping enable link-local (VSI view) display ipv6 nd snooping count vsi (any view) display ipv6 nd snooping vsi (any view) reset ipv6 nd snooping vsi (user view) |
| IPv6 basics: Enabling ND logging for user online and offline events | The ipv6 nd online-offline-log enable command was added. |
| IPv6 basics: Specifying the URL of the boot file in RA messages | The ipv6 nd ra boot-file-url command was added. |
| IPv6 basics: Specifying DNS suffix information to be advertised in RA messages | The ipv6 nd ra dns search-list command was added. |
| IPv6 basics: Enabling DNS suffix suppression in RA messages | The ipv6 nd ra dns search-list suppress command was added. |
| IPv6 basics: Specifying DNS server information to be advertised in RA messages | The ipv6 nd ra dns server command was added. |
| IPv6 basics: Enabling DNS server suppression in RA messages | The ipv6 nd ra dns server suppress command was added. |
| IPv6 basics: Enabling ND direct route advertisement | The ipv6 nd route-direct advertise command was added. |
| IPv6 basics: Specifying a prefix length for generating a network route for identified ND entries | The ipv6 nd route-direct prefix convert-length command was added. |
| IPv6 basics: Recording user IPv6 address conflicts | The following commands were added: ipv6 nd user-ip-conflict record enable display ipv6 nd user-ip-conflict record |
| IPv6 basics: Recording user port migrations | The following commands were added: ipv6 nd user-move record enable display ipv6 nd user-move record |
| IPv6 basics: Setting the aging timer for ND entries in stale state on an interface | The ipv6 neighbor timer stale-aging command was added. |
| IPv6 basics: Enabling Layer 3 packet statistics counting | The statistics l3-packet enable command was added. |
| DHCPv6: Enabling IPv6 address binding conversion for IP source guard. | The ipv6 dhcp server entry-convert enable command was added. |
| DHCPv6: Advertising the subnet assigned to DHCPv6 clients when the DHCPv6 server dynamically assigns IPv6 addresses in the subnet to clients | The export-route keyword was added to the network command. |
| DHCPv6 relay agent: Enabling the DHCPv6 relay agent to advertise host routes for IPv6 addresses assigned to DHCP clients | The ipv6 dhcp advertise address-route command was added. |
| DHCPv6 relay agent: Enabling the DHCPv6 relay agent to support Option 79 | The ipv6 dhcp relay client-link-address enable command was added. |
| DHCPv6 relay agent: Configuring the DHCPv6 relay agent to discard the DHCPv6 requests that are delivered from VXLAN tunnels | The ipv6 dhcp relay request-from-tunnel discard command was added. |
| DHCPv6 snooping: Disabling DHCP snooping | The ipv6 dhcp snooping disable command was added. |
| DHCPv6 snooping: Enabling DHCPv6 snooping for VLANs | The ipv6 dhcp snooping enable vlan command was added. |
| DHCPv6 snooping: Configuring a port in a VLAN as a DHCPv6 snooping trusted port | The ipv6 dhcp snooping trust interface command was added. |
| DHCPv6: Configuring DHCPv6 guard | The following commands were added: device-role display ipv6 dhcp guard policy if-match reply acl if-match server acl ipv6 dhcp guard apply policy ipv6 dhcp guard policy preference trust port |
| DHCPv6: Enabling the EUI-64 address allocation mode | The address-alloc-mode eui-64 command was added. |
| DHCPv6 snooping: Recording DHCPv6 snooping prefix entries | The following commands were added: ipv6 dhcp snooping pd binding record display ipv6 dhcp snooping pd binding reset ipv6 dhcp snooping pd binding |

New features: Layer 3—IP routing features

Table 5 describes the Layer 3—IP routing features added in this software version. For more information about the features and commands, see HPE FlexFabric 5950 Switch Series Layer 3—IP Routing Configuration Guide and HPE FlexFabric 5950 Switch Series Layer 3—IP Routing Command Reference.

Layer 3—IP routing features added in version R6301

| Feature | Command changes |
| --- | --- |
| IP routing basics: Clearing IPv4/IPv6 route statistics for all VPN instances or for the public network and all VPN instances | The all-routes and all-vpn-instance keywords were added to the following commands: reset ip routing-table statistics protocol reset ipv6 routing-table statistics protocol |
| IP routing basics: Displaying routing table information for all VPN instances or for the public network and all VPN instances | The all-routes and all-vpn-instance keywords were added to the following commands: display ip routing-table display ipv6 routing-table |
| IP routing basics: Enabling the RIB to flush route attribute information to the FIB | The flush route-attribute command was added. |
| IP routing basics: Setting the maximum number of active IPv4/IPv6 routes supported by the device. | The routing-table limit command was added. |
| Static routing: Specifying the index of the next hop when creating a static route. | The nexthop-index index-string option was added to the ip route-static command. |
| Static routing: Enabling periodic sending of ARP requests to the next hops of static routes. | The ip route-static arp-request command was added. |
| OSPF: Displaying OSPF neighbor state change log information for the specified neighbor on the specified IRF member device. | The neighbor-id argument and slot slot-number option were added to the display ospf event-log command. |
| OSPF: Displaying information about the hello packets sent to or received from neighbors. | The hello keyword was added to the display ospf peer command. |
| OSPF: Displaying statistics for sent or received hello packets. | The hello keyword was added to the display ospf statistics command. |
| OSPF: Displaying log information about received or sent hello packets. | The display ospf event-log hello command was added. |
| OSPF: Displaying information about hello packets sent by OSPF interfaces. | The display ospf interface hello command was added. |
| OSPF: Enabling OSPF to limit the LSU transmit rate | The ospf lsu-flood-control command was added. |
| OSPF: Clearing OSPF neighbor state change information about an IRF member device. | The slot keyword was added to the reset ospf event-log command. |
| OSPF: Clearing OSPF log information about received or sent hello packets. | The reset ospf event-log hello command was added. |
| BGP: Specifying the period after reboot within which the startup policy is effective | The bgp apply-policy on-startup duration command was added. |
| BGP: Setting the MED attribute value in the startup policy | The bgp policy on-startup med command was added. |
| BGP: Enabling BGP to send withdrawal messages of the default route prior to other routes | The default-route update-first command was added. |
| BGP: Setting the BGP route sending rate | The route-rate-limit command was added. |
| BGP: Disabling BGP session establishment with all peers and peer groups | The ignore all-peers command was added. |
| BGP: Disabling BGP session establishment with a peer or peer group | The graceful, community, local-preference, and med keywords were added to the peer ignore command. |
| BGP: Setting the session retry timer | The following commands were added: timer connect-retry peer timer connect-retry |
| BGP: Disabling route recursion policy control for routes received from the specified peer or peer group | The peer nexthop-recursive-policy disable command was added. |
| BGP: BGP ORF capabilities negotiation | The following commands were added: peer capability-advertise orf non-standard peer capability-advertise orf prefix-list display bgp peer received prefix-list |
| BGP: BGP additional path feature | The following commands were added: additional-paths select-best peer additional-paths peer advertise additional-paths best The Add-Path related fields were added to the output from the following commands: display bgp routing-table ipv4 unicast display bgp routing-table ipv6 unicast display bgp routing-table vpnv4 display bgp routing-table vpnv6 |
| BGP: BGP IPv4 RT filter related features | The following commands were added: address-family ipv4 rtfilter display bgp routing-table ipv4 rtfilter The rtfilter keyword was added to the following commands: display bgp group display bgp peer display bgp update-group refresh bgp reset bgp |
| BGP: Configuring BGP features in BGP IPv4 RT filter address family view | Support for BGP IPv4 RT filter address family view was added to the following commands: peer default-route-advertise peer enable peer reflect-client reflect between-clients reflector cluster-id |
| BGP: Enabling BGP to prefer routes with an IPv6 next hop during optimal route selection | The bestroute ipv6-nexthop command was added. |
| BGP: Enabling BGP to ignore router IDs during optimal route selection. | The bestroute router-id-ignore command was added. |
| BGP: Enabling BGP to immediately send route updates for routes that match an IPv6 prefix list | The bgp update-delay on-startup ipv6-prefix-list command was added. |
| BGP: Specifying existent and nonexistent policies to control route advertisement | The following commands were added: peer advertise-policy exist-policy peer advertise-policy non-exist-policy |
| BGP: Enabling BGP to ignore the first AS number of EBGP route updates received from a peer or peer group | The peer ignore-first-as command was added. |
| BGP: Enabling the route reflector to change the attributes of routes to be reflected | The reflect change-path-attribute command was added. |
| BGP: Configuring optimal route selection delay | The route-select delay command was added. |
| BGP: Applying route update interval setting to withdrawn routes | The route-update-interval withdrawn enable command was added. |
| BGP: Configuring RPKI related features | The following commands were added: rpki server tcp port passwords refresh-time response-time purge-time check-origin-validation peer advertise origin-as-validation bestroute origin-as-validation reset bgp rpki server display bgp rpki server display bgp rpki table |
| BGP: Configuring BGP BMP related features | The following commands were added: server connect-interface server vpn-instance route-mode adj-rib-out route-mode loc-rib |
| BGP: Enabling BGP to send withdrawal messages of routes matching the specified routing policy prior to other routes | The update-first route-policy command was added. |
| BGP: Displaying BGP IPv4 unicast routes, IPv4 multicast routes, IPv6 unicast routes, and IPv6 multicast routes that match an AS path list specified by its name. | The as-path-acl-name argument was added to the following commands: display bgp routing-table ipv4 unicast display bgp routing-table ipv4 multicast display bgp routing-table ipv6 unicast display bgp routing-table ipv6 multicast |
| BGP: Displaying BGP update group information for IPv6 EVPN address family. | The ipv6-address argument was added to the display bgp update-group l2vpn evpn command. |
| BGP: Displaying BGP update group information for MVPN address family | The mvpn keyword was added to the display bgp update-group ipv4 command. |
| BGP: Displaying route flap statistics for BGP routes that match an AS path list specified by its name. | The as-path-acl-name argument was added to the display bgp routing-table flap-info command. |
| BGP: Specifying an IPv4/IPv6 ACL by its name for the policy used to filter BGP routes. | The name ipv4-acl-name and name ipv6-acl-name options were added to the following commands: filter-policy import filter-policy export peer filter-policy |
| BGP: Removing the specified routing policy applied to routes incoming from or outgoing to a peer or peer group | The route-policy-name argument was added to the undo peer route-policy command. |
| BGP: Removing an IPv6 prefix list specified to filter routes received from or advertised to a peer or peer group | The ipv6-prefix-list-name argument was added to the undo peer prefix-list command. |
| BGP: Redistributing the local network specified in the public instance or a VPN instance | The import-route local-aggregate command was added. |
| BGP: Specifying an AS path list by its name to filter routes incoming from or outgoing to a peer or peer group | The as-path-acl-name argument was added to the peer as-path-acl command. |
| BGP: Support for IPv6 BGP peers of specific commands | The ipv6-address prefix-length argument was added to the following commands in BGP EVPN address family view: peer enable peer next-hop-local peer reflect-client peer route-policy peer advertise-community peer allow-as-loop |
| BGP: Configuring BGP features in BGP MVPN address family view | Support for BGP MVPN address family view was added to the following commands: peer reflect-client reflect between-clients reflector cluster-id refresh bgp reset bgp |
| BGP: Displaying information about dynamic IPv6 peers or peer groups for the EVPN address family and resetting or soft resetting the BGP session to an IPv6 peer for the EVPN address family | The ipv6-address prefix-length argument was added to the following commands: display bgp peer l2vpn evpn refresh bgp l2vpn evpn reset bgp l2vpn evpn |
| BGP: Clearing flap statistics for BGP routes that match an AS path list specified by its name. | The as-path-acl-name argument was added to the reset bgp flap-info command. |
| Policy-based routing: Setting the action that drops matching packets when all next hops on a policy node are invalid | The apply fail-action-drop next-hop command was added. |
| Policy-based routing: Specifying NULL0 as the output interface for IP packets | The apply output-interface NULL0 command was added. |
| Policy-based routing: Configuring a description for a policy node | The description text command was added. |
| Policy-based routing: Setting a local QoS ID match criterion for IP packets | The qppb-manipulation keyword was added to the if-match qos-local-id command. |
| IPv6 policy-based routing: Setting the action that drops matching IPv6 packets when all next hops on an IPv6 policy node are invalid | The apply fail-action-drop next-hop command was added. |
| IPv6 policy-based routing: Specifying NULL0 as the output interface for IPv6 packets | The apply output-interface NULL0 command was added. |
| IPv6 policy-based routing: Configuring a description for an IPv6 policy node | The description text command was added. |
| IPv6 policy-based routing: Setting a local QoS ID match criterion for IPv6 packets | The qppb-manipulation keyword was added to the if-match qos-local-id command. |
| Routing policy: Setting the SoO extended community attribute for BGP routes | The apply extcommunity soo command was added. |
| Routing policy: Matching BGP EVPN routes that have the specified L3VNI and setting an L3VNI for BGP EVPN routes | The following commands were added: if-match l3-vni apply l3-vni |
| Routing policy: Setting the BGP RPKI validation state match criterion | The if-match rpki command was added. |
| Routing policy: Configuring a route distinguisher (RD) list, matching routes whose RD matches the specified RD list, and displaying RD list information | The following commands were added: ip rd-list if-match rd-list display ip rd-list |
| Routing policy: Clearing BGP AS path list statistics | - The reset ip as-path command was added. |
| Routing policy: Setting the routing policy change delay timer | The route-policy-change delay-time command was added. |
| Routing policy: Matching IPv6 prefixes with the specified length whose last bit is an even number. | The if-match ipv6 even-prefix-length command was added. |
| Routing policy: Matching IPv6 prefixes with the specified length whose last bit is an odd number. | The if-match ipv6 odd-prefix-length command was added. |
| Routing policy: Matching BGP routes whose AS_PATH attribute matches an AS path list specified by its name, configuring an AS path list and specifying its name, and displaying information about a BGP AS path list specified by its name. | The as-path-name argument was added to the following commands: if-match as-path ip as-path display ip as-path |
| Routing policy: Matching BGP EVPN IMET routes, BGP EVPN IP prefix advertisement routes, and BGP EVPN MAC/IP advertisement routes. | The bgp-evpn-imet, bgp-evpn-ip-prefix, and bgp-evpn-mac-ip keywords were added to the if-match route-type command. |

New features: IP multicast features

Table 6 describes IP multicast features added in this software version. For more information about the features and commands, see HPE FlexFabric 5950 Switch Series IP Multicast Configuration Guide and HPE FlexFabric 5950 Switch Series IP Multicast Command Reference.

IP multicast features added in version R6301

| Feature | Command changes |
| --- | --- |
| Setting the DSCP value in multicast protocol packets | The dscp command was added to IGMP snooping view, IGMP view, MSDP view, MLD snooping view, MLD view, and IPv6 PIM view. |
| IGMP snooping: Sending IGMP general queries upon a path change | The send-query enable command was added. |
| IGMP snooping: Displaying information about dynamic IGMP snooping group entries | The display igmp-snooping group command was added. |
| IGMP snooping: Enabling IGMP snooping querier election for a VLAN or VSI | The igmp-snooping querier-election command was added. |
| Multicast routing and forwarding: Mtrace | The following commands were added: mtrace v1 mtrace v2 mtrace-service port |
| Multicast routing and forwarding: Setting the maximum number of cached unknown multicast packets | The following commands were added: multicast forwarding-table cache-unknown per-entry multicast forwarding-table cache-unknown total |
| PIM: Display PIM routing entries for MVPN extranet | The extranet { source-vpn-instance source-vpn-instance-name ¦ source-public-instance ¦ receive-vpn-instance receive-vpn-instance-name ¦ receive-public-instance option was added to the display pim routing-table command. |
| Multicast VPN: BGP IPv4 MVPN | The following commands were added: address-family ipv4 mvpn display bgp routing-table ipv4 mvpn policy vpn-target |
| Multicast VPN: Adding attributes Source AS Extended Community and RF Route Import Extended Community to routes that are sent to the BGP IPv4 MVPN neighbors | The mvpn-advertise-rt-import command was added. |
| Multicast VPN: Configuring an MVPN extranet RPF selection policy | The following commands were added: multicast extranet select-rpf ipv6 multicast extranet select-rpf |
| MLD snooping: Sending MLD general queries upon a path change | The send-query enable command was added. |
| MLD snooping: Displaying information about dynamic MLD snooping group entries | The display mld-snooping group command was added. |
| MLD snooping: Enabling MLD snooping querier election for a VLAN or VSI | The mld-snooping querier-election command was added. |
| IPv6 multicast routing and forwarding: IPv6 mtrace | The following commands were added: ipv6 mtrace-service port mtrace v2 ipv6 |
| IPv6 multicast routing and forwarding: Setting the maximum number of cached unknown IPv6 multicast packets | The following commands were added: ipv6 multicast forwarding-table cache-unknown per-entry ipv6 multicast forwarding-table cache-unknown total |
| IPv6 PIM: Embedded RP | The embedded-rp command was added. |

New features: MPLS features

Table 7 describes the security features added in this software version. For more information about the features and commands, see HPE FlexFabric 5950 Switch Series MPLS Configuration Guide and HPE FlexFabric 5950 Switch Series MPLS Command Reference.

MPLS features added in version R6301

| Feature | Command changes |
| --- | --- |
| Basic MPLS: Displaying information about IS-IS SRLSPs | The isis keyword was added to the display mpls lsp command. |
| Static LSP: Displaying information about static LSPs deployed by OpenFlow | The openflow keyword is added to the display mpls static-lsp command. |
| MPLS TE: Setting the maximum link bandwidth and maximum reservable bandwidth in percentage for MPLS TE traffic | The percent parameters were added to the following commands: mpls te max-link-bandwidth mpls te max-reservable-bandwidth mpls te max-reservable-bandwidth mam mpls te max-reservable-bandwidth rdm |
| Tunnel policy: Displaying tunnel policy information | The display tunnel-policy command was added. |
| MPLS L3VPN: Specifying the local network to be advertised in the public instance or a VPN instance | The network command was added. |
| MPLS L3VPN: Replicating routes from a VPN instance to the public network | The route-replicate from vpn-instance command was added to public instance IPv4 address family view and public instance IPv6 address family view. |
| MPLS L3VPN: Creating an OSPF sham link with the None authentication mode | The authentication-none keyword was added to the sham-link command. |
| MPLS L3VPN: Enabling a VPN instance to replicate routes from the public network or other VPN instances | The route-replicate command was added to VPN instance IPv6 address family view. |
| MPLS L3VPN: Enabling ECMP VPN route redistribution | The vpn-route cross multipath command was added. |
| MPLS L2VPN: Configuring the master/slave PW redundancy mode | The pw-redundancy command was added. |
| VPLS: Setting the maximum bandwidth for traffic on an AC | The bandwidth command was added to service instance view. |
| VPLS: Setting the expected bandwidth for a PW | The bandwidth command was added to VSI LDP PW view and VSI static PW view. |
| VPLS: Setting the maximum bandwidth for traffic in a VSI | The bandwidth command was added to VSI view. |
| VPLS: Configuring the master/slave PW redundancy mode | The pw-redundancy command was added. |
| MPLS OAM: PW connectivity verification | The following commands were added: bfd discriminator display l2vpn pw bfd ping mpls pw vccv bfd vccv cc |
| MPLS SR: Configuring an IS-IS prefix SID | The isis prefix-sid command was added. |
| MPLS SR: Enabling MPLS SR adjacency label allocation | The segment-routing adjacency enable command was added. |
| MPLS SR: Configuring the MPLS SRGB | The segment-routing global-block command was added. |
| MPLS SR: Enabling MPLS SR | The segment-routing mpls command was added. |
| MPLS SR: Configuring the device to prefer SRLSPs in traffic forwarding | The segment-routing sr-prefer command was added. |
| MPLS SR: Configuring a prefix segment for static MPLS SR | The static-sr-mpls prefix command was added. |

New features: ACL and QoS features

Table 8 describes the security features added in this software version. For more information about the features and commands, see HPE FlexFabric 5950 Switch Series ACL and QoS Configuration Guide and HPE FlexFabric 5950 Switch Series ACL and QoS Command Reference.

ACL and QoS features added in version R6301

| Feature | Command changes |
| --- | --- |
| ACL: Applying an ACL to a VLAN interface to filter packets | The packet-filter vlan-interface command was added. The vlan-interface parameter was added to the following commands: display packet-filter statistics display packet-filter verbose reset acl counter reset packet-filter statistics |
| ACL: Matching the local QoS ID in an advanced ACL rule | The qos-local-id parameter was added to the following commands: rule (IPv4 advanced ACL view) rule (IPv6 advanced ACL view) |
| ACL: Matching VXLAN packets in a rule | The following command was added to IPv4 advanced ACL view: rule [ rule-id ] { deny ¦ permit } vxlan [ destination { dest-address dest-wildcard ¦ any } ¦ source { source-address source-wildcard ¦ any } ¦ source-port operator port1 [ port2 ] ¦ vxlan-id vxlan-id ] * inner-protocol inner-protocol [ counting ¦ inner-destination { dest-address dest-wildcard ¦ any } ¦ inner-destination-port operator port1 [ port2 ] ¦ inner-established ¦ inner-source { source-address source-wildcard ¦ any } ¦ inner-source-port operator port1 [ port2 ] ¦ [ logging ] ¦ time-range time-range-name ] * - The following command was added to Layer 2 ACL view: rule [ rule-id ] { deny ¦ permit } vxlan [ counting ¦ dest-mac dest-address dest-mask ¦ inner-dest-mac inner-dest-address inner-dest-mask ¦ inner-source-mac inner-source-address inner-source-mask ¦ inner-type inner-protocol-type inner-protocol-type-mask ¦ source-mac source-address source-mask ¦ time-range time-range-name ¦ type protocol-type protocol-type-mask ¦ vxlan-id vxlan-id ] * |
| QoS: Matching the inner header information of VXLAN packets | The inner keyword was added to the if-match acl command. |
| QoS: Configuring a description for a traffic class | The description command was added. |
| QoS: Enabling MAC address learning for a traffic behavior | The mac-address mac-learning enable command was added. |
| QoS: Clearing queue-based outbound traffic statistics for interfaces | The reset qos queue-statistics interface outbound command was added. |
| QoS: Configuring trusting the 802.1p priority in the outer IP header of VXLAN packets | - The qos trust tunnel-dot1p command was added. |
| QoS: Applying accounting-type QoS policies and marking-type QoS policies globally | The accounting and remarking keywords were added to the following commands: qos apply policy global display qos policy diagnosis global display qos policy global reset qos policy global |

New features: Security features

Table 9 describes the security features added in this software version. For more information about the features and commands, see HPE FlexFabric 5950 Switch Series Security Configuration Guide and HPE FlexFabric 5950 Switch Series Security Command Reference.

Security features added in version R6301

| Feature | Command changes |
| --- | --- |
| AAA: Configuring password control attributes for network access users | The following commands were added to network access users: password-control length password-control complexity password-control composition |
| AAA: Configuring an EAP profile | The following commands were added: eap-profile method ca-file |
| AAA: Support for EAP-based RADIUS server status detection | The eap-profile eap-profile-name option was added to the radius-server test-profile command. |
| AAA: Specifying a source interface for outgoing RADIUS packets | The interface interface-type interface-number option was added to the following commands: radius nas-ip nas-ip (RADIUS scheme view) |
| AAA: Specifying a source interface for outgoing HWTACACS packets | The interface interface-type interface-number option was added to the following commands: hwtacacs nas-ip nas-ip (HWTACACS scheme view) |
| AAA: Specifying a RADIUS server selection mode for reauthentication | The reauthentication server-select command was added. |
| AAA: Setting the version of RADIUS server status change MIB nodes | The radius trap-version command was added. |
| AAA: Disabling the RADIUS service | - The [ undo ] radius enable command was added. |
| AAA: Displaying authentication and accounting load statistics for all RADIUS servers | The display radius server-load statistics command was added. |
| AAA: Clearing history authentication and accounting load statistics for all RADIUS servers | The reset radius server-load statistics command was added. |
| AAA: Configuring the AAA test feature | The following commands were added: radius attribute-test-group include exclude test-aaa |
| AAA: Configuring a connection recording policy | The following commands were added: aaa connection-recording policy accounting hwtacacs-scheme display aaa connection-recording policy |
| AAA: Forcibly sending stop-accounting RADIUS packets | The stop-accounting-packet send-force command was added. |
| AAA: Specifying a RADIUS or HWTACACS server by its host name | The host-name argument was added to the following commands: primary authentication (RADIUS scheme view) primary accounting (RADIUS scheme view) secondary authentication (RADIUS scheme view) secondary accounting (RADIUS scheme view) state secondary (RADIUS scheme view) primary authentication (HWTACACS scheme view) primary authorization (HWTACACS scheme view) primary accounting (HWTACACS scheme view) secondary accounting (HWTACACS scheme view) secondary authorization (HWTACACS scheme view) secondary authentication (HWTACACS scheme view) |
| AAA: Configuring a password in a test profile for detecting the RADIUS server status | The password { cipher ¦ simple } string parameter was added to the radius-server test-profile command. |
| 802.1X: Sending EAP-Success packets on assignment of users to the 802.1X Auth-Fail VLAN or VSI | The dot1x auth-fail eapol command was added. |
| 802.1X: Configuring 802.1X unauthenticated user aging | The dot1x unauthenticated-user aging enable command was added. The user-aging { auth-fail-vlan ¦ auth-fail-vsi ¦ critical-vlan ¦ critical-vsi ¦ guest-vlan ¦ guest-vsi } aging-time-value parameters were added to the dot1x timer command. |
| 802.1X: Enabling 802.1X online user synchronization | The dot1x server-recovery online-user-sync command was added. |
| 802.1X: Configuring 802.1X offline detection | The dot1x offline-detect enable command was added. The offline-detect offline-detect-value option was added to the dot1x timer command. |
| 802.1X: Setting the maximum size of EAP-TLS fragments sent to the server | The dot1x eap-tls-fragment to-server command was added. |
| 802.1X: Logging off 802.1X users | The reset dot1x access-user command was added. |
| 802.1X: CAR attribute assignment | N/A |
| 802.1X: Displaying the MAC addresses of 802.1X users in a type of 802.1X VLAN or VSI. | The display dot1x mac-address command was added. |
| 802.1X: Enabling 802.1X user logging | The dot1x access-user log enable command was added. |
| 802.1X: Configuring 802.1X MAC address binding | The following commands were added: dot1x mac-binding dot1x mac-binding enable |
| 802.1X: Support for VSI manipulation | The following commands were added: dot1x auth-fail vsi dot1x critical vsi dot1x guest-vsi dot1x guest-vsi-delay reset dot1x guest-vsi |
| 802.1X: Configuring 802.1X on Layer 2 aggregate interfaces | The following commands were added to Layer 2 aggregate interface view: dot1x dot1x auth-fail vsi dot1x critical vsi dot1x guest-vsi dot1x max-user |
| MAC authentication: Specifying an authentication method for MAC authentication | The mac-authentication authentication-method command was added. |
| MAC authentication: Configuring user aging for unauthenticated MAC authentication users | The mac-authentication unauthenticated-user aging enable command was added. The user-aging { critical-vlan ¦ critical-vsi ¦ guest-vlan ¦ guest-vsi } aging-time-value parameters were added to the mac-authentication timer command in system view. |
| MAC authentication: Configuring MAC authentication offline detection for a MAC authentication user | The mac-authentication offline-detect mac-address command was added. |
| MAC authentication: Enabling online user synchronization for MAC authentication | The mac-authentication server-recovery online-user-sync command was added. |
| MAC authentication: Specifying an ACL-based filter to identify source IP addresses that can or cannot trigger MAC authentication | The exclude-ip acl acl-number option was added to the mac-authentication carry user-ip command. |
| MAC authentication: Logging off MAC authentication users | The reset mac-authentication access-user command was added. |
| MAC authentication: CAR attribute assignment | N/A |
| MAC authentication: Displaying the MAC addresses of MAC authentication users in a type of MAC authentication VLAN or VSI. | The display mac-authentication mac-address command was added. |
| MAC authentication: Enabling MAC authentication user logging | The mac-authentication access-user log enable command was added. |
| MAC authentication: Support for VSI manipulation | The following commands were added: mac-authentication critical vsi mac-authentication guest-vsi mac-authentication guest-vsi auth-period reset mac-authentication critical vsi reset mac-authentication guest-vsi |
| MAC authentication: Configuring MAC authentication on Layer 2 aggregate interfaces | The following commands were added to Layer 2 aggregate interface view: mac-authentication mac-authentication critical vsi mac-authentication guest-vsi mac-authentication guest-vsi auth-period mac-authentication max-user |
| Portal: Enabling the device to check the issuing of category-2 portal filtering rules | The portal user-rule assign-check enable command was added. |
| Portal: Configuring the NAS-Port-Type attribute carried in outgoing RADIUS requests | The portal nas-port-type command was added. |
| Portal: Support for Web proxy | The portal web-proxy port command was added. |
| Web authentication | All Web authentication commands were newly added. |
| Triple authentication | N/A |
| Port security: Support for ntkauto mode of the need to known (NTK) feature | The ntkauto keyword was added to the port-security ntk-mode command. |
| Port security: Setting the block timer for blocked MAC addresses | The port-security timer blockmac command was added. |
| Port security: Logging the first access attempt from a new MAC address in a VLAN after port security's MAC address limit for that VLAN is reached | The vlan-mac-limit keyword was added to the port-security access-user log enable command. |
| Port security: Enabling port security user logging | The port-security access-user log enable command was added. |
| Port security: Enabling the quiet timer for 802.1X or MAC authentication users that are logged off by the authorization-fail-offline feature | The quiet-period keyword was added to the port-security authorization-fail offline command. |
| Port security: Setting port security's limit on the number of MAC addresses for specific VLANs on a port | The port-security mac-limit command was added. |
| Port security: Setting the secure MAC aging timer in seconds | The second keyword was added to the port-security timer autolearn aging command. |
| Port security: Configuring the escape critical VSI feature | The following commands were added: port-security global escape critical-vsi port-security escape critical-vsi |
| Password control: Setting the user authentication timeout time | The password-control authentication-timeout command was added. |
| Password control: Enabling the password change at first login feature | The password-control change-password first-login enable command was added. |
| Password control: Enabling password control globally for network access users | The network-class keyword was added to the password-control enable command. |
| Password control: Deleting history password records for network access users | The network-class [ user-name user-name ] parameters were added to the reset password-control history-record command. |
| IPsec: Specifying the ECDSA signature authentication method for an IKE proposal | The ecdsa-signature keyword was added to the authentication-method command. |
| IPsec: Specifying the DH group group19, group20, or group24 to be used for key negotiation in IKE phase 1 | In non-FIPS mode, the group19 and group20 keywords were added to the dh command. In FIPS mode, the group19, group20, and group24 keywords were added to the dh command. |
| SSH: Specifying keyboard-interactive authentication | The keyboard-interactive keyword was added to the ssh user command. |
| SSH: Support for non-interactive method to connect to the SCP server | The no-more-input keyword was added to the scp and scp ipv6 commands. |
| SSH: Disconnecting SSH sessions | The free ssh { user-ip { ip-address ¦ ipv6 ipv6-address } [ port port-number ] ¦ user-pid pid-number ¦ username username } command was added. |
| SSH: Logging for SSH login attempts that are denied by the SSH login control ACL | The ssh server acl-deny-log enable command was added. |
| SSH: Enabling SSH algorithm renegotiation and key re-exchange | The ssh server key-re-exchange enable command was added. |
| SSH: Specifying the SSH service port | The ssh server port command was added. |
| SSH: Deleting server public keys saved in the public key file on the Stelnet client | The delete ssh client server-public-key command was added. |
| SSH: Displaying server public key information saved in the public key file on the SSH client | The display ssh client server-public-key command was added. |
| SSH: Specifying and displaying the source IP address for outgoing SCP packets | The following commands were added: display scp client source scp client source scp client ipv6 source |
| SSL: Enabling the SSL server to send the complete certificate chain to the client during SSL negotiation | The certificate-chain-sending enable command was added. |
| Attack detection and prevention: Applying an attack defense policy to an interface | The attack-defense apply policy command was added. |
| Attack detection and prevention: Enabling the blacklist feature on an interface | The blacklist enable command was added. |
| IP source guard: Displaying IPv6SG prefix bindings | The display ipv6 source binding pd command was added. |
| ARP attack protection: Enabling logging for source MAC-based ARP attack detection | The arp source-mac log enable command was added. |
| ARP attack protection: Support for specifying an IRF member device if the specified interface is a virtual interface when displaying ARP attack entries detected by source MAC-based ARP attack detection | The slot slot-number option can be specified following the interface interface-type interface-number option in the display arp source-mac command if the specified interface is a virtual interface. |
| ARP attack protection: Specifying the maximum number of ARP attack detection logs for each log output | The number number option was added to the arp detection log enable command. |
| ARP attack protection: Specifying the rate at which the device sends ARP requests for ARP scanning | The send-rate pps option was added to the arp scan command. |
| ARP attack protection: Setting the interval for sending ARP detection logs to the information center | The interval interval option was added to the arp detection log enable command. |
| ND attack defense: Enabling ND attack detection logging | The ipv6 nd detection log enable command was added. |
| ND attack defense: ND attack detection | The following commands were added: ipv6 nd detection enable ipv6 nd detection trust display ipv6 nd detection statistics reset ipv6 nd detection statistics |
| MFF | All MFF commands were newly added. |
| MACsec: Setting the MKA life time | The mka timer mka-life command was added. |

New features: High availability features

Table 10 describes the high availability features added in this software version. For more information about the features and commands, see HPE FlexFabric 5950 Switch Series High Availability Configuration Guide and HPE FlexFabric 5950 Switch Series High Availability Command Reference.

High availability features added in version R6301

| Feature | Command changes |
| --- | --- |
| RRPP: Setting the link-up delay timer | The linkup-delay-timer command was added. |
| DLDP: Enabling DLDP on a port | The initial-unidirectional-delay keyword was added to the dldp enable command. |
| BFD: Configuring the BFD authentication mode for single-hop BFD control packets | The hmac-md5, hmac-mmd5, hmac-msha1, and hmac-sha1 keywords were added to the bfd authentication-mode command. |
| BFD: Configuring BFD session flapping suppression | The bfd dampening command was added. |
| BFD: Creating a BFD session for detecting the local interface state | The template template-name option was added to the bfd detect-interface source-ip command. |
| BFD: Configuring the timer that delays reporting the first BFD session establishment failure to the data link layer | The bfd detect-interface first-fail-timer command was added. |
| BFD: Enabling special processing for BFD sessions | The bfd detect-interface special-processing command was added. |
| BFD: Configuring the authentication mode for multihop BFD control packets | The hmac-md5, hmac-mmd5, hmac-msha1, and hmac-sha1 keywords were added to the bfd multi-hop authentication-mode command. |
| BFD: Setting the local discriminator for the reflector | The sbfd local-discriminator command was added. |
| BFD: Displaying SBFD session information | The display sbfd session command was added. |
| VRRP: Specifying IPv4 VRRP control VLANs. | The vrrp dot1q command was added. |
| VRRP: Specifying IPv6 VRRP control VLANs | The vrrp ipv6 dot1q command was added. |
| VRRP: Specifying IPv4 VRRP control VLANs in 25GE interface view | Support for 25GE interface view was added to the vrrp dot1q command. |
| VRRP: Specifying IPv6 VRRP control VLANs in 25GE interface view | Support for 25GE interface view was added to the vrrp ipv6 dot1q command. |
| Track: Creating a track entry and associating it with the neighbor availability status of an LLDP interface | The track lldp neighbor command was added. |
| Track: Configuring a tracked list. | The following commands were added: - delay - object - threshold percentage - threshold weight - track list boolean - track list threshold percentage - track list threshold weight |

New features: Network management and monitoring features

Table 11 describes the network management and monitoring features added in this software version. For more information about the features and commands, see HPE FlexFabric 5950 Switch Series Network Management and Monitoring Configuration Guide and HPE FlexFabric 5950 Switch Series Network Management and Monitoring Command Reference.

Network management and monitoring features added in R6301

| Feature | Command changes |
| --- | --- |
| Information center: Displaying information about log output filters. | The display info-center filter command was added. |
| Information center: Displaying the content of the log file buffer. | The display logfile buffer command was added. |
| Information center: Displaying the content of the security log file buffer. | The display security-logfile buffer command was added. |
| Information center: Creating a log output filter. | The info-center filter filter-name { module-name ¦ default } { deny ¦ level severity } command was added. |
| Information center: Specifying a log host and configuring output parameters. | The filter filter-name option was added to the info-center loghost command. |
| Information center: Displaying log buffer information and buffered logs. | The last-mins mins option was added to the display logbuffer command. |
| Information center: Specifying a log host and to configure output parameters. | The dscp dscp-value option was added to the info-center loghost [ vpn-instance vpn-instance-name ] { hostname ¦ ipv4-address ¦ ipv6 ipv6-address } [ port port-number ] [ dscp dscp-value ] [ facility local-number ]command. |
| SNMP: Creating an SNMP community, group, or user. | Support for specifying an advanced ACL was added to the following commands: snmp-agent community snmp-agent group snmp-agent usm-user { v1 ¦ v2c } snmp-agent usm-user v3 |
| SNMP: Setting the DSCP priority for SNMP responses. | The snmp-agent packet response dscp command was added. |
| SNMP: Configuring an SNMP notification target host. | The dscp dscp-value option was added to the following commands: In non-FIPS mode: snmp-agent target-host trap address udp-domain { ipv4-address ¦ ipv6 ipv6-address } [ udp-port port-number ] [ dscp dscp-value ] [ vpn-instance vpn-instance-name ] params securityname security-string [ v1 ¦ v2c ¦ v3 [ authentication ¦ privacy ] ] In FIPS mode: snmp-agent target-host trap address udp-domain { ipv4-address ¦ ipv6 ipv6-address } [ udp-port port-number ] [ dscp dscp-value ] [ vpn-instance vpn-instance-name ] params securityname security-string v3 { authentication ¦ privacy } |
| EAA: Adding a reboot action to a monitor policy. | The subslot keyword was added to the action reboot command. |
| Ansible | N/A |
| NETCONF: Specifying the error-when-rollback attribute in the <Configuration> element. | N/A |
| NETCONF: Configuring the device to use module-specific namespaces. | The netconf capability specific-namespace command was added. |

New features: FC and FCoE features

Table 12 describes the security features added in this software version. For more information about the features and commands, see HPE FlexFabric 5950 Switch Series FC and FCoE Configuration Guide and HPE FlexFabric 5950 Switch Series FC and FCoE Command Reference.

FC and FCoE features added in version R6301

| Feature | Command changes |
| --- | --- |
| Enabling area ID-to-F_Port binding | The following commands were added: fc port-bind-area enable display fc port-bind-area [ vsan vsan-id ] |
| Enabling BB_Credit recovery for an FC interface | The fcb2bcredit recovery enable command was added. The following information was added to the display interface fc command output: BB_Credit Recovery is enabled. BB_Credit Recovery is disabled. |

New features: OpenFlow features

Table 13 describes OpenFlow features added in this software version. For more information about the features and commands, see HPE FlexFabric 5950 Switch Series OpenFlow Configuration Guide and HPE FlexFabric 5950 Switch Series OpenFlow Command Reference.

OpenFlow features added in version R6301

| Feature | Command changes |
| --- | --- |
| Configuring an MPLS flow table for an OpenFlow instance | The mpls keyword was added to the flow-table command. |
| Configuring controllers to which ARP packets are forbidden to be reported | The forbidden packet-in arp controller command was added. |
| Excluding the specified VLANs from the VLANs in which traffic is forwarded in the OpenFlow forwarding process | The openflow normal-forward vlan command was added. |

New features: VXLAN features

Table 14 describes the VXLAN features added in this software version. For more information about the features and commands, see HPE FlexFabric 5950 Switch Series VXLAN Configuration Guide and HPE FlexFabric 5950 Switch Series VXLAN Command Reference.

VXLAN features added in version R6301

| Feature | Command changes |
| --- | --- |
| Displaying ND flood suppression entries | The display ipv6 nd suppression vsi command was added. |
| Disabling flooding for a VSI | The all-direction and dci keywords were added to the flooding disable command. |
| Enabling ND flood suppression | The ipv6 nd suppression enable command was added. |
| Clearing ND flood suppression entries on VSIs | The reset ipv6 nd suppression vsi command was added. |
| Configuring the VLAN tag processing rule for incoming traffic | The rewrite inbound tag command was added. |
| Configuring the VLAN tag processing rule for outgoing traffic | The rewrite outbound tag command was added. |
| Enabling packet statistics for a manually created VXLAN or VXLAN-DCI tunnel | The statistics enable command was added in tunnel interface view. |
| Enabling packet statistics for automatically created VXLAN tunnels | The tunnel statistics vxlan auto command was added. |
| Disabling the ACLs issued by the OVSDB controller | The vtep acl disable command was added. |
| Disabling remote ND learning for VXLANs | The vxlan tunnel nd-learning disable command was added. |
| Configuring VLAN-based VXLAN assignment | The vxlan vlan-based and vxlan vni commands were added. |
| Enabling packet statistics for Ethernet service instances of a VLAN | The ac statistics enable command was added. |
| Assigning backup VXLAN tunnels to a VXLAN | The tunnel command has the following changes: The backup-tunnel keyword was added. The remote-vni keyword was removed. |
| Displaying detailed information about MAC address entries for VSIs | The verbose keyword was added to the display l2vpn mac-address command. |
| Testing the reachability of a remote VM | The following commands were added: emulate-ping vxlan emulate-ping vxlan enable |

New features: EVPN features

Table 15 describes the EVPN features added in this software version. For more information about the features and commands, see HPE FlexFabric 5950 Switch Series EVPN Configuration Guide and HPE FlexFabric 5950 Switch Series EVPN Command Reference.

EVPN features added in version R6301

| Feature | Command changes |
| --- | --- |
| Enabling BGP EVPN route advertisement for the BGP VPNv4 or VPNv6 address family | The advertise evpn route command was added. |
| Enabling BGP VPNv4 or VPNv6 route advertisement for the BGP EVPN address family | The advertise l3vpn route command was added. |
| Disabling ARP information advertisement for an EVPN instance | The arp-advertising disable command was added. |
| Displaying information about IPv4 peers that are automatically discovered through MAC/IP advertisement routes | The mac-ip keyword was added to the display evpn auto-discovery command. |
| Displaying EVPN ND flood suppression entries | The display evpn route nd suppression command was added. |
| Specifying the IP addresses of the VTEPs in a DR system | The evpn drni local command was added. |
| Configuring the EVPN global MAC address | The evpn global-mac command was added. |
| Enabling the device to redistribute received MAC/IP advertisement routes that contain ARP or ND information into a BGP unicast routing table | The import evpn mac-ip command was added. |
| Disabling generation of IP prefix advertisement routes for the subnets of a VSI interface | The ip-prefix-route generate disable command was added. |
| Enabling conversational learning for remote MAC address entries | The mac-address forwarding-conversational-learning command was added. |
| Disabling an EVPN instance from learning MAC addresses from ND information | The nd mac-learning disable command was added. |
| Suppressing the advertisement of specific BGP EVPN routes to a peer or peer group | The peer advertise evpn-route suppress command was added. |
| Replacing the L3 VXLAN ID and RD of IP prefix advertisement routes | The peer re-originated command was added. |
| Automatically generating an RD based on the router ID | The router-id keyword was added to the route-distinguisher command. |

Modified feature: ISSU by using issu commands

Feature change description

You can use issu commands to display or install patch images.

Command changes

Modified command: display version comp-matrix

Old syntax

display version comp-matrix file { boot filename | system filename | feature filename&<1-30> } *

display version comp-matrix file ipe ipe-filename

New syntax

display version comp-matrix file { boot filename | system filename | feature filename&<1-30> | patch filename&<1-30> } *

display version comp-matrix file ipe ipe-filename [ patch filename&<1-30> ]

Views

Any view

Parameters

patch: Specifies a space-separated list of up to 30 patch image files.

filename: Specifies a .bin file in the filesystemname/filename.bin format. The file must be stored in the root directory of a file system on the device. The value string excluding the file system location section (if any) can have a maximum of 63 characters. For more information about specifying a file, see file system management in Fundamentals Configuration Guide.

Change description

Before modification: The command does not support patch image files.

After modification: The command supports patch image files.

Modified command: issu load

Old syntax

issu load file { boot filename | system filename | feature filename&<1-30> } * slot slot-number&<1-9> [ reboot ]

issu load file ipe ipe-filename slot slot-number&<1-9> [ reboot ]

New syntax

issu load file { boot filename | system filename | feature filename&<1-30> | patch filename&<1-30> } * slot slot-number&<1-9> [ reboot ]

issu load file ipe ipe-filename [ patch filename&<1-30> ] slot slot-number&<1-9> [ reboot ]

Views

User view

Parameters

patch: Specifies a space-separated list of up to 30 patch image files.

filename: Specifies a .bin file in the filesystemname/filename.bin format. The file must be stored in the root directory of a file system on the device. The value string excluding the file system location section (if any) can have a maximum of 63 characters. For more information about specifying a file, see file system management in Fundamentals Configuration Guide.

Change description

Before modification: The command does not support patch image files.

After modification: The command supports patch image files.

Modified command: issu one-step

Old syntax

issu one-step file { boot filename | system filename | feature filename&<1-30> } * [ slot slot-number&<1-9> ] [ reboot ]

issu one-step file ipe ipe-filename slot slot-number&<1-9> [ reboot ]

New syntax

issu one-step file { boot filename | system filename | feature filename&<1-30> | patch filename&<1-30> } * [ slot slot-number&<1-9> ] [ reboot ]

issu one-step file ipe ipe-filename [ patch filename&<1-30> ] slot slot-number&<1-9> [ reboot ]

Views

User view

Parameters

patch: Specifies a space-separated list of up to 30 patch image files.

filename: Specifies a .bin file in the filesystemname/filename.bin format. The file must be stored in the root directory of a file system on the device. The value string excluding the file system location section (if any) can have a maximum of 63 characters. For more information about specifying a file, see file system management in Fundamentals Configuration Guide.

Change description

Before modification: The command does not support patch image files.

After modification: The command supports patch image files.

Modified feature: Automatic configuration

Feature change description

Before modification: The device supports automatic configuration only on IPv4 networks.

After modification: The device supports automatic configuration on both IPv4 and IPv6 networks.

Command changes

None

Modified feature: Setting the timestamp format for logs sent to log hosts

Feature change description

From this release, the timestamp for logs output to log hosts can be accurate to milliseconds.

Command changes

Modified command: info-center timestamp loghost

Old syntax

info-center timestamp loghost { date | iso [ with-timezone ] | no-year-date | none }

undo info-center timestamp loghost

New syntax

info-center timestamp loghost { date [ with-milliseconds ] | iso [ with-milliseconds | with-timezone ] * | no-year-date | none }

undo info-center timestamp loghost

Change description

Before modification: The log timestamp cannot be accurate to milliseconds.

After modification: The log timestamp can be set to be accurate to milliseconds. The millisecond value is appended to the time information in the timestamp with a dot as the separator.

Modified feature: Configuring an interface event for a CLI-defined EAA monitor policy

Feature change description

This release added support of monitoring traffic statistics on multiple interfaces. The interfaces in an interface range must be the same type.

Command changes

Modified command: event-interface

Old syntax

event interface interface-type interface-number monitor-obj monitor-obj start-op start-op start-val start-val restart-op restart-op restart-val restart-val [ interval interval ]

New syntax

event interface interface-list monitor-obj monitor-obj start-op start-op start-val start-val restart-op restart-op restart-val restart-val [ interval interval ]

Change description

Before modification: Only one interface can be monitored.

After modification: Multiple interfaces can be monitored. The interfaces in an interface range must be the same type. The start interface number must be smaller than the end interface number.

Modified feature: Configuring an EAA monitor policy by using Tcl

Feature change description

In this release, the system uses the platformtools module rather than the Comware module to create a Tcl script file.

Command changes

N/A

Modified feature: Displaying the operating status and information of an interface

  - Feature change description

In this version and later, the current system time and the last time when the physical state of an interface changed to up or down are displayed in the operating status and information of the interface.

  - Command changes

    - Modified command: display interface

Syntax

display interface [ interface-type [ interface-number | interface-number.subnumber ] ] [ brief [ description | down ] ]

Views

Any view

Command output after modification

# Display the operating status and information of Ten-GigabitEthernet 1/0/1.

<Sysname> display interface ten-gigabitethernet 1/0/1

Ten-GigabitEthernet1/0/1

Current state: Administratively DOWN

Line protocol state: DOWN

Description: Ten-GigabitEthernet 1/0/1 Interface

Bandwidth: 10000000 kbps

Maximum transmission unit: 1500

Allow jumbo frames to pass

Broadcast max-ratio: 100%

Multicast max-ratio: 100%

Unicast max-ratio: 100%

Internet protocol processing: Disabled

IP packet frame type: Ethernet II, hardware address: 3822-d666-bd0c

IPv6 packet frame type: Ethernet II, hardware address: 3822-d666-bd0c

Media type is twisted pair

Port priority: 2

Unknown-speed mode, unknown-duplex mode

Link speed type is autonegotiation, link duplex type is autonegotiation

Flow-control is not enabled

The maximum frame length is 9416

Last link flapping: 6 hours 39 minutes 28 seconds

Last clearing of counters: Never

Current system time:2019-02-10 14:56:12

Last time when physical state changed to up:-

Last time when physical state changed to down:2019-02-10 14:55:25

Change description

Before modification: The current system time and the last time when the physical state of an interface changed to up or down are not displayed in the operating status and information of the interface.

After modification: The current system time and the last time when the physical state of an interface changed to up or down are displayed in the operating status and information of the interface.

Modified feature: Displaying PFC information of all interfaces

  - Feature change description

In this version and later, the PFC information displayed for all interfaces is modified.

  - Command changes

    - Modified command: display priority-flow-control interface

Syntax

display priority-flow-control interface [ interface-type [ interface-number ] ]

Views

Any view

Change description

Before modification:

# Display PFC information of all interfaces.

<Sysname> display priority-flow-control interface

<Sysname> display priority-flow-control interface

Interface           AdminMode  OperMode  Dot1pList   Prio  Recv       Send

--------------------------------------------------------------------------------

XGE1/0/1            Auto       Disabled  0,2-3,5-6   0     178        43

After modification:

# Display PFC information of all interfaces.

<Sysname> display priority-flow-control interface

Conf -- Configured mode   Ne -- Negotiated mode   P -- Priority

Interface     Conf Ne  Dot1pList   P Recv       Sent

XGE1/0/1      Auto On  0,2-3,5-6   0 178        43

Modified feature: Link state change suppression on an interface

  - Feature change description

In this version, the syntax for configuring link state change suppression on an Ethernet interface is modified. When the link state change suppression interval is configured in seconds on an Ethernet interface, the value range is modified.

  - Command changes

    - Modified command: link-delay

Old syntax

link-delay [ msec ] delay-time [ mode { up | updown } ]

undo link-delay [ msec ] delay-time [ mode { up | updown } ]

New syntax

link-delay { down | up } [ msec ] delay-time

undo link-delay { down | up }

Views

Ethernet interface view

Change description

Before modification:

- If the mode keyword is not specified, the link-down events are suppressed.

- If the mode up keyword combination is specified, the link-up events are suppressed.

- If the mode updown keyword combination is specified, both link-down and link-up events are suppressed.

- If the suppression interval configured in the command without the mode keyword specified is the same as the suppression interval configured in the command with the mode up keyword combination specified on an interface, the two commands are automatically merged into the command with the mode updown keyword combination specified in the configuration file of the interface.

- If the msec keyword is not specified, the link state change suppression interval is configured in seconds, and the value range is 0 to 30.

After modification:

- If the down keyword is not specified, the link-down events are suppressed.

- If the up keyword is specified, the link-up events are suppressed.

- You can set different link state change suppression intervals for link-down events and link-up events.

- If the msec keyword is not specified, the link state change suppression interval is configured in seconds, and the value range is 0 to 300.

Modified feature: MAC-to-VLAN entries

  - Feature change description

In this version and later, the keyword for configuring 802.1p priorities in MAC-to-VLAN entries is modified to dot1p.

  - Command changes

    - Modified command: mac-vlan mac-address

Old syntax

mac-vlan mac-address mac-address [ mask mac-mask ] vlan vlan-id [ dot1q priority ]

New syntax

mac-vlan mac-address mac-address [ mask mac-mask ] vlan vlan-id [ dot1p priority ]

Views

System view

Change description

Before modification: The keyword for configuring 802.1p priorities in MAC-to-VLAN entries is dot1q.

After modification: The keyword for configuring 802.1p priorities in MAC-to-VLAN entries is dot1p.

    - Modified command: display mac-vlan

Syntax

display mac-vlan { all | dynamic | mac-address mac-address [ mask mac-mask ] | static | vlan vlan-id }

Views

Any view

Old command output

# Display all MAC-to-VLAN entries.

<Sysname> display mac-vlan all

The following MAC VLAN entries exist:

State: S - Static, D - Dynamic

MAC address        Mask                VLAN ID   Dot1q      State

0008-0001-0000     ffff-ff00-0000      5         3          S

0002-0001-0000     ffff-ffff-ffff      5         3          S&D

Total MAC VLAN entries count: 2

New command output

# Display all MAC-to-VLAN entries.

<Sysname> display mac-vlan all

The following MAC VLAN entries exist:

State: S - Static, D - Dynamic

MAC address        Mask                VLAN ID   Dot1p      State

0008-0001-0000     ffff-ff00-0000      5         3          S

0002-0001-0000     ffff-ffff-ffff      5         3          S&D

Total MAC VLAN entries count: 2

Change description

Before modification: The field for displaying the 802.1p priority of a VLAN was Dot1q.

After modification: The field for displaying the 802.1p priority of a VLAN was Dot1p.

Modified feature: Collision handling process of LACP MAD and BFD MAD

Feature change description

Before modification, LACP MAD and BFD MAD use the following process to handle a multi-active collision:

Compare the number of members in each fabric.

Set all fabrics to the Recovery state except the one that has the most members.

Compare the member IDs of the masters if all IRF fabrics have the same number of members.

Set all fabrics to the Recovery state except the one that has the lowest numbered master.

After modification, LACP MAD and BFD MAD use the following process to handle a multi-active collision:

Compare the health states (chip forwarding performance states) of split fabrics.

Set all fabrics to the Recovery state except the healthiest one.

Compare the number of members in each fabric if all IRF fabrics are in the same health state.

Set all fabrics to the Recovery state except the one that has the most members.

Compare the member IDs of their masters if all IRF fabrics have the same number of members.

Set all fabrics to the Recovery state except the one that has the lowest numbered master.

Command changes

None.

Modified feature: Collision handling process of ND MAD

Feature change description

Before modification:

ND MAD uses the following process to handle a multi-active collision:

Compares the member IDs of the masters in the IRF fabrics.

Sets all fabrics to the Recovery state except the one that has the lowest numbered master.

ND MAD cannot be configured together with ARP MAD because they handle collisions differently.

After modification:

ND MAD uses the following process to handle a multi-active collision:

Compares the health states (chip forwarding performance states) of split fabrics.

Sets all fabrics to the Recovery state except the healthiest one.

Compares the member IDs of the masters if all IRF fabrics are in the same health state.

Sets all fabrics to the Recovery state except the one that has the lowest numbered master.

ND MAD can be configured together with ARP MAD because they use the same collision handling process.

Command changes

None.

Modified feature: Displaying global link aggregation load sharing modes

Feature change description

The default global load sharing modes were modified in the output from the display link-aggregation load-sharing mode command.

Command changes

Modified command: display link-aggregation load-sharing mode

Syntax

display link-aggregation load-sharing mode [ interface [ { bridge-aggregation | route-aggregation } interface-number ] ]

Views

Any view

Change description

Before modification: If the default global load sharing modes are used, only Default is displayed.

# Display the global link aggregation load sharing modes. This example displays the default settings.

<Sysname> display link-aggregation load-sharing mode

Link-Aggregation Load-Sharing Algorithm:

Default

Link-Aggregation Load-Sharing Seed:

Default

Link-aggregation load-sharing mode:

Layer 2 traffic: packet type-based sharing

Layer 3 traffic: packet type-based sharing

After modification: If the default global load sharing modes are used, the default values are displayed.

# Display the global link aggregation load sharing modes. This example displays the default settings.

<Sysname> display link-aggregation load-sharing mode

Link-aggregation load-sharing algorithm:

5 (default)

Link-aggregation load-sharing offset:

0 (default)

Link-aggregation load-sharing seed:

0x0 (default)

Tunneled traffic load-sharing mode:

Outer (default)

Link-aggregation load-sharing mode:

Layer 2 traffic:   packet type-based sharing

Layer 3 traffic:   packet type-based sharing

Modified feature: Configuring a link aggregation load sharing hash seed

Feature change description

The value range for the load sharing hash seed was changed to 1 to FFFFFFFF.

Command changes

Modified command: link-aggregation global load-sharing seed

Syntax

link-aggregation global load-sharing seed seed-number

Views

Any view

Change description

Before modification: The value range for the seed-number argument is 0 to FFFFFFFF.

After modification: The value range for the seed-number argument is 1 to FFFFFFFF.

Modified feature: Displaying detailed information about the IPP and DR interfaces of DRNI

Feature change description

The interface numbers of aggregation member ports were added for the local IPP and DR interfaces in the output from the display drni verbose command.

Command changes

Modified command: display drni verbose

Syntax

display drni verbose [ interface interface-type interface-number ]

Views

Any view

Change description

Before modification:

# Display detailed information about DR interface Bridge-Aggregation 1.

<Sysname> display drni verbose bridge-aggregation 1

Flags: A -- Home_Gateway, B -- Neighbor_Gateway, C -- Other_Gateway,

D -- IPP_Activity, E -- DRCP_Timeout, F -- Gateway_Sync,

G -- Port_Sync, H -- Expired

DR interface/DR group ID: BAGG1/1

State: UP

Local state/Peer state: ABDFG/ABDFG

Local Selected ports Index: 16385, 16386

Peer Selected ports Index: 32769, 32770

# Display detailed information about IPP Bridge-Aggregation 2.

<Sysname> display drni verbose bridge-aggregation 2

Flags: A -- Home_Gateway, B -- Neighbor_Gateway, C -- Other_Gateway,

D -- IPP_Activity, E -- DRCP_Timeout, F -- Gateway_Sync,

G -- Port_Sync, H -- Expired

IPP/IPP ID: BAGG2/1

State: UP

Local state/Peer state: ABDFG/ABDFG

Local Selected ports Index: 16385, 16386

Peer Selected ports Index: 32769, 32770

After modification:

# Display detailed information about DR interface Bridge-Aggregation 1.

<Sysname> display drni verbose interface bridge-aggregation 1

Flags: A -- Home_Gateway, B -- Neighbor_Gateway, C -- Other_Gateway,

D -- IPP_Activity, E -- DRCP_Timeout, F -- Gateway_Sync,

G -- Port_Sync, H -- Expired

DR interface/DR group ID: BAGG1/1

State: UP

Local state/Peer state: ABDFG/ABDFG

Local Selected ports (index): XGE1/0/1 (16385), XGE1/0/2 (16386)

Peer Selected ports indexes: 32769, 32770

# Display detailed information about IPP Bridge-Aggregation 2.

<Sysname> display drni verbose interface bridge-aggregation 2

Flags: A -- Home_Gateway, B -- Neighbor_Gateway, C -- Other_Gateway,

D -- IPP_Activity, E -- DRCP_Timeout, F -- Gateway_Sync,

G -- Port_Sync, H -- Expired

IPP/IPP ID: BAGG2/1

State: UP

Local state/Peer state: ABDFG/ABDFG

Local Selected ports (index): XGE1/0/1 (16385), XGE1/0/2 (16386)

Peer Selected ports indexes: 32769, 32770

Modified feature: Configuring the advertisable TLVs for LLDP

Feature change description

From this release, link aggregation TLVs in the IEEE 802.3 organizationally specific TLV set and DCBX TLVs are supported.

Table 16 shows the IEEE 802.3 organizationally specific TLVs.

The Power Stateful Control TLV is defined in IEEE P802.3at D1.0 and is not supported in later versions. The device sends this type of TLVs only after receiving them.

IEEE 802.3 organizationally specific TLVs

| Type | Description |
| --- | --- |
| MAC/PHY Configuration/Status | Contains the bit-rate and duplex capabilities of the port, support for autonegotiation, enabling status of autonegotiation, and the current rate and duplex mode. |
| Link Aggregation | Indicates whether the port supports link aggregation, and if yes, whether link aggregation is enabled. |
| Power Via MDI | Contains the power supply capabilities of the port: - Port class (PSE or PD). - Power supply mode. - Whether PSE power supply is supported. - Whether PSE power supply is enabled. - Whether pair selection can be controlled. - Power supply type. - Power source. - Power priority. - PD requested power. - PSE allocated power. |
| Maximum Frame Size | Indicates the supported maximum frame size. |
| Power Stateful Control | Indicates the power state control configured on the sending port, including the following: - Power supply mode of the PSE/PD. - PSE/PD priority. - PSE/PD power. |
| Energy-Efficient Ethernet | Indicates Energy Efficient Ethernet (EEE). |

Command changes

Modified command: lldp tlv-enable

Old syntax

In Layer 2 Ethernet interface view:

- For nearest bridge agents:

lldp tlv-enable { basic-tlv { all | port-description | system-capability | system-description | system-name | management-address-tlv [ ipv6 ] [ ip-address | interface loopback interface-number ] } | dot1-tlv { all | congestion-notification | port-vlan-id | link-aggregation | dcbx | protocol-vlan-id [ vlan-id ] | vlan-name [ vlan-id ] | management-vid [ mvlan-id ] } | dot3-tlv { all | mac-physic | max-frame-size | power } | med-tlv { all | capability | inventory | network-policy [ vlan-id ] | power-over-ethernet | location-id { civic-address device-type country-code { ca-type ca-value }&<1-10> | elin-address tel-number } } }

undo lldp tlv-enable { basic-tlv { all | port-description | system-capability | system-description | system-name | management-address-tlv [ ipv6 ] [ ip-address | interface loopback interface-number ] } | dot1-tlv { all | congestion-notification | port-vlan-id | link-aggregation | dcbx | protocol-vlan-id | vlan-name | management-vid } | dot3-tlv { all | mac-physic | max-frame-size | power } | med-tlv { all | capability | inventory | network-policy [ vlan-id ] | power-over-ethernet | location-id } }

- For nearest non-TPMR bridge agents:

lldp agent nearest-nontpmr tlv-enable { basic-tlv { all | port-description | system-capability | system-description | system-name | management-address-tlv [ ipv6 ] [ ip-address ] } | dot1-tlv { all | congestion-notification | port-vlan-id | link-aggregation } }

lldp tlv-enable dot1-tlv { protocol-vlan-id [ vlan-id ] | vlan-name [ vlan-id ] | management-vid [ mvlan-id ] }

undo lldp agent nearest-nontpmr tlv-enable { basic-tlv { all | port-description | system-capability | system-description | system-name | management-address-tlv [ ipv6 ] [ ip-address ] } | dot1-tlv { all | congestion-notification | port-vlan-id | link-aggregation } }

undo lldp tlv-enable dot1-tlv { protocol-vlan-id | vlan-name | management-vid }

- For nearest customer bridge agents:

lldp agent nearest-customer tlv-enable { basic-tlv { all | port-description | system-capability | system-description | system-name | management-address-tlv [ ipv6 ] [ ip-address ] } | dot1-tlv { all | congestion-notification | port-vlan-id | link-aggregation } }

lldp tlv-enable dot1-tlv { protocol-vlan-id [ vlan-id ] | vlan-name [ vlan-id ] | management-vid [ mvlan-id ] }

undo lldp agent nearest-customer tlv-enable { basic-tlv { all | port-description | system-capability | system-description | system-name | management-address-tlv [ ipv6 ] [ ip-address ] } | dot1-tlv { all | congestion-notification | port-vlan-id | link-aggregation } }

undo lldp tlv-enable dot1-tlv { protocol-vlan-id | vlan-name | management-vid }

In Layer 3 Ethernet interface view:

lldp tlv-enable { basic-tlv { all | port-description | system-capability | system-description | system-name | management-address-tlv [ ipv6 ] [ ip-address | interface loopback interface-number ] } | dot1-tlv { all | link-aggregation } | dot3-tlv { all | mac-physic | max-frame-size | power } | med-tlv { all | capability | inventory | power-over-ethernet | location-id { civic-address device-type country-code { ca-type ca-value }&<1-10> | elin-address tel-number } }

lldp agent { nearest-nontpmr | nearest-customer } tlv-enable { basic-tlv { all | port-description | system-capability | system-description | system-name | management-address-tlv [ ipv6 ] [ ip-address ] } | dot1-tlv { all | link-aggregation } }

undo lldp tlv-enable { basic-tlv { all | port-description | system-capability | system-description | system-name | management-address-tlv [ ipv6 ] [ ip-address | interface loopback interface-number ] } | dot1-tlv { all | link-aggregation } | dot3-tlv { all | mac-physic | max-frame-size | power } | med-tlv { all | capability | inventory | power-over-ethernet | location-id } }

undo lldp agent { nearest-nontpmr | nearest-customer } tlv-enable { basic-tlv { all | port-description | system-capability | system-description | system-name | management-address-tlv [ ipv6 ] [ ip-address ] } | dot1-tlv { all | link-aggregation } }

In management Ethernet interface view:

lldp tlv-enable { basic-tlv { all | port-description | system-capability | system-description | system-name | management-address-tlv [ ipv6 ] [ ip-address ] } | dot1-tlv { all | link-aggregation } | dot3-tlv { all | mac-physic | max-frame-size | power } | med-tlv { all | capability | inventory | power-over-ethernet | location-id { civic-address device-type country-code { ca-type ca-value }&<1-10> | elin-address tel-number } } }

lldp agent { nearest-nontpmr | nearest-customer } tlv-enable { basic-tlv { all | port-description | system-capability | system-description | system-name | management-address-tlv [ ipv6 ] [ ip-address ] } | dot1-tlv { all | link-aggregation } }

undo lldp tlv-enable { basic-tlv { all | port-description | system-capability | system-description | system-name | management-address-tlv [ ipv6 ] [ ip-address ] } | dot1-tlv { all | link-aggregation } | dot3-tlv { all | mac-physic | max-frame-size | power } | med-tlv { all | capability | inventory | power-over-ethernet | location-id } }

undo lldp agent { nearest-nontpmr | nearest-customer } tlv-enable { basic-tlv { all | port-description | system-capability | system-description | system-name | management-address-tlv [ ipv6 ] [ ip-address ] } | dot1-tlv { all | link-aggregation } }

In Layer 2 aggregate interface view:

lldp agent nearest-nontpmr tlv-enable { basic-tlv { all | management-address-tlv [ ipv6 ] [ ip-address ] | port-description | system-capability | system-description | system-name } | dot1-tlv { all | port-vlan-id } }

lldp agent nearest-customer tlv-enable { basic-tlv { all | management-address-tlv [ ipv6 ] [ ip-address ] | port-description | system-capability | system-description | system-name } | dot1-tlv { all | port-vlan-id } }

lldp tlv-enable dot1-tlv { protocol-vlan-id [ vlan-id ] | vlan-name [ vlan-id ] | management-vid [ mvlan-id ] }

undo lldp agent nearest-nontpmr tlv-enable { basic-tlv { all | management-address-tlv [ ipv6 ] [ ip-address ] | port-description | system-capability | system-description | system-name } | dot1-tlv { all | port-vlan-id } }

undo lldp agent nearest-customer tlv-enable { basic-tlv { all | management-address-tlv [ ipv6 ] [ ip-address ] | port-description | system-capability | system-description | system-name } | dot1-tlv { all | port-vlan-id } }

undo lldp tlv-enable dot1-tlv { protocol-vlan-id | vlan-name | management-vid }

In Layer 3 aggregate interface view:

lldp agent { nearest-customer | nearest-nontpmr } tlv-enable basic-tlv { all | management-address-tlv [ ipv6 ] [ ip-address ] | port-description | system-capability | system-description | system-name }

undo lldp agent { nearest-customer | nearest-nontpmr } tlv-enable basic-tlv { all | management-address-tlv [ ipv6 ] [ ip-address ] | port-description | system-capability | system-description | system-name }

In IRF physical interface view:

lldp tlv-enable basic-tlv { port-description | system-capability | system-description | system-name }

undo lldp tlv-enable basic-tlv { port-description | system-capability | system-description | system-name }

New syntax

In Layer 2 Ethernet interface view:

- For nearest bridge agents:

lldp tlv-enable { basic-tlv { all | port-description | system-capability | system-description | system-name | management-address-tlv [ ipv6 ] [ ip-address | interface loopback interface-number ] } | dot1-tlv { all | congestion-notification | port-vlan-id | link-aggregation | dcbx | protocol-vlan-id [ vlan-id ] | vlan-name [ vlan-id ] | management-vid [ mvlan-id ] } | dot3-tlv { all | link-aggregation | mac-physic | max-frame-size | power } | med-tlv { all | capability | inventory | network-policy [ vlan-id ] | power-over-ethernet | location-id { civic-address device-type country-code { ca-type ca-value }&<1-10> | elin-address tel-number } } }

undo lldp tlv-enable { basic-tlv { all | port-description | system-capability | system-description | system-name | management-address-tlv [ ipv6 ] [ ip-address | interface loopback interface-number ] } | dot1-tlv { all | congestion-notification | port-vlan-id | link-aggregation | dcbx | protocol-vlan-id | vlan-name | management-vid } | dot3-tlv { all | link-aggregation | mac-physic | max-frame-size | power } | med-tlv { all | capability | inventory | network-policy [ vlan-id ] | power-over-ethernet | location-id } }

- For nearest non-TPMR bridge agents:

lldp agent nearest-nontpmr tlv-enable { basic-tlv { all | port-description | system-capability | system-description | system-name | management-address-tlv [ ipv6 ] [ ip-address ] } | dot1-tlv { all | congestion-notification | port-vlan-id | link-aggregation } | dot3-tlv { all | link-aggregation } }

lldp tlv-enable dot1-tlv { protocol-vlan-id [ vlan-id ] | vlan-name [ vlan-id ] | management-vid [ mvlan-id ] }

undo lldp agent nearest-nontpmr tlv-enable { basic-tlv { all | port-description | system-capability | system-description | system-name | management-address-tlv [ ipv6 ] [ ip-address ] } | dot1-tlv { all | congestion-notification | port-vlan-id | link-aggregation } | dot3-tlv { all | link-aggregation } }

undo lldp tlv-enable dot1-tlv { protocol-vlan-id | vlan-name | management-vid }

- For nearest customer bridge agents:

lldp agent nearest-customer tlv-enable { basic-tlv { all | port-description | system-capability | system-description | system-name | management-address-tlv [ ipv6 ] [ ip-address ] } | dot1-tlv { all | congestion-notification | port-vlan-id | link-aggregation } | dot3-tlv { all | link-aggregation } }

lldp tlv-enable dot1-tlv { protocol-vlan-id [ vlan-id ] | vlan-name [ vlan-id ] | management-vid [ mvlan-id ] }

undo lldp agent nearest-customer tlv-enable { basic-tlv { all | port-description | system-capability | system-description | system-name | management-address-tlv [ ipv6 ] [ ip-address ] } | dot1-tlv { all | congestion-notification | port-vlan-id | link-aggregation } | dot3-tlv { all | link-aggregation } }

undo lldp tlv-enable dot1-tlv { protocol-vlan-id | vlan-name | management-vid }

In Layer 3 Ethernet interface view:

lldp tlv-enable { basic-tlv { all | port-description | system-capability | system-description | system-name | management-address-tlv [ ipv6 ] [ ip-address | interface loopback interface-number ] } | dot1-tlv { all | link-aggregation } | dot3-tlv { all | link-aggregation | mac-physic | max-frame-size | power } | med-tlv { all | capability | inventory | power-over-ethernet | location-id { civic-address device-type country-code { ca-type ca-value }&<1-10> | elin-address tel-number } } }

lldp agent { nearest-nontpmr | nearest-customer } tlv-enable { basic-tlv { all | port-description | system-capability | system-description | system-name | management-address-tlv [ ipv6 ] [ ip-address ] } | dot1-tlv { all | link-aggregation } | dot3-tlv { all | link-aggregation } }

undo lldp tlv-enable { basic-tlv { all | port-description | system-capability | system-description | system-name | management-address-tlv [ ipv6 ] [ ip-address | interface loopback interface-number ] } | dot1-tlv { all | link-aggregation } | dot3-tlv { all | link-aggregation | mac-physic | max-frame-size | power } | med-tlv { all | capability | inventory | power-over-ethernet | location-id } }

undo lldp agent { nearest-nontpmr | nearest-customer } tlv-enable { basic-tlv { all | port-description | system-capability | system-description | system-name | management-address-tlv [ ipv6 ] [ ip-address ] } | dot1-tlv { all | link-aggregation } | dot3-tlv { all | link-aggregation } }

In management Ethernet interface view:

lldp tlv-enable { basic-tlv { all | port-description | system-capability | system-description | system-name | management-address-tlv [ ipv6 ] [ ip-address ] } | dot1-tlv { all | link-aggregation } | dot3-tlv { all | link-aggregation | mac-physic | max-frame-size | power } | med-tlv { all | capability | inventory | power-over-ethernet | location-id { civic-address device-type country-code { ca-type ca-value }&<1-10> | elin-address tel-number } } }

lldp agent { nearest-nontpmr | nearest-customer } tlv-enable { basic-tlv { all | port-description | system-capability | system-description | system-name | management-address-tlv [ ipv6 ] [ ip-address ] } | dot1-tlv { all | link-aggregation } | dot3-tlv { all | link-aggregation } }

undo lldp tlv-enable { basic-tlv { all | port-description | system-capability | system-description | system-name | management-address-tlv [ ipv6 ] [ ip-address ] } | dot1-tlv { all | link-aggregation } | dot3-tlv { all | link-aggregation | mac-physic | max-frame-size | power } | med-tlv { all | capability | inventory | power-over-ethernet | location-id } }

undo lldp agent { nearest-nontpmr | nearest-customer } tlv-enable { basic-tlv { all | port-description | system-capability | system-description | system-name | management-address-tlv [ ipv6 ] [ ip-address ] } | dot1-tlv { all | link-aggregation } | dot3-tlv { all | link-aggregation } }

In Layer 2 aggregate interface view:

lldp agent nearest-nontpmr tlv-enable { basic-tlv { all | management-address-tlv [ ipv6 ] [ ip-address ] | port-description | system-capability | system-description | system-name } | dot1-tlv { all | port-vlan-id } }

lldp agent nearest-customer tlv-enable { basic-tlv { all | management-address-tlv [ ipv6 ] [ ip-address ] | port-description | system-capability | system-description | system-name } | dot1-tlv { all | port-vlan-id } }

lldp tlv-enable dot1-tlv { protocol-vlan-id [ vlan-id ] | vlan-name [ vlan-id ] | management-vid [ mvlan-id ] }

undo lldp agent nearest-nontpmr tlv-enable { basic-tlv { all | management-address-tlv [ ipv6 ] [ ip-address ] | port-description | system-capability | system-description | system-name } | dot1-tlv { all | port-vlan-id } }

undo lldp agent nearest-customer tlv-enable { basic-tlv { all | management-address-tlv [ ipv6 ] [ ip-address ] | port-description | system-capability | system-description | system-name } | dot1-tlv { all | port-vlan-id } }

undo lldp tlv-enable dot1-tlv { protocol-vlan-id | vlan-name | management-vid }

In Layer 3 aggregate interface view:

lldp agent { nearest-customer | nearest-nontpmr } tlv-enable basic-tlv { all | management-address-tlv [ ipv6 ] [ ip-address ] | port-description | system-capability | system-description | system-name }

undo lldp agent { nearest-customer | nearest-nontpmr } tlv-enable basic-tlv { all | management-address-tlv [ ipv6 ] [ ip-address ] | port-description | system-capability | system-description | system-name }

In IRF physical interface view:

lldp tlv-enable basic-tlv { port-description | system-capability | system-description | system-name }

undo lldp tlv-enable basic-tlv { port-description | system-capability | system-description | system-name }

Change description

The link-aggregation and dcbx keywords were added.

Modified feature: Setting the aging timer for dynamic ARP entries

Feature change description

In this version and later, you can set the aging timer for dynamic ARP entries in the following views:

Layer 3 Ethernet interface/subinterface view

Layer 3 aggregate interface/subinterface view

VSI interface view

VLAN interface view

Command changes

Modified command: arp timer aging

Syntax

arp timer aging { aging-minutes | second aging-seconds }

undo arp timer aging

Views

System view

Layer 3 Ethernet interface/subinterface view

Layer 3 aggregate interface/subinterface view

VSI interface view

VLAN interface view

Change description

Before modification: This command is supported only in system view.

After modification: This command is supported in system view and the following interface view:

Layer 3 Ethernet interface/subinterface view

Layer 3 aggregate interface/subinterface view

VSI interface view

VLAN interface view

Modified feature: ARP snooping

Feature change description

In this version and later, support for enabling ARP snooping for a VXLAN was added. If you enable ARP snooping for a VXLAN, ARP packets received in the VXLAN are redirected to the CPU. For the VXLAN, the CPU uses the sender IP and MAC addresses of the ARP packets, VSI name, and link ID to create ARP snooping entries.

Command changes

Modified command: arp snooping enable

Syntax

arp snooping enable

Views

VLAN view

VSI view

Change description

Before modification: This command is supported only in VLAN view.

After modification: This command is supported in both VLAN view and VSI view.

Modified command: display arp snooping

Old syntax

display arp snooping [ vlan vlan-id ] [ slot slot-number ] [ count ]

display arp snooping ip ip-address [ slot slot-number ]

New syntax

display arp snooping { vlan [ vlan-id ] | vsi [vsi-name ] } [ slot slot-number ] [ count ]

display arp snooping vlan ip ip-address [ slot slot-number ]

Views

Any view

Change description

Before modification: Displaying ARP snooping entries for a VSI is not supported.

After modification: Displaying ARP snooping entries for a VSI is supported.

Modified command: reset arp snooping

Old syntax

reset arp snooping [ ip ip-address | vlan vlan-id ]

New syntax

reset arp snooping { vlan [ vlan-id ] | vsi [ vsi-name ] }

reset arp snooping vlan ip ip-address

Views

Any view

Change description

Before modification: You can clear ARP snooping entries only for a VLAN, ARP snooping entries for the specified IP address in VLANs, or all ARP snooping entries. The syntax for the command to clear ARP snooping entries for the specified IP address in VLANs is reset arp snooping ip ip-address.

After modification: You can also clear ARP snooping entries for a VSI, for all VSIs, or for all VLANs. The syntax for the command to clear ARP snooping entries for the specified IP address in VLANs is reset arp snooping vlan ip ip-address.

Modified feature: Specifying DHCP servers for a DHCP relay address pool

Feature change description

This release supports specifying DHCP servers in VPN instances for a DHCP relay address pool.

Command changes

Modified command: remote-server

Old syntax

remote-server ip-address&<1-8>

New syntax

remote-server ip-address&<1-8> [ public | vpn-instance vpn-instance-name ]

undo remote-server [ ip-address&<1-8> ]

Views

DHCP address pool view

Change description

Before modification: The command supports only specifying DHCP servers on the public network for a DHCP relay address pool.

After modification: The command supports specifying DHCP servers on the public network and VPN instances for a DHCP relay address pool.

Modified feature: Displaying DHCP snooping and DHCPv6 snooping trusted ports

Feature change description

This release added support for displaying DHCP snooping and DHCPv6 snooping trusted ports in VSIs.

Command changes

Modified command: display dhcp snooping trust

Syntax

display dhcp snooping trust

Views

Any view

Change description

Before modification: This command displays trusted ports only in VLANs.

# Display information about trusted ports.

<Sysname> display dhcp snooping trust

DHCP snooping is enabled.

Interface                                       Trusted          VLAN

=========================                       ============     =======

Ten-GigabitEthernet1/0/1                        Trusted

Ten-GigabitEthernet1/0/2                        -                100

Ten-GigabitEthernet1/0/3                        -                100, 200

        - Command output

| Field | Description |
| --- | --- |
| Interface | Interface name. |
| Trusted | For a DHCP snooping trusted port specified in the global DHCP snooping configuration, this field displays Trusted. For a trusted port specified in VLAN-based DHCP snooping configuration, this field displays a hyphen (-). |
| VLAN | VLAN to which the trusted port belongs. If the trusted port is specified in global DHCP snooping configuration, the field value is empty. |

After modification: This command supports displaying trusted ports in VLANs and VSIs.

# Display information about trusted ports.

<Sysname> display dhcp snooping trust

DHCP snooping is enabled.

Interface                                        Trusted  VLAN

============================                     =======  ================

XGE1/0/1                                         Trusted  -

XGE1/0/2                                         -        100

VSI name                                        Tunnel trusted

============================                    ==============

a                                               Trusted

AC                                              Trusted

============================                    ============

XGE1/0/1 srv 1                                  Trusted

        - Command output

| Field | Description |
| --- | --- |
| Interface | Interface name. |
| Trusted | For a DHCP snooping trusted port specified in the global DHCP snooping configuration, this field displays Trusted. For a trusted port specified in VLAN-based DHCP snooping configuration, this field displays a hyphen (-). |
| VLAN | VLAN to which the trusted port belongs. If the trusted port is specified in global DHCP snooping configuration, this field displays a hyphen (-). |
| VSI name | VSI name of the VXLAN tunnel interface. This field is available when you configure the tunnel interface assigned to the VSI as a DHCP snooping trusted interface by using the dhcp snooping trust tunnel command. |
| Tunnel trusted | Trusted tunnel interface specified in VXLAN-based DHCP snooping configuration. |
| AC | AC name, which is indicated by the interface name and Ethernet service instance name. This field is available when you configure the AC as the DHCP snooping trusted interface by using the dhcp snooping trust command in Ethernet service instance view. |
| Trusted | Trusted AC specified in VXLAN-based DHCP snooping configuration. |

Modified command: display ipv6 dhcp snooping trust

Syntax

display ipv6 dhcp snooping trust

Views

Any view

Change description

Before modification: This command displays information about trusted ports only in VLANs.

# Display information about trusted ports.

<Sysname> display ipv6 dhcp snooping trust

DHCPv6 snooping is enabled.

Interface                                       Trusted

=========================                       ============

Ten-GigabitEthernet1/0/1                        Trusted

VSI(Trust tunnel)                               Trusted

=========================                       ============

Interface                     SrvID             Trusted

===================================             ============

XGE1/0/1                      1                 Trusted

        - Command output

| Field | Description |
| --- | --- |
| Interface | Interface name. |
| VSI(Trust tunnel) | This field is not supported in the current software version. VSI name. This field is available when you configure the VXLAN tunnel interfaces assigned to the VSI as a DHCP snooping trusted interface by using the ipv6 dhcp snooping trust tunnel command. |
| SrvID | This field is not supported in the current software version. ID of the Ethernet service instance on which the mapped AC is configured as a DHCP snooping trusted interface. |
| Trusted | DHCP snooping trusted interface. This field displays Trusted if the interface is configured as trusted after the DHCPv6 snooping is enabled. |

After modification: This command supports displaying trusted ports in VLANs and VSIs.

# Display information about trusted ports.

<Sysname> display ipv6 dhcp snooping trust

DHCPv6 snooping is enabled.

Interface                              Trusted             VLAN

============================           ============        =======

XGE1/0/1                               -                   100

XGE1/0/2                               Trusted             -

VSI name                               Tunnel trusted

============================           ============

a                                      Trusted

AC                                     Trusted

============================           ============

XGE1/0/1 srv 1                         Trusted

Command output

| Field | Description |
| --- | --- |
| Interface | Interface name. |
| Trusted | Trusted port specified in global DHCPv6 snooping configuration. If the trusted port is specified in VLAN-based DHCPv6 snooping configuration, this field displays a hyphen (-). |
| VLAN | VLAN to which the trusted port belongs. If the trusted port is specified in global DHCPv6 snooping configuration, this field displays a hyphen (-). |
| VSI name | VSI name of the VXLAN tunnel interface. This field is available when you configure the tunnel interface assigned to the VSI as a DHCP snooping trusted interface by using the ipv6 dhcp snooping trust tunnel command. |
| Tunnel trusted | Trusted tunnel interface specified in VXLAN-based DHCPv6 snooping configuration. |
| AC | AC name, which is indicated by the interface name and Ethernet service instance name. This field is available when you configure the AC as the DHCPv6 snooping trusted interface by using the ipv6 dhcp snooping trust command in Ethernet service instance view. |
| Trusted | Trusted AC specified in VXLAN-based DHCPv6 snooping configuration. |

Modified feature: Displaying DHCP address pool information

Feature change description

The display dhcp server pool command output supports displaying the time unit for the lease duration.

Command changes

Modified command: display dhcp server pool

Syntax

display dhcp server pool [ pool-name | vpn-instance vpn-instance-name ]

Views

Any view

Change description

The time unit was added to the lease duration in the command output.

# Display information about all DHCP pools.

<Sysname> display dhcp server pool

Pool name: 0

Network 20.1.1.0 mask 255.255.255.0

class a range 20.1.1.50 20.1.1.60

bootfile-name abc.cfg

dns-list 20.1.1.66 20.1.1.67 20.1.1.68

domain-name www.aabbcc.com

bims-server ip 192.168.0.51 sharekey cipher $c$3$K13OmQPi791YvQoF2Gs1E+65LOU=

option 2 ip-address 1.1.1.1

expired day 1 hour 2 minute 3 second 0

Pool name: 1

Network 20.1.2.0 mask 255.255.255.0

secondary networks:

20.1.3.0 mask 255.255.255.0

20.1.4.0 mask 255.255.255.0

bims-server ip 192.168.0.51 port 50 sharekey cipher $c$3$K13OmQPi791YvQoF2Gs1E+65LOU=

forbidden-ip 20.1.1.22 20.1.1.36 20.1.1.37

forbidden-ip 20.1.1.22 20.1.1.23 20.1.1.24

gateway-list 20.1.1.1 20.1.1.2 20.1.1.4

nbns-list 20.1.1.5 20.1.1.6 20.1.1.7

netbios-type m-node

option 2 ip-address 1.1.1.1

expired day 1 hour 0 minute 0 second 0

Pool name: 2

Network 20.1.3.0 mask 255.255.255.0

address range 20.1.3.1 to 20.1.3.15

class departmentA range 20.1.3.20 to 20.1.3.29

class departmentB range 20.1.3.30 to 20.1.3.40

next-server 20.1.3.33

tftp-server domain-name www.dian.org.cn

tftp-server ip-address 192.168.0.120

voice-config ncp-ip 20.1.3.2

voice-config as-ip 20.1.3.5

voice-config voice-vlan 3 enable

voice-config fail-over 20.1.3.6 123*

option 2 ip-address 20.1.3.10

expired day 1 hour 0 minute 0 second 0

Pool name: 3

static bindings:

ip-address 10.10.1.2 mask 255.0.0.0

hardware-address 00e0-00fc-0001 ethernet

ip-address 10.10.1.3 mask 255.0.0.0

client-identifier aaaa-bbbb

expired unlimited

Command output

| Field | Description |
| --- | --- |
| Pool name | Name of an address pool. If the address pool is assigned by the OVSDB controller, the pool name starts with a question mark (?). For more information about OVSDB, see OVSDB VTEP configuration in VXLAN Configuration Guide. |
| Network | Assignable network. |
| secondary networks | Assignable secondary networks. |
| address range | Assignable address range. |
| class class-name range | DHCP user class and its address range. |
| static bindings | Static IP-to-MAC/client ID bindings. |
| option | Customized DHCP option. |
| expired | Lease duration. |
| bootfile-name | Boot file name |
| dns-list | DNS server IP address. |
| domain-name | Domain name suffix. |
| bims-server | BIMS server information. |
| forbidden-ip | IP addresses excluded from dynamic allocation. |
| gateway-list | Gateway addresses. |
| nbns-list | WINS server addresses. |
| netbios-type | NetBIOS node type. |
| next-server | Next server IP address. |
| tftp-server domain-name | TFTP server name. |
| tftp-server ip-address | TFTP server address. |
| voice-config ncp-ip | Primary network calling processor address. |
| voice-config as-ip | Backup network calling processor address. |
| voice-config voice-vlan | Voice VLAN. |
| voice-config fail-over | Failover route. |

Modified feature: Displaying DHCPv6 address pool information

Feature change description

The display ipv6 dhcp pool command output supports displaying the time unit for the preferred lifetime and valid lifetime.

Command changes

Modified command: display ipv6 dhcp pool

Syntax

display ipv6 dhcp pool [ pool-name | vpn-instance vpn-instance-name ]

Views

Any view

Examples

After modification, the time unit is added to the preferred lifetime and valid lifetime.

The following shows the examples.

# Display information about DHCPv6 address pool 1.

<Sysname> display ipv6 dhcp pool 1

DHCPv6 pool: 1

Network: 3FFE:501:FFFF:100::/64

Preferred lifetime 604800 seconds, valid lifetime 2592000 seconds

Prefix pool: 1

Preferred lifetime 24000 seconds, valid lifetime 36000 seconds

Addresses:

Range: from 3FFE:501:FFFF:100::1

to 3FFE:501:FFFF:100::99

Preferred lifetime 70480 seconds, valid lifetime 200000 seconds

Total address number: 153

Available: 153

In-use: 0

Temporary addresses:

Range: from 3FFE:501:FFFF:100::200

to 3FFE:501:FFFF:100::210

Preferred lifetime 60480 seconds, valid lifetime 259200 seconds

Total address number: 17

Available: 17

In-use: 0

Static bindings:

DUID: 0003000100e0fc000001

IAID: 0000003f

Prefix: 3FFE:501:FFFF:200::/64

Preferred lifetime 604800 seconds, valid lifetime 2592000 seconds

DUID: 0003000100e0fc00cff1

IAID: 00000001

Address: 3FFE:501:FFFF:2001::1/64

Preferred lifetime 604800 seconds, valid lifetime 2592000 seconds

DNS server addresses:

2::2

Domain name:

aaa.com

SIP server addresses:

5::1

SIP server domain names:

bbb.com

# Display information about DHCPv6 address pool 1, which contains an ineffective subnet prefix.

<Sysname> display ipv6 dhcp pool 1

DHCPv6 pool: 1

Network: Not-available

Preferred lifetime 604800 seconds, valid lifetime 2592000 seconds

# Display information about DHCPv6 address pool 1, in which the subnet prefix is ineffective after a configuration recovery.

<Sysname> display ipv6 dhcp pool 1

DHCPv6 pool: 1

Network: 1::/64(Zombie)

Preferred lifetime 604800 seconds, valid lifetime 2592000 seconds

Command output

| Field | Description |
| --- | --- |
| DHCPv6 pool | Name of the DHCPv6 address pool. |
| Network | IPv6 subnet for dynamic IPv6 address allocation. If the subnet prefix is ineffective, this field displays Not-available. If the subnet prefix becomes ineffective after a configuration recovery (for example, a switchover from the backup to the master), the prefix is marked (Zombie). |
| Prefix pool | Prefix pool referenced by the address pool. |
| Preferred lifetime | Preferred lifetime in seconds. |
| valid lifetime | Valid lifetime in seconds. |
| Addresses | Non-temporary IPv6 address range. |
| Range | IPv6 address range for dynamic allocation. |
| Total address number | Total number of IPv6 addresses. |
| Available | Total number of available IPv6 addresses. |
| In-use | Total number of assigned IPv6 addresses. |
| Temporary addresses | Temporary IPv6 address range for dynamic allocation. |
| Static bindings | Static bindings configured in the address pool. |
| DUID | Client DUID. |
| IAID | Client IAID. If no IAID is configured, this field displays Not configured. |
| Prefix | IPv6 address prefix. |
| Address | Static IPv6 address. |
| DNS server addresses | DNS server address. |
| Domain name | Domain name. |
| SIP server addresses | SIP server address. |
| SIP server domain names | Domain name of the SIP server. |

Modified feature: Enabling recording DHCPv6 snooping address entries in VLAN view

Feature change description

This release added support of enabling recording DHCPv6 snooping address entries in VLAN view.

Command changes

Modified command: ipv6 dhcp snooping binding record

Syntax

ipv6 dhcp snooping binding record

undo ipv6 dhcp snooping binding record

Change description

Before modification: This command is available in Layer 2 Ethernet interface view and Layer 2 aggregate interface view.

After modification: This command is available in Layer 2 Ethernet interface view, Layer 2 aggregate interface view, and VLAN view.

Modified feature: Setting the aging timer for ND entries in stale state

Feature change description

This release supports setting the aging timer in seconds for ND entries in stale state.

Command changes

Old syntax

ipv6 neighbor stale-aging aging-time

New syntax

ipv6 neighbor stale-aging { aging-minutes | second aging-seconds }

Parameters

aging-minutes: Specifies the aging timer in minutes for ND entries in stale state, in the range of 1 to 1440.

second aging-seconds: Specifies the aging timer in seconds for ND entries in stale state, in the range of 60 to 86400.

Change description

Before modification: The aging timer can be set in minutes for ND entries in stale state.

After modification: The aging timer can be set in minutes or in seconds for ND entries in stale state.

Modified feature: Displaying member interfaces shut down by Monitor Link

Feature change description

From this release, the display monitor-link group command displays the member interfaces shut down by Monitor Link.

Command changes

Modified command: display monitor-link group

Syntax

display monitor-link group { group-id | all }

Views

Any view

Change description

Before modification: The display monitor-link group command does not display the member interfaces shut down by Monitor Link.

After modification: The display monitor-link group command displays the member interfaces shut down by Monitor Link.

Modified feature: Displaying DLDP configuration

Feature change description

The following fields were added to the command output:

DLDP initial-unidirectional-delay.

Neighbor echo time.

Command changes

Modified command: display dldp

Syntax

display dldp [ interface interface-type interface-number ]

Views

Any view

Change description

After modification, the command displays the DLDP initial-unidirectional-delay and Neighbor echo time fields.

<Sysname> display dldp

DLDP global status: Enabled

DLDP advertisement interval: 5s

DLDP authentication-mode: Simple

DLDP authentication-password: ******

DLDP unidirectional-shutdown mode: Auto

DLDP delaydown-timer value: 1s

Number of enabled ports: 2

Interface Ten-GigabitEthernet1/0/1

DLDP port state: Bidirectional

DLDP port unidirectional-shutdown mode: None

DLDP initial-unidirectional-delay: 100s

Number of the port’s neighbors: 1

Neighbor MAC address: 0023-8956-3600

Neighbor port index: 79

Neighbor state: Confirmed

Neighbor aged time: 13s

Neighbor echo time: -

        - Command output

| Field | Description |
| --- | --- |
| DLDP initial-unidirectional-delay | Delay time in seconds for DLDP to block the interface upon an Initial-to-Unidirectional state transition. |
| Neighbor echo time | Number of seconds remaining before the Echo timer expires. The Echo timer starts when the Entry timer expires. The neighbor information is deleted when the Echo timer expires. A hyphen (-) indicates that the Echo timer has not started. |

Modified feature: Configuring routing policy-based recursive lookup

Feature change description

Support for IPv6 BGP routes in RIB IPv6 address family view was added to this release.

Command changes

Modified command: protocol nexthop recursive-lookup

Syntax

protocol protocol nexthop recursive-lookup route-policy route-policy-name

undo protocol protocol nexthop recursive-lookup route-policy

Views

RIB IPv4 address family view

RIB IPv6 address family view

Change description

Before modification: The command supports only BGP and static routes in RIB IPv4 address family view.

After modification: The command also supports IPv6 BGP routes in RIB IPv6 address family view.

Modified feature: Associating Track with the output interface for a static route

Feature change description

In this release, you can associate a track entry with the output interface for a static route.

Command changes

Modified command: ip route-static

Old syntax

ip route-static { dest-address { mask-length | mask } | group group-name } { interface-type interface-number [ next-hop-address ] [ backup-interface interface-type interface-number [ backup-nexthop backup-nexthop-address ] [ permanent ] | bfd { control-packet | echo-packet } | permanent ] | next-hop-address [ recursive-lookup host-route ] [ bfd control-packet bfd-source ip-address | permanent | track track-entry-number ] | vpn-instance d-vpn-instance-name next-hop-address [ recursive-lookup host-route ] [ bfd control-packet bfd-source ip-address | permanent | track track-entry-number ] } [ preference preference ] [ tag tag-value ] [ description text ]

ip route-static vpn-instance s-vpn-instance-name { dest-address { mask-length | mask } | group group-name } { interface-type interface-number [ next-hop-address ] [ backup-interface interface-type interface-number [ backup-nexthop backup-nexthop-address ] [ permanent ] | bfd { control-packet | echo-packet } | permanent ] | next-hop-address [ recursive-lookup host-route ] [ public ] [ bfd control-packet bfd-source ip-address | permanent | track track-entry-number ] | vpn-instance d-vpn-instance-name next-hop-address [ recursive-lookup host-route ] [ bfd control-packet bfd-source ip-address | permanent | track track-entry-number ] } [ preference preference ] [ tag tag-value ] [ description text ]

New syntax

ip route-static { dest-address { mask-length | mask } | group group-name } { interface-type interface-number [ next-hop-address ] [ backup-interface interface-type interface-number [ backup-nexthop backup-nexthop-address ] [ permanent ] | bfd { control-packet | echo-packet } | permanent | track track-entry-number ] | next-hop-address [ recursive-lookup host-route ] [ bfd control-packet bfd-source ip-address | permanent | track track-entry-number ] | vpn-instance d-vpn-instance-name next-hop-address [ recursive-lookup host-route ] [ bfd control-packet bfd-source ip-address | permanent | track track-entry-number ] } [ preference preference ] [ tag tag-value ] [ description text ]

ip route-static vpn-instance s-vpn-instance-name { dest-address { mask-length | mask } | group group-name } { interface-type interface-number [ next-hop-address ] [ backup-interface interface-type interface-number [ backup-nexthop backup-nexthop-address ] [ permanent ] | bfd { control-packet | echo-packet } | permanent | track track-entry-number ] | next-hop-address [ recursive-lookup host-route ] [ public ] [ bfd control-packet bfd-source ip-address | permanent | track track-entry-number ] | vpn-instance d-vpn-instance-name next-hop-address [ recursive-lookup host-route ] [ bfd control-packet bfd-source ip-address | permanent | track track-entry-number ] } [ preference preference ] [ tag tag-value ] [ description text ]

Views

System view

Change description

Before modification: You cannot associate a track entry with the output interface for a static route.

After modification: You can associate a track entry with the output interface for a static route.

Modified feature: Configuring OSPF area authentication

Feature change description

From this release, a key ID becomes optional in HMAC-MD5 and MD5 authentication modes, and a key string becomes optional in simple authentication mode.

Command changes

Modified command: authentication-mode

Old syntax

In MD5/HMAC-MD5 authentication mode:

authentication-mode { hmac-md5 | md5 } key-id { cipher | plain } string

undo authentication-mode [ { hmac-md5 | md5 } key-id ]

In simple authentication mode:

authentication-mode simple { cipher | plain } string

undo authentication-mode

New syntax

In MD5/HMAC-MD5 authentication mode:

authentication-mode { hmac-md5 | md5 } [ key-id { cipher | plain } string ]

undo authentication-mode [ { hmac-md5 | md5 } [ key-id ] ]

In simple authentication mode:

authentication-mode simple [ { cipher | plain } string ]

undo authentication-mode

Views

OSPF area view

Change description

Before modification: A key ID is required in HMAC-MD5 and MD5 authentication modes, and a key string is required in simple authentication mode.

After modification: A key ID is optional in HMAC-MD5 and MD5 authentication modes, and a key string is optional in simple authentication mode.

Modified feature: Configuring OSPF interface authentication

Feature change description

From this release, a key ID becomes optional in HMAC-MD5 and MD5 authentication modes, a key string becomes optional in simple authentication mode, and the none authentication mode is supported.

Command changes

Modified command: ospf authentication-mode

Old syntax

In MD5/HMAC-MD5 authentication mode:

ospf authentication-mode { hmac-md5 | md5 } key-id { cipher | plain } string

undo ospf authentication-mode { hmac-md5 | md5 } key-id

In simple authentication mode:

ospf authentication-mode simple { cipher | plain } string

undo ospf authentication-mode simple

New syntax

In MD5/HMAC-MD5 authentication mode:

ospf authentication-mode { hmac-md5 | md5 } [ key-id { cipher | plain } string ]

undo ospf authentication-mode { hmac-md5 | md5 } [ key-id ]

In simple authentication mode:

ospf authentication-mode simple [ { cipher | plain } string ]

undo ospf authentication-mode simple

In none authentication mode:

ospf authentication-mode none

undo ospf authentication-mode none

Views

Interface view

Change description

Before modification: A key ID is required in HMAC-MD5 and MD5 authentication modes, a key string is required in simple authentication mode, and the none authentication mode is not supported.

After modification: A key ID is optional in HMAC-MD5 and MD5 authentication modes, a key string is optional in simple authentication mode, and the none authentication mode is supported.

Modified feature: Configuring a virtual link

Feature change description

From this release, a key ID becomes optional in HMAC-MD5 and MD5 authentication modes, a key string becomes optional in simple authentication mode, and the none authentication mode is supported.

Command changes

Modified command: vlink-peer

Old syntax

vlink-peer router-id [ dead seconds | hello seconds | { { hmac-md5 | md5 } key-id { cipher | plain } string | keychain keychain-name | simple { cipher | plain } string } | retransmit seconds | trans-delay seconds ] *

undo vlink-peer router-id [ dead | hello | { hmac-md5 | md5 } key-id | keychain | retransmit | simple | trans-delay ] *

New syntax

vlink-peer router-id [ dead seconds | hello seconds | [ authentication-none | { hmac-md5 | md5 } [ key-id { cipher | plain } string ] | keychain keychain-name | simple [ { cipher | plain } string ] ] | retransmit seconds | trans-delay seconds ] *

undo vlink-peer router-id [ dead | hello | [ authentication-none | { hmac-md5 | md5 } [ key-id ] | keychain ] | retransmit | simple | trans-delay ] *

Views

Interface view

Change description

Before modification: A key ID is required in HMAC-MD5 and MD5 authentication modes, a key string is required in simple authentication mode, and the none authentication mode is not supported.

After modification: A key ID is optional in HMAC-MD5 and MD5 authentication modes, a key string is optional in simple authentication mode, and the none authentication mode is supported.

Modified feature: Setting the number of OSPF logs

Feature change description

From this release, you can set the number of logs for the following types of hello packets:

Received hello packets.

Received abnormal hello packets.

Received hello packets that were dropped.

Sent hello packets.

Sent abnormal hello packets.

Hello packets that failed to be sent.

The maximum number of LSA aging, neighbor state change, or route calculation logs that OSPF can generate by default is also changed in this release.

Command changes

Modified command: event-log

Old syntax

event-log { lsa-flush | peer | spf } size count

undo event-log { lsa-flush | peer | spf } size

New syntax

event-log { hello { received [ abnormal | dropped ] | sent [ abnormal | failed ] } | lsa-flush | peer | spf } size count

undo event-log { hello { received [ abnormal | dropped ] | sent [ abnormal | failed ] } | lsa-flush | peer | spf } size

Views

OSPF view

Change description

Before modification:

The hello, received, sent, abnormal, dropped, and failed keywords are not supported. You cannot set the number of logs for hello packets.

OSPF can generate a maximum of 10 LSA aging, neighbor state change, or route calculation logs by default.

After modification:

The hello, received, sent, abnormal, dropped, and failed keywords are added to this command to enable you to set the number of logs for hello packets.

OSPF can generate a maximum of 100 LSA aging, neighbor state change, or route calculation logs by default.

Modified feature: Displaying IS-IS LSP log information

Feature change description

From this release, you must specify the refreshed or purged keyword when displaying IS-IS LSP log information.

Command changes

Old syntax

display isis event-log lsp [ level-1 | level-2 ] [ process-id ]

New syntax

display isis event-log lsp { purged | refreshed } [ level-1 | level-2 ] [ process-id ]

Change description

Before modification, the refreshed and purged keywords are not available.

After modification, you must specify the refreshed or purged keyword for the command.

Modified feature: Clearing IS-IS LSP log information

Feature change description

From this release, you must specify the refreshed or purged keyword when clearing IS-IS LSP log information.

Command changes

Old syntax

reset isis event-log lsp [ process-id ]

New syntax

reset isis event-log lsp { purged | refreshed } [ process-id ]

Change description

Before modification, the refreshed and purged keywords are not available.

After modification, you must specify the refreshed or purged keyword for the command.

Modified feature: Specifying a label allocation mode

Feature change description

From this release, BGP supports allocating a label to each VPN instance.

Command changes

Modified command: label-allocation-mode

Old syntax

label-allocation-mode per-prefix

New syntax

label-allocation-mode { per-prefix | per-vrf }

Views

BGP instance view

Change description

Before modification, BGP does not support allocating a label to each VPN instance.

After modification, BGP supports allocating a label to each VPN instance. When you specify the per-VPN instance label allocation mode, do not execute the vpn popgo command because it is mutually exclusive with the label-allocation-mode per-vrf command. The egress PE will pop the label for each packet and forward the packet through the FIB table.

Modified feature: Displaying BGP peer or peer group information

Feature change description

Support for displaying peer or peer group information of all VPN instances was added to BGP.

Command changes

Modified command: display bgp peer

Old syntax

display bgp [ instance instance-name ] peer ipv4 [ unicast ] vpn-instance vpn-instance-name [ [ ipv4-address ] verbose ]

display bgp [ instance instance-name ] peer ipv6 [ unicast ] vpn-instance vpn-instance-name [ [ ipv6-address ] verbose ]

New syntax

display bgp [ instance instance-name ] peer ipv4 [ unicast ] { vpn-instance vpn-instance-name [ [ ipv4-address ] verbose ] | vpn-instance-all [ verbose ] }

display bgp [ instance instance-name ] peer ipv6 [ unicast ] { vpn-instance vpn-instance-name [ [ ipv6-address ] verbose ] | vpn-instance-all [ verbose ] }

Views

BGP instance view

Change description

Before modification: BGP cannot display peer or peer group information for all VPN instances.

After modification: BGP can display peer or peer group information for all VPN instances.

Modified feature: Displaying AS path information for BGP routes

Feature change description

A maximum of 16 AS numbers can be displayed for the Path/Ogn field in the output from the following commands (exceeding AS numbers are omitted and are available by displaying detailed routing information):

display bgp link-state

display bgp routing-table dampened

display bgp routing-table flap-info

display bgp routing-table ipv4 multicast

display bgp routing-table ipv4 rtfilter

display bgp routing-table ipv4 unicast

display bgp routing-table ipv6 multicast

display bgp routing-table ipv6 unicast

Command changes

Syntax

display bgp [ instance instance-name ] link-state [ ls-prefix | peer { ipv4-address | ipv6-address } { advertised | received } [ statistics ] | statistics ]

display bgp [ instance instance-name ] routing-table dampened { ipv4 | ipv6 } { multicast | [ unicast ] [ vpn-instance vpn-instance-name ] }

display bgp [ instance instance-name ] routing-table flap-info ipv4 { multicast | [ unicast ] [ vpn-instance vpn-instance-name ] } [ ipv4-address [ { mask-length | mask } [ longest-match ] ] | as-path-acl as-path-acl-number ]

display bgp [ instance instance-name ] routing-table flap-info ipv6 { multicast | [ unicast ] [ vpn-instance vpn-instance-name ] } [ ipv6-address prefix-length | as-path-acl as-path-acl-number ]

display bgp [ instance instance-name ] routing-table ipv4 multicast [ ipv4-address [ { mask-length | mask } [ longest-match ] ] | ipv4-address [ mask-length | mask ] advertise-info | as-path-acl as-path-acl-number | community-list { { basic-community-list-number | comm-list-name } [ whole-match ] | adv-community-list-number } | peer ipv4-address { advertised-routes | received-routes } [ ipv4-address [ mask-length | mask ] | statistics ] | statistics ]

display bgp [ instance instance-name ] routing-table ipv4 [ unicast ] [ vpn-instance vpn-instance-name ] [ ipv4-address [ { mask-length | mask } [ longest-match ] ] | ipv4-address [ mask-length | mask ] advertise-info | as-path-acl as-path-acl-number | community-list { { basic-community-list-number | comm-list-name } [ whole-match ] | adv-community-list-number } | peer ipv4-address { advertised-routes | received-routes } [ ipv4-address [ mask-length | mask ] | statistics ] | statistics ]

display bgp [ instance instance-name ] routing-table ipv6 multicast  [ ipv6-address prefix-length [ advertise-info ] | as-path-acl as-path-acl-number | community-list { { basic-community-list-number | comm-list-name } [ whole-match ] | adv-community-list-number } | peer ipv6-address { advertised-routes | received-routes } [ ipv6-address prefix-length | statistics ] | statistics ]

display bgp [ instance instance-name ] routing-table ipv6 [ unicast ] [ vpn-instance vpn-instance-name ] [ ipv6-address prefix-length [ advertise-info ] | as-path-acl as-path-acl-number | community-list { { basic-community-list-number | comm-list-name } [ whole-match ] | adv-community-list-number } | peer ipv6-address { advertised-routes | received-routes } [ ipv6-address prefix-length | statistics ] | statistics ]

display bgp [ instance instance-name ] routing-table ipv6 [ unicast ] peer ipv4-address { advertised-routes | received-routes } [ ipv6-address prefix-length | statistics ]

Views

Any view

Change description

Before modification, all AS numbers are displayed for the Path/Ogn field in the command output, as shown in the following example.

# Display brief information about all BGP IPv4 unicast routes.

<Sysname> display bgp routing-table ipv4

Total number of routes: 4

BGP local router ID is 192.168.100.1

Status codes: * - valid, > - best, d - dampened, h - history,

s - suppressed, S - stale, i - internal, e - external

a – additional-path

Origin: i - IGP, e - EGP, ? - incomplete

Network            NextHop         MED        LocPrf     PrefVal Path/Ogn

* >  10.2.1.0/24        10.2.1.1        0                     0       i

e                    10.2.1.2        0                     0       200 201

202 203

204 205

206 207

208 209

210 211

212 213

214 215

216 217

218i

* >  192.168.1.0        192.168.1.135   0                     0       i

*  e                    10.2.1.2        0                     0       200i

After modification, a maximum of 16 AS numbers can be displayed for the Path/Ogn field in the command output. Exceeding AS numbers are omitted and are available by displaying detailed routing information.

# Display brief information about all BGP IPv4 unicast routes.

<Sysname> display bgp routing-table ipv4

Total number of routes: 4

BGP local router ID is 192.168.100.1

Status codes: * - valid, > - best, d - dampened, h - history,

s - suppressed, S - stale, i - internal, e - external

a – additional-path

Origin: i - IGP, e - EGP, ? - incomplete

Network            NextHop         MED        LocPrf     PrefVal Path/Ogn

* >  10.2.1.0/24        10.2.1.1        0                     0       i

e                    10.2.1.2        0                     0       200 201

202 203 204 205 206 207 208 209 210 211 212 213 214 215 ...i

* >  192.168.1.0        192.168.1.135   0                     0       i

*  e                    10.2.1.2        0                     0       200i

Modified feature: Displaying RPF information for an IPv6 multicast source

Feature change description

Support for displaying the AS number of the source-side PE and the route target attribute of the C-multicast route was added.

Command changes

Modified command: display ipv6 multicast rpf-info

Syntax

display ipv6 multicast [ vpn-instance vpn-instance-name ] rpf-info ipv6-source-address [ ipv6-group-address ]

Views

Any view

Change description

Before modification: The Source AS and C-multicast route target fields are not supported.

# Display RPF information for IPv6 multicast source 2001::101 on the public network.

<Sysname> display ipv6 multicast rpf-info 2001::101

RPF information about source 2001::101:

RPF interface: Vlan-interface1, RPF neighbor: FE80::A01:101:1

Referenced prefix/prefix length: 2001::/64

Referenced route type: igp

Route selection rule: preference-preferred

Load splitting rule: disable

After modification: The Source AS and C-multicast route target fields are supported.

# Display RPF information for IPv6 multicast source 2001::101 on the public network.

<Sysname> display ipv6 multicast rpf-info 2001::101

RPF information about source 2001::101:

RPF interface: Vlan-interface1, RPF neighbor: FE80::A01:101:1

Referenced prefix/prefix length: 2001::/64

Referenced route type: igp

Route selection rule: preference-preferred

Load splitting rule: disable

Source AS: 0

C-multicast route target: 0x0000000000000000

Command output

| Field | Description |
| --- | --- |
| RPF information about source 2001::101 | RPF information of the IPv6 multicast source 2001::101. |
| RPF interface | Type and number of the RPF interface. |
| RPF neighbor | IPv6 address (link-local address) of the RPF neighbor. |
| Referenced prefix/prefix length | Referenced route and its prefix length. |
| Referenced route type | Type of the referenced route: - igp—IPv6 IGP unicast route. - egp—IPv6 EGP unicast route. - unicast (direct) —IPv6 directly connected unicast route. - unicast—Other IPv6 unicast route, such as IPv6 unicast static route. - mbgp—IPv6 MBGP route. |
| Route selection rule | RPF route selection rule: - Route preference. - Longest prefix match. |
| Load splitting rule | Whether load splitting is enabled. |
| Source AS | AS number of the source-side PE. NOTE: This field is supported in this version and later. |
| C-multicast route target | Route target attribute value of the C-multicast route. NOTE: This field is supported in this version and later. |

Modified feature: Displaying statistics for the IGMP messages and PIMv2 hello messages learned through IGMP snooping

Feature change description

The layout of command output was changed and support for displaying the number of IGMP messages received on the DRNI IPP was added.

Command changes

Modified command: display igmp-snooping statistics

Syntax

display igmp-snooping statistics

Views

Any view

Change description

Before modification: The DRNI field is not supported.

# Display statistics for the IGMP messages and PIMv2 hello messages learned through IGMP snooping.

<Sysname> display igmp-snooping statistics

Received IGMP general queries:  0

Received IGMPv1 reports:  0

Received IGMPv2 reports:  19

Received IGMP leaves:  0

Received IGMPv2 specific queries:  0

Sent     IGMPv2 specific queries:  0

Received IGMPv3 reports:  1

Received IGMPv3 reports with right and wrong records:  0

Received IGMPv3 specific queries:  0

Received IGMPv3 specific sg queries:  0

Sent     IGMPv3 specific queries:  0

Sent     IGMPv3 specific sg queries:  0

Received PIMv2 hello:  0

Received error IGMP messages:  19

After modification: The DRNI field is supported.

# Display statistics for the IGMP messages and PIMv2 hello messages learned through IGMP snooping.

<Sysname> display igmp-snooping statistics

Received IGMP messages:

IGMP general queries: 19

DRNI: 19

IGMPv2 specific queries: 10

DRNI: 0

IGMPv3 specific queries: 0

DRNI: 0

IGMPv3 specific sg queries: 0

DRNI: 0

IGMPv1 reports: 0

DRNI: 0

IGMPv2 reports: 19

DRNI: 19

IGMPv3 reports: 0

DRNI: 0

IGMPv3 reports with right and wrong records: 0

DRNI: 0

IGMP leaves: 5

DRNI: 5

Error IGMP messages: 1

DRNI: 1

Sent IGMP messages:

IGMPv2 specific queries: 0

IGMPv3 specific queries: 0

IGMPv3 specific sg queries: 0

Received PIMv2 hello : 0

Command output

| Field | Description |
| --- | --- |
| general queries | Number of IGMP general queries. |
| specific queries | Number of IGMP group-specific queries. |
| reports | Number of IGMP reports. |
| leaves | Number of IGMP leave messages. |
| reports with right and wrong records | Number of IGMP reports with correct and incorrect records. |
| specific sg queries | Number of IGMP group-and-source-specific queries. |
| error IGMP messages | Number of IGMP messages with errors. |
| DRNI | IGMP messages received on the DRNI IPP. NOTE: This field is supported in this version and later. |

Modified feature: Displaying statistics for the MLD messages and IPv6 PIM hello messages learned through MLD snooping

Feature change description

The layout of the command output was changed and support for displaying the number of MLD messages received on the DRNI IPP was added.

Command changes

Modified command: display mld-snooping statistics

Syntax

display mld-snooping statistics

Views

Any view

Change description

Before modification: The DRNI field is not supported.

# Display statistics for the MLD messages and IPv6 PIM hello messages learned through MLD snooping.

<Sysname> display mld-snooping statistics

Received MLD general queries:  0

Received MLDv1 specific queries:  0

Received MLDv1 reports:  0

Received MLD dones:  0

Sent     MLDv1 specific queries:  0

Received MLDv2 reports:  0

Received MLDv2 reports with right and wrong records:  0

Received MLDv2 specific queries:  0

Received MLDv2 specific sg queries:  0

Sent     MLDv2 specific queries:  0

Sent     MLDv2 specific sg queries:  0

Received IPv6 PIM hello:  0

Received error MLD messages:  0

After modification: The DRNI field is supported.

# Display statistics for the MLD messages and IPv6 PIM hello messages learned through MLD snooping.

<Sysname> display mld-snooping statistics

Received MLD messages:

MLD general queries: 19

DRNI: 19

MLDv1 specific queries: 0

DRNI: 0

MLDv2 specific queries: 0

DRNI: 0

MLDv2 specific sg queries: 0

DRNI: 0

MLDv1 reports: 19

DRNI: 19

MLDv2 reports: 0

DRNI: 0

MLDv2 reports with right and wrong records: 0

DRNI: 0

MLD dones: 5

DRNI: 5

Error MLD messages: 1

DRNI: 1

Sent MLD messages:

MLDv1 specific queries: 0

MLDv2 specific queries: 0

MLDv2 specific sg queries: 0

Received IPv6 PIM hello: 0

Command output

| Field | Description |
| --- | --- |
| general queries | Number of MLD general queries. |
| specific queries | Number of MLD multicast-address-specific queries. |
| reports | Number of MLD reports. |
| dones | Number of MLD done messages. |
| reports with right and wrong records | Number of MLD reports with correct and incorrect records. |
| specific sg queries | Number of MLD multicast-address-and-source-specific queries. |
| error MLD messages | Number of MLD messages with errors. |
| DRNI | MLD messages received on the DRNI IPP. NOTE: This field is supported in this version and later. |

Modified feature: Enabling a VPN instance to replicate routes from the public network or other VPN instances

Feature change description

This release allows a VPN instance to advertise the BGP routes replicated from the public network or another VPN instance.

Command changes

Modified command: route-replicate

Old syntax

route-replicate from { public | vpn-instance vpn-instance-name } protocol bgp as-number [ route-policy route-policy-name ]

New syntax

route-replicate from { public | vpn-instance vpn-instance-name } protocol bgp as-number [ advertise ] [ route-policy route-policy-name ]

Views

VPN instance IPv4 address family view

Change description

Before modification: The advertise keyword is not available when you enable a VPN instance to replicate BGP routes from the public network or another VPN instance.

After modification: The advertise keyword is available when you enable a VPN instance to replicate BGP routes from the public network or another VPN instance. This keyword allows the VPN instance to advertise the replicated BGP routes.

Modified feature: Specifying outgoing labels for a static SRLSP

Feature change description

From this release, the maximum number of outgoing labels that can be specified for a static SRLSP was changed to 4.

Command changes

Modified command: static-sr-mpls lsp

Syntax

static-sr-mpls lsp lsp-name out-label out-label-value&<1-n>

Views

System view

Change description

The value for the n argument was changed from 6 to 4.

Modified feature: Creating an ACL

Feature change description

This software version allows you to specify both a name and a number when creating an ACL.

Command changes

Modified command: acl

Old syntax

acl [ ipv6 ] { advanced | basic } { acl-number | name acl-name } [ match-order { auto | config } ]

acl mac { acl-number | name acl-name } [ match-order { auto | config } ]

acl user-defined { acl-number | name acl-name }

undo acl [ ipv6 ] { all | { advanced | basic } { acl-number | name acl-name } }

undo acl mac { all | acl-number | name acl-name }

undo acl user-defined { all | acl-number | name acl-name }

New syntax

Command set 1:

acl [ ipv6 ] { name acl-name | number acl-number [ name acl-name ] [ match-order { auto | config } ] }

undo acl [ ipv6 ] { all | name acl-name | number acl-number }

Command set 2:

acl [ ipv6 ] { advanced | basic } { acl-number | name acl-name } [ match-order { auto | config } ]

acl mac { acl-number | name acl-name } [ match-order { auto | config } ]

acl user-defined { acl-number | name acl-name }

undo acl [ ipv6 ] { all | { advanced | basic } { acl-number | name acl-name } }

undo acl mac { all | acl-number | name acl-name }

undo acl user-defined { all | acl-number | name acl-name }

Views

System view

Change description

Before modification: Command set 1 is not supported. You can create an ACL by specifying a name or a number, but not both. When creating a Layer 2 or user-defined ACL, you must specify the mac or user-defined keyword.

After modification: Command set 1 is supported. You can create an ACL by specifying both a name and a number. You can create a Layer 2 or user-defined ACL without specifying the mac or user-defined keyword.

Modified feature: Referencing an ACL in QoS or packet filtering

Feature change description

This software version allows you to reference a Layer 2 or user-defined ACL in QoS or packet filtering without specifying the mac or user-defined keyword..

Command changes

Modified command: none

Change description

The feature change applies to the following commands:

acl copy

display acl

display packet-filter statistics

display packet-filter statistics sum

display packet-filter verbose

if-match acl

packet-filter (Ethernet service instance view)

packet-filter (interface view)

packet-filter vlan-interface

reset acl counter

reset packet-filter statistics

Before modification: When referencing a Layer 2 or user-defined ACL in QoS or packet filtering, you must specify the mac or user-defined keyword.

After modification: You can reference a Layer 2 or user-defined ACL without specifying the mac or user-defined keyword.

Modified feature: Enabling hardware-count for the packet filtering default action

Feature change description

In this release, this feature can be configured in Ethernet service instance view.

Command changes

Modified command: packet-filter default hardware-count

Syntax

packet-filter default { inbound | outbound } hardware-count

undo packet-filter default { inbound | outbound } hardware-count

Views

Layer 2 Ethernet interface view

Layer 2 aggregate interface view

Layer 3 Ethernet interface view

Layer 3 Ethernet subinterface view

Layer 3 aggregate interface view

VLAN interface view

VSI interface view

Ethernet service instance view

Change description

Before modification: The packet-filter default hardware-count command cannot be configured in Ethernet service instance view.

After modification: The packet-filter default hardware-count command can be configured in Ethernet service instance view.

Modified feature: Configuring drop-level-based parameters for a queue in a WRED table

Feature change description

This release added support for the drop-level drop-level option in the undo form of the command. You can restore the default for the command based on drop levels.

Command changes

Modified command: queue

Old syntax

queue queue-id [ drop-level drop-level ] low-limit low-limit high-limit high-limit [ discard-probability discard-prob ]

undo queue { queue-id | all }

New syntax

queue queue-id [ drop-level drop-level ] low-limit low-limit high-limit high-limit [ discard-probability discard-prob ]

undo queue { queue-id [ drop-level drop-level ] | all }

Views

WRED table view

Change description

Before modification: The drop-level drop-level option is not available in the undo form of the command.

After modification: The drop-level drop-level option is available in the undo form of the command.

Modified feature: Configuring binding attributes for a local user

Feature change description

Support of the binding interface attribute was added to device management users.

Command changes

Modified command: bind-attribute

Syntax

bind-attribute { ip ip-address | location interface interface-type interface-number | mac mac-address | vlan vlan-id } *

Views

Local user view

Change description

Before modification: The binding interface attribute is applicable only to network access users (LAN and portal users).

After modification: The binding interface attribute is applicable to both device management users and network access users (LAN and portal users).

Modified feature: Password handling when global password control is enabled

Feature change description

The following changes were added:

The way that the device handles passwords of device management users when global password control is enabled.

Before modification:

A password set in plaintext form is saved in encrypted form and a password set in hashed form is not saved.

If a user changes its own password in plaintext form, the new password must have a minimum of four characters different from the current password and any password in the history records. If the user changes its own password in hashed form, the system does not compare the new password with the current password or passwords in the history records.

If a user deletes its own password, the system does not request the user to enter the current plaintext password.

In FIPS mode, if a user with the network-admin user role changes its password, the system does not request the user to enter the current plaintext password.

After modification:

All passwords in the history records are saved in hashed form.

If a user changes its own password in plaintext form, the system requests the user to enter the current plaintext password. The new password must be different from all passwords in the history records and the current password. In addition, the new password must have a minimum of four characters different from the current password.

If a user changes the password for another user in plaintext form, the new password must be different from the latter user's all passwords in the history records and current password.

If a user deletes its own password, the system requests the user to enter the current plaintext password.

Except the above listed situations, the system does not request a user to enter the current plaintext password or compare the new password with passwords in the history records and the current password.

The way the device handles super passwords when global password control is enabled:

Before modification:

If a super new password is set in plaintext form, the password is saved in encrypted form. If a new super password is set in hashed form, the password is not saved.

If a super password is changed in plaintext form, the new password must have a minimum of four characters different from the current password and any password in the history records. If a super password is changed in hashed form, the system does not compare the new password with the current password and passwords in the history records.

After modification:

All super passwords in the history records are saved in hashed form.

If a super password is changed in plaintext form, the new password must be different from all passwords in the history records and the current password. If a super password is changed in hashed form, the system does not compare the new password with the current one and those stored in the history password records.

Command changes

None.

Modified feature: Setting the quiet timer for RADIUS servers

Feature change description

The minimum value of the RADIUS server quiet timer was changed from 1 minute to 0 minutes.

Command changes

Modified command: timer quiet (RADIUS scheme view)

Syntax

timer quiet minutes

undo timer quiet

Views

RADIUS scheme view

Change description

Before modification: The value range for the minutes argument is 1 to 255 minutes.

After modification: The value range for the minutes argument is 0 to 255 minutes. If you set this argument to 0, the device does not change the state of the current server for a user when the server is unreachable. It sends an authentication or accounting request of the user to the next server in active state. For an authentication or accounting request of a new user, it still tries to send the request to the current server because the current server is in active state.

Modified feature: Configuring MAC-based MAC authentication user accounts

Feature change description

Support for password configuration was added to MAC-based MAC authentication user accounts.

Command changes

Modified command: mac-authentication user-name-format

Old syntax

mac-authentication user-name-format { fixed [ account name ] [ password { cipher | simple } string ] | mac-address [ { with-hyphen | without-hyphen } [ lowercase | uppercase ] ] }

New syntax

mac-authentication user-name-format { fixed [ account name ] | mac-address [ { with-hyphen | without-hyphen } [ lowercase | uppercase ] ] } [ password { cipher | simple } string ]

Views

System view

Change description

Before modification: You cannot specify a password for MAC-based MAC authentication user accounts. The MAC address of each user is used as the password.

After modification: You can specify a password for all MAC-based MAC authentication user accounts by using the password { cipher | simple } string option. If you do not specify a password, each user uses its own MAC address as the password.

Modified feature: MAC authentication VLAN mode

Feature change description

As from this version, the device changes its manner in handling an online authenticated user that meets the following requirements:

The user accesses a port in MAC authentication single-VLAN mode from a VLAN different from the VLAN where it was authenticated the last time.

The user has been assigned an authorization VLAN.

Command changes

Modified command: mac-authentication host-mode

Syntax

mac-authentication host-mode multi-vlan

Views

Layer 2 Ethernet interface view

Change description

As from this version, the device changes its manner in handling an online authenticated user that meets the following requirements:

The user accesses a port in MAC authentication single-VLAN mode from a VLAN different from the VLAN where it was authenticated the last time.

The user has been assigned an authorization VLAN.

Before modification: When the user accesses the port from a new VLAN, the device does not authenticate the user or log off the user from the authorization VLAN. The user is online in the authorization VLAN.

After modification: When the user accesses the port from a new VLAN, the device handles the user depending on the status of the port security MAC move feature.

If port security MAC move is disabled, the user cannot pass authentication and come online from the new VLAN until after it goes offline from the port.

If port security MAC move is enabled, the user can pass authentication on the new VLAN and come online without having to first go offline from the port. After the user passes authentication on the new VLAN, the original authentication session of the user is deleted from the port.

No changes were introduced to the command syntax.

|  | NOTE: To enable single-VLAN mode, execute the undo form of the mac-authentication host-mode multi-vlan command. To enable the port security MAC move feature, use the port-security mac-move permit command. |
| --- | --- |

Modified feature: Port security MAC move

Feature change description

As from this version, the port security MAC move feature takes effect on users that move between VLANs on a port in addition to users that move between ports.

Command changes

Modified command: port-security mac-move permit

Syntax

port-security mac-move permit

Views

System view

Change description

Before modification: Port security MAC move setting takes effect only on users that move between ports on the device.

After modification: Port security MAC move setting also take effect on users that move between VLANs on a port in a VXLAN environment. In additional, user packets are VLAN tagged.

If this feature is disabled, authenticated users must go offline from the original VLAN first before they can be reauthenticated successfully on the new VLAN and come online.

If this feature is enabled, authenticated users can be reauthenticated successfully on the new VLAN without having to go offline from the original VLAN. The port will remove the users from the original VLAN immediately after the users are reauthenticated successfully on the new VLAN.

No changes were introduced to the command syntax.

|  | NOTE: MAC authentication multi-VLAN mode has higher priority than MAC move for users moving between VLANs on a port. If MAC authentication multi-VLAN mode is enabled, these users can come online in the new VLAN without being reauthenticated. To enable MAC authentication multi-VLAN mode, use the mac-authentication host-mode multi-vlan command. |
| --- | --- |

Modified feature: Web authentication support for HTTPS redirection

Before modification: Web authentication supports only HTTP redirection. It does not support HTTPS redirection.

After modification: Web authentication supports both HTTP redirection and HTTPS redirection. To redirect HTTPS requests of users, execute the http-redirect https-port command in system view to specify an HTTPS redirect listening port number.

Modified feature: RSA key modulus length

Feature change description

The supported RSA key modulus length was changed.

Command changes

Modified command: public-key local create

Syntax

In non-FIPS mode:

public-key local create { dsa | ecdsa [ secp192r1 | secp256r1 | secp384r1 | secp521r1 ] | rsa } [ name key-name ]

In FIPS mode:

public-key local create { dsa | ecdsa [ secp256r1 | secp384r1 | secp521r1 ] | rsa } [ name key-name ]

Views

System view

Change description

Before modification, the key modulus length of a local RSA key pair is as follows:

In non-FIPS mode:

Value range: 512 to 2048 bits.

Default: 1024 bits.

To ensure security, use a minimum of 768 bits.

In FIPS mode: 2048 bits.

After modification, the key modulus length of a local RSA key pair is as follows:

In non-FIPS mode:

Value range: 512 to 4096 bits.

Default: 1024 bits.

To ensure security, use a minimum of 768 bits.

In FIPS mode:

Value range: A multiple of 256 bits in the range of 2048 to 4096 bits.

Default: 2048 bits.

Modified feature: RSA key modulus length used for certification request in a PKI domain

Feature change description

The value range for the RSA key modulus length used for certificate request in a PKI domain was changed.

Command changes

Modified command: public-key rsa

Syntax

public-key rsa { { encryption name encryption-key-name [ length key-length ] | signature name signature-key-name [ length key-length ] } * | general name key-name [ length key-length ] }

Views

PKI domain view

Feature change description

Before modification:

In non-FIPS mode, the value range for the key-length argument is 512 to 2048 bits, and the default is 1024 bits.

In FIPS mode, the value for the key-length argument must be 2048 bits.

After modification:

In non-FIPS mode, the value range for the key-length argument is 512 to 4096 bits, and the default is 1024 bits.

In FIPS mode, the value for the key-length argument must be a multiple of 256 in the range of 2048 to 4096, and the default is 2048.

Modified feature: Displaying IPv4SG bindings

Feature change description

This feature has the following changes:

Support for displaying IPv4SG bindings generated based on ARP snooping for VLANs was added.

Support for displaying IPv4SG bindings generated based on ARP snooping for VSIs was added.

Support for display IPv4SG bindings generated based on the ARP flood suppression module was removed.

Command changes

Modified command: display ip source binding

Old syntax

display ip source binding [ static | [ vpn-instance vpn-instance-name ] [ arp-snooping | arp-suppression | dhcp-relay | dhcp-server | dhcp-snooping | dot1x ] ] [ ip-address ip-address ] [ mac-address mac-address ] [ vlan vlan-id ] [ interface interface-type interface-number ] [ slot slot-number ]

New syntax

display ip source binding [ static | [ vpn-instance vpn-instance-name ] [ arp-snooping-vlan | arp-snooping-vsi | dhcp-relay | dhcp-server | dhcp-snooping | dot1x ] ] [ ip-address ip-address ] [ mac-address mac-address ] [ vlan vlan-id ] [ interface interface-type interface-number ] [ slot slot-number ]

Views

Any view

Change description

The arp-snooping keyword was changed to the arp-snooping-vlan and arp-snooping-vsi keywords, and the arp-suppression keyword was removed

arp-snooping-vlan: Specifies IPv4SG bindings generated based on ARP snooping for VLANs.

arp-snooping-vsi: Specifies IPv4SG bindings generated based on ARP snooping for VSIs.

Modified feature: Displaying IPv6SG bindings

Feature change description

This feature has the following changes:

Support for displaying IPv6SG bindings generated based on ND snooping for VLANs was added.

Support for displaying IPv6SG bindings generated based on ND snooping for VSIs was added.

Support for display IPv6SG bindings generated based on the ND flood suppression module was removed.

Command changes

Modified command: display ipv6 source binding

Old syntax

display ipv6 source binding [ static | [ vpn-instance vpn-instance-name ] [ dhcpv6-relay | dhcpv6-snooping | dot1x | nd-snooping ] ] [ ip-address ipv6-address ] [ mac-address mac-address ] [ vlan vlan-id ] [ interface interface-type interface-number ] [ slot slot-number ]

New syntax

display ipv6 source binding [ static | [ vpn-instance vpn-instance-name ] [ dhcpv6-relay | dhcpv6-server | dhcpv6-snooping | dot1x | nd-snooping-vlan | nd-snooping-vsi ] ] [ ip-address ipv6-address ] [ mac-address mac-address ] [ vlan vlan-id ] [ interface interface-type interface-number ] [ slot slot-number ]

Views

Any view

Change description

The nd-snooping keyword was changed to the nd-snooping-vlan and nd-snooping-vsi keywords, and the arp-suppression keyword was removed

nd-snooping-vlan: Specifies IPv6SG bindings generated based on ND snooping for VLANs.

nd-snooping-vsi: Specifies IPv6SG bindings generated based on ND snooping for VSIs.

Modified feature: Displaying the MFF configuration for a VLAN

Feature change description

From this release, the display mac-forced-forwarding vlan command does not support displaying the MFF operating mode.

Command changes

Modified command: display mac-forced-forwarding vlan

Syntax

display mac-forced-forwarding vlan vlan-id

Views

Any view

Predefined user roles

network-admin

network-operator

Change description

Before modification: The MFF operating mode is displayed in the command output.

# Display the MFF configuration for VLAN 2.

<Sysname> display mac-forced-forwarding vlan 2

VLAN 2

Mode: Manual/Single

Gateway:

--------------------------------------------------------------------------

192.168.1.42         000f-e200-8046

Server:

--------------------------------------------------------------------------

192.168.1.48         192.168.1.49

        - Command output

| Field | Description |
| --- | --- |
| VLAN 2 | ID of the VLAN to which the gateways belong. |
| Mode | MFF operating mode: Manual—Manual mode. Single—Single-gateway mode. |
| Gateway | IP and MAC addresses of gateways. If no address is learned, this field displays N/A. |
| Server | Server IP addresses. |

After modification: The command does not support displaying the MFF operating mode.

# Display the MFF configuration for VLAN 2.

<Sysname> display mac-forced-forwarding vlan 2

VLAN 2

Gateway:

--------------------------------------------------------------------------

192.168.1.42         000f-e200-8046

Server:

--------------------------------------------------------------------------

192.168.1.48         192.168.1.49

        - Command output

| Field | Description |
| --- | --- |
| VLAN 2 | ID of the VLAN to which the gateways belong. |
| Gateway | IP and MAC addresses of gateways. If no address is learned, this field displays N/A. |
| Server | Server IP addresses. |

Modified feature: Associating Track with application modules

Feature change description

From this release, you cannot configure the notification delay when associating Track with application modules. Creating a track entry associated with an application module enters Track view. You can configure the delay only in Track view for notifying the application module of track entry state changes.

Command changes

Modified command: track bfd ctrl

Old syntax

track track-entry-number bfd ctrl [ interface interface-type interface-number | vpn-instance vpn-instance-name ] remote ip remote-ip-address local ip local-ip-address [ delay { negative negative-time | positive positive-time } * ]

New syntax

track track-entry-number bfd ctrl [ interface interface-type interface-number | vpn-instance vpn-instance-name ] remote ip remote-ip-address local ip local-ip-address

Change description

Before modification: This command allows you to set the notification delay when associating Track with the application module, and does not enter Track view.

After modification: Creating a track entry associated with the application module enters Track view. You can configure the notification delay in Track view.

Modified command: track bfd echo

Old syntax

track track-entry-number bfd echo interface interface-type interface-number remote ip remote-ip-address local ip local-ip-address [ delay { negative negative-time | positive positive-time } * ]

New syntax

track track-entry-number bfd echo interface interface-type interface-number remote ip remote-ip-address local ip local-ip-address

Change description

Before modification: This command allows you to set the notification delay when associating Track with the application module, and does not enter Track view.

After modification: Creating a track entry associated with the application module enters Track view. You can configure the notification delay in Track view.

Modified command: track cfd

Old syntax

track track-entry-number cfd cc service-instance instance-id mep mep-id [ delay { negative negative-time | positive positive-time } * ]

New syntax

track track-entry-number cfd cc service-instance instance-id mep mep-id

Change description

Before modification: This command allows you to set the notification delay when associating Track with the application module, and does not enter Track view.

After modification: Creating a track entry associated with the application module enters Track view. You can configure the notification delay in Track view.

Modified command: track interface

Old syntax

track track-entry-number interface interface-type interface-number [ delay { negative negative-time | positive positive-time } * ]

New syntax

track track-entry-number interface interface-type interface-number

Change description

Before modification: This command allows you to set the notification delay when associating Track with the application module, and does not enter Track view.

After modification: Creating a track entry associated with the application module enters Track view. You can configure the notification delay in Track view.

Modified command: track interface physical

Old syntax

track track-entry-number interface interface-type interface-number physical [ delay { negative negative-time | positive positive-time } * ]

New syntax

track track-entry-number interface interface-type interface-number physical

Change description

Before modification: This command allows you to set the notification delay when associating Track with the application module, and does not enter Track view.

After modification: Creating a track entry associated with the application module enters Track view. You can configure the notification delay in Track view.

Modified command: track interface protocol

Old syntax

track track-entry-number interface interface-type interface-number protocol { ipv4 | ipv6 } [ delay { negative negative-time | positive positive-time } * ]

New syntax

track track-entry-number interface interface-type interface-number protocol { ipv4 | ipv6 }

Change description

Before modification: This command allows you to set the notification delay when associating Track with the application module, and does not enter Track view.

After modification: Creating a track entry associated with the application module enters Track view. You can configure the notification delay in Track view.

Modified command: track ip route reachability

Old syntax

track track-entry-number ip route [ vpn-instance vpn-instance-name ] ip-address { mask-length | mask } reachability [ delay { negative negative-time | positive positive-time } * ]

New syntax

track track-entry-number ip route [ vpn-instance vpn-instance-name ] ip-address { mask-length | mask } reachability

Change description

Before modification: This command allows you to set the notification delay when associating Track with the application module, and does not enter Track view.

After modification: Creating a track entry associated with the application module enters Track view. You can configure the notification delay in Track view.

Modified command: track lldp neighbor

Old syntax

track track-entry-number lldp neighbor interface interface-type interface-number [ delay { negative negative-time | positive positive-time } * ]

New syntax

track track-entry-number lldp neighbor interface interface-type interface-number

Change description

Before modification: This command allows you to set the notification delay when associating Track with the application module, and does not enter Track view.

After modification: Creating a track entry associated with the application module enters Track view. You can configure the notification delay in Track view.

Modified command: track nqa

Old syntax

track track-entry-number nqa entry admin-name operation-tag reaction item-number [ delay { negative negative-time | positive positive-time } * ]

New syntax

track track-entry-number nqa entry admin-name operation-tag reaction item-number

Change description

Before modification: This command allows you to set the notification delay when associating Track with the application module, and does not enter Track view.

After modification: Creating a track entry associated with the application module enters Track view. You can configure the notification delay in Track view.

Modified feature: Specifying the length of ICMP echo requests sent by an IPv4 or IPv6 ping operation

Feature change description

The maximum length of ICMP echo requests that can be sent by an IPv4 or IPv6 ping operation was changed from 8100 bytes to 9600 bytes.

Command changes

Modified command: ping

Syntax

ping [ ip ] [ -a source-ip | -c count | -f | -h ttl | -i interface-type interface-number | -m interval | -n | -p pad | -q | -r | -s packet-size | -t timeout | -tos tos | -v | -vpn-instance vpn-instance-name ] * host

Views

Any view

Change description

Before modification: The value range for the packet-size argument is 20 to 8100 bytes.

After modification: The value range for the packet-size argument is 20 to 9600 bytes.

Modified command: ping ipv6

Syntax

ping ipv6 [ -a source-ipv6 | -c count | -i interface-type interface-number | -m interval | -q | -s packet-size | -t timeout | -tc traffic-class | -v | -vpn-instance vpn-instance-name ] * host

Views

Any view

Change description

Before modification: The value range for the packet-size argument is 20 to 8100 bytes.

After modification: The value range for the packet-size argument is 20 to 9600 bytes.

Modified feature: Removing a TCP or UDP listening service for a VPN instance

Feature change description

From this release, you can remove a TCP or UDP listening service for the specified VPN instance.

Command changes

Modified command: nqa server tcp-connect

Old syntax

nqa server tcp-connect ip-address port-number [ vpn-instance vpn-instance-name ] [ tos tos ]

undo nqa server tcp-connect ip-address port-number

New syntax

nqa server tcp-connect ip-address port-number [ vpn-instance vpn-instance-name ] [ tos tos ]

undo nqa server tcp-connect ip-address port-number [ vpn-instance vpn-instance-name ]

Views

System view

Change description

The vpn-instance vpn-instance-name option was added to the undo form of the command.

vpn-instance vpn-instance-name: Specifies an MPLS L3VPN instance by its name, a case-sensitive string of 1 to 31 characters. If you do not specify this option, the NQA server listens to the TCP services on an IP address on the public network.

Modified command: nqa server udp-echo

Old syntax

nqa server udp-echo ip-address port-number [ vpn-instance vpn-instance-name ] [ tos tos ]

undo nqa server udp-echo ip-address port-number

New syntax

nqa server udp-echo ip-address port-number [ vpn-instance vpn-instance-name ] [ tos tos ]

undo nqa server udp-echo ip-address port-number [ vpn-instance vpn-instance-name ]

Views

System view

Change description

The vpn-instance vpn-instance-name option was added to the undo form of the command.

vpn-instance vpn-instance-name: Specifies an MPLS L3VPN instance by its name, a case-sensitive string of 1 to 31 characters. If you do not specify this option, the NQA server listens to UDP services on an IP address on the public network.

Modified feature: Specifying the source IP address for NTP messages

Feature change description

You can configure the source IP address for NTP messages directly or by specifying an interface. Before the modification, the source IP address can be configured only by specifying an interface.

Command changes

Modified command: ntp-service source

Old syntax

ntp-service source interface-type interface-number

New syntax

ntp-service source { interface-type interface-number | ip-address }

Views

System view

Parameter

interface-type interface-number: Specifies an interface by its type and number. The device uses the primary address of the specified source interface as the source address to send NTP messages. The destination address of the NTP response messages is the primary address of the specified source interface.

ip-address: Specifies the source IP address for NTP messages.

Change description

The ip-address argument was added to the command.

New feature: Specifying the NTP time-offset thresholds for log and trap outputs

Specifying the NTP time-offset thresholds for log and trap outputs

About the NTP time-offset thresholds for log and trap outputs

By default, the system synchronizes the NTP client's time to the server and outputs a log and a trap when the time offset exceeds 128 ms for multiple times.

After you set the NTP time-offset thresholds for log and trap outputs, the system synchronizes the client's time to the server when the time offset exceeds 128 ms for multiple times, but outputs logs and traps only when the time offset exceeds the specified thresholds, respectively.

Procedure

            - Enter system view.

system-view

            - Specify the NTP time-offset thresholds for log and trap outputs.

ntp-service time-offset-threshold { log log-threshold | trap trap-threshold } *

By default, no NTP time-offset thresholds are set for log and trap outputs.

Command reference

ntp-service time-offset-threshold

Use ntp-service time-offset-threshold to set the NTP time-offset thresholds for log and trap outputs.

Use undo ntp-service time-offset-threshold to restore the default.

Syntax

ntp-service time-offset-threshold { log log-threshold | trap trap-threshold } *

undo ntp-service time-offset-threshold

Default

No NTP time-offset thresholds are set for log and trap outputs.

Views

System view

Predefined user roles

network-admin

mdc-admin

Parameters

log log-threshold: Specifies the NTP time-offset threshold for log output. The value range for the log-threshold argument is 128 to 60000, in milliseconds.

trap trap-threshold: Specifies the NTP time-offset threshold for trap output. The value range for the trap-threshold argument is 128 to 60000, in milliseconds.

Usage guidelines

By default, the system synchronizes the NTP client's time to the server and outputs a log and a trap when the time offset exceeds 128 ms for multiple times.

After you set the thresholds, the system synchronizes the client's time to the server when the time offset exceeds 128 ms for multiple times, but outputs logs and traps only when the time offset exceeds the specified thresholds, respectively.

Examples

# Set the NTP time-offset thresholds for log and trap outputs to 500 ms and 600 ms, respectively.

<Sysname> system-view

[Sysname] ntp-service time-offset-threshold log 500 trap 600

New feature: Specifying the SNTP time-offset thresholds for log and trap outputs

Specifying the SNTP time-offset thresholds for log and trap outputs

      - About SNTP time-offset thresholds for log and trap outputs

By default, the system synchronizes the SNTP client's time to the server and outputs a log and a trap when the time offset exceeds 128 ms for multiple times.

After you set the SNTP time-offset thresholds for log and trap outputs, the system synchronizes the client's time to the server when the time offset exceeds 128 ms for multiple times, but outputs logs and traps only when the time offset exceeds the specified thresholds, respectively.

      - Procedure

            - Enter system view.

system-view

            - Specify the SNTP time-offset thresholds for log and trap outputs.

sntp time-offset-threshold { log log-threshold | trap trap-threshold } *

By default, no SNTP time-offset thresholds are set for log and trap outputs

Command reference

    - sntp time-offset-threshold

Use sntp time-offset-threshold to specify the SNTP time-offset thresholds for log and trap outputs.

Use undo sntp time-offset-threshold to restore the default.

Syntax

sntp time-offset-threshold { log log-threshold | trap trap-threshold } *

undo sntp time-offset-threshold

Default

No SNTP time-offset thresholds are set for log and trap outputs.

Views

System view

Predefined user roles

network-admin

mdc-admin

Parameters

log log-threshold: Specifies the SNTP time-offset threshold for log output. The value range for the log-threshold argument is 128 to 60000, in milliseconds.

trap trap-threshold: Specifies the SNTP time-offset threshold for trap output. The value range for the trap-threshold argument is 128 to 60000, in milliseconds.

Usage guidelines

By default, the system synchronizes the SNTP client's time to the server and outputs a log and a trap when the time offset exceeds 128 ms for multiple times.

After you set the thresholds, the system synchronizes the client's time to the server when the time offset exceeds 128 ms for multiple times, but outputs logs and traps only when the time offset exceeds the specified thresholds, respectively.

Examples

# Set the SNTP time-offset thresholds for log and trap outputs to 500 ms and 600 ms, respectively.

<Sysname> system-view

[Sysname] sntp time-offset-threshold log 500 trap 600

Modified feature: Creating a sampler

  - Feature change description

The value range for the sampling rate of a sampler was modified.

  - Command changes

    - Modified command: sampler

Syntax

sampler sampler-name mode random packet-interval n-power rate

Views

System view

Change description

Before modification: The value range for the rate argument is 1 to 13.

After modification: The value range for the rate argument is 0 to 13.

Modified feature: sFlow counter sampling

  - Feature change description

In this version and later, you can specify an sFlow instance for counter sampling.

  - Command changes

    - Modified command: sflow counter collector

Old syntax

sflow counter collector collector-id

undo sflow counter collector

New syntax

sflow counter [ instance instance-id ] collector collector-id

undo sflow counter [ instance instance-id ] collector

Views

Layer 2 Ethernet interface view

Layer 3 Ethernet interface view

Parameters

instance instance-id: Specifies an sFlow instance by its ID in the range of 1 to 4. The default ID for an sFlow instance is 1. If you do not specify an sFlow instance, this command specifies sFlow instance 1 for counter sampling.

Change description

Before modification: You can specify only an sFlow collector for counter sampling.

After modification: You can specify an sFlow instance and an sFlow collector for counter sampling.

Modified feature: sFlow flow sampling

  - Feature change description

In this version and later, you can specify an sFlow instance for flow sampling.

  - Command changes

    - Modified command: sflow counter collector

Old syntax

sflow flow collector collector-id

undo sflow flow collector

New syntax

sflow flow [ instance instance-id ] collector collector-id

undo sflow flow [ instance instance-id ] collector

Views

Layer 2 Ethernet interface view

Layer 3 Ethernet interface view

Parameters

instance instance-id: Specifies an sFlow instance by its ID in the range of 1 to 4. The default ID for an sFlow instance is 1. If you do not specify an sFlow instance, this command specifies sFlow instance 1 for flow sampling.

Change description

Before modification: You can specify only an sFlow collector for flow sampling.

After modification: You can specify an sFlow instance and an sFlow collector for flow sampling.

Modified feature: Configuring a backup PW for a cross-connect

Feature change description

The value range for the outgoing label of a backup PW was changed.

Command changes

Modified command: backup-peer

Syntax

backup-peer ip-address pw-id pw-id [ in-label label-value out-label label-value ] [ pw-class class-name | tunnel-policy tunnel-policy-name ] *

Views

Cross-connect PW view

Change description

Before modification: The value range for the outgoing label of a backup PW is 16 to 1023.

After modification: The value range for the outgoing label of a backup PW is 16 to 1048575.

Modified feature: Configuring a backup PW for a VSI

Feature change description

The value range for the outgoing label of a backup PW was modified.

Command changes

Modified command: backup-peer

Syntax

In VSI LDP PW view:

backup-peer ip-address [ pw-id pw-id ] [ pw-class class-name | tunnel-policy tunnel-policy-name ] *

In VSI static PW view:

backup-peer ip-address [ pw-id pw-id ] in-label label-value out-label label-value [ pw-class class-name | tunnel-policy tunnel-policy-name ] *

Views

VSI LDP PW view

VSI static PW view

Change description

Before modification: The value range for the outgoing label of a backup PW is 16 to 1023.

After modification: The value range for the outgoing label of a backup PW is 16 to 1048575.

Modified feature: Change of the bandwidth limit value range for VSIs

Feature change description

The value range for the VSI bandwidth limit was changed.

Command changes

Modified command: bandwidth

Syntax

bandwidth bandwidth

undo bandwidth

Views

VSI view

Change description

Before modification: The value range for the bandwidth argument is 64 to 4194303 kbps.

After modification: The value range for the bandwidth argument is 64 to 167772159 kbps.

Modified feature: Value range change for the broadcast, multicast, or unknown unicast restraint bandwidth of VSIs

Feature change description

The value range for the broadcast, multicast, or unknown unicast restraint bandwidth of VSIs was changed.

Command changes

Modified command: restrain

Syntax

restrain { broadcast | multicast | unknown-unicast } bandwidth

undo restrain { broadcast | multicast | unknown-unicast }

Views

VSI view

Change description

Before modification: The value for the bandwidth argument can be 0 or in the range of 64 to 4194303 kbps.

After modification: The value for the bandwidth argument can be 0 or in the range of 64 to 167772159 kbps.

Modified feature: Frame match criteria of VXLAN Ethernet service instances

Feature change description

The frame match criteria were changed for Ethernet service instances in VLAN access mode.

Command changes

Modified command: encapsulation

Syntax

encapsulation s-vid vlan-id [ c-vid { vlan-id-list | all } | only-tagged ]

encapsulation s-vid vlan-id-list [ c-vid vlan-id-list ]

encapsulation { default | tagged | untagged }

undo encapsulation

Views

Ethernet service instance view

Change description

Before modification: On a Layer 2 Ethernet or aggregate interface, if you configure the encapsulation default criterion for an Ethernet service instance in VLAN access mode, make sure no other Ethernet service instances exist on the interface.

After modification: On a Layer 2 Ethernet or aggregate interface, if an Ethernet service instance in VLAN access mode uses the encapsulation default criterion, you can create other Ethernet service instances on the interface. The interface matches traffic to the Ethernet service instances in the following criterion order:

- encapsulation { tagged | untagged }

- encapsulation default

- encapsulation s-vid vlan-id [ c-vid { vlan-id | all } | only-tagged ] and encapsulation s-vid vlan-id-list [ c-vid vlan-id-list ]

Modified feature: Displaying EVPN routing table information

Feature change description

ECMP route flags were added to the output from the display evpn routing-table command.

Command changes

Modified command: display evpn routing-table

Syntax

display evpn routing-table [ ipv6 ] { public-instance | vpn-instance vpn-instance-name } [ count ]

Views

Any view

Change description

The Flags field was added to the command output to display ECMP route flags.

E—The route carries a valid ESI.

A—All Ethernet auto-discovery routes are received. The ECMP routes for the next hop can be issued.

L—An active local ESI exists. Remote routes are not issued.

-—The MAC/IP advertisement route does not have a valid ESI. ECMP routes are not supported.

- Modified feature: NETCONF logging

  - Feature change description

The default setting for NETCONF logging was changed.

Before modification: NETCONF logging is disabled.

After modification: NETCONF logging is enabled only for row operations for <action> and <edit-config> operations.

  - Command changes

    - Modified command: netconf log

Old syntax

netconf log source { all | { agent | soap } * } { protocol-operation { all | { action | config | get | set | session | syntax | others } * } | row-operation | verbose }

undo netconf log source { all | { agent | soap } * } { protocol-operation { all | { action | config | get | session | set | syntax | others } * } | row-operation | verbose }

New syntax

netconf log source { all | { agent | soap } * } { protocol-operation { all | { action | config | get | set | session | syntax | others } * } | verbose }

undo netconf log source { all | { agent | soap } * } { protocol-operation { all | { action | config | get | session | set | syntax | others } * } | verbose }

Views

System view

Change description

Before modification: NETCONF logging is disabled by default. To enable the device to log row operations for <action> and <edit-config> operations, you must execute the netconf log source { all | { agent | soap } * } protocol-operation command.

After modification: NETCONF logging is enabled only for row operations of <action> and <edit-config> operations by default. You do not need to execute the netconf log source { all | { agent | soap } * } protocol-operation command.

Modified feature: Specifying the role of the device in the VCF fabric

Feature change description

In this version and later, a data center VCF fabric supports the spine-aggregate-leaf topology. Residing on the distribution layer, an aggregate node is between leaf nodes and spine nodes. In such a VCF fabric, OSPF runs on the Layer 3 networks between the spine and aggregate nodes and between the aggregate and leaf nodes. The default role of the device in the VCF fabric is aggregate.

Automated overlay network deployment is not supported on aggregate nodes.

Command reference

Modified command: vcf-fabric role

Old syntax

vcf-fabric role { access | leaf | spine }

undo vcf-fabric role

New syntax

vcf-fabric role { access | aggr | leaf | spine }

undo vcf-fabric role

Views

System view

Change description

The aggr keyword was added. The default role of the device in the VCF fabric was changed to aggregate.

aggr: Specifies the aggregate node.

Modified command: display vcf-fabric role

Syntax

display vcf-fabric role

Views

Any view

Change description

The aggr value was added to the Default role and Current role fields.

Before modification:

# Display the role of the device in the VCF fabric.

<System> display vcf-fabric role

Default role: leaf

Current role: leaf

After modification:

# Display the role of the device in the VCF fabric.

<System> display vcf-fabric role

Default role: aggr

Current role: aggr

Command output

| Field | description |
| --- | --- |
| Default role | Default role of the device in the VCF fabric: - spine—The device is a spine node. - aggr—The device is an aggregate node. - leaf—The device is a leaf node. - access—The device is an access node. NOTE: The aggr value was added in this version. |
| Current role | Current role of the device in the VCF fabric: - spine—The device is a spine node. - aggr—The device is an aggregate node. - leaf—The device is a leaf node. - access—The device is an access node. NOTE: The aggr value was added in this version. |

Related commands

vcf-fabric role

Modified command: display vcf-fabric underlay autoconfigure

Syntax

display vcf-fabric underlay autoconfigure

Views

Any view

Change description

Before modification: VCF fabric does not support the spine-aggregate-leaf topology.

After modification: VCF fabric supports the spine-aggregate-leaf topology. If you use this command on a master spine node, BGP peer information on all spine and leaf nodes will be displayed.

# Display information about automated underlay network deployment.

<Sysname> display vcf-fabric underlay autoconfigure

success command:

#

system

clock timezone beijing add 08:00:00

#

system

lldp global enable

lldp compliance cdp

#

system

ospf 1

graceful-restart ietf

area 0.0.0.0

#

system

interface LoopBack0

#

system

l2vpn enable

#

system

vxlan tunnel mac-learning disable

vxlan tunnel arp-learning disable

#

system

stp global enable

#

system

ntp-service enable

ntp-service unicast-peer 172.16.1.136

#

system

netconf soap https enable

restful https enable

#

system

info-center loghost 172.16.1.136

#

system

local-user aaa

password ******

service-type https

authorization-attribute user-role network-admin

#

system

line vty 0 63

authentication-mode scheme

user-role network-admin

#

system

bgp 100

graceful-restart

address-family l2vpn evpn

#

system

vcf-fabric topology enable

#

system

neutron

rabbit user openstack

rabbit password ******

rabbit host ip 172.16.1.136

restful user aaa password ******

vpn-target 1:1 export-extcommunity

vsi-mac 789c-2f5f-0200

network-type distributed-vxlan

proxy-arp enable

l2agent enable

l3agent enable

#

system

snmp-agent

snmp-agent packet max-size 4096

snmp-agent community read public

snmp-agent community write private

snmp-agent sys-info version all

#

Uplink interface:

Ten-GigabitEthernet1/0/1

Ten-GigabitEthernet1/0/2

Downlink interface:

Ten-GigabitEthernet1/0/3

Ten-GigabitEthernet1/0/4

Loopback0 IP allocation:

Device_MAC         Loopback_IP       Management_IP     State

a43c-adae-0400     200.1.1.6         172.16.1.193      Up

a43c-9aa7-0100     200.1.1.4         172.16.1.152      Up

Loopback1 IP allocation:

Device_MAC         Loopback_IP       Management_IP     State

a43c-adae-0400     100.1.1.6         172.16.1.193      Up

a43c-9aa7-0100     100.1.1.4         172.16.1.152      Up

Aggregation configuration:

AggrID: 7

interface: Ten-GigabitEthernet1/0/5

interface: Ten-GigabitEthernet1/0/6

IRF allocation:

Self Bridge Mac: a43c-adae-0400

IRF Status: Yes

Member List: [1, 2]

BGP peer configuration:

Device_MAC: 265b-208d-0300

Device_role:leaf

BGP peer list:

200.1.4.5

Device_MAC: 1ea6-06a4-0500

Device_role:spine

BGP peer list:

200.1.4.4

200.1.4.7

Device_MAC: 1ead-aed1-0600

Device_role:leaf

BGP peer list:

200.1.4.5

Command output

| Field | Description |
| --- | --- |
| success command | Commands that have been successfully executed during automated underlay network deployment. |
| Uplink interface | Uplink interfaces of the device. |
| Downlink interface | Downlink interfaces of the device. |
| Loopback0 IP allocation | IP addresses assigned to Loopback 0. |
| Loopback1 IP allocation | IP addresses assigned to Loopback 1. |
| Aggregation configurations | Information about Layer 2 aggregation groups. |
| IRF allocation | IRF configurations, including IRF bridge MAC address of the IRF fabric, IRF status, and the IRF member ID of the device. |
| BGP peer configuration | Information about BGP peers of the device. NOTE: In this version and later, BGP peer information on all spine and leaf nodes is displayed only when this command is executed on the master spine node in a spine-aggregate-leaf VCF fabric. |