HPE 5950-CMW710-R6301P02 Release Notes

The information in this document is subject to change without notice. © Copyright 2023 Hewlett Packard Enterprise Development LP

Contents

Introduction	1

Version information	1

Version number	1

Version history	1

Hardware and software compatibility matrix	8

ISSU compatibility list	10

Upgrade restrictions and guidelines	10

Hardware feature updates	11

R6301P02	11

R6301P01	11

R6301	11

F6207	11

F6206	11

R6205P03	11

F6205P02	11

F6205	11

F6203	11

F6202	12

R6125	12

R6123	12

R6106P01	13

R6106	13

R6105	13

Software feature and command updates	13

MIB updates	13

Operation changes	15

Operation changes in F6301P02	15

Operation changes in F6301P01	15

Operation changes in R6301	15

Operation changes in F6207	16

Operation changes in F6206	16

Operation changes in R6205P03	17

Operation changes in F6205P02	17

Operation changes in F6205	18

Operation changes in F6203	18

Operation changes in F6202	18

Operation changes in R6125	19

Operation changes in R6123	19

Operation changes in R6106P01	19

Operation changes in R6106	19

Operation changes in R6105	19

Restrictions and cautions	20

Open problems and workarounds	20

List of resolved problems	20

Resolved problems in R6301P02	20

Resolved problems in R6301P01	21

Resolved problems in R6301	21

Resolved problems in F6207	28

Resolved problems in F6206	30

Resolved problems in R6205P03	32

Resolved problems in F6205P02	33

Resolved problems in F6205	34

Resolved problems in F6203	35

Resolved problems in F6202	36

Resolved problems in R6125	37

Resolved problems in R6123	38

Resolved problems in R6106P01	39

Resolved problems in R6106	40

Resolved problems in R6105	40

Support and other resources	40

Accessing Hewlett Packard Enterprise Support	40

Documents	41

Related documents	41

Documentation feedback	41

Appendix A Feature list	42

Hardware features	42

Software features	44

Appendix B Upgrading software	48

System software file types	48

System startup process	48

Upgrade methods	49

Upgrading from the CLI	50

Preparing for the upgrade	50

Downloading software to the master switch	51

Upgrading the software images	53

Installing a patch package	55

Upgrading from the Boot menu	56

Prerequisites	56

Accessing the Boot menu	57

Accessing the basic Boot menu	58

Accessing the extended Boot menu	59

Using TFTP to upgrade software images through the management Ethernet port	60

Using FTP to upgrade software through the management Ethernet port	62

Using XMODEM to upgrade software through the console port	64

Using TFTP to upgrade Boot ROM through the management Ethernet port	68

Using FTP to upgrade Boot ROM through the management Ethernet port	69

Using XMODEM to upgrade Boot ROM through the console port	71

Managing files from the Boot menu	75

Displaying all files	75

Deleting files	76

Changing the attribute of software images	76

Handling software upgrade failures	78

List of tables

Table 1 Version history	1

Table 2 Hardware and software compatibility matrix	8

Table 3 ISSU compatibility list	10

Table 4 MIB updates	13

Table 5 5950 series hardware features	42

Table 6 Software features of the MSR 20/MSR 30/MSR 50 series	44

Table 7 Shortcut keys	57

Table 8 Basic Boot ROM menu options	58

Table 9 BASIC ASSISTANT menu options	59

Table 10 Extended Boot ROM menu options	60

Table 11 EXTENDED ASSISTANT menu options	60

Table 12 TFTP parameter description	61

Table 13 FTP parameter description	63

Table 14 TFTP parameter description	69

Table 15 FTP parameter description	70

Introduction

This document describes the features, restrictions and guidelines, open problems, and workarounds for version R6301P02. Before you use this version on a live network, back up the configuration and test the version to avoid software upgrade affecting your live network.

Use this document in conjunction with HPE 5950-CMW710-R6301P02 Release Notes (Software Feature Changes) and the documents listed in "Related documents."

Version information

Version number

HPE Comware Software, Version 7.1.070, Release 6301P02

Note: You can see the version number with the display version command in any view. Please see Note ①.

Version history

|  | IMPORTANT: The software feature changes listed in the version history table for each version are not complete. To obtain complete information about all software feature changes in each version, see the Software Feature Changes document for this release notes. |
| --- | --- |

Version history

| Version number | Last version | Release date | Release type | Remarks |
| --- | --- | --- | --- | --- |
| 5950-CMW710-R6301P02 | 5950-CMW710-R6301P01 | 2023-04-28 | Release version | - MIB updates. |
| 5950-CMW710-R6301P01 | 5950-CMW710-R6301 | 2022-12-31 | Release version | - Fixed bugs. |
| 5950-CMW710-R6301 | 5950-CMW710-F6207 | 2019-06-12 | Release version | New feature includes: - New features: Fundamentals features - New features: Virtual technologies features - New features: Layer 2-LAN switching features - New features: Layer 3-IP services features - New features: Layer 3-IP routing features - New features: IP multicast features - New features: MPLS features - New features: ACL and QoS features - New features: Security features - New features: High availability features - New features: Network management and monitoring features - New features: FC and FCoE features - New features: OpenFlow features - New features: VXLAN features - New features: EVPN features - New feature: Specifying the NTP time-offset thresholds for log and trap outputs - New feature: Specifying the SNTP time-offset thresholds for log and trap outputs Modified features includes: - Modified feature: ISSU by using issu commands - Modified feature: Automatic configuration - Modified feature: Setting the timestamp format for logs sent to log hosts - Modified feature: Configuring an interface event for a CLI-defined EAA monitor policy - Modified feature: Configuring an EAA monitor policy by using Tcl - Modified feature: Displaying the operating status and information of an interface - Modified feature: Displaying PFC information of all interfaces - Modified feature: Link state change suppression on an interface - Modified feature: MAC-to-VLAN entries - Modified feature: Collision handling process of LACP MAD and BFD MAD - Modified feature: Collision handling process of ND MAD - Modified feature: Displaying global link aggregation load sharing modes - Modified feature: Configuring a link aggregation load sharing hash seed - Modified feature: Displaying detailed information about the IPP and DR interfaces of DRNI - Modified feature: Configuring the advertisable TLVs for LLDP - Modified feature: Setting the aging timer for dynamic ARP entries - Modified feature: ARP snooping - Modified feature: Specifying DHCP servers for a DHCP relay address pool - Modified feature: Displaying DHCP snooping and DHCPv6 snooping trusted ports - Modified feature: Displaying DHCP address pool information - Modified feature: Displaying DHCPv6 address pool information - Modified feature: Enabling recording DHCPv6 snooping address entries in VLAN view - Modified feature: Setting the aging timer for ND entries in stale state - Modified feature: Displaying member interfaces shut down by Monitor Link - Modified feature: Displaying DLDP configuration - Modified feature: Configuring routing policy-based recursive lookup - Modified feature: Associating Track with the output interface for a static route - Modified feature: Configuring OSPF area authentication - Modified feature: Configuring OSPF interface authentication - Modified feature: Configuring a virtual link - Modified feature: Setting the number of OSPF logs - Modified feature: Displaying IS-IS LSP log information - Modified feature: Clearing IS-IS LSP log information - Modified feature: Specifying a label allocation mode - Modified feature: Displaying BGP peer or peer group information - Modified feature: Displaying AS path information for BGP routes - Modified feature: Displaying RPF information for an IPv6 multicast source - Modified feature: Displaying statistics for the IGMP messages and PIMv2 hello messages learned through IGMP snooping - Modified feature: Displaying statistics for the MLD messages and IPv6 PIM hello messages learned through MLD snooping - Modified feature: Enabling a VPN instance to replicate routes from the public network or other VPN instances - Modified feature: Specifying outgoing labels for a static SRLSP - Modified feature: Creating an ACL - Modified feature: Referencing an ACL in QoS or packet filtering - Modified feature: Enabling hardware-count for the packet filtering default action - Modified feature: Configuring drop-level-based parameters for a queue in a WRED table - Modified feature: Configuring binding attributes for a local user - Modified feature: Password handling when global password control is enabled - Modified feature: Setting the quiet timer for RADIUS servers - Modified feature: Configuring MAC-based MAC authentication user accounts - Modified feature: MAC authentication VLAN mode - Modified feature: Port security MAC move - Modified feature: Web authentication support for HTTPS redirection - Modified feature: RSA key modulus length - Modified feature: RSA key modulus length used for certification request in a PKI domain - Modified feature: Displaying IPv4SG bindings - Modified feature: Displaying IPv6SG bindings - Modified feature: Displaying the MFF configuration for a VLAN - Modified feature: Associating Track with application modules - Modified feature: Specifying the length of ICMP echo requests sent by an IPv4 or IPv6 ping operation - Modified feature: Removing a TCP or UDP listening service for a VPN instance - Modified feature: Specifying the source IP address for NTP messages - Modified feature: Creating a sampler - Modified feature: sFlow counter sampling - Modified feature: sFlow flow sampling - Modified feature: Configuring a backup PW for a cross-connect - Modified feature: Configuring a backup PW for a VSI - Modified feature: Change of the bandwidth limit value range for VSIs - Modified feature: Value range change for the broadcast, multicast, or unknown unicast restraint bandwidth of VSIs - Modified feature: Frame match criteria of VXLAN Ethernet service instances - Modified feature: Displaying EVPN routing table information - Modified feature: NETCONF logging - Modified feature: Specifying the role of the device in the VCF fabric - Fixed bugs. |
| 5950-CMW710-F6207 | 5950-CMW710-F6206 | 2019-01-28 | Feature version | This version fixed bugs and introduced feature changes. New features include: - Changing the next hop address of VPNv4 routes to a VPN address - There are also modified features. |
| 5950-CMW710-F6206 | 5950-CMW710-R6205P03 | 2018-08-15 | Feature version | This version fixed bugs and introduced feature changes. New features include: - Setting fan tray serial numbers - Simple multichassis link aggregation (S-MLAG) - Configuring PFC deadlock detection There are also modified features. |
| 5950-CMW710-R6205P03 | 5950-CMW710-F6205P02 | 2017-11-13 | Feature version | This version fixed bugs and introduced feature changes. New features include: - Enabling STP dispute guard - Enabling DNS spoofing - Enabling symmetric load sharing There are also modified features. |
| 5950-CMW710-F6205P02 | 5950-CMW710-F6205 | 2017-10-30 | Feature version | This version fixed bugs and introduced feature changes. New features include: Setting PFC thresholds Local-MAC address learning Conversational learning for remote MAC address entries Removed features include: - Ignoring port speed in setting the aggregation states of member ports - PBB - SPBM There are also modified features. |
| 5950-CMW710-F6205 | 5950-CMW710-F6203 | 2017-6-28 | Feature version | This version fixed bugs and introduced feature changes. New features include: One-step ISSU Specifying ignored packet fields for the default link-aggregation load sharing Static SR over MPLS There are also modified features. |
| 5950-CMW710-F6203 | 5950-CMW710-F6202 | 2017-4-27 | Feature version | This version fixed bugs and introduced feature changes. New features include: DRNI |
| 5950-CMW710-F6202 | 5950-CMW710-R6125 | 2017-3-28 | Feature version | This version fixed bugs and introduced feature changes. New features include: FC and FCoE MACsec EEE PBB MPLS L2VPN VPLS SPBM ERPS FNA(HPE FlexFabric Network Analytics) DCBX There are also modified features. |
| 5950-CMW710-R6125 | 5950-CMW710-R6123 | 2017-1-13 | Release version | - Fixed bugs. - Modified features. |
| 5950-CMW710-R6123 | 5950-CMW710-R6106P01 | 2016-9-30 | Release version | Added feature and new devices. |
| 5950-CMW710-R6106P01 | 5950-CMW710-R6106 | 2016-9-11 | Release version | Added feature: New feature: Load balancing among BGP routes with different AS_PATH attributes of the same length |
| 5950-CMW710-R6106 | 5950-CMW710-R6105 | 2016-5-31 | Release version | Fixed bugs. |
| 5950-CMW710-R6105 | First release | 2016-4-11 | Release version | None |

Hardware and software compatibility matrix

|  | CAUTION: To avoid an upgrade failure, use Table 2 to verify the hardware and software compatibility before performing an upgrade. |
| --- | --- |

Hardware and software compatibility matrix

| Item | Specifications |
| --- | --- |
| Product family | HPE 5950 Series |
| Hardware platform | HPE FlexFabric 5950 32QSFP28 Switch JH321A HPE FlexFabric 5950 32QSFP28 TAA-compliant Switch JH322A HPE FlexFabric 5950 48SFP28 8QSFP28 Switch JH402A HPE FlexFabric 5950 4-slot Switch JH404A |
| Memory | 4GB |
| Flash | 1GB |
| Boot ROM version | Version 106 or higher (Note: Perform the command display version command in any view to view the version information. Please see Note②) |
| Host software | 5950-CMW710-R6301P02.ipe |
| iMC version | iMC BIMS 7.3(E0501) iMC EAD 7.3 (E0502) iMC EIA 7.3(E0503) iMC TAM 7.3 (E0503) iMC UAM 7.3 (E0503) iMC MVM 7.3 (E0501) iMC NTA 7.3 (E0502) iMC PLAT 7.3 (E0605) iMC QoSM 7.3 (E0502) iMC-RAM 7.3(E0501) iMC UBA 7.3 (E0502) iMC VFM 7.3 (E0502) |
| iMC version for FNA | iNode PC 7.3(E0504) |
| iNode version | None |
| Web version | None |
| OAA version | None |

To display version information for the system software and Boot ROM of 5950:

<HPE> display version

HPE Comware Software, Version 7.1.070, Release 6301P02           ------- Note①

Copyright (c) 2010-2023 Hewlett Packard Enterprise Development LP

HPE FF 5950 32Q28 Switch uptime is 0 weeks, 0 days, 1 hour, 16 minutes

Last reboot reason : Auto Update reboot

Boot image: flash:/s6820-cmw710-boot-r6301p02.bin

Boot image version: 7.1.070, Release 6301P02

Compiled Mar 22 2023 11:00:00

System image: flash:/s6820-cmw710-system-r6301p02.bin

System image version: 7.1.070, Release 6301P02

Compiled Mar 22 2023 11:00:00

Slot 1:

Uptime is 0 weeks,2 days,3 hours,5 minutes

FF 5950 32Q28 Switch with 2 Processor

BOARD TYPE:         FF 5950 32Q28 Switch

DRAM:               3584M bytes

FLASH:              1024M bytes

PCB 1 Version:      VER.B

PCB 2 Version:      VER.B

FPGA Version:       NONE

Bootrom Version:    106                                      ------ Note②

CPLD 1 Version:     001

CPLD 2 Version:     001

Release Version:    HPE FF 5950 32Q28 Switch-6301P02

Patch Version:      None

Reboot Cause:       UserReboot

[SubSlot 0] 32QSFP28 + 2SFP PLUS

ISSU compatibility list

ISSU compatibility list

| Current version | Earlier version | ISSU compatibility |
| --- | --- | --- |
| 5950-CMW710-R6301P02 | 5950-CMW710-R6301P01 | No |
|  | 5950-CMW710-R6301 | No |
|  | 5950-CMW710-F6207 | Yes |
|  | 5950-CMW710-F6206 | Yes |
|  | 5950-CMW710-R6205P03 | Yes |
|  | 5950-CMW710-F6205P02 | Yes |
|  | 5950-CMW710-F6205 | Yes |
|  | 5950-CMW710-F6203 | Yes |
|  | 5950-CMW710-F6202 | Yes |
|  | 5950-CMW710-R6125 | Yes |
|  | 5950-CMW710-R6123 | No |
|  | 5950-CMW710-R6106P01 | No |
|  | 5950-CMW710-R6106 | No |
|  | 5950-CMW710-R6105 | No |

Upgrade restrictions and guidelines

Before performing a software upgrade, it is important to refer to the Software Feature Changes document for any feature changes in the new version. Also check the most recent version of the related documents (see "Related documents") available on the HPE website for more information about feature configuration and commands.

Because Release 6106 and later versions support new signatures, the software cannot be downgraded to Release 6105 after it is upgraded to Release 6106 or later.

When the software is upgraded to or from version F6202/F6203, the port media-type configuration might change. If the port media type configuration changes, you must configure the port media type again as needed. This command is available only on the lowest-numbered eight 25-GE interfaces and highest-numbered eight 25-GE interfaces on the HPE FlexFabric 5950 48SFP28 8QSFP28 Switch (JH402A) and the 25-GE interfaces on the LSWM124TG2H interface module of the HPE FlexFabric 5950 4-slot Switch (JH404A).

If the device is configured with FlexFabric Network Analytics (FNA), do not use ISSU to upgrade the software to version F6205.

When the software is upgraded to or from version F6205 by using ISSU, IRF split might occur to cause software upgrade failure. As a best practice, do not use ISSU to upgrade the software to version F6205 or upgrade the software from version F6205 to another version.

Hardware feature updates

R6301P02

- None.

R6301P01

- None.

R6301

- None.

F6207

The following transceiver modules are supported:

QSFP-100G-ER4L-WDM1300

F6206

The following transceiver modules are supported:

HPE X150 100G QSFP28 LC SWDM4 100m MM Transceiver(JH419A)

R6205P03

None.

F6205P02

The HPE FlexFabric 5950 4-slot Switch supports the new subcard LSWM18CQMSEC (JH957A).

F6205

None.

F6203

None.

F6202

The following transceiver modules are supported:

HPE X190 8G/4G/2G SFP+ LC Short Wave Transceiver JG879A

HPE X190 8G/4G/2G SFP+ LC Long Wave Transceiver JG880A

The Mellanox 655874-B21 QSFP+ to SFP+ adapter is supported. When a 100G QSFP28 interface or 40G QSFP+ interface has this adapter installed, the interface is compatible with a 10G SFP+ transceiver module. LSWM116Q Interface Card (JH405A) does not support this adapter.

R6125

None.

R6123

The following devices are supported:

HPE FlexFabric 5950 48SFP28 8QSFP28 Switch JH402A

HPE FlexFabric 5950 4-slot Switch JH404A

The following expansion cards are supported:

LSWM18CQ

LSWM116Q

LSWM124TG2H

LSWM124XGT2Q

LSWM124XG2QFC

LSWM18QC

LSWM124XG2Q

LSWM124XG2QL

The following transceiver modules are supported:

HPE X150 100G QSFP28 LC LR4 10km SM Transceiver(JL275A)

HPE X240 100G QSFP28 to QSFP28 5m Direct Attach Copper Cable(JL273A)

HPE X2A0 100G QSFP28 to QSFP28 7m Active Optical Cable(JL276A)

HPE X2A0 100G QSFP28 to QSFP28 10m Active Optical Cable(JL277A)

HPE X2A0 100G QSFP28 to QSFP28 20m Active Optical Cable(JL278A)

HPE X240 QSFP28 4xSFP28 1m DAC Cable(JL282A)

HPE X240 QSFP28 4xSFP28 3m DAC Cable(JL283A)

HPE X2A0 40G QSFP+ to QSFP+ 7m Active Optical Cable(JL287A)

HPE X2A0 40G QSFP+ to QSFP+ 10m Active Optical Cable(JL288A)

HPE X2A0 40G QSFP+ to QSFP+ 20m Active Optical Cable(JL289A)

HPE X190 25G SFP28 LC SR 100m MM XCVR(JL293A)

HPE X240 25G SFP28 to SFP28 1m DAC(JL294A)

HPE X240 25G SFP28 to SFP28 3m DAC(JL295A)

R6106P01

None.

R6106

The following transceiver modules are supported:

HPE X2A0 10G SFP+ to SFP+ 7m Active Optical Cable (JL290A)

HPE X2A0 10G SFP+ to SFP+ 10m Active Optical Cable(JL291A)

HPE X2A0 10G SFP+ to SFP+ 20m Active Optical Cable(JL292A)

R6105

First release.

Software feature and command updates

For more information about the software feature and command update history, see HPE 5950-CMW710-R6301P02 Release Notes (Software Feature Changes).

MIB updates

MIB updates

| Item | MIB file | Module | Description |
| --- | --- | --- | --- |
| 5950-CMW710-R6301P02 |  |  |  |
| New | hh3c-arp-suppression.mib | HH3C-ARP-SUPPRESSION-MIB | Added hh3cARPSuppressionChassis hh3cARPSuppressionSlot hh3cARPSuppressionVsiName hh3cARPSuppressionIpAddr hh3cARPSuppressionMacAddr hh3cARPSuppressionLinkID hh3cARPSuppressionAging |
| Modified | None | None | None |
| 5950-CMW710-R6301P01 |  |  |  |
| New | None | None | None |
| Modified | None | None | None |
| 5950-CMW710-R6301 |  |  |  |
| New | hh3c-stack.mib | HH3C-STACK-MIB | Added hh3cStackTrapOjbects |
| Modified | sflow.mib hh3c-bgp-evpn-mib.mib | SFLOW-MIB HH3C-BGP-EVPN-MIB | Modified sFlowCpInstance sFlowFsInstance hh3cBgpEvpnPAtrAddrPrefix hh3cBgpEvpnPAtrPeer |
| 5950-CMW710-F6207 |  |  |  |
| New | HH3C-BGP-EVPN-MIB.docx | None | None |
| Modified | HH3C-TRANSCEIVER-INFO-MIB.docx | hh3cTransceiverInfoTable | Added: hh3cTransceiverPartNumber hh3cTransceiverProductCode |
| 5950-CMW710-F6206 |  |  |  |
| New | None | None | None |
| Modified | HH3C-IF-EXT-MIB.docx | hh3cIfTable | Added: hh3cIfFwdErrDiscards |
| 5950-CMW710-R6205P03 |  |  |  |
| New | None | None | None |
| Modified | None | None | None |
| 5950-CMW710-F6205P02 |  |  |  |
| New | None | None | None |
| Modified | None | None | None |
| 5950-CMW710-F6205 |  |  |  |
| New | None | None | None |
| Modified | None | None | None |
| 5950-CMW710-F6203 |  |  |  |
| New | None | None | None |
| Modified | None | None | None |
| 5950-CMW710-F6202 |  |  |  |
| New | None | None | None |
| Modified | None | None | None |
| 5950-CMW710-R6125 |  |  |  |
| New | None | None | None |
| Modified | None | None | None |
| 5950-CMW710-R6123 |  |  |  |
| New | None | None | None |
| Modified | None | None | None |
| 5950-CMW710-R6106P01 |  |  |  |
| New | None | None | None |
| Modified | None | None | None |
| 5950-CMW710-R6106 |  |  |  |
| New | None | None | None |
| Modified | None | None | None |
| 5950-CMW710-R6105 |  |  |  |
| New | First release | First release | First release |
| Modified | First release | First release | First release |

Operation changes

Operation changes in F6301P02

None.

Operation changes in F6301P01

None.

Operation changes in R6301

- [201810100437]Added support for using existent and nonexistent policies to filter the BGP routes to be advertised to a peer or peer group.

- [201710190032]Added support for MACsec 256-bit encryption to the LSWM18CQMSEC(JH957A) interface module.

Before modification: The LSWM18CQMSEC(JH957A) interface module only supports the default MACsec 128-bit encryption.

After modification: The macsec cipher-suite gcm-aes-256 and macsec cipher-suite gcm-aes-128 commands were added. You can use these commands to enable 256-bit encryption and 128-bit encryption on the LSWM18CQMSEC(JH957A) interface module.

- [201811260646]Removed the I - Invalid value from the Flags field in the output from the display evpn route nd command.

- [201708150654]Added DHCP support for OVSDB.

- [201809260394]Added the DRNI MIB.

- [201711140340]Added the EVPN MIB.

- [201811090674]Added the watchdog timer reset time to the last system reboot information.

- [201901090330]Added CPU model information to the output from the display version command.

- [201805220468]Added support for configuring OSPF commands in VSI interface view and disabling a VSI interface from receiving and sending OSPF packets.

- [201904130555]Added SNMP and CLI support for reading the current, voltage, fan direction, and power of a 650 W power supply.

- [201804030211]Added 802.1X MAC address information to the output from the display mac-address interface command.

- [201804270716]Added NETCONF support for reading packet statistics about VSI interfaces used for EVPN or VXLAN Layer 3 forwarding.

- [201904090672]Added support for inbound and outbound rate limiting for Layer 3 subinterfaces.

Operation changes in F6207

None.

Operation changes in F6206

- The default maximum size of a PIM and IPv6 PIM join or prune message

The default maximum size of a PIM and IPv6 PIM join or prune message was changed from 8100 bytes to 1200 bytes. For more information, see HPE 5950-CMW710-F6206 Release Notes (Software Feature Changes).

- VLAN-based VXLAN assignment

Before modification: If you enable VLAN-based VXLAN assignment and map a VLAN to a VXLAN, the device automatically performs the following operations:

    - Creates an Ethernet service instance that uses the VLAN ID as its instance ID on each interface in the VLAN. The matching outer VLAN ID of the Ethernet service instances is the VLAN ID.

    - Maps the Ethernet service instances to the VSI of the VXLAN.

After modification:

  - If you enable VLAN-based VXLAN assignment and map a VLAN to a VXLAN, the device automatically performs the following operations:

    - Creates an Ethernet service instance that uses the VLAN ID as its instance ID on each interface in the VLAN.

    - Maps the Ethernet service instances to the VSI of the VXLAN.

  - On an interface, the frame match criterion of the Ethernet service instance is set as follows:

    - If the VLAN ID is the PVID of the interface, the Ethernet service instance matches untagged frames.

    - If the VLAN ID is not the PVID of the interface, the Ethernet service instance matches frames tagged with an outer VLAN ID that is same as that VLAN ID.

- Modify the number of VRRP groups that a device can support to 1024.

Before modification: A device supports a maximum of 256 VRRP groups.

After modification: A device supports a maximum of 1024 VRRP groups.

- Creating an ACL

This software version allows you to specify both a name and a number when creating an ACL. For more information, see HPE 5950-CMW710-F6206 Release Notes (Software Feature Changes)

- DRNI configuration consistency check

VLAN information was added to DRNI configuration consistency check. For more information, see HPE 5950-CMW710-F6206 Release Notes (Software Feature Changes).

- Assigning IP addresses to management Ethernet interfaces

Before modification: The IP addresses assigned to the management Ethernet interfaces of all IRF member devices must be in the same subnet.

After modification: The IP addresses must be in the same subnet if you assign them through the same management Ethernet interface of the master device. The IP addresses must be in different subnets if you assign them through different management Ethernet interfaces of the master device.

- Displaying recommended ISSU methods

This release deleted support for displaying version compatibility information. For more information, see HPE 5950-CMW710-F6206 Release Notes (Software Feature Changes). To identify whether the software versions support ISSU compatible upgrade, see ISSU compatibility list in the release notes.

Operation changes in R6205P03

None.

Operation changes in F6205P02

- Modified interface QoS policy application settings

Before modification: You can apply only one QoS policy to each direction of an interface, and only one traffic behavior action can be performed on traffic matching a traffic class.

After modification: You can apply a maximum of three QoS policies of different types to the same direction of an interface: one generic, one accounting-type, and one marking-type. For more information, see HPE 5950-CMW710-F6205P02 Release Notes (Software Feature Changes).

- ARP MAD collision handling process

Before modification: ARP MAD uses the following process to handle a multi-active collision:

Compares the member IDs of the masters in the split IRF fabrics.

Sets all fabrics to the Recovery state except the one that has the lowest numbered master.

After modification: ARP MAD uses the following process to handle a multi-active collision:

Compares the forwarding performance of the chips in each split fabric.

Sets all fabrics to the Recovery state except the one that has the best chip forwarding performance.

Compares the member IDs of the masters if all IRF fabrics have the same chip forwarding performance.

Sets all fabrics to the Recovery state except the one that has the lowest numbered master.

- DRNI and EVPN distributed relay

The distributed-relay keyword in DRNI commands and EVPN distributed relay commands were changed to drni. For more information, see HPE 5950-CMW710-F6205P02 Release Notes (Software Feature Changes).

- Multicast VPN

MD view was changed to MVPN view and the multicast-domain keyword in commands was changed to multicast-vpn. For more information, see HPE 5950-CMW710-F6205P02 Release Notes (Software Feature Changes).

- Configuration rollback by using NETCONF

Suppression for the <edit-config> operation during a configuration rollback was added. To allow the <edit-config> operation to be performed during a configuration rollback, you must perform an <action> operation to change the value of the DisableEditConfigWhenRollback attribute to false. For more information, see HPE 5950-CMW710-F6205P02 Release Notes (Software Feature Changes).

Operation changes in F6205

- Configuring the FlexFabric Network Analytics (FNA) feature

This release added support for configuring the FNA feature on an IRF fabric.

- Monitoring CPU usage

The minor alarm threshold, severe alarm threshold, and recovery threshold were added to monitor CPU usage in finer granularity. For more information, see HPE 5950-CMW710-F6205 Release Notes (Software Feature Changes).

- Configuring IRF bridge MAC persistence

The MAC address persistent time changed from 10 minutes to 12 minutes for the irf mac-address persistent timer command.

Operation changes in F6203

None.

Operation changes in F6202

- Support of 10-GE and 25-GE interfaces for the 1Gbps speed

Before modification: 10-GE and 25-GE interfaces on HPE FlexFabric 5950 48SFP28 8QSFP28 Switch and HPE FlexFabric 5950 4-slot Switch do not support the 1Gbps speed or transceiver module.

After modification: 10-GE and 25-GE interfaces on HPE FlexFabric 5950 48SFP28 8QSFP28 Switch and HPE FlexFabric 5950 4-slot Switch support the 1Gbps speed or transceiver module.

- Excluding a service interface from the IRF MAD shutdown action by the system

When the IRF fabric transits to the Recovery state, the system automatically excludes the following service interfaces from being shut down:

Before modification:

IRF physical interfaces.

Member interfaces of an aggregate interface if the aggregate interface is excluded from being shut down.

After modification:

IRF physical interfaces.

Interfaces used for BFD MAD.

Member interfaces of an aggregate interface if the aggregate interface is excluded from being shut down.

- Aging time for ARP entries in the output of the display arp command

Before modification: The command displays the ARP entry aging time in minutes.

After modification: The command displays the ARP entry aging time in seconds.

- Value range for port security's limit on the number of secure MAC addresses

The value range for the max-count argument of the port-security max-mac-count max-count [ vlan [ vlan-id-list ] ] command changed.

Before modification: The value range for the max-count argument is 1 to 4294967295.

After modification: The value range for the max-count argument is 1 to 2147483647.

- Enabling a BGP instance

Support for session multithreading was removed. You can create the same public IPv4 unicast address family on different BGP instances. For more information, see HPE 5950-CMW710-F6202 Release Notes (Software Feature Changes).

- Specifying a RabbitMQ server

In this software version, you can specify the port number through which the device communicates with a RabbitMQ server when you specify the RabbitMQ server. You can specify the MPLS L3VPN instance to which a RabbitMQ server belongs when you remove the RabbitMQ server. For more information, see HPE 5950-CMW710-F6202 Release Notes (Software Feature Changes).

- IChanged the default load sharing mode for IP tunneled traffic for the ip load-sharing mode command

Before modification: If you do not specify the tunnel { all | inner | outer } parameters in the ip load-sharing mode command, IP tunneled traffic is distributed based on the outer IP header.

After modification: If you do not specify the tunnel { all | inner | outer } parameters in the ip load-sharing mode command, IP tunneled traffic is distributed based on the inner IP header.

Operation changes in R6125

- Number of subinterfaces supported

Before modification, you can create 500 Layer 3 Ethernet subinterfaces and Layer 3 aggregate subinterfaces in total.

After modification, you can create 1024 Layer 3 Ethernet subinterfaces and Layer 3 aggregate subinterfaces in total.

- CL72 negotiation

Before modification, CL72 negotiation is enabled by default.

After modification, CL72 negotiation is disabled by default.

Operation changes in R6123

- Configuring IRF bridge MAC persistence

The MAC address persistent time changed from 6 minutes to 10 minutes for the irf mac-address persistent timer command.

Operation changes in R6106P01

None.

Operation changes in R6106

None.

Operation changes in R6105

First release.

Restrictions and cautions

To connect an interface on the LSWM124XG2QL(JH180A) interface module or a 25-GE interface with a GE transceiver module to another device, you must perform the following tasks on the interface:

Set the interface speed to 1000 Mpbs by using the speed 1000 command.

Configure the interface to operate in full duplex mode by using the duplex full command.

When multiple queues on one interface or multiple interfaces exceed the buffer usage threshold in FNA at the same time, alarms are not reported for some queues, and the corresponding queue monitor graphs are incorrect.

Multicast packets cannot trigger FNA alarms on the following interfaces:

10-GE interfaces on the HPE FlexFabric 5950 32QSFP28 Switch (JH321A) and HPE FlexFabric 5950 32QSFP28 TAA-compliant Switch (JH322A).

GE interfaces on the HPE FlexFabric 5950 48SFP28 8QSFP28 Switch (JH402A) and HPE FlexFabric 5950 4-slot Switch (JH404A).

Open problems and workarounds

      - 201612230336

Symptom: After a tunnel interface comes up, the interface cannot decapsulate and forward packets received from VTEPs.

Condition: This symptom occurs if multiple tunnels exist and the source IP address of one tunnel is the global source IP address. Then, the tunnel that is not configured with a source IP address can come up, but the tunnel interface cannot decapsulate and forward packets received from VTEPs.

Workaround: Make sure the source IP address configured for a tunnel is different from the global source IP address.

      - 201808170465

Symptom: NetStream is unavailable on a shutdown GigabitEthernet interface on the rear panel.

Condition: This symptom might occur if NetStream is enabled on a shutdown GigabitEthernet interface on the rear panel.

Workaround: Bring up the GigabitEthernet interface.

      - 201901150116

Symptom: In a VPLS network, if RSVP is used to distribute labels for MPLS TE tunnels, an MPLS TE tunnel cannot switch traffic from the failed primary path to the backup path.

Condition: This symptom might occur if RSVP is used to distribute labels for MPLS TE tunnels in a VPLS network.

Workaround: None.

List of resolved problems

Resolved problems in R6301P02

None.

Resolved problems in R6301P01

      - 202212280349/202212021236

- Symptom: A VLAN interface fails to forward traffic at Layer 3 during an ISSU.

- Condition: This symptom occurs when an ISSU is performed.

Resolved problems in R6301

      - 201905200485/201901090410

- Symptom: On the IRF fabric, the management address fails to be displayed in the LLDP information received from the neighboring devices.

- Condition: This symptom might occur if the following conditions exist:

VLAN interfaces are created on the IRF fabric and IP addresses are assigned to the interfaces.

An IRF subordinate device reboots.

      - 201812060001

- Symptom: The XMLCFGD process creates a core file unexpectedly.

- Condition: This symptom might occur if a NETCONF connection is established to the device to manage the device and NETCONF is used to reboot the device.

      - 201809290321

- Symptom: On a DRNI network, a device reboots because of memory exhaustion.

- Condition: This symptom might occur if the following conditions exist:

The keepalive timeout timer on the secondary DR member device is set to the maximum value.

A configuration rollback is performed on the primary DR member device to cancel the DRNI configuration and then another configuration rollback is performed to recover the DRNI configuration.

      - 201902010798

- Symptom: A device management user fails to obtain another user role by using the super command.

- Condition: This symptom might occur if the device management user logs in to the device after passing HWTACACS authentication and executes the super command to obtain another user role.

      - 201904010489

- Symptom: The device fails to forward traffic correctly.

- Condition: This symptom might occur if a loop exists on the device, which causes the ARP table to update repeatedly and then causes FIB table update failure.

      - 201903211294

- Symptom: The device reboots unexpectedly.

- Condition: This symptom might occur if the control plane deploys entries that contain unassigned IP addresses to the data plane on a control-/data-plane separated network.

      - 201807190673

- Symptom: The ofcd process fails because of exception.

- Condition: This symptom might occur if the established OpenFlow tunnel is attacked by exception OpenFlow packets in which the length of the protocol header field is 0.

      - 201809110564

- Symptom: The cp process still remains on the device after the connection to the controller is terminated.

- Condition: This symptom might occur if the controller deploys the save command through NETCONF to save the running configuration and then terminates the connection to the device.

      - 201811060548

- Symptom: The CPU usage rises rapidly during inter-VPN traffic forwarding.

- Condition: This symptom might occur if BGP redirects direct routes between multiple VPN instances.

      - 201809200079

- Symptom: The RADIUS server fails to assign an authorization VLAN name to a user after the user passes authentication.

- Condition: This symptom might occur if the authorization VLAN name is in the format of \000XXXXX\000.

      - 201904010490

- Symptom: The device reboots unexpectedly.

- Condition: This symptom might occur if ARP entries are deleted when SNMP is walking the ARP table.

      - 201904020841

- Symptom: The device reboots unexpectedly.

- Condition: This symptom might occur if TCP MSS is set on a subinterface and the subinterface is repeatedly deleted and created when SLB traffic is forwarded.

      - 201807300378/201905090714

- Symptom: A memory leak occurs on the SNMP process.

- Condition: This symptom occurs if the following conditions exist:

SNMP notifications for system logs are disabled.

The NMS walks the SYSLOG-MSG-MIB to obtain data.

      - 201811070579

- Symptom: The lauthd process creates a core file unexpectedly.

- Condition: This symptom might occur if the local-user-export class network guest url b command is executed consecutively several times.

      - 201811060248

- Symptom: The IMC server forcibly logs out a portal user after the user passes portal authentication.

- Condition: This symptom might occur if the portal authentication server runs IMC PLAT 7.3 and security policy confirmation (such as ACL and VLAN) is deployed on the IMC server.

      - 201810230548/201809120806

- Symptom: A memory leakage occurs on a subordinate device in an IRF fabric.

- Condition: This symptom might occur if portal users that obtain IP addresses through DHCP carries Option 82 or Option 18 when they come online.

      - 201809200058

- Symptom: The Aaad process on an IRF fabric creates a core file unexpectedly.

- Condition: This symptom might occur if the following conditions exist:

  - A large number of IPoE users come online through the IRF fabric.

  - Master/subordinate switchover repeatedly takes place.

  - The AAA process reboots repeatedly.

      - 201812070009/201812061078

- Symptom: Specific UDP packets get lost during forwarding.

- Condition: This symptom might occur if a UDP packet has the following characteristics:

  - The packet is a fragment packet.

  - The packet carries MPLS labels.

  - The third and fourth bytes in the IP header of non-first fragment packets is 0D AF.

      - 201811060034

- Symptom: An IPsec SA is established between the device and the peer device through IKEv2 negotiation and the security protocol is ESP. IPsec protocol packets from the peer device are discarded because the packet length exceeds the port MTU.

- Condition: This symptom might occur if TFC padding is enabled and IPsec packet fragmentation is disabled on the peer device.

      - 201903211236

- Symptom: The CLI of a device in an IRF fabric gets stuck and no commands can be input.

- Condition: This symptom might occur if a large number of tunnels flap and IRF master/subordinate switchover repeatedly takes place.

      - 201902020055

- Symptom: IS-IS neighbor relationship cannot be established.

- Condition: This symptom occurs if the following operations are performed:

Configure the network type as P2P and enable IS-IS on an interface.

Reboot the device.

      - 201904020277

- Symptom: ARP entries become blackhole entries, and packets are lost.

- Condition: This symptom occurs if the following operations are performed:

Multiple Layer 2 aggregation groups exist in the network, and loops exist in some aggregation groups.

Enable ARP active acknowledgement.

Configure static routes on a Layer 3 interface. Shut down and then bring up the Layer 3 interface, or MAC address moves occur on the Layer 3 interface.

      - 201902020232

- Symptom: The master IRF member device might reboot unexpectedly at a low probability.

- Condition: This symptom occurs if the following operations are performed:

Set a small idle timeout value for TCP connections.

Initiate a large number of TCP connections for services using TCP (for example, BGP and HTTP) on the local end.

      - 201811060022

- Symptom: The memory leaks for the IPFS module.

- Condition: This symptom occurs if the following conditions exist:

  - A large amount of traffic with varying quintuples is forwarded by software.

  - The fast forwarding entries age out.

      - 201902020140

- Symptom: After the TCP client connection is closed, the memory leaks.

- Condition: This symptom occurs if the following operations are performed:

The client sends a large amount of data to the server. The server cannot process so much data, so the server responds with Zero Window.

The client starts the persist timer after receiving Zero Window.

The client actively closes the connection.

      - 201902020187

- Symptom: The CPU usage might be high at a low probability.

- Condition: This symptom occurs if a large number of packets are transmitted when a user logs in through nested Telnet.

      - 201812070478

- Symptom: An interface on a subordinate IRF member device cannot join a voice VLAN again after leaving the voice VLAN.

- Condition: This symptom occurs if the following operations are performed:

Enable LLDP on an interface on a subordinate IRF member device, and configure a voice VLAN on the interface. Connect the interface to a voice device supporting LLDP/CDP.

Establish or disconnect the LLDP neighbor relationship on the subordinate IRF member device.

      - 201811060177

- Symptom: After an IP phone successfully comes online, the gateway cannot ping the IP phone for a period of time.

- Condition: This symptom occurs if the following operations are performed:

Connect an interface to a Cisco IP phone, enable CDP-compatible LLDP on the interface, and assign the IP phone to a voice VLAN.

The interface repeatedly comes up and goes down.

      - 201811060399

- Symptom: A DHCP client cannot obtain an IP address.

- Condition: This symptom occurs if the device acts as a DHCP sever, multiple address pools are configured, and some address pools are configured with address ranges for dynamic allocation by using the address range command.

      - 201812060884

- Symptom: The XMLCFGD process exits exceptionally.

- Condition: This symptom occurs if the following operations are performed:

The device acts as a DHCP Sever. In a DHCP address pool, configure more than 13 static IP address bindings.

Use SoapUI to get the data of the DHCP/DHCPServerPoolStatic table.

      - 201810290644

- Symptom: During auto upgrade, the using tengige command is mistakenly executed. As a result, the comsh process becomes abnormal, and related interfaces disappear.

- Condition: This symptom occurs because the using tengige command is mistakenly executed during the configuration recovery process. On the device, the using tengige command takes effect in real time, but the configuration file incorrectly contains the command.

      - 201903290556

- Symptom: Interface flapping causes the CPU usage to reach 100%.

- Condition: This symptom occurs if the following operations are performed:

Multiple routes of BGP neighbors are configured with FRR. The active and backup next hops of FRR are reverse for two routes (for example, the active and backup next hops of route A are 1 and 2, and the active and backup next hops of route B are 2 and 1), and the next hops 1 and 2 are in the network segments of routes A and B.

Shut down the interfaces corresponding to the two next hops in sequence.

      - 201903290558

- Symptom: When the spanning tree mode is switched to PVST, the device will be stuck for a period of time.

- Condition: This symptom occurs if a large number of VLANs and interfaces exist on the device and the spanning tree mode is switched to PVST.

      - 201811060535

- Symptom: When an interface card is unplugged and plugged, the aggregate interface creation event on the interface card is not reported. As a result, the aggregate interface on the interface card is not set to the drive, and the aggregate interface member ports cannot forward traffic.

- Condition: This symptom occurs because the interface management module does not report the aggregate interface creation event during the startup process when an interface card is plugged.

- Occurrence probability: This symptom occurs only when interface events are not reported. In an environment, there are a large number of interface events. In a complicated environment, the occurrence probability is high. In a test environment, the occurrence probability is low.

      - 201807060250

- Symptom: Some traffic is broadcast on a DR interface.

- Condition: This symptom occurs if an aggregate interface leaves and then joins a DR group and continuously receives traffic.

      - 201903110087

- Symptom: The BFD session on a Layer 3 aggregate interface flaps.

- Condition: This symptom occurs if the following operations have been performed:

Configure a Layer 3 aggregate interface with member ports on different cards, enable BFD for OSPF, and use MD5 authentication for BFD control packets.

Remove a member port from the Layer 3 aggregation group and then add it back to the aggregation group.

      - 201806040598

- Symptom: The secure MAC address entry is not removed from the display mac-address command after a user goes offline.

- Condition: This symptom occurs if port security is configured and the user goes offline after passing authentication.

      - 201701100257

- Symptom: Traffic detection fails in a Fabric Director scenario.

- Condition: This symptom occurs if a QoS policy is issued multiple times.

      - 201806070741

- Symptom: The remark dscp command issued by OpenFlow does not take effect.

- Condition: This symptom occurs if the Output action is issued by OpenFlow at the same time.

      - 201904020301

- Symptom: The relevant MAC address entry is not removed from the display mac-address command after an 802.1X user moves to a different VLAN on the same port.

- Condition: This symptom occurs if an 802.1X user moves to a different VLAN on the same port.

      - 201904020262

- Symptom: In an EVPN distributed relay environment, the interface where a single-armed AC is configured cannot forward packets.

- Condition: This symptom occurs if the IPP interface setting is cancelled and then restored for a tunnel interface .

      - 201904110239

- Symptom: A DR system fails to be established.

- Condition: This symptom occurs if a manually created tunnel interface is used as the IPL.

      - 201903150058

- Symptom: In a DRNI network, the DR interface of the secondary DR device is still up after the IPP interface is brought down.

- Condition: This symptom occurs if the secondary DR device is in DRNI MAD DOWN state.

      - 201903210720

- Symptom: In an EVPN distributed relay environment, the DR system sends out multiple copies of unknown unicast packets.

- Condition: This symptom might occur if the following operations have been performed:

Use a VXLAN tunnel as the IPL and reboot the DR system.

Receive unknown unicast packets from the remote AC.

      - 201812060999

- Symptom: In a DRNI network, the DR interface is set to DRNI DOWN state.

- Condition: This symptom might occur if the IPP interface flaps.

- 201903080004/201903070270

- Symptom: In an MPLS network, a P device drops packets continuously.

- Condition: This symptom might occur if the link between the P device and another P device or a PE device flaps for a long time more than once.

- 201805040745

- Symptom: In a multiple VSC environment, the device cannot connect to the primary VSC.

- Condition: This symptom might occur if the OVSDB process is restarted.

- 201902140542

- Symptom: In an EVPN distributed relay environment, the IPL cannot work correctly.

- Condition: This symptom might occur if you configure VLAN-based VXLAN assignment and then configure EVPN distributed relay.

- 201810300310

- Symptom: The management Ethernet port goes down in an IRF fabric.

- Condition: This symptom might occur after a master/subordinate switchover is performed.

- 201711070993

- Symptom: In a VXLAN network, VMs in different network segments cannot communicate.

- Condition: This symptom occurs if a VXLAN gateway group is used as the gateway.

- 201805020138/201805020139

- Symptom: An additional coldStart log is printed every time the switch sends a trap.

- Condition: This symptom occurs after the switch reboots.

      - 201904020313

- Symptom: A user can join and leave the multicast group without passing authentication.

- Condition: This symptom occurs if both MLD and IPv6 portal authentication are configured on the VLAN interface.

      - 201903180860

- Symptom: A serial port hangs in a DRNI network.

- Condition: This symptom might occur if the following operations have been performed:

Enable and disable configuration consistency check repeatedly.

Execute the display drni consistency type2 global command.

      - 201810100474

- Symptom: ICMPv6 packets are counted into the IP-other protocol type.

- Condition: This symptom occurs when the switch receives ICMPv6 packets.

      - 201811090192

- Symptom: The MAC address entry is not removed from the display mac-address command after a MAC authentication user goes offline.

- Condition: This symptom occurs if the MAC authentication user comes online and then goes offline.

      - 201812110026

- Symptom: In an EVPN network, an access port sends packets with VLAN tags.

- Condition: This symptom might occur if two route reflectors are used and link switchover between them has occurred.

      - 201904030323

- Symptom: The remote host has the TCP timestamps vulnerability.

- Condition: This symptom occurs if the host implements RFC 1323.

      - 201812061014

- Symptom：HPE Comware 7 stored and reflected XSS Vulnerability

- Condition：An xss reflected in the web portal of the appliance HP Comware switch 7.1.045. Attackers can exploit this issue to open a web browser and log in to the application using valid or not credentials.

      - 201902010459

- Symptom：CVE-2018-5407

- Condition：OpenSSL is prone to a local information-disclosure vulnerability.  Local attackers

- can exploit this issue to obtain sensitive information. This may aid in further attacks.

      - 201812050851

- Symptom: Files in the flash might fail to be deleted at a low probability.

- Condition: This symptom occurs if multiple consoles operate the device simultaneously.

      - 201811230657

- Symptom：CVE-2018-15473

- Condition：OpenSSH is prone to a user-enumeration vulnerability. An attacker may leverage this issue to harvest valid user accounts, which may aid in brute-force attacks. OpenSSH through 7.7 are vulnerable; other versions may also be affected.

      - 201903140269/201904020861

- Symptom: After the operating mode of a device is switched from L3GW to L2GW, the L3VNI configuration remains.

- Condition: This symptom occurs if the following operations are performed:

Configure the device to operate in L3GW mode, and configure L3VNIs.

Configure the device to operate in L2GW mode, save the configuration, and reboot the device.

      - 201903280399

- Symptom: When EVPN and DRNI are used together on the switch, frequent tunnel interface flapping might cause traffic interruption.

- Condition: This symptom might occur if frequent tunnel interface flapping occurs.

Resolved problems in F6207

      - 201808290235

- Symptom: Symmetric load sharing might not take effect.

- Condition: This symptom occurs if symmetric load sharing is configured on the device.

      - 201811010044

- Symptom: The panel of the LSWM116Q interface module is displayed incorrectly on IMC.

- Condition: This symptom occurs if the LSWM116Q interface module is installed in subslot 4 of the device and managed by IMC.

      - 201811090022

- Symptom: When an aggregation group has more than eight member ports, the excessive member ports do not have traffic.

- Condition: This symptom occurs if more than eight ports on a single device are added to an aggregation group and the corresponding aggregate interface receives unknown unicast traffic.

      - 201811130637

- Symptom: When a specific port acts as the monitor port of Layer 2 remote port mirroring, no traffic is mirrored to the monitor port.

- Condition: This symptom occurs if the following conditions exist:

On an IRF fabric, Layer 2 remote port mirroring is configured.

In the remote probe VLAN, the monitor port and the reflector port reside on different IRF member devices and have the same port number.

      - 201812050136

- Symptom：CVE-2018-15473

- Condition：OpenSSH is prone to a user-enumeration vulnerability. An attacker may leverage this issue to harvest valid user accounts, which may aid in brute-force attacks. OpenSSH through 7.7 are vulnerable; other versions may also be affected.

      - 201812190652

- Symptom: A PBR policy fails to be applied because the ACL resources are insufficient.

- Condition: This symptom occurs if the following conditions exist:

The PBR policy is applied to interfaces belonging to different interface groups.

When you apply the PBR policy, the next hop of the PBR policy does not exist in the routing table.

The next hop of the PBR policy becomes reachable in the routing table.

      - 201812270200

- Symptom: A 40-GE interface is split into 10-GE breakout interfaces, and these 10-GE interfaces are configured as Layer 3 interfaces. Some breakout interfaces might be in an incorrect STP state and fail to learn ARP entries.

- Condition: This symptom occurs if the following operations are performed:

Split a 40-GE interface into 10-GE breakout interfaces.

Configure these 10-GE breakout interfaces as Layer 3 interfaces.

      - 201812290518

- Symptom: Layer 3 packets sent out of a specific interface are dropped.

- Condition: This symptom occurs if a specific interface forwards Layer 3 packets.

      - 201901110111

- Symptom: The CPU usage of the COPPD process becomes high.

- Condition: This symptom occurs if the following conditions exist:

Configure the RSVP protocol, and enable BFD for RSVP.

The device resides in the middle of the tunnel, and the downstream device is unreachable.

      - 201901110113

- Symptom: The Comsh task remains, and the CPU usage becomes high.

- Condition: This symptom occurs if the following operations are performed:

Log in to the device through Telnet.

Use the lock-key command to set the user line locking key.

Press this shortcut key to lock the remote connection.

Disconnect the remote connection when the remote connection is still locked.

      - 201901170294

- Symptom: Packet loss might occur.

- Condition: This symptom occurs if link-aggregation traffic redirection is configured and some slots are rebooted.

Resolved problems in F6206

      - 201708310741

- Symptom: In a DRNI network, the keepalive link goes down.

- Condition: This symptom occurs if the device is configured with both DRNI and OpenFlow.

      - 201712180820

- Symptom: In an EVPN DRNI network, the downlink interface receives redundant traffic.

- Condition: This symptom occurs if a DR interface on a DR device is shut down when the downlink host is silent.

      - 201807120129

- Symptom: MPLS traffic cannot be forwarded.

- Condition: This symptom occurs if the following conditions exist in a CTOC network:

Primary and backup tunnels are configured.

The traffic is switched to the backup tunnel when a node fails.

The traffic is switched back to the primary tunnel when the link restores.

      - 201807190749

- Symptom: CRC error packets appear when FC traffic is being forwarded.

- Condition: This symptom occurs if a 16G FC module is used to transmit the FC traffic.

      - 201801190600

- Symptom: 10-GE interfaces on the LSWM124XG2Q and LSWM124XG2QFC interface modules do not come up.

- Condition: This symptom occurs if the HPE FlexFabric 5950 4-slot Switch has LSWM124XG2Q and LSWM124XG2QFC interface modules installed.

      - 201806120020

- Symptom: The RSVP process exits exceptionally after certain operations.

- Condition: This symptom occurs if the public network tunnels are configured with TE FRR and IP address conflicts exist in a CTOC network.

      - 201806120097

- Symptom: The device might reboot unexpectedly at a low probability.

- Condition: This symptom occurs if the HPE FlexFabric 5950 4-slot Switch uses an LSWM116Q interface module.

      - 201805220109

- Symptom: When an IRF physical interface goes down, the other interfaces in the same group as the IRF physical interface flap.

- Condition: This symptom occurs if a 40-GE interface on an LSWM116Q interface module of the HPE FlexFabric 5950 4-slot Switch is used as an IRF physical interface.

      - 201803190071

- Symptom: When ACLs are issued to multiple Layer 3 aggregate interfaces, the ACLs take effect on only one Layer 3 aggregate interface.

- Condition: This symptom occurs if ACLs are issued to multiple Layer 3 aggregate interfaces and rules are added to or deleted from these ACLs.

      - 201803170047

- Symptom: Issued ACLs might not take effect.

- Condition: This symptom occurs if ACLs are repeatedly issued or repeatedly dynamically modified.

      - 201802050250

- Symptom: The device reboots unexpectedly.

- Condition: This symptom occurs if the shutdown and undo shutdown commands are executed on the FC interface connecting to a server or storage device.

      - 201711290143

- Symptom: A user cannot come online.

- Condition: This symptom occurs if the switch is configured to operate in FCF mode.

      - 201803290351

- Symptom: PFC deadlock detection takes effect only on the first interface where it is configured, and does not take effect on the subsequent interfaces where it is configured.

- Condition: This symptom occurs if PFC deadlock detection is configured on two or more interfaces of the device.

- 201711030407

Symptom: CVE-2017-1000253

Condition: Local attackers may exploit this issue to gain root privileges.

- 201712220137

Symptom: CVE-2017-3736

Condition: An attacker can exploit this issue to gain access to sensitive information that may aid in further attacks.

- 201712190381

Symptom: CVE-2017-12190

Condition: Local attacker can exploit these issues to obtain sensitive information that may lead to further attacks.

Symptom: CVE-2017-12192

Condition: Attackers can exploit this issue to cause denial-of-service conditions. Due to the nature of this issue, arbitrary code execution may be possible but this has not been confirmed.

Symptom: CVE-2017-15274

Condition: An attacker can exploit this issue to cause a local denial-of-service condition.

Symptom: CVE-2017-15299

Condition: An attacker can exploit this issue to trigger a kernel panic, denying service to legitimate users.

- 201802060169

Symptom: CVE-2017-15896

Condition: An attacker can exploit this issue to bypass TLS validate and encrypt, send application data to Node.js.

Symptom: CVE-2017-3737

Condition:  Successfully exploiting this issue will allow attackers to bypass security restrictions and perform unauthorized actions; this may aid in launching further attacks.

Symptom: CVE-2017-3738

Condition: An attacker can exploit this issue to gain access to sensitive information that may aid in further attacks.

- 201805250759

Symptom: CVE-2016-9586

Condition: Attackers can exploit this issue to execute arbitrary code in the context of the affected application. Failed exploit attempts will result in denial-of-service conditions.

- 201710180649

Symptom: MACsec connections cannot be set up if a transceiver module or subcard is removed and re-installed when MACsec traffic exists.

Condition: This symptom might occur if a transceiver module or subcard is removed and re-installed when MACsec traffic exists.

      - 201710240487

Symptom: EVPN traffic forwarding fails if the remote VTEP switches between tunnels.

Condition: This symptom might occur if the remote VTEP switches between tunnels.

      - 201710270200

Symptom: An interface cannot forward traffic after its MACsec configuration is removed.

Condition: This symptom might occur if MACsec configuration is removed from an interface.

      - 201706270683

Symptom: NetStream fails to collect traffic statistics on an interface when NetStream is disabled on other interfaces.

Condition: This symptom occurs if NetStream is configured on multiple interfaces and then NetStream is disabled on one of these interfaces.

      - 201609230034

- Symptom: BFD session flapping occurs when SSL VPN AC interfaces are shut down.

- Condition: This symptom might occur if SSL VPN AC interfaces are shut down.

      - 201708310704

Symptom: A 10-GE copper interface cannot come up.

Condition: This symptom occurs if an external loopback test is performed on the 10-GE copper interface.

      - 201709260138

Symptom:  CVE-2017-3735

Condition: Successfully exploiting this issue will allow attackers to bypass security restrictions and perform unauthorized actions; this may aid in launching further attacks.

- 201711270371

Symptom: The management Ethernet interface might fail to be pinged.

Condition: This symptom occurs if a master/subordinate switchover occurs to an IRF fabric.

Resolved problems in R6205P03

      - 201709260138

Symptom:  CVE-2017-3735

Condition: Successfully exploiting this issue will allow attackers to bypass security restrictions and perform unauthorized actions; this may aid in launching further attacks.

      - 201710180649

Symptom: MACsec connections cannot be set up if a transceiver module or subcard is removed and re-installed when MACsec traffic exists.

Condition: This symptom might occur if a transceiver module or subcard is removed and re-installed when MACsec traffic exists.

      - 201710240487

Symptom: EVPN traffic forwarding fails if the remote VTEP switches between tunnels.

Condition: This symptom might occur if the remote VTEP switches between tunnels.

      - 201710270200

Symptom: An interface cannot forward traffic after its MACsec configuration is removed.

Condition: This symptom might occur if MACsec configuration is removed from an interface.

      - 201706270683

Symptom: NetStream fails to collect traffic statistics on an interface when NetStream is disabled on other interfaces.

Condition: This symptom occurs if NetStream is configured on multiple interfaces and then NetStream is disabled on one of these interfaces.

      - 201609230034

- Symptom: BFD session flapping occurs when SSL VPN AC interfaces are shut down.

- Condition: This symptom might occur if SSL VPN AC interfaces are shut down.

      - 201708310704

Symptom: A 10-GE copper interface cannot come up.

Condition: This symptom occurs if an external loopback test is performed on the 10-GE copper interface.

Resolved problems in F6205P02

      - 201706050458

Symptom: In an EVPN network, traffic is duplicated.

Condition: This symptom occurs if the following conditions exist:

In an EVPN network, the outgoing interface of a VXLAN tunnel is a Layer 3 aggregate interface.

The shutdown or undo shutdown command is executed on an interface transmitting traffic.

      - 201706090010

Symptom: Residual rawip sockets exist and occupy the memory.

Condition: This symptom occurs if the switch performs NQA operations for a long time.

      - 201705250358

Symptom: An FC aggregate interface cannot come up.

Condition: This symptom occurs if the member ports of the FC aggregate interface are all on subordinate IRF member devices.

      - 201705060255

Symptom: NetStream fails to collect traffic statistics on an interface.

Condition: This symptom occurs if IPv6 NetStream filtering is enabled on the interface.

      - 201609060180

- Symptom: The following symptoms occur on an interface that hosts an AC:

  - If the priority trust mode is not set, the interface cannot perform EXP value mapping correctly for MPLS packets with an EXP value of 0, and the 802.1p priority of the packets is set to 0 after priority mapping.

  - If the priority trust mode is set to 802.1p or DSCP, the 802.1p priority of packets is set to 0 after priority mapping.

- Condition: This symptom might occur if the priority trust mode is not set or the qos trust dot1p or qos trust dscp command is executed on an interface that hosts an AC.

      - 201708250543

Symptom: On a DR interface, traffic among different service instances associated with the same VSI cannot be forwarded.

Condition: This symptom occurs if the DR interface is configured with ACs and the shutdown and undo shutdown commands are executed on the member ports of the DR interface.

      - 201708310748

Symptom: NetStream fails to collect traffic statistics on an interface.

Condition: This symptom occurs if IPv6 NetStream filtering is enabled on the interface.

      - 201709080177

Symptom: A fiber management interface cannot come up.

Condition: This symptom occurs if the transceiver module is removed from and then installed into a fiber management interface on a subordinate IRF member device.

      - 201708310760

Symptom: MACsec cannot operate correctly on an MKA-enabled interface.

Condition: This symptom occurs if the speed command is used to set the speed for an interface supporting MACsec.

Resolved problems in F6205

201705230173

- Symptom: On an EVPN network with distributed EVPN gateways, outgoing packets on an Ethernet service instance carry an 802.1Q VLAN tag unexpectedly.

- Condition: This symptom might occur if the device acts as an EVPN gateway to perform Layer 3 forwarding and the Ethernet service instance is configured to match frames that do not have an 802.1Q VLAN tag.

201612050269

- Symptom: CVE-2016-7427

- Condition: An attacker with access to the NTP broadcast domain can periodically inject specially crafted broadcast mode NTP packets into the broadcast domain which, while being logged by ntpd, can cause ntpd to reject broadcast mode packets from legitimate NTP broadcast servers

- Symptom: CVE-2016-7428

- Condition: An attacker with access to the NTP broadcast domain can send specially crafted broadcast mode NTP packets to the broadcast domain which, while being logged by ntpd, will cause ntpd to reject broadcast mode packets from legitimate NTP broadcast servers

- Symptom: CVE-2016-7431

- Condition: Zero Origin timestamp problems were fixed by Bug 2945 in ntp-4.2.8p6. However, subsequent timestamp validation checks introduced a regression in the handling of some Zero origin timestamp checks.

201702220570

- Symptom: CVE-2017-3730

- Condition: OpenSSL is prone to denial-of-service vulnerability. Attackers can exploit this issue to cause a denial-of-service condition.

- Symptom: CVE-2017-3731

- Condition: OpenSSL is prone to denial-of-service vulnerability. An attacker may exploit this issue to crash the application, resulting in denial-of-service condition.

- Symptom: CVE-2017-3732

- Condition: OpenSSL is prone to an information-disclosure vulnerability. An attacker can exploit this issue to gain access to sensitive information that may aid in further attacks.

201704280171

Symptom: CVE-2015-3405

Condition: An attacker can exploit the ntp-keygen utility to spoof an NTP client or server.

Symptom: CVE-2014-9297

Condition: An attacker can exploit this issue. When an NTP client decrypted a secret received from an NTP server.

Symptom: CVE-2015-9298

Condition: An attacker could bypass source IP restrictions and send malicious control and configuration packets.

201704280531

Symptom: CVE-2017-64558

Condition: NTP are prone to a buffer-overflow vulnerability because it fails to properly bounds-check user-supplied data before copying it into an insufficiently sized buffer.

Symptom: CVE-2016-9042

Condition: NTP is prone to a denial-of-service vulnerability. An attacker can exploit this issue to cause a denial-of-service condition, denying service to legitimate users.

Resolved problems in F6203

201704010616

- Symptom: An IRF fabric splits when the IRF fabric has 8K VPLS PWs and the public network interface on the IRF fabric flaps.

- Condition: This symptom might occur if the IRF fabric has 8K VPLS PWs and the public network interface on the IRF fabric flaps.

201703310499

- Symptom: FNA is not available for GE interfaces on the FlexFabric 5950 48SFP28 8QSFP28 or FlexFabric 5950 4-slot switch.

- Condition: This symptom might occur if FNA is configured on GE interfaces of the FlexFabric 5950 48SFP28 8QSFP28 or FlexFabric 5950 4-slot switch.

201704240426

Symptom: A memory exhaustion occurs on the switch when the switch is enabled with FNA and keeps generating notification messages for a long time.

Condition: This symptom might occur if the switch is enabled with FNA and keeps generating notification messages for a long time.

201703160569

- Symptom: A 10-GE interface that is installed with a GE transceiver module and connected to another device cannot come up if the 10-GE interface is on one of the following hardware:

LSWM124XG2QL(JH180A) interface module.

FlexFabric 5950 32QSFP28 switch.

FlexFabric 5950 32QSFP28 TAA-compliant switch.

- Condition: This symptom might occur if the 10-GE interface is installed with a GE transceiver module and connected to another device and the 10-GE interface is on one of the following hardware:

  - LSWM124XG2QL(JH180A) interface module.

  - FlexFabric 5950 32QSFP28 switch.

  - FlexFabric 5950 32QSFP28 TAA-compliant switch.

201704250571

Symptom: In an IRF fabric, forwarded Layer 2 packets arrive at the incoming interface of the packets again.

Condition: This symptom occurs with a low probability if an interface in the IRF fabric is split into breakout interfaces and then the breakout interfaces are combined.

Resolved problems in F6202

201612050269

Symptom: CVE-2016-7427

Condition: An attacker with access to the NTP broadcast domain can periodically inject specially crafted broadcast mode NTP packets into the broadcast domain which, while being logged by ntpd, can cause ntpd to reject broadcast mode packets from legitimate NTP broadcast servers

Symptom: CVE-2016-7428

Condition: An attacker with access to the NTP broadcast domain can send specially crafted broadcast mode NTP packets to the broadcast domain which, while being logged by ntpd, will cause ntpd to reject broadcast mode packets from legitimate NTP broadcast servers

Symptom: CVE-2016-7431

Condition: Zero Origin timestamp problems were fixed by Bug 2945 in ntp-4.2.8p6. However, subsequent timestamp validation checks introduced a regression in the handling of some Zero origin timestamp checks.

      - 201611030036

- Symptom: Flow mirroring does not take effect.

- Condition: This symptom occurs if flow mirroring is configured in the outbound direction of a Layer 3 interface or Layer 3 aggregate interface.

Resolved problems in R6125

201610100291

Symptom: On a VXLAN or EVPN network, unknown unicast, broadcast, or multicast packets are duplicated or lost after they are forwarded.

Condition: This symptom occurs if the Ethernet service instance of a VSI is created on a cross-chassis Layer 2 aggregate interface, of which member ports are on different IRF member devices.

201612270330

Symptom: The system prompts that the memory is insufficient when the BGP neighborship information is displayed.

Condition: This symptom occurs if SNMP is used to read BGP information multiple times and causes memory leak.

201610240176

Symptom: Inter-data center Layer 2 forwarding fails in an EVPN-DCI network.

Condition: This symptom might occur if Layer 2 traffic is forwarded between two data centers of an EVPN-DCI network.

201609060256

Symptom: Frequent LDP protocol flapping causes hardware nexthop entry leakage and forwarding failure.

Condition: This symptom might occur if the LDP protocol flaps frequently.

201610140317

Symptom: CVE-2016-6304

Condition: OpenSSL is prone to denial-of-service vulnerability. An attacker may exploit this issue to cause a denial-of-service condition.

Symptom: CVE-2016-6306

Condition: OpenSSL is prone to a local denial-of-service vulnerability. A local attacker can exploit this issue to cause a denial-of-service condition.

201611070341

Symptom: CVE-2016-8858

Condition: A remote user can send specially crafted data during the key exchange process to trigger a flaw in kex_input_kexinit() and consume excessive memory on the target system. This can be exploited to consume up to 384 MB per connection.

201611080162

Symptom: CVE-2016-5195

Condition: An unprivileged local user could use this flaw to gain write access to otherwise read-only memory mappings and thus increase their privileges on the system.

201609140425

Symptom: After L2VPN is disabled, the EVPN-enabled device cannot forward traffic correctly because some EVPN ARP entries are not deleted.

Condition: This symptom might occur if L2VPN is disabled on the EVPN-enabled device.

Resolved problems in R6123

201609180002

Symptom: CVE-2016-2105

Condition: Fixed vulnerability in “EVP Encode” in OpenSSL before 1.0.1t and 1.0.2 before 1.0.2h allows remote attackers to cause a denial of service (heap memory corruption) via a large amount of binary data.

Symptom: CVE-2016-2106

Condition: Fixed vulnerability in “EVP Encrypt” in OpenSSL before 1.0.1t and 1.0.2 before 1.0.2h allows remote attackers to cause a denial of service (heap memory corruption) via a large amount of binary data.

Symptom: CVE-2016-2107

Condition: Fixed vulnerability in OpenSSL before 1.0.1t and 1.02h allows remote attackers to obtain sensitive cleartext information via a padding-oracle attack against an AES CBC session.

Symptom: CVE-2016-2108

Condition: Fixed vulnerability in OpenSSL before 1.0.1o and 1.0.2 before 1.0.2c allows remote attackers to execute arbitrary code or cause a denial of service (buffer underflow and memory corruption).

Symptom: CVE-2016-2109

Condition: Fixed vulnerability in “asn” before 1.0.1t and 1.0.2 before 1.0.2h allows remote attackers to cause a denial of service (memory consumption) via a short invalid encoding.

Symptom: CVE-2016-2176

Condition: Fixed vulnerability in “X509” in OpenSSL before 1.0.1t and 1.0.2 before 1.0.2h allows remote attackers to obtain sensitive information from memory or cause a denial of service.

201607290021

Symptom: CVE-2016-2177

Condition: Fixed vulnerability in s3_srvr.c, ssl_sess.c, and t1_lib.c functions in OpenSSL through 1.0.2h that allows remote attackers to cause a denial of service (integer overflow and application crash), or possibly have an unspecified other impact by leveraging unexpected malloc behavior.

201606240193

Symptom: CVE-2016-4953

Condition: Fixed vulnerability in NTP 4.x before 4.2.8p8 allows remote attackers to cause a denial of service by sending a spoofed packet with incorrect authentication data at a certain time.

Symptom: CVE-2016-4954

Condition: Fixed vulnerability in ntpd in NTP 4.x before 4.2.8p8 allows remote attackers to cause a denial of service by sending spoofed packets from source IP addresses in a certain scenario.

Symptom: CVE-2016-4956

Condition: Fixed vulnerability in NTP 4.x before 4.2.8p8 allows remote attackers to cause a denial of service via a spoofed broadcast packet.

201609010406

Symptom: CVE-2009-3238

Condition: The get_random_int function in the Linux kernel before 2.6.30 produces insufficiently random numbers, which allows attackers to predict the return value, and possibly defeat protection mechanisms

201603310333

Symptom: The port mirroring configuration fails to be issued.

Condition: This symptom occurs if an aggregate interface is configured as the monitor port of a mirroring group.

201604080607

Symptom: VXLAN traffic cannot be forwarded when the switch acts as a VXLAN IP gateway and has sFlow configured.

- Condition: This symptom occurs if sFlow is enabled on the physical interface corresponding to the VXLAN tunnel.

201604140250

- Symptom: In a VXLAN network, the routing protocols flap.

Condition: This symptom occurs if the following conditions exist:

The switch acts as a VXLAN IP gateway.

A large number of VXLAN tunnels are created on the switch.

The switch is sending and receiving a large number of ARP packets.

201604200574

- Symptom: OSPF neighborship flaps.

- Condition: This symptom occurs if an IRF splits.

201512230507

- Symptom: The configuration might fail to be saved.

- Condition: This symptom might occur if you save the configuration.

Resolved problems in R6106P01

None.

Resolved problems in R6106

201604060275

- Symptom: The PBR configuration on a VSI interface does not take effect.

- Condition: This symptom occurs if PBR is configured on a VSI interface of a VXLAN IP gateway.

201604010510

- Symptom: The DHCP snooping entries are not complete.

- Condition: This symptom occurs if both DHCP relay agent and DHCP snooping are configured on the switch in a Layer 3 network.

201604050208

- Symptom: The sub-VLANs of a super VLAN do not send IGMP/MLD queries.

- Condition: This symptom occurs if a super VLAN is configured and IGMP/MLD is configured on the VLAN interface of the super VLAN.

201603310127

- Symptom: Unknown unicast traffic and broadcast traffic are dropped in the outbound direction of an interface.

- Condition: This symptom occurs if the interface is configured as an IRF physical interface and then configured as a common interface.

Resolved problems in R6105

First release.

Support and other resources

Accessing Hewlett Packard Enterprise Support

For live assistance, go to the Contact Hewlett Packard Enterprise Worldwide website:

www.hpe.com/assistance

To access documentation and support services, go to the Hewlett Packard Enterprise Support Center website:

www.hpe.com/support/hpesc

Information to collect:

Technical support registration number (if applicable).

Product name, model or version, and serial number.

Operating system name and version.

Firmware version.

Error messages.

Product-specific reports and logs.

Add-on products or components.

Third-party products or components.

Documents

To find related documents, see the Hewlett Packard Enterprise Support Center website at http://www.hpe.com/support/hpesc.

Enter your product name or number and click Go. If necessary, select your product from the resulting list.

For a complete list of acronyms and their definitions, see HPE FlexNetwork technology acronyms.

Related documents

The following documents provide related information:

- HPE FlexFabric 5950 Command References-Release 620x

- HPE FlexFabric 5950 Configuration Guides-Release 620x

- HPE FlexFabric 5950 Switch Series Installation Guide

- HPE X711 Frt(prt)-Bck(pwr) HV2 Fan Tray (JH388A) & HPE X712 Bck(pwr)-Frt(prt) HV2 Fan Tray (JH389A) User Guide

- HPE A58x0AF 650W AC (JC680A) & 650W DC (JC681A) Power Supplies User Guide

- HPE 5930-4Slot Fan Trays (JH185A & JH186A) User Guide

- HPE LSWM18CQ Interface Module (JH406A) User Guide

- HPE LSWM116Q Interface Module (JH405A) User Guide

- HPE LSWM124TG2H Interface Module (JH450A) User Guide

- HPE LSWM18QC Interface Module (JH183A) User Guide

- HPE LSWM124XG2Q & LSWM124XG2QL Interface Modules User Guide

- HPE LSWM124XGT2Q Interface Module (JH182A) User Guide

- HPE LSWM124XG2QFC Interface Module (JH184A) User Guide

Documentation feedback

Hewlett Packard Enterprise is committed to providing documentation that meets your needs. To help us improve the documentation, send any errors, suggestions, or comments to Documentation Feedback (docsfeedback@hpe.com). When submitting your feedback, include the document title, part number, edition, and publication date located on the front cover of the document. For online help content, include the product name, product version, help edition, and publication date located on the legal notices page.

Feature list

Hardware features

5950 series hardware features

| Item | HPE FlexFabric 5950 32QSFP28 Switch JH321A HPE FlexFabric 5950 32QSFP28 TAA-compliant Switch JH322A | HPE FlexFabric 5950 48SFP28 8QSFP28 Switch JH402A | HPE FlexFabric 5950 4-slot Switch JH404A |
| --- | --- | --- | --- |
| Dimensions (H × W × D) | 43.6 × 440 × 540 mm (1.72 × 17.32 × 21.26 in) | 43.6×440×460 (1.72 × 17.32 × 18.11 in) | 88.1×440×660 (1.72 × 17.32 × 25.98 in) |
| Weight | ≤ 15.0 kg (33.07 lb) | ≤ 15kg (33.07 lb) | ≤ 18kg (39.68 lb) |
| Console ports | 1 |  |  |
| Mini USB(Console) | 1 |  |  |
| Management Ethernet ports | - 10M/100M/1000M Base-T copper port: 1 - SFP port: 1 |  |  |
| USB ports | 1 |  |  |
| SFP ports | N/A | 2 | 2 |
| SFP+ ports | 2 | N/A | N/A |
| SFP28 ports | N/A | 48 | N/A |
| QSFP28 ports | 32 | 8 | N/A |
| Expansion cards | N/A | N/A | LSWM18CQ LSWM116Q LSWM124TG2H LSWM124XGT2Q LSWM124XG2QFC LSWM18QC LSWM124XG2Q LSWM124XG2QL LSWM18CQMSEC |
| Fan tray slots | 6 | 5 | 2 |
| Fan trays | HPE X712 Bck(pwr)-Frt(prt) HV2 Fan Tray (JH389A) HPE X711 Frt(prt)-Bck(pwr) HV2 Fan Tray (JH388A) | HPE X712 Bck(pwr)-Frt(prt) HV2 Fan Tray (JH389A) HPE X711 Frt(prt)-Bck(pwr) HV2 Fan Tray (JH388A) | HPE 5930 4-slot B(pwr) F(prt) Fan Tray(JH185A) HPE 5930 4-slot F(prt) B(pwr) Fan Tray(JH186A) |
| Power module slots | 2 | 2 | 4 |
| Power modules | A58x0AF 650W AC Power Supply (JC680A) A58x0AF 650W DC Power Supply (JC681A) |  |  |
| AC-input voltage | Rated voltage: 100 VAC to 240 VAC @ 50 or 60 Hz Max voltage: 90 VAC to 264 VAC @ 47 to 63 Hz |  |  |
| DC-input voltage | Rated voltage: –48 VDC to –60 VDC Max voltage: –40 VDC to –72 VDC |  |  |
| Minimum power consumption | Single-AC: 280W Dual-AC: 286W Single-DC: 270W Dual-DC: 275W | Single-AC: 188W Dual-AC: 200W Single-DC: 180W Dual-DC: 195W | Dual-AC: 180W Triple AC: 197W Quadruple AC: 213W Dual-DC: 187W Triple DC: 202W Quadruple DC: 213W |
| Maximum power consumption | Single-AC: 586W Dual-AC: 590W Single-DC: 582W Dual-DC: 572W | Single-AC: 405W Dual-AC: 413W Single-DC: 400W Dual-DC: 408W | Dual-AC: 612W Triple AC: 618W Quadruple AC: 635W Dual-DC: 610W Triple DC: 612W Quadruple DC: 613W |
| Chassis leakage current compliance | UL60950-1 EN60950-1 IEC60950-1 GB4943 |  |  |
| Melting current of power supply fuse | 650W AC power supply: 10 A @ 250 VAC 650W DC power supply: 30 A @ 250 VDC |  |  |
| Operating temperature | 0°C to 45°C (32°F to 113°F) |  |  |
| Operating humidity | 10% to 90%, noncondensing |  |  |
| Fire resistance compliance | UL60950-1 EN60950-1 IEC60950-1 GB4943 |  |  |

Software features

Software features of the MSR 20/MSR 30/MSR 50 series

| Feature | HPE FlexFabric 5950 32QSFP28 Switch JH321A HPE FlexFabric 5950 32QSFP28 TAA-compliant Switch JH322A | HPE FlexFabric 5950 48SFP28 8QSFP28 Switch JH402A | HPE FlexFabric 5950 4-slot Switch JH404A |
| --- | --- | --- | --- |
| Full duplex Wire speed L2 switching capacity | 6400Gbps | 4000Gbps | 6400Gbps |
| Whole system Wire speed L2 switching Packet forwarding rate | 3169.6Mpps | 3169.6Mpps | 3169.6Mpps |
| Forwarding mode | Store-forward and cut-through |  |  |
| Link aggregation | - Aggregation of 10-GE ports - Aggregation of 25-GE ports - Aggregation of 40-GE ports - Aggregation of 100-GE ports - Static link aggregation - Dynamic link aggregation - When stacked, supports up to 256 aggregation groups, each supporting up to 32 ports |  |  |
| Flow control | IEEE 802.3x flow control and back pressure |  |  |
| Jumbo Frame | Supports maximum frame size of 9416 |  |  |
| MAC address table | - 136K MAC addresses - 8K static MAC addresses - Blackhole MAC addresses - MAC address learning limit on a port |  |  |
| VLAN | - Port-based VLANs (4094 VLANs) |  |  |
| QinQ | Supported |  |  |
| VLAN Mapping | 1;1 VLAN Mapping 1:2 VLAN Mapping 2:2 VLAN Mapping |  |  |
| ARP | - 136K entries - 1K static entries - Gratuitous ARP - Standard proxy ARP and local proxy ARP - ARP source suppression - ARP black hole - ARP detection (based on DHCP snooping entries/802.1x security entries/static IP-to-MAC bindings) |  |  |
| ND | - 68K entries - 1K static entries |  |  |
| VLAN virtual interface | 2K |  |  |
| DHCP | - DHCP client - DHCP snooping - DHCP relay agent - DHCP server - DHCPv6 server |  |  |
| UDP helper | - Supported |  |  |
| DNS | - Dynamic domain name resolution - Dynamic domain name resolution client - IPv4/IPv6 addresses |  |  |
| IPv4 routing | - 4K static routes - RIP(Routing Information Protocol) v1/2; up to 4K IPv4 routes - OSPF (Open Shortest Path First) v1/v2; up to 128K IPv4 routes - IS-IS(Intermediate System to Intermediate system); up to 128K IPv4 routes - BGP (Border Gateway Protocol); up to 128K IPv4 routes - Configurable maximum number of equal-cost routes; up to 2K equal-cost routes - Routing policy - VRRP - PBR |  |  |
| IPv6 routing | - 2K static routes - RIPng: Supports up to 2K IPv6 routes - OSPF v3: Supports up to 84K IPv6 routes - IPv6 IS-IS: Supports up to 84K IPv6 routes - BGP4+: Supports up to 84K IPv6 routes - Up to 2K ECMP routes; each ECMP route supports up to 128 next hops - Routing policy - VRRP - PBR |  |  |
| Multicast | - IGMP snooping - MLD snooping - IPv4 and IPv6 multicast VLAN IPv4 and IPv6 PIM snooping IGMP and MLD PIM and IPv6 PIM MSDP Multicast VPN |  |  |
| MPLS | - Static LSP - LDP - MPLS TE - Static CRLSP - RSVP - Tunnel policy - MPLS OAM - MPLS L3VPN - MCE |  |  |
| BFD | RIP/RIPng - OSPF/OSPFv3 - IS-IS/IPv6 IS-IS - BGP/BGP4+ - Static route/IPv6 static route |  |  |
| Tunnel | - IPv4 over IPv4 tunnel - IPv4 over IPv6 tunnel - IPv6 over IPv4 manual tunnel - IPv6 over IPv4 6to4 tunnel - IPv6 over IPv4 ISATAP tunnel - IPv6 over IPv6 tunnel - GRE tunnel |  |  |
| MSTP | - STP/RSTP/MSTP protocol - STP Root Guard - BPDU Guard |  |  |
| QoS/ACL | - Restriction of the rates at which a port sends and receives packets, with a granularity of 8 kbps. - Packet redirect - Committed access rate (CAR), with a granularity of traffic limit 8 kbps. - Eight output queues for each port - Flexible queue scheduling algorithms based on port and queue, including strict priority (SP), Weighted Deficit Round Robin (WDRR), Weighted Fair Queuing (WFQ), SP + WDRR, and SP + WFQ. - Remarking of 802.1p and DSCP priorities - Packet filtering at L2 (Layer 2) through L4 (Layer 4); flow classification based on source MAC address, destination MAC address, source IP (IPv4/IPv6) address, destination IP (IPv4/IPv6) address, port, protocol, and VLAN. - Time range - Weighted Random Early Detection (WRED) - Queue shaping - User profile - COPP - Explicit Congestion Notification (ECN) |  |  |
| Mirroring | - Flow mirroring - Port mirroring - Multiple mirror observing port |  |  |
| Remote mirroring | - Port remote mirroring (RSPAN) |  |  |
| Security | - Hierarchical management and password protection of users - AAA authentication - RADIUS - HWTACACS - SSH 2.0 - PKI SSL HTTPs Attack detection and prevention IP Source Guard uRPF Keychain |  |  |
| Traffic Management | sFlow Netstream(HPE FlexFabric 5950 48SFP28 8QSFP28 Switch and HPE FlexFabric 5950 4-slot Switch) |  |  |
| Loading and upgrading | - Loading and upgrading through XModem protocol - Loading and upgrading through FTP - Loading and upgrading through the trivial file transfer protocol (TFTP) |  |  |
| Management | - Configuration at the command line interface - Remote configuration through Telnet - Configuration through Console port - Simple network management protocol (SNMP) - System log - Hierarchical alarms - NTP - Power supply alarm function - Fan and temperature alarms |  |  |
| Maintenance | - Debugging information output - Ping and Tracert - Remote maintenance through Telnet - DLDP - File download and upload through USB port |  |  |

Upgrading software

The following information describes how to upgrade software while the router is operating normally or when the router cannot correctly start up.

System software file types

Software required for starting up the switch includes:

- Boot ROM image—A .bin file that comprises a basic section and an extended section. The basic section is the minimum code that bootstraps the system. The extended section enables hardware initialization and provides system management menus. You can use these menus to load software and the startup configuration file or manage files when the switch cannot correctly start up.

- Software images—Includes boot images and system images.

  - Boot image—A .bin file that contains the operating system kernel. It provides process management, memory management, file system management, and the emergency shell.

  - System image—A .bin file that contains the main application code required for device operation.  This includes device management, interface management, configuration management, and routing management.

The software images that have been loaded are called "current software images." The software images specified to load at next startup are called "startup software images."

These images might be released separately or as a whole in one .ipe package file. If an .ipe file is used, the system automatically decompresses the file, loads the .bin boot and system images in the file and sets them as startup software images. Typically, the Boot ROM and software images for this switch series are released in an .ipe file named main.ipe.

In addition to these images, HP irregularly releases patch packages for you to fix bugs without rebooting the switch. A patch package does not add new features or functions.

System startup process

Upon power-on, the Boot ROM image runs to initialize hardware and then the software images run to start up the entire system, as shown in Figure 1.

System startup process

Upgrade methods

You can upgrade system software by using one of the following methods:

| Upgrading method | Software types | Remarks |
| --- | --- | --- |
| Upgrading from the CLI | Software images | - You must reboot the switch to complete the upgrade. - This method can interrupt ongoing network services. |
|  | Patch packages | The upgrade does not interrupt ongoing services. Make sure the patch packages match the current software images. A patch package can fix bugs only for its matching software image version. |
| Upgrading from the Boot menu | - Boot ROM image - Software images | Use this method when the switch cannot correctly start up. CAUTION: Upgrading an IRF fabric from the CLI rather than the Boot menu. The Boot menu approach requires that you upgrade the member switches one by one and has larger impact on services than the CLI approach. |

The output in this document is for illustration only and might vary with software releases. For example, this document uses boot.bin and system.bin to represent boot and system image names, whereas the actual software image name format is chassis_software platform version_image type_release, for example, 5950-cmw710-boot-r6105.bin and 5950-cmw710-system-r6105.bin.

Upgrading from the CLI

This section uses a two-member IRF fabric as an example to describe how to upgrade software from the CLI. If you have more than two subordinate switches, repeat the steps for the subordinate switch to upgrade their software. If you are upgrading a standalone switch, ignore the steps for upgrading the subordinate switch.

Preparing for the upgrade

Before you upgrade software, complete the following tasks:

Log in to the IRF fabric through Telnet or the console port (details not shown).

Perform the display irf command in any view to identify the number of IRF members, each member switch’s role and IRF member ID.

<Sysname> display irf

MemberID   Role   Priority  CPU-Mac         Description

*+1      Master  5         0023-8927-afdc  ---

2      Standby 1         0023-8927-af43  ---

--------------------------------------------------

* indicates the device is the master.

+ indicates the device through which the user logs in.

The Bridge MAC of the IRF is: 0023-8927-afdb

Auto upgrade                : no

Mac persistent              : 6 min

Domain ID                   : 0

Perform the dir command in user view to identify the free storage space of each member switch.

Identify the free Flash space of the master switch.

<Sysname> dir

Directory of flash:

0      -rw-       41424  Aug 23 2013 00:33:57     startup.mdb

1      -rw-        3792  Aug 23 2013 00:33:56     startup.cfg

2      -rw-    53555200  Aug 23 2013 16:04:08     system.bin

3      drw-           -  Aug 23 2013 00:03:07     seclog

4      drw-           -  Aug 23 2013 00:03:07     diagfile

5      drw-           -  Aug 23 2013 00:03:07     logfile

6      -rw-     9959424  Aug 23 2013 16:04:08     boot.bin

7      -rw-     9012224  Aug 21 2013 09:54:27     backup.bin

1048576 KB total (977704 KB free)

Identify the free Flash space of each subordinate switch, for example, switch 2.

<Sysname> dir slot2#flash:/

Directory of slot2#flash:/

0      -rw-       41424  Aug 23 2013 00:33:57     startup.mdb

1      -rw-        3792  Aug 23 2013 00:33:56     startup.cfg

2      -rw-    93871104  Aug 23 2013 16:00:08     system.bin

3      drw-           -  Aug 23 2013 00:03:07     seclog

4      drw-           -  Aug 23 2013 00:03:07     diagfile

5      drw-           -  Aug 23 2013 00:03:07     logfile

6      -rw-    13611008  Aug 23 2013 15:59:00     boot.bin

7      -rw-     9012224  Aug 21 2013 09:54:27     backup.bin

1048576 KB total (934767 KB free)

Compare the free Flash space of each member switch with the size of the software file to load. If the space is sufficient, start the upgrade process. If not, go to the next step.

Delete obsolete files in Flash to free space:

|  | CAUTION: - To avoid data loss, do not delete the current configuration file. For information about the current configuration file, perform the display startup command. Hewlett Packard Enterprise recommends that you preferentially delete obsolete software images. To avoid inadvertent delete of the current software images, perform the display boot-loader command in any view to identify them. - The delete /unreserved file command deletes a file permanently and the action cannot be undone. - The delete file command moves a file to the recycle bin and the file still occupies storage space. To permanently delete the file from the recycle bin, first perform the undelete command to restore the file and then perform the delete /unreserved file command. |
| --- | --- |

Delete obsolete files from the Flash memory of the master switch.

<Sysname> delete /unreserved flash:/backup.bin

The file cannot be restored. Delete flash:/backup.bin?[Y/N]:y

Deleting the file permanently will take a long time. Please wait...

Deleting file flash:/backup.bin...Done.

Delete obsolete files from the Flash memory of the subordinate switch.

<Sysname> delete /unreserved slot2#flash:/backup.bin

The file cannot be restored. Delete slot2#flash:/backup.bin?[Y/N]:y

Deleting the file permanently will take a long time. Please wait...

Deleting file slot2#flash:/backup.bin...Done.

Downloading software to the master switch

Before you start upgrading software images or patch packages, make sure you have downloaded the upgrading software files to the root directory in Flash memory. This section describes downloading an .ipe software file as an example.

The following are ways to download, upload, or copy files to the master switch:

- FTP download from a server

- FTP upload from a client

- TFTP download from a server

- Copying files from a USB flash drive

Prerequisites

If FTP or TFTP is used, the IRF fabric and the PC working as the FTP/TFTP server or FTP client can reach each other.

Prepare the FTP server or TFTP server program yourself for the PC. The switch series does not come with these software programs.

FTP download from a server

You can use the switch as an FTP client to download files from an FTP server.

To download a file from an FTP server, for example, the server at 10.10.110.1:

Run an FTP server program on the server, configure an FTP username and password, specify the working directory and copy the file, for example, newest.ipe, to the directory.

Perform the ftp command in user view on the IRF fabric to access the FTP server.

<Sysname> ftp 10.10.110.1

Trying 10.10.110.1...

Press CTRL+K to abort

Connected to 10.10.110.1

220 FTP service ready.

User(10.10.110.1:(none)):username

331 Password required for username.

Password:

230 User logged in

Enable the binary transfer mode.

ftp> binary

200 Type set to I.

Perform the get command in FTP client view to download the file from the FTP server.

ftp> get newest.ipe

227 Entering Passive Mode (10,10,110,1,17,97).

125 BINARY mode data connection already open, transfer starting for /newest.ipe

226 Transfer complete.

63521792 bytes received in 35 seconds (896. 0 kbyte/s)

ftp> bye

221 Server closing.

FTP upload from a client

You can use the IRF fabric as an FTP server and upload files from a client to the IRF fabric.

To FTP upload a file from a client:

On the IRF fabric:

Enable FTP server.

<Sysname> system-view

[Sysname] ftp server enable

Add a local FTP user account, set its password and access service type, and assign it to the user role network-admin for uploading file to the working directory of the server.

[Sysname] local-user abc

[Sysname-luser-manage-abc] password simple pwd

[Sysname-luser-manage-abc] service-type ftp

[Sysname-luser-manage-abc] authorization-attribute user-role network-admin

[Sysname-luser-manage-abc] quit

[Sysname] quit

On the PC:

FTP to the IRF fabric (the FTP server).

c:\> ftp 1.1.1.1

Connected to 1.1.1.1.

220 FTP service ready.

User(1.1.1.1:(none)):abc

331 Password required for abc.

Password:

230 User logged in.

Enable the binary file transfer mode.

ftp> binary

200 TYPE is now 8-bit binary.

Upload the file (for example, newest.ipe) to the root directory in the Flash memory of the master switch.

ftp> put newest.ipe

200 PORT command successful

150 Connecting to port 10002

226 File successfully transferred

ftp: 63521792 bytes sent in 64.58 secs (497.60 Kbytes/sec).

TFTP download from a server

To download a file from a TFTP server, for example, the server at 10.10.110.1:

Run a TFTP server program on the server, specify the working directory, and copy the file, for example, newest.ipe, to the directory.

On the IRF fabric, perform the tftp command in user view to download the file to the root directory in the Flash memory of the master switch.

<Sysname> tftp 10.10.110.1 get newest.ipe

Press CTRL+C to abort.

% Total    % Received % Xferd  Average Speed   Time    Time     Time  Current

Dload  Upload   Total   Spent    Left  Speed

100 60.5M    0 60.5M    0     0   143k      0 --:--:--  0:03:38 --:--:--  142k

Copying files from a USB flash drive

Every 5950 switch provides a USB port for you to copy files from a USB flash drive.

To copy a file from a USB flash drive to the Flash memory of the master switch:

Plug the USB flash drive in the USB port of the switch.

Copy the file (for example, newest.ipe) to the Flash memory of the switch.

<Sysname> cd usba:

<Sysname> copy usba:/newest.ipe newest.ipe

Copy usba:/newest.ipe to flash:/newest.ipe?[Y/N]:y

Start to copy usba:/newest.ipe to flash:/newest.ipe... Done.

Upgrading the software images

To upgrade the software images:

Specify the upgrading image file (newest.ipe in this example) used at the next startup for the master switch, and assign the M attribute to the boot and system images in the file.

<Sysname> boot-loader file flash:/newest.ipe slot 1 main

Verifying the file flash:/newest.ipe on slot 1....Done..

Images in IPE:

boot.bin

system.bin

Decompressing file boot.bin to flash:/boot.bin................Done.

Decompressing file system.bin to flash:/system.bin.............................Done.

Decompression completed.

You are recommended to delete the .ipe file after you set startup software images for all slots.

Do you want to delete flash:/newest.ipe now? [Y/N]:n

Verifying the file flash:/boot.bin on slot 1...Done.

Verifying the file flash:/system.bin on slot 1...Done.

The images that have passed all examinations will be used as the backup startup software images at the next reboot on slot 1

Specify the upgrading image file used at next startup for the subordinate switch, and assign the M attribute to the boot and system images in the file. (As a result, the subordinate switch automatically copies the file to the root directory in its Flash memory.)

<Sysname> boot-loader file flash:/newest.ipe slot 2 main

Verifying the file flash:/newest.ipe on slot 2....Done..

Images in IPE:

boot.bin

system.bin

Decompressing file boot.bin to flash:/boot.bin................Done.

Decompressing file system.bin to flash:/system.bin.............................Done.

Decompression completed.

You are recommended to delete the .ipe file after you set startup software images for all slots.

Do you want to delete flash:/newest.ipe now? [Y/N]:n

Verifying the file flash:/boot.bin on slot 2...Done.

Verifying the file flash:/system.bin on slot 2...Done.

The images that have passed all examinations will be used as the backup startup software images at the next reboot on slot 2

(Optional) If the IRF fabric size has a lot of members, enable the software auto-update function.

<Sysname> system-view

[Sysname] irf auto-update enable

[Sysname] quit

Software auto-update is typically used for synchronizing the software images of the master switch to new member switches when you expand the IRF fabric. This function enables a subordinate switch to compare its main startup software image version with that of the IRF master. If the versions are different, the subordinate switch automatically downloads the current software images from the master, sets the downloaded images as the main software images at the next reboot, and automatically reboots with the new images to re-join the IRF fabric. In this upgrade process, the function avoids the failure of assign all the subordinate switch the same main software image file as the master switch causing an upgrade failure.

Save the current configuration in any view to prevent data loss.

<Sysname> save

The current configuration will be written to the device. Are you sure? [Y/N]:y

Please input the file name(*.cfg)[flash:/startup.cfg]

(To leave the existing filename unchanged, press the enter key):

flash:/startup.cfg exists, overwrite? [Y/N]:y

Validating file. Please wait.................

Saved the current configuration to mainboard device successfully.

Slot 2:

Save next configuration file successfully.

Reboot the IRF fabric to complete the upgrade.

<Sysname> reboot

Start to check configuration with next startup configuration file, please wait.

........DONE!

This command will reboot the device. Continue? [Y/N]:y

Now rebooting, please wait...

The system automatically loads the .bin boot and system images in the .ipe file and sets them as the startup software images.

Perform the display version command in any view to verify that the current main software images have been updated (details not shown).

|  | NOTE: The system automatically checks the compatibility of the Boot ROM image and the boot and system images during the reboot. If you are prompted that the Boot ROM image in the upgrading image file is different than the current Boot ROM image, upgrade both the basic and extended sections of the Boot ROM image for compatibility. If you choose to not upgrade the Boot ROM image, the system will ask for an upgrade at the next reboot performed by powering on the switch or rebooting from the CLI (promptly or as scheduled). If you fail to make any choice in the required time, the system upgrades the entire Boot ROM image. |
| --- | --- |

Installing a patch package

To install a patch package, for example, system-patch.bin:

Activate the patch package on the master switch and the subordinate switch.

<Sysname> install activate patch flash:/system-patch.bin slot 1

<Sysname> install activate patch flash:/system-patch.bin slot 2

Verify that the patch package has been activated.

<Sysname> display install active

Active packages on slot 1:

flash:/boot.bin

flash:/system.bin

flash:/system-patch.bin

Active packages on slot 2:

flash:/boot.bin

flash:/system.bin

flash:/system-patch.bin

Commit the installation so the patch package continues to take effect after a reboot.

<Sysname> install commit

Verify that the patch package installation has been committed.

<Sysname> display install committed

Committed packages on slot 1:

flash:/boot.bin

flash:/system.bin

flash:/system-patch.bin

Committed packages on slot 2:

flash:/boot.bin

flash:/system.bin

flash:/system-patch.bin

For more information about installing patch packages, see HP FlexFabric 5950 Switch Series Fundamentals Configuration Guide.

Upgrading from the Boot menu

You can upgrade the Boot ROM image and software images but not patch packages from the Boot menu.

In this approach, you must access the Boot menu of each member switch to upgrade their software one by one. If you are upgrading software images for an IRF fabric, using the CLI is a better choice.

The following sections describe the methods of upgrading software images:

- Using TFTP to upgrade software images through the management Ethernet port

- Using FTP to upgrade software through the management Ethernet port

- Using XMODEM to upgrade software through the console port

The following sections describe the methods of upgrading Boot ROM images:

- Using TFTP to upgrade Boot ROM through the management Ethernet port

- Using FTP to upgrade Boot ROM through the management Ethernet port

- Using XMODEM to upgrade Boot ROM through the console port

|  | TIP: Upgrading through an Ethernet port is faster than through the console port. |
| --- | --- |

Prerequisites

Make sure that the prerequisites are met before you start upgrading software from the Boot menu.

Upgrading environment

Use a console cable to connect the console terminal, for example, a PC, to the console port on the switch. Run a terminal emulator program on the console terminal and set the following terminal settings:

- Bits per second—9,600

- Data bits—8

- Parity—None

- Stop bits—1

- Flow control—None

- Emulation—VT100

TFTP/FTP download

To use TFTP or FTP:

- Run a TFTP or FTP server program on the file server or the console terminal.

- Copy the upgrade file to the file server.

- Correctly set the working directory on the TFTP or FTP server.

- Make sure that the file server and the switch can reach each other.

Storage space

Make sure that sufficient space is available for the upgrading software file. If no sufficient space is available, delete obsolete files as described in "Managing files from the Boot menu."

Upgrading time

Make sure that the upgrade has minimal impact on the network services. During the upgrade, the switch cannot provide any services.

Accessing the Boot menu

Power on the switch (for example, an HP FF 5950-32QSFP28 Switch), and you can see the following information:

Starting......

Press Ctrl+D to access BASIC BOOT MENU

Press Ctrl+T to start memory test

********************************************************************************

*                                                                              *

*               HPE FF 5950 32QSFP28 Switch BOOTROM, Version 106               *

*                                                                              *

********************************************************************************

Copyright (c) 2010-2016 Hewlett Packard Enterprise Development LP

Creation Date       : Apr  7 2016

CPU Type            : P2020

CPU L1 Cache        : 32KB

CPU Clock Speed     : 1200MHz

Memory Type         : DDR3 SDRAM

Memory Size         : 4096MB

Memory Speed        : 800MHz

Flash Size          : 1024MB

CPLD Version        : 1.0

PCB Version         : Ver.B

Mac Address         : 6000D246D4F8

Press Ctrl+B to access EXTENDED BOOT MENU...0

Press one of the shortcut key combinations at prompt.

Shortcut keys

| Shortcut keys | Prompt message | Function | Remarks |
| --- | --- | --- | --- |
| Ctrl+B | Press Ctrl+B to enter Extended Boot menu... | Accesses the extended Boot menu. | Press the keys within 1 second (in fast startup mode) or 5 seconds (in full startup mode) after the message appears. You can upgrade and manage system software and Boot ROM from this menu. |
| Ctrl+D | Press Ctrl+D to access BASIC BOOT MENU | Accesses the basic Boot menu. | Press the keys within 1 seconds after the message appears. You can upgrade Boot ROM or access the extended Boot ROM segment from this menu. |
| Ctrl+T | Press Ctrl+T to start heavy memory test | Performs a RAM pressure test. | Press the keys within 1 second after the message appears. |

Accessing the basic Boot menu

If the extended Boot ROM segment has corrupted, you can repair or upgrade it from the basic Boot menu.

Press Ctrl+D within 1 seconds after the "Press Ctrl+D to access BASIC BOOT MENU" prompt message appears. If you fail to do this within the time limit, the system starts to run the extended Boot ROM segment.

********************************************************************************

*                                                                              *

*                             BASIC BOOTROM, Version 101                       *

*                                                                              *

********************************************************************************

BASIC BOOT MENU

1. Update full BootRom

2. Update extended BootRom

3. Update basic BootRom

4. Boot extended BootRom

0. Reboot

Ctrl+U: Access BASIC ASSISTANT MENU

Enter your choice(0-4):

Basic Boot ROM menu options

| Option | Task |
| --- | --- |
| 1. Update full BootRom | Update the entire Boot ROM, including the basic segment and the extended segment. To do so, you must use XMODEM and the console port. For more information, see Using XMODEM to upgrade Boot ROM through the console port. |
| 2. Update extended BootRom | Update the extended Boot ROM segment. To do so, you must use XMODEM and the console port. For more information, see Using XMODEM to upgrade Boot ROM through the console port. |
| 3. Update basic BootRom | Update the basic Boot ROM segment. To do so, you must use XMODEM and the console port. For more information, see Using XMODEM to upgrade Boot ROM through the console port. |
| 4. Boot extended BootRom | Access the extended Boot ROM segment. For more information, see Accessing the extended Boot menu. |
| 0. Reboot | Reboot the switch. |
| Ctrl+U: Access BASIC ASSISTANT MENU | Press Ctrl + U to access the BASIC ASSISTANT menu (see Table 9). |

BASIC ASSISTANT menu options

| Option | Task |
| --- | --- |
| 1. RAM Test | Perform a RAM self-test. |
| 0. Return to boot menu | Return to the basic Boot menu. |

Accessing the extended Boot menu

Press Ctrl+B within 1 second (in fast startup mode) or 5 seconds (in full startup mode) after the "Press Ctrl-B to enter Extended Boot menu..." prompt message appears. If you fail to do this, the system starts decompressing the system software.

Alternatively, you can enter 4 in the basic Boot menu to access the extended Boot menu.

The "Password recovery capability is enabled." or "Password recovery capability is disabled." message appears, followed by the extended Boot menu. Availability of some menu options depends on the state of password recovery capability (see Table 10). For more information about password recovery capability, see HP FlexFabric 5950 Switch Series Fundamentals Configuration Guide.

Password recovery capability is enabled.

EXTENDED BOOT MENU

1. Download image to flash

2. Select image to boot

3. Display all files in flash

4. Delete file from flash

5. Restore to factory default configuration

6. Enter BootRom upgrade menu

7. Skip current system configuration

8. Set switch startup mode

0. Reboot

Ctrl+Z: Access EXTENDED ASSISTANT MENU

Ctrl+F: Format file system

Ctrl+P: Change authentication for console login

Ctrl+R: Download image to SDRAM and run

Enter your choice(0-8):

Extended Boot ROM menu options

| Option | Tasks |
| --- | --- |
| 1. Download image to flash | Download a software image file to the flash. |
| 2. Select image to boot | - Specify the main and backup software image file for the next startup. - Specify the main and backup configuration files for the next startup. This task can be performed only if password recovery capability is enabled. |
| 3. Display all files in flash | Display files on the flash. |
| 4. Delete file from flash | Delete files to free storage space. |
| 5. Restore to factory default configuration | Delete the current next-startup configuration files and restore the factory-default configuration. This option is available only if password recovery capability is disabled. |
| 6. Enter BootRom upgrade menu | Access the Boot ROM upgrade menu. |
| 7. Skip current system configuration | Start the switch without loading any configuration file. This is a one-time operation and takes effect only for the first system boot or reboot after you choose this option. This option is available only if password recovery capability is enabled. |
| 8. Set switch startup mode | Set the startup mode to fast startup mode or full startup mode. |
| 0. Reboot | Reboot the switch. |
| Ctrl+F: Format file system | Format the current storage medium. |
| Ctrl+P: Change authentication for console login | Skip the authentication for console login. This is a one-time operation and takes effect only for the first system boot or reboot after you choose this option. This option is available only if password recovery capability is enabled. |
| Ctrl+R: Download image to SDRAM and run | Download a system software image and start the switch with the image. This option is available only if password recovery capability is enabled. |
| Ctrl+Z: Access EXTENDED ASSISTANT MENU | Access the EXTENDED ASSISTANT MENU. For options in the menu, see Table 11. |

EXTENDED ASSISTANT menu options

| Option | Task |
| --- | --- |
| 1. Display Memory | Display data in the memory. |
| 2. Search Memory | Search the memory for a specific data segment. |
| 0. Return to boot menu | Return to the extended Boot ROM menu. |

Using TFTP to upgrade software images through the management Ethernet port

Enter 1 in the Boot menu to access the file transfer protocol submenu.

1. Set TFTP protocol parameters

2. Set FTP protocol parameters

3. Set XMODEM protocol parameters

0. Return to boot menu

Enter your choice(0-3):

Enter 1 to set the TFTP parameters.

Load File Name      :update.ipe

Server IP Address   :192.168.0.3

Local IP Address    :192.168.0.2

Subnet Mask         :255.255.255.0

Gateway IP Address  :0.0.0.0

TFTP parameter description

| Item | Description |
| --- | --- |
| Load File Name | Name of the file to download (for example, update.ipe). |
| Server IP Address | IP address of the TFTP server (for example, 192.168.0.3). |
| Local IP Address | IP address of the switch (for example, 192.168.0.2). |
| Subnet Mask | Subnet mask of the switch (for example, 255.255.255.0). |
| Gateway IP Address | IP address of the gateway (in this example, no gateway is required because the server and the switch are on the same subnet). |

|  | NOTE: - To use the default setting for a field, press Enter without entering any value. - If the switch and the server are on different subnets, you must specify a gateway address for the switch. |
| --- | --- |

Enter all required parameters, and enter Y to confirm the settings. The following prompt appears:

Are you sure to download file to flash? Yes or No (Y/N):Y

Enter Y to start downloading the image file. To return to the Boot menu, enter N.

Loading.........................................................................

................................................................................

................................................................................

................................................................Done!

Enter the M (main), B (backup), or N (none) attribute for the images. In this example, assign the main attribute to the images.

Please input the file attribute (Main/Backup/None) M

Image file boot.bin is self-decompressing...

Free space: 534980608 bytes

Writing flash...................................................................

................................................................................

...................................................................Done!

Image file system.bin is self-decompressing...

Free space: 525981696 bytes

Writing flash...................................................................

................................................................................

................................................................................

................................................................................

................................................................................

................................................................................

.......................................................................Done!

|  | NOTE: - The switch always attempts to boot first with the main images, and if the attempt fails, for example, because the main images are not available, the switch tries to boot with the backup images. An image with the none attribute is just stored in Flash memory for backup and you must change its attribute to make it usable at reboot. - If an image with the same attribute as the image you are loading is already in Flash memory, the attribute of the old image changes to none after the new image becomes valid. |
| --- | --- |

Enter 0 in the Boot menu to reboot the switch with the new software images.

EXTENDED BOOT MENU

1. Download image to flash

2. Select image to boot

3. Display all files in flash

4. Delete file from flash

5. Restore to factory default configuration

6. Enter BootRom upgrade menu

7. Skip current system configuration

8. Set switch startup mode

0. Reboot

Ctrl+Z: Access EXTENDED ASSISTANT MENU

Ctrl+F: Format file system

Ctrl+P: Change authentication for console login

Ctrl+R: Download image to SDRAM and run

Enter your choice(0-8):0

Using FTP to upgrade software through the management Ethernet port

Enter 1 in the Boot menu to access the file transfer protocol submenu.

1. Set TFTP protocol parameters

2. Set FTP protocol parameters

3. Set XMODEM protocol parameters

0. Return to boot menu

Enter your choice(0-3):

Enter 2 to set the FTP parameters.

Load File Name      :update.ipe

Server IP Address   :192.168.0.3

Local IP Address    :192.168.0.2

Subnet Mask         :255.255.255.0

Gateway IP Address  :0.0.0.0

FTP User Name       :switch

FTP User Password   :***

FTP parameter description

| Item | Description |
| --- | --- |
| Load File Name | Name of the file to download (for example, update.ipe). |
| Server IP Address | IP address of the FTP server (for example, 192.168.0.3). |
| Local IP Address | IP address of the switch (for example, 192.168.0.2). |
| Subnet Mask | Subnet mask of the switch (for example, 255.255.255.0). |
| Gateway IP Address | IP address of the gateway (in this example, no gateway is required because the server and the switch are on the same subnet). |
| FTP User Name | Username for accessing the FTP server, which must be the same as configured on the FTP server. |
| FTP User Password | Password for accessing the FTP server, which must be the same as configured on the FTP server. |

|  | NOTE: - To use the default setting for a field, press Enter without entering any value. - If the switch and the server are on different subnets, you must specify a gateway address for the switch. |
| --- | --- |

Enter all required parameters, and enter Y to confirm the settings. The following prompt appears:

Are you sure to download file to flash? Yes or No (Y/N):Y

Enter Y to start downloading the image file. To return to the Boot menu, enter N.

Loading.........................................................................

................................................................................

................................................................................

................................................................Done!

Enter the M (main), B (backup), or N (none) attribute for the images. In this example, assign the main attribute to the images.

Please input the file attribute (Main/Backup/None) M

Image file boot.bin is self-decompressing...

Free space: 534980608 bytes

Writing flash...................................................................

................................................................................

...................................................................Done!

Image file system.bin is self-decompressing...

Free space: 525981696 bytes

Writing flash...................................................................

................................................................................

................................................................................

................................................................................

................................................................................

................................................................................

.......................................................................Done!

EXTENDED BOOT MENU

1. Download image to flash

2. Select image to boot

3. Display all files in flash

4. Delete file from flash

5. Restore to factory default configuration

6. Enter BootRom upgrade menu

7. Skip current system configuration

8. Set switch startup mode

0. Reboot

Ctrl+Z: Access EXTENDED ASSISTANT MENU

Ctrl+F: Format file system

Ctrl+P: Change authentication for console login

Ctrl+R: Download image to SDRAM and run

Enter your choice(0-8):0

|  | NOTE: - The switch always attempts to boot first with the main images, and if the attempt fails, for example, because the main images not available, the switch tries to boot with the backup images. An image with the none attribute is just stored in Flash memory for backup and you must change its attribute to make it usable at reboot. - If an image with the same attribute as the image you are loading is already in Flash memory, the attribute of the old image changes to none after the new image becomes valid. |
| --- | --- |

Enter 0 in the Boot menu to reboot the switch with the new software images.

Using XMODEM to upgrade software through the console port

XMODEM download through the console port is slower than TFTP or FTP download through the management Ethernet port. To save time, use the management Ethernet port as long as possible.

Enter 1 in the Boot menu to access the file transfer protocol submenu.

1. Set TFTP protocol parameters

2. Set FTP protocol parameters

3. Set XMODEM protocol parameters

0. Return to boot menu

Enter your choice(0-3):

Enter 3 to set the XMODEM download baud rate.

Please select your download baudrate:

1.* 9600

2.  19200

3.  38400

4.  57600

5.  115200

0.  Return to boot menu

Enter your choice(0-5):5

Select an appropriate download rate, for example, enter 5 to select 115200 bps.

Download baudrate is 115200 bps

Please change the terminal's baudrate to 115200 bps and select XMODEM protocol

Press enter key when ready

Set the serial port on the terminal to use the same baud rate and protocol as the console port. If you select 9600 bps as the download rate for the console port, skip this task.

Select Call > Disconnect in the HyperTerminal window to disconnect the terminal from the switch.

Disconnecting the terminal from the switch

Select File > Properties, and in the Properties dialog box, click Configure.

Properties dialog box

Select 115200 from the Bits per second list and click OK.

Modifying the baud rate

Select Call > Call to reestablish the connection.

Reestablishing the connection

Press Enter. The following prompt appears:

Are you sure to download file to flash? Yes or No (Y/N):Y

Enter Y to start downloading the file. (To return to the Boot menu, enter N.)

Now please start transfer file with XMODEM protocol

If you want to exit, Press <Ctrl+X>

Loading ...CCCCCCCCCCCCCCCCCCCCCCCCC

Select Transfer > Send File in the HyperTerminal window.

Transfer menu

In the dialog box that appears, click Browse to select the source file, and select Xmodem from the Protocol list.

File transmission dialog box

Click Send. The following dialog box appears:

File transfer progress

Enter the M (main), B (backup), or N (none) attribute for the images. In this example, assign the main attribute to the images.

Please input the file attribute (Main/Backup/None) m

The boot.bin image is self-decompressing...

At the Load File name prompt, enter a name for the Boot image to be saved to Flash memory.

Load File name  : default_file boot-update.bin

Free space: 470519808 bytes

Writing flash...................................................................

.............Done!

The system-update.bin image is self-decompressing...

At the Load File name prompt, enter a name for the system image to be saved to Flash memory.

Load File name  : default_file system-update.bin

Free space: 461522944 bytes

Writing flash...................................................................

.............Done!

Your baudrate should be set to 9600 bps again!

Press enter key when ready

|  | NOTE: - The switch always attempts to boot first with the main images, and if the attempt fails, for example, because the main images not available, the switch tries to boot with the backup images. An image with the none attribute is just stored in Flash memory for backup and you must change its attribute to make it usable at reboot. - If an image with the same attribute as the image you are loading is already in Flash memory, the attribute of the old image changes to none after the new image becomes valid. |
| --- | --- |

If the baud rate of the HyperTerminal is not 9600 bps, restore it to 9600 bps. If the baud rate is 9600 bps, skip this step.

To access the switch through the console port after a reboot, you must perform this step, because the console port rate reverts to 9600 bps at a reboot.

Press Enter to access the Boot menu.

EXTENDED BOOT MENU

1. Download image to flash

2. Select image to boot

3. Display all files in flash

4. Delete file from flash

5. Restore to factory default configuration

6. Enter BootRom upgrade menu

7. Skip current system configuration

8. Set switch startup mode

0. Reboot

Ctrl+Z: Access EXTENDED ASSISTANT MENU

Ctrl+F: Format file system

Ctrl+P: Change authentication for console login

Ctrl+R: Download image to SDRAM and run

Enter your choice(0-8):0

Enter 0 to reboot the system with the new software images.

Using TFTP to upgrade Boot ROM through the management Ethernet port

Enter 6 in the Boot menu to access the Boot ROM update menu.

1. Update full BootRom

2. Update extended BootRom

3. Update basic BootRom

0. Return to boot menu

Enter your choice(0-3):

Enter 1 in the Boot ROM update menu to upgrade the full Boot ROM.

The file transfer protocol submenu appears:

1. Set TFTP protocol parameters

2. Set FTP protocol parameters

3. Set XMODEM protocol parameters

0. Return to boot menu

Enter your choice(0-3):

Enter 1 to set the TFTP parameters.

Load File Name      :update.btm

Server IP Address   :192.168.0.3

Local IP Address    :192.168.0.2

Subnet Mask         :255.255.255.0

Gateway IP Address  :0.0.0.0

TFTP parameter description

| Item | Description |
| --- | --- |
| Load File Name | Name of the file to download (for example, update.btm). |
| Server IP Address | IP address of the TFTP server (for example, 192.168.0.3). |
| Local IP Address | IP address of the switch (for example, 192.168.0.2). |
| Subnet Mask | Subnet mask of the switch (for example, 255.255.255.0). |
| Gateway IP Address | IP address of the gateway (in this example, no gateway is required because the server and the switch are on the same subnet). |

|  | NOTE: - To use the default setting for a field, press Enter without entering any value. - If the switch and the server are on different subnets, you must specify a gateway address for the switch. |
| --- | --- |

Enter all required parameters and press Enter to start downloading the file.

Loading.................................................Done!

Enter Y at the prompt to upgrade the basic Boot ROM section.

Will you Update Basic BootRom? (Y/N):Y

Updating Basic BootRom...........Done.

Enter Y at the prompt to upgrade the extended Boot ROM section.

Updating extended BootRom? (Y/N):Y

Updating extended BootRom.........Done.

Enter 0 in the Boot ROM update menu to return to the Boot menu.

1. Update full BootRom

2. Update extended BootRom

3. Update basic BootRom

0. Return to boot menu

Enter your choice(0-3):

Enter 0 in the Boot menu to reboot the switch with the new Boot ROM image.

Using FTP to upgrade Boot ROM through the management Ethernet port

Enter 6 in the Boot menu to access the Boot ROM update menu.

1. Update full BootRom

2. Update extended BootRom

3. Update basic BootRom

0. Return to boot menu

Enter your choice(0-3):

Enter 1 in the Boot ROM update menu to upgrade the full Boot ROM.

The file transfer protocol submenu appears:

1. Set TFTP protocol parameters

2. Set FTP protocol parameters

3. Set XMODEM protocol parameters

0. Return to boot menu

Enter your choice(0-3):

Enter 2 to set the FTP parameters.

Load File Name     :update.btm

Server IP Address  :192.168.0.3

Local IP Address   :192.168.0.2

Subnet Mask        :255.255.255.0

Gateway IP Address :0.0.0.0

FTP User Name      :switch

FTP User Password  :***

FTP parameter description

| Item | Description |
| --- | --- |
| Load File Name | Name of the file to download (for example, update.btm). |
| Server IP Address | IP address of the FTP server (for example, 192.168.0.3). |
| Local IP Address | IP address of the switch (for example, 192.168.0.2). |
| Subnet Mask | Subnet mask of the switch (for example, 255.255.255.0). |
| Gateway IP Address | IP address of the gateway (in this example, no gateway is required because the server and the switch are on the same subnet). |
| FTP User Name | Username for accessing the FTP server, which must be the same as configured on the FTP server. |
| FTP User Password | Password for accessing the FTP server, which must be the same as configured on the FTP server. |

|  | NOTE: - To use the default setting for a field, press Enter without entering any value. - If the switch and the server are on different subnets, you must specify a gateway address for the switch. |
| --- | --- |

Enter all required parameters and press Enter to start downloading the file.

Loading.................................................Done!

Enter Y at the prompt to upgrade the basic Boot ROM section.

Will you Update Basic BootRom? (Y/N):Y

Updating Basic BootRom...........Done.

Enter Y at the prompt to upgrade the extended Boot ROM section.

Updating extended BootRom? (Y/N):Y

Updating extended BootRom.........Done.

Enter 0 in the Boot ROM update menu to return to the Boot menu.

1. Update full BootRom

2. Update extended BootRom

3. Update basic BootRom

0. Return to boot menu

Enter your choice(0-3):

Enter 0 in the Boot menu to reboot the switch with the new Boot ROM image.

Using XMODEM to upgrade Boot ROM through the console port

XMODEM download through the console port is slower than TFTP or FTP download through the management Ethernet port. To save time, use the management Ethernet port as long as possible.

Enter 6 in the Boot menu to access the Boot ROM update menu.

1. Update full BootRom

2. Update extended BootRom

3. Update basic BootRom

0. Return to boot menu

Enter your choice(0-3):

Enter 1 in the Boot ROM update menu to upgrade the full Boot ROM.

The file transfer protocol submenu appears:

1. Set TFTP protocol parameters

2. Set FTP protocol parameters

3. Set XMODEM protocol parameters

0. Return to boot menu

Enter your choice(0-3):

Enter 3 to set the XMODEM download baud rate.

Please select your download baudrate:

1.* 9600

2.  19200

3.  38400

4.  57600

5.  115200

0.  Return to boot menu

Enter your choice(0-5):5

Select an appropriate download rate, for example, enter 5 to select 115200 bps.

Download baudrate is 115200 bps

Please change the terminal's baudrate to 115200 bps and select XMODEM protocol

Press enter key when ready

Set the serial port on the terminal to use the same baud rate and protocol as the console port. If you select 9600 bps as the download rate for the console port, skip this task.

Select Call > Disconnect in the HyperTerminal window to disconnect the terminal from the switch.

Disconnecting the terminal from the switch

Select File > Properties, and in the Properties dialog box, click Configure.

Properties dialog box

Select 115200 from the Bits per second list and click OK.

Modifying the baud rate

Select Call > Call to reestablish the connection.

Reestablishing the connection

Press Enter to start downloading the file.

Now please start transfer file with XMODEM protocol

If you want to exit, Press <Ctrl+X>

Loading ...CCCCCCCCCCCCCCCCCCCCCCCCC

Select Transfer > Send File in the HyperTerminal window.

Transfer menu

In the dialog box that appears, click Browse to select the source file, and select Xmodem from the Protocol list.

File transmission dialog box

Click Send. The following dialog box appears:

File transfer progress

Enter Y at the prompt to upgrade the basic Boot ROM section.

Loading ...CCCCCCCCCCCCCC  ...Done!

Will you Update Basic BootRom? (Y/N):Y

Updating Basic BootRom...........Done.

Enter Y at the prompt to upgrade the extended Boot ROM section.

Updating extended BootRom? (Y/N):Y

Updating extended BootRom.........Done.

If the baud rate of the HyperTerminal is not 9600 bps, restore it to 9600 bps at the prompt. If the baud rate is 9600 bps, skip this step.

Please change the terminal's baudrate to 9600 bps, press ENTER when ready.

To access the switch through the console port after a reboot, you must perform this step, because the console port rate reverts to 9600 bps at a reboot.

Press Enter to access the Boot ROM update menu.

Enter 0 in the Boot ROM update menu to return to the Boot menu.

1. Update full BootRom

2. Update extended BootRom

3. Update basic BootRom

0. Return to boot menu

Enter your choice(0-3):

Enter 0 in the Boot menu to reboot the switch with the new Boot ROM image.

Managing files from the Boot menu

From the Boot menu, you can display files in Flash memory to check for obsolete files, incorrect files, or space insufficiency, delete files to release storage space, or change the attributes of software images.

Displaying all files

Enter 3 in the Boot menu to display all files in Flash memory and identify the free space size.

EXTENDED BOOT MENU

1. Download image to flash

2. Select image to boot

3. Display all files in flash

4. Delete file from flash

5. Restore to factory default configuration

6. Enter BootRom upgrade menu

7. Skip current system configuration

8. Set switch startup mode

0. Reboot

Ctrl+Z: Access EXTENDED ASSISTANT MENU

Ctrl+F: Format file system

Ctrl+P: Change authentication for console login

Ctrl+R: Download image to SDRAM and run

Enter your choice(0-8): 3

The following is a sample output:

Display all file(s) in flash:

File Number    File Size(bytes)     File Name

================================================================================

1              8177                 flash:/testbackup.cfg

2(*)           53555200             flash:/system.bin

3(*)           9959424              flash:/boot.bin

4              3678                 flash:/startup.cfg_backup

5              30033                flash:/default.mdb

6              42424                flash:/startup.mdb

7              18                   flash:/.pathfile

8              232311               flash:/logfile/logfile.log

9              5981                 flash:/startup.cfg_back

10(*)          6098                 flash:/startup.cfg

11             20                   flash:/.snmpboots

Free space: 1009906637 bytes

The current image is boot.bin

(*)-with main attribute

(b)-with backup attribute

(*b)-with both main and backup attribute

Deleting files

If storage space is insufficient, delete obsolete files to free up storage space.

To delete files:

Enter 4 in the Boot menu:

Deleting the file in flash:

File Number    File Size(bytes)     File Name

================================================================================

1              8177                 flash:/testbackup.cfg

2(*)           53555200             flash:/system.bin

3(*)           9959424              flash:/boot.bin

4              3678                 flash:/startup.cfg_backup

5              30033                flash:/default.mdb

6              42424                flash:/startup.mdb

7              18                   flash:/.pathfile

8              232311               flash:/logfile/logfile.log

9              5981                 flash:/startup.cfg_back

10(*)          6098                 flash:/startup.cfg

11             20                   flash:/.snmpboots

Free space: 1009906637 bytes

The current image is boot.bin

(*)-with main attribute

(b)-with backup attribute

(*b)-with both main and backup attribute

Enter the number of the file to delete. For example, enter 1 to select the file testbackup.cfg.

Please input the file number to change: 1

Enter Y at the confirmation prompt.

The file you selected is testbackup.cfg,Delete it? (Y/N):Y

Deleting....................................Done!

Changing the attribute of software images

Software image attributes include main (M), backup (B), and none (N). System software and boot software can each have multiple none-attribute images but only one main image and one backup image on the switch. You can assign both the M and B attributes to one image. If the M or B attribute you are assigning has been assigned to another image, the assignment removes the attribute from that image. If the removed attribute is the sole attribute of the image, its attribute changes to N.

For example, the system image system.bin has the M attribute and the system image system-update.bin has the B attribute. After you assign the M attribute to system-update.bin, the attribute of system-update.bin changes to M+B and the attribute of system.bin changes to N.

To change the attribute of a system or boot image:

Enter 2 in the Boot menu.

EXTENDED BOOT MENU

1. Download image to flash

2. Select image to boot

3. Display all files in flash

4. Delete file from flash

5. Restore to factory default configuration

6. Enter BootRom upgrade menu

7. Skip current system configuration

8. Set switch startup mode

0. Reboot

Ctrl+Z: Access EXTENDED ASSISTANT MENU

Ctrl+F: Format file system

Ctrl+P: Change authentication for console login

Ctrl+R: Download image to SDRAM and run

Enter your choice(0-8): 2

Enter 1 or 2 at the prompt to set the attribute of a software image. (The following output is based on the option 2. To set the attribute of a configuration file, enter 3.)

1. Set image file

2. Set bin file

3. Set configuration file

0. Return to boot menu

Enter your choice(0-3): 2

File Number    File Size(bytes)     File Name

================================================================================

1(*)              53555200              flash:/system.bin

2(*)              9959424              flash:/boot.bin

3                 13105152              flash:/boot-update.bin

4                 91273216             flash:/system-update.bin

Free space: 905848832 bytes

(*)-with main attribute

(b)-with backup attribute

(*b)-with both main and backup attribute

Note:Select .bin files. One but only one boot image and system image must be included.

Enter the number of the file you are working with. For example, enter 3 to select the boot image boot-update.bin and enter 4 to select the system image system-update.bin.

Enter file No.(Allows multiple selection):3

Enter another file No.(0-Finish choice):4

Enter 0 to finish the selection.

Enter another file No.(0-Finish choice):0

You have selected:

flash:/boot-update.bin

flash:/system-update.bin

Enter M or B to change its attribute to main or backup. If you change its attribute to M, the attribute of boot.bin changes to none.

Please input the file attribute (Main/Backup) M

This operation may take several minutes. Please wait....

Next time, boot-update.bin will become default boot file!

Next time, system-update.bin will become default boot file!

Set the file attribute success!

Handling software upgrade failures

If a software upgrade fails, the system runs the old software version.

To handle a software upgrade failure:

Verify that the software release is compatible with the switch model and the correct file is used.

Verify that the software release and the Boot ROM release are compatible. For software and Boot ROM compatibility, see the hardware and software compatibility matrix in the correct release notes.

Check the physical ports for a loose or incorrect connection.

If you are using the console port for file transfer, check the HyperTerminal settings (including the baud rate and data bits) for any wrong setting.

Check the file transfer settings:

  - If XMODEM is used, you must set the same baud rate for the terminal as for the console port.

  - If TFTP is used, you must enter the same server IP addresses, file name, and working directory as set on the TFTP server.

  - If FTP is used, you must enter the same FTP server IP address, source file name, working directory, and FTP username and password as set on the FTP server.

Check the FTP or TFTP server for any incorrect setting.

Check that the storage device has sufficient space for the upgrade file.